3GPP TSG RAN WG1 Meeting #116														            R1-2400928

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		9.5.2

Title:						On-demand SIB1 for NES

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The Release 19 WI for Network Energy Saving (NES) was approved in RAN#102 [1]. One of the objectives is the following study:

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. Checkpoint for normative work in RAN#105

This contribution focuses on some of the relevant aspects concerning the above stated study of the NES technique of on-demand SIB1 transmissions for UEs in idle/inactive mode.

# Overview of on-demand SIB-1 and its potential

While on-demand SIB-1 is mentioned alongside on-demand SSB as part of technique A-5 in TR 38.864 [2], the focus of the discussion, the simulation results and Rel-18 contributions prior to the TR were on on-demand SSB.  Therefore, it is important to understand the commonalities and differences between on-demand SSB and on-demand SIB-1. On-demand SSB refers in most contexts to omitting both SSB and SIB-1 and that a trigger signal such as an uplink wake-up signal would restore both SSB and SIB-1 transmission. Instead, on-demand SIB-1 can be defined as an operation where SSBs are transmitted regularly but SIB-1 is omitted and it is only transmitted on-demand after a UE triggers SIB-1 transmission, e.g. via an uplink wake-up signal. Only a UE which needs to read SIB-1 would trigger SIB-1 transmission. The remaining time without such demand from any UE, energy can be saved at the gNB by omitting SIB-1 transmission.

Naturally, on-demand SSB provides larger energy savings than on-demand SIB-1, but there are several advantages of on-demand SIB-1 over on-demand SSB, which makes on-demand SIB-1 more practical and easily deployed and co-exist with legacy UEs. Namely:

- In on-demand SIB-1 operation, the cell can still be found during initial cell selection and cell re-selection. There is no discovery issue, as identified for on-demand SSB.

- As SSBs are transmitted the design of uplink wake-up signal for SIB-1 on-demand may consider some degree of time and frequency synchronization.

- RRM measurements can still be performed on the cell and be reported in other cells.

Given these characteristics defining and standardizing on-demand SIB-1 for idle/inactive operation could be much simpler than standardizing on-demand SSB for idle/inactive operation. The latter may still be justified for future releases given the huge potential on energy savings, but for Rel-19 the adoption of on-demand SIB-1 could be a very important step on the right direction. The question remains of which energy savings are attainable for on-demand SIB-1 operation.

The potential of omitting SIB-1 was mostly investigated in Rel-18 SI on the context of SIB-1-less operation with a cross-carrier SIB-1 transmitted in another co-located cell. The energy savings potential of on-demand SIB-1 goes beyond that because energy does need to be spent on the SIB-1 transmitted on the other carrier. Instead, SIB-1 is only transmitted when strictly needed.

In order to understand why omitting SIB-1 leads to energy savings, one needs to observe the following characteristics of SIB-1:

- SIB-1 must be decodable in the complete coverage of the cell. Therefore:

  - SIB-1 should use the most robust modulation and coding scheme

  - Multiple SIB-1s need to be transmitted separately in time on every analog beamforming direction (each direction corresponding to an SSB on a SSB burst)

  - The PDCCH part of SIB-1 needs enough CCEs for good coverage

- While transmitting SIB-1 the whole system bandwidth is not available for utilization (before reading SIB-1 the same CORESET#0 numerology of 24, 48 or 96 RBs is used instead, depending on the operating band)

- SIB-1 size depends on the fields, but in practice it may be significantly large (e.g. 200 bytes)

These characteristics together typically lead to a SIB-1 occupying a whole slot in the time domain. Even though the total PRB occupation may not be high on large bandwidths, time occupation (on slot) is considerable due to repetitions to support analog beamforming and initial beam selection. As an example, in a C-band deployment using SCS=30kHz, up to 8 beams may be used. This means in case of SSB 8x 4 symbols – a total of 32 symbols. But for SIB-1, occupying a full slot, this means a total of 8 slots only for SIB-1 transmission, in a system which only has a total 40 slots on the typical 20 ms SSB period. At zero load, this is by far the largest contributor to time occupancy of RBs and therefore, a large contributor to energy consumption.

- Observation 1: SIB-1 transmissions may occupy resources for a considerable time (e.g. 8 out of 40 slots). Consequently, SIB-1 transmission is a main contributor to gNB energy consumption at zero and low load.

This behavior and the potential of on-demand SIB-1 was studied with system-level simulations following the TR 38.864 energy consumption model, for different number of target SSB/SIB-1 beams (corresponding to analog beamforming directions). Further simulation parameters are summarized on appendix A.  Three options were simulated, at zero traffic load:

- A baseline where regular SSB and SIB-1 transmissions is done every 20 ms period (SIB-1 period of 160 ms and 8 repetitions per period)

- SSB is regularly transmitted but SIB-1 is fully omitted

- SSB is regularly transmitted, SIB-1 is omitted, but another signal is transmitted every 20 ms to carry a wake-up-signal configuration provisioning to UE (e.g. a 1 symbol DCI over CORESET#0)

Figure 1 – Potential energy saving gains for on-demand SIB-1 at zero load, for different SSB/SIB-1 beam configurations

From the simulation results illustrated on  Figure 1, the following conclusions can be drawn:

- Even though the energy saving gains without analog beamforming (1 SSB, 1 SIB-1) are modest they grow significantly as the number of beams increase (up to more than 40%).

- Even if another signal needs to be transmitted, to provide wake-up-signal configuration provisioning to UE, the energy saving gains are significantly large.

Note that in the simulations where a 1-symbol DCI was assumed to be transmitted every 20 ms to carry wake-up signal configuration, no changes to CORESET#0 were assumed. Even more efficient operation could be achieved by packing the signals further in time (e.g. new multiplexing patterns).

- Observation 2: The energy saving potential of on demand SIB-1 is significant. Depending on the number of beams (on SSB and SIB-1 transmission), energy savings between 13.4% and 42.3% are possible.

- Proposal 1: TR 38.864 is updated with further description and simulation results of on-demand SIB-1.

# Triggering Method by UL WUS

As described in the study objective, in order to define on-demand SIB-1 operation, for idle/inactive UEs, a triggering method by uplink wake-up-signal shall be defined. Preferably, the wake-up signal should reuse an existing signal/channel.

In legacy operation, before a UE in idle/inactive mode can transmit any uplink signal, it needs to obtain necessary minimum system information (MSI) regarding uplink transmissions, which is currently distributed in the MIB and SIB-1. Specifically, the MSI provides the UE with information about precise  frequency position, location in time and amount of transmit power for UL transmissions.  This means, techniques involving uplink transmissions from the UE before receiving the SIB-1 and/or the MIB becomes challenging as the UE lacks enough SI to do the UL transmissions precisely. Therefore, RAN1 should study solutions for ensuring that the gNB effectively detects the UL WUS transmissions from idle/inactive UEs.

- Observation 3: Techniques involving UL transmissions before the UE receives SIB-1 become challenging as the UE lacks enough SI to do the UL transmissions precisely.

- Proposal 2: RAN1 should study solutions for ensuring that the gNB receiver can effectively detect the UL WUS transmissions from idle/inactive UEs.

- Timing Advance (TA) determination is part of random access procedure. The reception of most UL channels depends on TA, but for PRACH the timing is more relaxed. Therefore, when the UE needs to send a wake-up signal it cannot apply TA yet. Doing a random access procedure at this point is also not possible, as the UE needs information from SIB-1 to perform that procedure. These 2 characteristics (no TA and inability to perform RA) points to PRACH as the main candidate to be used as a basis for an uplink cell wake-up signal.

- Observation 4: Prior to random access procedure the UE cannot apply a precise time advance, and a random access procedure cannot be performed by UEs without obtaining SIB-1.

- Proposal 3: PRACH is used as the basis for an uplink cell wake-up signal.

# Provisioning UL cell wake-up signal configuration to the UE

As described in the previous section, in order to transmit an UL cell wake-up signal the UE needs to know which frequency and time resources to use, which power can be used for transmission and the waveform format (e.g. based on PRACH). Such configuration, if flexible, needs to be provisioned to the UE.

The objective explicitly states, “No modification of SSB will be discussed under this objective”. This rules out extensions to SSB/MIB to provision UL cell wake-up signal configuration. The two remaining options are:

- UL cell wake-up signal configurations are predefined, in a way that no provisioning is needed.

- When SIB-1 is omitted (for energy saving) the gNB transmits another signal (e.g. a DCI on CORESET#0) which carries the configuration of UL cell wake-up signal.

In fact, these two options correspond to the simulated cases shown on Figure 1 and as discussed on section 2 the energy savings are very significant in both cases (obviously, larger if no other signal is transmitted).

- Proposal 4: RAN1 further investigates these 2 options:

- UL cell wake-up signal configurations are predefined, in a way that no provisioning is needed.

- When SIB-1 is omitted (for energy saving) the gNB transmits another signal (e.g. a DCI on CORESET#0) which carries the configuration of UL cell wake-up signal.

# Conclusions

In this contribution, we discussed some aspects of on-demand SIB1 for NES. The following observations and proposals have been made:

- Observation 1: SIB-1 transmissions may occupy resources for a considerable time (e.g. 8 out of 40 slots). Consequently, SIB-1 transmission is a main contributor to gNB energy consumption at zero and low load.

- Observation 2: The energy saving potential of on demand SIB-1 is significant. Depending on the number of beams (on SSB and SIB-1 transmission), energy savings between 13.4% and 42.3% are possible.

- Observation 3: Techniques involving UL transmissions before the UE receives SIB-1 become challenging as the UE lacks enough SI to do the UL transmissions precisely.

- Observation 4: Prior to random access procedure the UE cannot apply a precise time advance, and a random access procedure cannot be performed by UEs without obtaining SIB-1.

- Proposal 1: TR 38.864 is updated with further description and simulation results of on-demand SIB-1.

- Proposal 2: RAN1 should study solutions for ensuring that the gNB receiver can effectively detect the UL WUS transmissions from idle/inactive UEs.

- Proposal 3: PRACH is used as the basis for an uplink cell wake-up signal.

- Proposal 4: RAN1 further investigates these 2 options:

- UL cell wake-up signal configurations are predefined, in a way that no provisioning is needed.

- When SIB-1 is omitted (for energy saving) the gNB transmits another signal (e.g. a DCI on CORESET#0) which carries the configuration of UL cell wake-up signal.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson (Moderator), 3GPP TSG RAN Meeting #102	 Edinburgh, Scotland, December 2023.

- TR 38.864, “Study on network energy savings for NR”, V1.0.0, December 2022.

# Appendix A – Simulation parameters and assumptions

The system level simulation results shown in Figure 1 were produced according to the guidelines and models in TR 38.864 [2]. The gNB consumption model was set to the parameters of BS Category 1, Set 1. The simulation sets were set to the 3-sector deployment of a TDD system on 4 GHz (Set for system-level simulation).

Coreset#0 was set to 3 symbols, 1 symbol gap between PDCCH and PDSCH for SIB-1 and SIB-1 with 200 bytes. Both PDCCH and PDSCH of SIB-1 were assumed with 48 RBs.