3GPP TSG RAN WG1 Meeting #116														            R1-2400929

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		9.5.3

Title:						Adaptation of Common Signals and Channels for NES

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

The Release 19 WI for Network Energy Saving (NES) was approved in RAN#102 [1]. One of the objectives for specification is as follows:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution focuses on relevant aspects concerning the adaptation of common signals and channels.

# Adaptation of periodicity of SSB

During the study phase, 3GPP identified significant potential in adapting the transmission rate of common signals, namely Synchronization Signal Blocks (SSB), System Information Block Type 1 (SIB-1) and paging occasions. Also, in relation to that, the reception rate of RACH may be adapted accordingly.

Different views emerged during the study phase. Some contributions considered each signal being adapted completely independently while other contributions considered joint adaptation of the different common signals. Solely adjusting the periodicity of SSB without adapting the other signals may not be sufficient for maximizing energy savings. In addition to modifying the SSB periodicity, it is essential to adapt the periodicity (including repetition period) of SIB-1 accordingly. Adjusting their periodicities in tandem is crucial for optimizing energy savings and maintaining network performance.

- Observation 1: Solely adjusting the periodicity of SSB without adapting the other signals may not be sufficient for maximizing energy savings. It is essential to adapt the periodicity (including repetition period) of SIB-1 accordingly.

Paging and RACH could be adapted more independently to the adaptation of SSB/SIB-1. The reason is that, for paging, the typical cycles can be already sparser to accommodate SSB/SIB-1 adaptations (e.g. between 20 ms and 160 ms). In the case of RACH, the need to adapt together or not with SSB/SIB-1 depends on the duplexing type (TDD vs FDD), the UL/DL ratio in TDD, gNB power consumption and the support for initial beam management.

Adjusting the periodicity of SSB/SIB-1 holds promise for substantial energy savings by enabling gNBs to enter deeper sleep modes. Typically, with SSB/SIB-1 transmissions occurring every 20 ms, gNBs can only achieve light sleep mode. However, to access deep sleep mode and unlock greater energy savings, the interval between transmissions would need to exceed, for example, 50 ms as per the Category 1 settings assumed in [2]. Such adjustment of SSB/SIB-1 periodicity allows the gNBs to optimize energy consumption effectively. As shown in Figure 1, in scenarios with no network load, increasing the periodicity of SSB/SIB-1 transmissions to 160 ms from the current 20 ms can result in energy savings of up to 70% for the gNB. However, when the network load increases to 10%, this saving may decrease to 20%, but it remains a substantial reduction in energy consumption.

Figure 1 – Energy saving gains for joint SSB/SIB-1 period adaptation compared to a 20 ms baseline for 2 different PRB occupations.

- Observation 2: Adaptation the periodicity of SSB and SIB-1 can result in high energy savings at low to medium loads.

- Proposal 1: Joint SSB/SIB-1 period adaptation should be considered for Rel-19.

# On the backward compatibility of dynamic SSB/SIB-1 time domain adaptation

During the study phase, one of the main concerns raised for the Technique A1-3 in TR 38.864 [2] was no backward compatibility. However, while this is a legitimate concern, it is not insurmountable. In fact, even Release-15 UEs can camp on cells with different SSB periodicities: 5 ms, 10 ms, 20 ms (the typical case), 40 ms, 80 ms and 160 ms. This opens possibility for a backward compatible design of SSB/SIB-1 adaptation. What really would cause difficulties to legacy UEs is not a sparse energy saving period, but frequent changes. A legacy UE would not be able to cope, for example, with SSB which vanishes suddenly. A backward-compatible design of dynamic SSB/SIB-1 time domain adaptation needs to ensure legacy UEs perceive a constant SSB period.

Observation 3: A backward-compatible design of dynamic SSB/SIB-1 time domain adaptation needs to ensure legacy UEs perceive a constant SSB period.

Many of the discussions of dynamic SSB/SIB-1 adaptation revolved around having a default SSB period (e.g. 20 ms) and during periods of low load SSB transmissions are omitted to save energy. That creates difficulties and would not be understood by legacy UEs. Instead, in order to achieve a backward-compatible design, the default behavior, at least in SCells, should be to have a sparse SSB period (e.g. 80 or 160 ms) and when the load increases further SSB occasions are added to make sure the best performance is attainable. The complete envisioned operation is as follows:

- On a coverage layer (e.g. PCell), SSB periods are transmitted with legacy 20 ms period and SIB-1 at legacy 160 ms period with 8 repetitions, in order to make sure there are no issues on initial cell search.

- On capacity layers (SCells): 
The baseline configuration defines only sparse SSBs with a large periodicity (e.g. 80 ms or 160 ms)

- SIB-1 signals as usual, the configuration understood by legacy UEs, i.e. 80 ms or 160 ms on ssbperiodicityServingCell

- During times the network has more load, complementary SSBs are also transmitted to achieve the typical performance needed (e.g. an effective 20 ms period) in initial access and measurements. By complementary SSBs we mean SSBs which complement the already existing period of Sparse SSBs to achieve a shorter periodicity. This concept is illustrated in Figure 2.

- These complementary SSBs may be based on NCD-SSBs. This would make sure that legacy UEs can still find them, measure them, and synchronize to them but make them search for the sparse SSBs to obtain System Information with the proper periodicity information.

Figure 2 – Sparse SSBs are always transmitted at constant rate to serve legacy UEs. Complementary SSBs are added to dynamically adapt the SSB rate.

Such an operation allows for a high degree of backward compatibility. First and foremost, legacy UEs can always be served by the coverage layer and the coverage layer can attain initial access performance. Second, legacy UEs can still access the capacity cells and can be camped on it. The legacy UEs perceive the energy saving cells as legacy cell. Last, but not least, the network can adapt and save a lot of energy by adding or removing complementary SSBs dynamically. Rel-19 UEs may understand this adaptation, e.g. to optimize the schedule of measurements, but legacy UEs can also operate normally.

- Proposal 2: The adaptation of SSB periodicity is made backward compatible by defining complementary SSB occasions that can be added when the network needs to serve more traffic.

# Conclusions

In this contribution, we discussed some aspects of Common signals adaptation for NES. The following observations and proposals have been made:

- Observation 1: Solely adjusting the periodicity of SSB without adapting the other signals may not be sufficient for maximizing energy savings. It is essential to adapt the periodicity (including repetition period) of SIB-1 accordingly.

- Observation 2: Adaptation the periodicity of SSB and SIB-1 can result in high energy savings at low to medium loads.

Observation 3: A backward-compatible design of dynamic SSB/SIB-1 time domain adaptation needs to ensure legacy UEs perceive a constant SSB period.

- Proposal 1: Joint SSB/SIB-1 period adaptation should be considered for Rel-19.

- Proposal 2: The adaptation of SSB periodicity is made backward compatible by defining complementary SSB occasions that can be added when the network needs to serve more traffic.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson (Moderator), 3GPP TSG RAN Meeting #102	 Edinburgh, Scotland, December 2023.

- TR 38.864, “Study on network energy savings for NR”, V1.0.0, December 2022.