3GPP TSG RAN WG1 #116	R1-2400932

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.7.2
Source:	Toyota InfoTechnology Center
Title:	Discussion on ISAC channel modelling

Document for:	Discussion and Decision

# Introduction

In RAN #102 meeting, the SID for Rel-19 channel modelling for Integrated Sensing And Communication (ISAC) for NR was approved [1]. One of the objectives of the SID is to define channel modelling details for ISAC as shown below.

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

In this contribution, we provide our views on channel modeling for ISAC.

# Discussion on channel modelling for ISAC

In this section, we discuss channel modelling for ISAC.

## Stochastic model and map-based hybrid model

TR 38.901 [2] defines two types of channel models as shown below:

- Stochastic model:

  - This model is based on statistical modeling of radio channels. The radio channels are generated based on various channel parameters for path loss, delay spread, angular spread, Rician K factor, shadow fading, etc.

  - This model has less computational complexity, compared with the map-based hybrid model below. However, the current model is not suitable for ISAC because channel realizations may not be accurate or consistent with the target object’s mobility.

- Map-based hybrid model:

  - This is based on a deterministic component and a stochastic component following mainly the stochastic model above. The radio channels are created using the deterministic ray-tracing upon a digitized map and emulating certain stochastic components according to the statistic parameters.

  - By applying deterministic ray-tracing for paths associated with the target object, this model can provide more accurate and consistent channel realizations for object detection and/or tracking in ISAC.

For ISAC study, channel realizations associated with a target object need to be accurate and consistent in time, frequency, and space. In that sense, the map-based hybrid model is more suitable for ISAC channel modelling. Therefore, we propose to enhance the map-based hybrid model for ISAC channel modelling by applying deterministic ray-tracing for paths associated with the target object. In addition, for calibration and consistent simulation evaluation among different companies, common reference scenarios (e.g., outdoor and indoor scenarios) for the map-based hybrid model should be defined by taking into account target use cases and scenarios.

Observation 1: For ISAC study, applying deterministic ray-tracing for paths associated with a target object is suitable for accurate and consistent channel models.

Proposal 1: RAN1 to enhance the map-based hybrid model in TR 38.901 for ISAC channel modelling.

Proposal 2: RAN1 to apply deterministic ray-tracing for paths associated with a target object in the map-based hybrid model in TR 38.901.

Proposal 3: RAN1 to define common reference scenarios (e.g., outdoor and indoor scenarios) for the map-based hybrid model in TR 38.901 for ISAC channel modelling by taking into account target use cases and scenarios.

## Radar cross-section (RCS)

For ISAC study, consideration of radar cross-section (RCS) is important because object detection/tracking is performed based on received signals reflected by the object. However, the current channel models in TR 38.901 [2] do not consider RCS. Therefore, it is important to define the RCS model for each target object type.

Roughly speaking, the RCS of an object is the measure of the object's ability to reflect signals to the direction of the receiver. The RCS of an object depends on various factors such as:

- The size of the object

- The material of the object

- The shape of the object

- Frequency

- Incident and reflected angles.

Considering these aspects, to reduce the workload and complexity in ISAC channel modelling, it would be preferable to define the RCS models for selected target object types (e.g., vehicles, UAVs, humans, AGVs, objects creating hazards on roads/railways, etc.).

Observation 2: For ISAC channel modelling, taking into account radar cross-section (RCS) is necessary.

Observation 3: TR 38.901 does not consider radar cross-section (RCS).

Proposal 4: RAN1 to introduce radar cross-section (RCS) for ISAC channel modelling.

Proposal 5: RAN1 to define the radar cross-section (RCS) models for selected target object types (e.g., vehicles, UAVs, humans, AGVs, objects creating hazards on roads/railways, etc.).

# Conclusion

In this contribution we discussed our views on channel modelling for ISAC. Our observations and proposals are summarized as follows:

Observation 1: For ISAC study, applying deterministic ray-tracing for paths associated with a target object is suitable for accurate and consistent channel models.

Observation 2: For ISAC channel modelling, taking into account radar cross-section (RCS) is necessary.

Observation 3: TR 38.901 does not consider radar cross-section (RCS).

Proposal 1: RAN1 to enhance the map-based hybrid model in TR 38.901 for ISAC channel modelling.

Proposal 2: RAN1 to apply deterministic ray-tracing for paths associated with a target object in the map-based hybrid model in TR 38.901.

Proposal 3: RAN1 to define common reference scenarios (e.g., outdoor and indoor scenarios) for the map-based hybrid model in TR 38.901 for ISAC channel modelling by taking into account target use cases and scenarios.

Proposal 4: RAN1 to introduce radar cross-section (RCS) for ISAC channel modelling.

Proposal 5: RAN1 to define the radar cross-section (RCS) models for selected target object types (e.g., vehicles, UAVs, humans, AGVs, objects creating hazards on roads/railways, etc.).

# Reference

- RP-234069, “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR,” RAN #102, Dec. 2023.

- TR 38.901 V17.1.0, “Study on channel model for frequencies from 0.5 to 100 GHz,” Dec. 2023.