3GPP TSG RAN WG1 Meeting #116 	R1-2400933

Athens, Greece, February 26th – March 1st, 2024

Source:	   RUijie networks

Title:	   Discussions on enhancements for UE-initiated/event-driven beam

management

Agenda Item: 	   9.2.1

Document for: 	Discussion and decision

Introduction

In RAN#102 meeting, the NR MIMO phase 5 [1] was discussed. The following was agreed on the objective of enhancements for UE-initiated/event-driven beam management:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In legacy beam management procedures, the network may configure/activate frequent periodic or semi-persistent beam reporting (e.g., N best beams and corresponding L1-RSRPs) or triggers frequent aperiodic beam reporting to timely acquire the best/preferred beam for data/control transmissions. However, this clearly results in large UL reporting overhead and control signaling overhead. At the same time, if less frequent beam reporting is configured, the network could not always acquire ‘best/preferred’ beam(s) as the beam reporting by the UE may be outdated, thus leading to performance degradation. Given that UE has better and more-timely knowledge of beam quality changes, UE-initiated beam reporting procedure can lead to more timely beam reports yet with reduced reporting overhead. Under such a procedure, if the UE determines that e.g., current beam(s) quality becomes poor, UE can trigger beam reporting without the network needing to configure or trigger frequent reporting.

In this contribution, we present our views on enhancements for UE-initiated/event-driven beam management and proposals for moving forward.

Enhancements for UL signaling content

When a UE determines that e.g., current beam(s) quality becomes poor, e.g., via measurement result of L1-RSRP/RSSI/RSRQ or L1-SINR/SNR or hypothetical PDDCCH BLER or other performance metrics, UE can immediately initiate CSI or beam update request, where UE-preferred beam ID or beam group ID should be included. Assisted information about beam structure including beam grouping, beam coverage area, beamwidth or beam emission/incidence angle, may be optionally included in the UL signaling.

Based on the above analysis, we propose the following:

Proposal 1: UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching should include at least UE-preferred beam ID or beam group ID information, and optionally assisted information about beam structure.

Enhancements for UL signaling medium/container

The enhanced UL signaling can be transmitted in reserved UL resources, e.g., grant-free transmissions, or in the latest upcoming UL resources to facilitate fast beam switching. In either reserved UL resources or latest upcoming UL resources, the mapping between the resources and beams (e.g., beam ID or beam group ID) may be defined to assist fast beam switching. There are the following cases that may need further down-selection:

- Case 1: the UE may simultaneously transmit the UL signaling/reporting and switch to its preferred beam(s) without the network’s confirmation to be able to timely use the most suitable beam(s) for transmission/reception while reducing overhead, especially in fast-changing environments.

- Case 2: the UE could wait for a pre-defined period of time (e.g., BeamAppTime) before it switches to its preferred beam(s) for transmission/reception.

- Case 3: the UE could wait for the confirmation (ACK/NACK) of its transmitted UL signaling/reporting from the network before switching to its preferred beam(s).

The enhanced UL signaling could be transmitted in PUCCH or PUSCH, while the confirmation (ACK/NACK) of the UL signaling from the network could be transmitted in DCI or MAC-CE.

It may happen that the UE preferred beam(s) are different from the beam(s) indicated in TCI, the priority rule should be defined for this case. The details need further study.

Based on the above analysis, we propose the following:

Proposal 2: For beam switching in UE-initiated/event-driven beam management, the following three options should be further studied and down-selected:

- Case 1: the UE may simultaneously transmit the UL signaling/reporting and switch to its preferred beam(s) without the network’s confirmation to be able to timely use the most suitable beam(s) for transmission/reception while reducing overhead, especially in fast-changing environments.

- Case 2: the UE could wait for a pre-defined period of time (e.g., BeamAppTime) before it switches to its preferred beam(s) for transmission/reception.

- Case 3: the UE could wait for the confirmation (ACK/NACK) of its transmitted UL signaling/reporting from the network before switching to its preferred beam(s).

Proposal 3: The enhanced UL signaling could be transmitted in PUCCH or PUSCH, while the confirmation (ACK/NACK) of the UL signaling from the network could be transmitted in DCI or MAC-CE.

Proposal 4: The priority rule should be defined when the UE preferred beam(s) are different from the beam(s) indicated in TCI.

Conclusions

In this contribution, we present our views on UE-initiated/event-driven beam management. Based on the discussions in the previous sections we propose the following:

Proposal 1: UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching should include at least UE-preferred beam ID or beam group ID information, and optionally assisted information about beam structure.

Proposal 2: For beam switching in UE-initiated/event-driven beam management, the following three options should be further studied and down-selected:

- Case 1: the UE may simultaneously transmit the UL signaling/reporting and switch to its preferred beam(s) without the network’s confirmation to be able to timely use the most suitable beam(s) for transmission/reception while reducing overhead, especially in fast-changing environments.

- Case 2: the UE could wait for a pre-defined period of time (e.g., BeamAppTime) before it switches to its preferred beam(s) for transmission/reception.

- Case 3: the UE could wait for the confirmation (ACK/NACK) of its transmitted UL signaling/reporting from the network before switching to its preferred beam(s).

Proposal 3: The enhanced UL signaling could be transmitted in PUCCH or PUSCH, while the confirmation (ACK/NACK) of the UL signaling from the network could be transmitted in DCI or MAC-CE.

Proposal 4: The priority rule should be defined when the UE preferred beam(s) are different from the beam(s) indicated in TCI.

- References

RP-234007, “New WID: NR MIMO Phase 5”, Samsung, RAN#102.