3GPP TSG RAN WG1 Meeting #116		R1-2400940

Athens, Greece, February 26th – March 1st, 2024

Agenda item:       9.2.3

Source: 	Nokia, Nokia Shanghai Bell

Title: 	On the support for 3-antenna-port codebook based transmissions

Document for:     Discussion and Decision

# 1	Introduction

In RAN#102 one objective was agreed for 3Tx UL MIMO enhancements in Rel-19 WID [1]. The specific objective in the WID RP-234007 is:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported.

This contribution provides initial study on the 3Tx codebook design, on non-coherent codebook. System level simulations are also conducted to compare performance between 2Tx and 3Tx with coherent and non-coherent setup. The SLS results indicate that 3Tx non-coherent is not always better than 2-antenna-port in uplink. Related observations and proposals are provided in the contribution.

# 2	Non-coherent 3Tx codebook design

The starting point for non-coherent 3Tx codebook design shall follow the design principle for non-coherent 8Tx design in Rel-18. As agreed in RAN1 during the 8Tx study, all possible combinations of non-coherent 8-port precoders are supported up to 8 layers. RAN1 #113 meeting reached this agreement for 8Tx non-coherent:

Agreement For non-coherent uplink precoding by an 8TX UE, support Alt1., - Alt1. – All 255 combinations from 8 non-coherent rank1 precoders are supported.

With this agreement, Rel-18 8Tx non-coherent codebook (codebook4) is thus defined, as listed here based on TS 38.211:

Table 6.3.1.5-47: Precoding matrix W  for codebook4 and transmission using eight antenna ports. Up to 8 layers are supported with transform precoding disabled and up to one layer with transform precoding enabled.

| TPMI index | W |
| --- | --- |
| 0 – Δν-1 | W=122[ep0…epν-1] where column i of W, denoted ei, has an element 1 on the row corresponding to the port pi on which layer i is to be transmitted, and element 0 in all other rows, pi<pi+1, L=p=07δ(p)2p, where δp=1 if a layer is to be transmitted on port p and δp=0 otherwise, and Δz=k=1zC(8,k) for z≥1, where Cx,y is defined by Table 5.2.2.2.5-4 of [6, TS 38.214]. TPMI indices 0 to Δν-1 are mapped to values of L, first by increasing values of the number of transmitted layers, and then by increasing values of L for a given number of layers. |

For 3Tx, the total number of non-coherent uplink precoding would be 23-1=7. The exhaustive list of possible combinations for all 3 layers is shown in Table 1, following the same design principle for 8Tx codebook4.

Table 1    3Tx non-coherent codebook

| Rank | 3Tx Precoders | TPMI Size |
| --- | --- | --- |
| 1 | 100, 010, 001 | 3 |
| 2 | 100100, 100001, 001001 | 3 |
| 3 | 100010001, | 1 |

With the total 7 number of precoders, a maximum 3-bit TPMI field in DCI is sufficient to indicate these precoders, depending on maximum rank configuration from 1 to 3.

We thus propose:

Proposal 1: Support the 3Tx non-coherent codebook as listed in Table 1.

Proposal 2: Support a maximum 3-bit TPMI field for non-coherent 3Tx.

# 3	SRS and PT-RS for 3Tx

Uplink codebook-based transmission needs its corresponding SRS as well. Given that there is no 3-port SRS in NR, we shall have a 3-port SRS design to enable uplink 3Tx. There are two options for 3-port SRS design:

- Option 1: Define a new 3-port SRS

- Option 2: Re-use existing 4-port SRS for 3-port operation.

Given the limited WID scope with the indication “without enhancement on SRS resource”, Option 2 would have less specification impact, although Option 2 would also be considered as an “enhancement” to the existing SRS resource. We have this observation:

Observation 1: It would have less specification impact if we re-use NR existing 4-port SRS resource for 3Tx SRS design.

There is another potential impact to PT-RS for 3Tx operation. The existing PT-RS design would be re-usable for 3Tx, with modification of PT-RS mapping.

Proposal 3: Consider re-using PT-RS design for 3Tx operation.

# 4	3Tx performance

One motivation of using 3Tx over the existing 2Tx and 4Tx would be that the 3Tx can be considered as an “enhancement” over 2Tx UE implementation. The question is how much benefit can be achieved from using 3Tx vs 2Tx, given that there were few performance studies for 3Tx uplink. To answer this question, we studied the 3Tx system-level performance, where we used the SLS simulation assumptions used for Rel-18 UL 8Tx as a starting point. The simulation assumptions are listed below:

Table 2    3Tx and 2Tx system level simulation assumptions

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 OFDM symbol slot SCS , 30 KHz |
| Scenario | UMa (ISD = 200 m), 80% Indoor |
| Channel model | 38.901 |
| System bandwidth | 20 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| gNB antenna radiation pattern parameters | 38.901 Table 7.3-1, 8 dBi , 65° HPBW |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation: For 2TX UEs: max rank=2. For 3TX UEs: max rank = 3 |
| UE speed | 3 Km/h |
| UE TX antenna configuration | 2TX UEs: UEx: two x-pol antennas UEvv: two vertically polarized elements 3TX UEs: UExv: x-pol plus a vertically polarized element; UEvvv: three vertically polarized elements |
| Traffic model | Full buffer |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -80 dBm |
| UE power rating | 23 dBm (UE, 38.101): Full power mode-0 |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

The detailed performance results are captured in this table and in the subsequent two figures:

Table 3    Performance results of system level simulations

|  | Sector SE | Edge UE SE | Mean Rank |
| --- | --- | --- | --- |
| 2TX: comparing coherent vs non-coherent |  |  |  |
| Gain of coherent 2TX over non-coherent 2TX: (vv) | 13.8% | 9.2% | 0.1% |
| Gain of coherent 2TX over non-coherent 2TX: (x) | 2.0% | -3.6% | -1.2% |
| 2TX comparing (x) vs (vv) |  |  |  |
| Coherent 2TX: gain of (x) over (vv): | 8.7% | 4.8% | 11.3% |
| Noncoherent 2TX: gain from (x) over (vv): | 21% | 19% | 13% |
| Noncoherent: comparing 3TX with 2TX |  |  |  |
| Gain of noncoherent 3TX (xv) over noncoherent 2TX (x): | 6.7% | 4.0% | 35.6% |
| Gain of noncoherent 3TX (vvv) over noncoherent 2TX (vv): | 7.2% | -0.9% | 29.1% |
| Comparing non-coherent 3TX with coherent 2TX |  |  |  |
| gain of noncoherent 3TX (xv) over coherent 2TX (x): | 4.6% | 7.9% | 37.3% |
| gain of noncoherent 3TX (vvv) over coherent 2TX (vv): | -5.8% | -9.2% | 28.9% |

Figure 1: Relative mean UE spectral efficiency and Edge UE spectral efficiency: full buffer traffic.  Comparing 2TX with 3TX.

Figure 2: Mean scheduled rank per UE: full buffer traffic.  Comparing 2TX with 3TX.

Comparing coherent CBs and non-coherent CBs: the results are dependent on the UE configuration, but some trends can be seen from the cases that were run so far.  For Section SE, these are our observations:

Observation 2: Gain of 3TX non-coherent over 2TX non-coherent is less than 7% for the antenna configurations evaluated.

Observation 3: Gain of 3TX non-coherent over 2TX coherent:  for the antenna configurations evaluated, the gain ranges between a 6% loss to an 8% gain depending on the UE antenna configuration.

Note that the performance was evaluated with full power mode 0 for all UE cases.  Note that the spec support for full power mode 0 with UL 3TX is currently out of scope for the Rel-19 WID, so simulation results without full power mode 0 would be very helpful.

# 5	Conclusions

Initial study on the 3Tx codebook design for the Rel-19 WID is provided in this contribution. Here is the summary of our proposals and observations.

Proposal 1: Support the 3Tx non-coherent codebook as listed in Table 1.

Table 4    3Tx non-coherent codebook

| Rank | 3Tx Precoders | TPMI Size |
| --- | --- | --- |
| 1 | 100, 010, 001 | 3 |
| 2 | 100100, 100001, 001001 | 3 |
| 3 | 100010001, | 1 |

Proposal 2: Support a maximum 3-bit TPMI field for non-coherent 3Tx.

Observation 1: It would have less specification impact if we re-use NR existing 4-port SRS resource for 3Tx SRS design.

Proposal 3: Consider re-using PT-RS design for 3Tx operation.

Detailed system-level simulation results are provided on 3Tx vs 2Tx with these observations:

Observation 2: Gain of 3TX non-coherent over 2TX non-coherent is less than 7% for the antenna configurations evaluated.

Observation 3: Gain of 3TX non-coherent over 2TX coherent:  for the antenna configurations evaluated, the gain ranges between a 6% loss to an 8% gain depending on the UE antenna configuration.

# References

- RP-234007, “New WID: NR MIMO Phase 5,” RAN#102, Edinburgh, Scotland, Dec 11-15, 2023