3GPP TSG RAN WG1 #116	R1-2400941

Athens, Greece. 26th February – 1st March 2023

Agenda item:		9.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Document for:		Discussion and Decision

# Introduction

A New Work Item for Rel19 MIMO evolution was approved in RAN#102 [1].

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In the 5G networks, the UL capacity is limited due to various factors, such as power and computational constraints of the UEs. Many new vertical use cases and applications (e.g., industrial and vehicular) require larger UL capacity than in the traditional deployments. Operators are looking for ways to enhance the UL capacity. Introducing asymmetrical DL and UL with UL-only TRPs will help to significantly enhance the UL capacity in terms of system throughput and cell edge throughput. When introducing UL only TRPs within the coverage of a cell area, the required UL transmission power level would be lower than the legacy UL thus, doesn’t incur additional UL interference. The UL-only TRPs are more cost-effective than small cells for improving UL capacity. The small cells normally come with both the UL and DL capability, thus, more expensive and cause interference in the DL.

Introducing the asymmetrical DL and UL with UL-only TRPs introduces several technical challenges. As the UL-only TRPs won’t transmit any DL reference signals, UEs have no means to estimate the path loss between UE and the UL-only TRP. Thus, the transmitter power level may not be selected based on existing procedures. If the DL pathloss between UE and serving sTRP is used for setting the transmit power level between UE and UL-only TRP, the receiver at the UL-only TRP would be overwhelmed. The asymmetrical deployment will impact the TDD operation. The reciprocity-based channel measurements may not be applicable. Thus, additional measurement and feedback mechanisms are needed. The UL-only TRPs require low latency, high-capacity links with one or more serving TRPs (DL/UL capable). The optimal number of UL-only TRPs needs to be deployed in the cell based on the requirements and capacity demand to maximize the benefits. Otherwise, it may increase the CAPEX and OPEX for the network operator.

Figure 1 depicts the scenario defined in the WID [1]. The DL transmission is from only single TRP. This TRP is also capable of UL reception. Several UL-only TRPs are deployed in the area closer to the cell edge. The DL and UL capable TRP functions as an anchor point and has the backhaul connection with all UL-only TRPs. The UL-only TRPs won’t be able to transmit reference signals in the DL. The UEs may listen to the SSB and DL control information from the single serving TRP and transmit data and control information to serving sTRP or UL-only TRP or both depending on the deployed MIMO transmission reception technique. Both the FR1 and FR2 bands can be used in the deployment.

In addition, several existing features in 5G NR rel. 17 and 18 could be leveraged as building blocks to enable the UL-only TRPs. They are single DCI-based unified TCI for mTRP, 2TA enhancements, etc.

Figure 1: UL-only TRP deployment

# Discussion

## Power Control

Figure 2: Uplink transmission in the presence of UL-only TRP

In the 5G NR, two power control adjustment states are specified for SRS by the higher layers in the SRS-ResourceSet information element. They are the same as one of the PUSCH power control adjustment states and a separate CL PC state for SRS. For the PUSCH, there are two power control adjustment states specified by higher layer parameter twoPUSCH-PC-AdjustmentStates, and SRS PC adjustment states may follow one. When the non-co-located UL-Only TRPs are introduced in a cell, we may use separate SRS resources towards the serving gNB and UL-Only TRP. Thus, we may require two separate SRS power control loops to facilitate the channel and pathloss estimations.

Proposal 1: Agree on having two separate closed-loop power control adjustment states for SRS to facilitate UL-Only TRP deployments.

In legacy, with DCI format 2_3, a single TPC command can be indicated per CC. It’s worth recalling that DCI Format 2_3, which is a group common DCI, may be applicable for uplink carrier(s) of serving cells where a UE is not configured for PUSCH/PUCCH transmission or when srs-PowerControlAdjustmentStates indicates a separate power control adjustment state between SRS transmissions and PUSCH transmissions.

Now, considering two separate closed loops or adjustment states for SRS, one aspect that would need to be discussed is the indication of a second TPC for the same CC using DCI format 2_3.

Proposal 2: Considering two power control adjustment states for SRS for scenarios including UL-only TRP(s), discuss how to enable the indication of a second TPC command in DCI format 2_3.

As depicted in Figures 1 and 2 and in the introduction, when the UL-only TRPs are deployed, it may improve the average and cell-edge spectral efficiencies significantly. It may be helpful to establish the performance gains using some performance evaluations.

Proposal 3. Study the UL system-level performance with and without UL-only TRPs to establish the performance gains. Genie-aided TPC may be assumed towards UL-only TRPs.

As the UL-only TRP doesn’t have the capability to transmit (no DL TX RF chains) in the DL to the UEs, the UEs don’t have the means to estimate the channel and calculate pathloss between UL-only TRP and UE. However, UE can estimate the channel state information and pathloss between serving gNB (TRP) to UE. If the UEs transmit at the power level intended to reach the serving gNB, it would overwhelm the power amplifier of the UL-only TRPs; thus, the benefits of deploying UL-only TRPs may not be obtained. It may be beneficial to see how much potential performance gain is lost when the UE-to-serving gNB pathloss is used for power control towards UL-Only TRP.

Proposal 4. Study and compare the UL system-level performance in the presence of UL-only TRPs with a) genie aided TPC towards UL-only TRPs, and DL serving gNB to UE pathloss used for TPC towards serving gNB, and b) DL serving gNB to UE pathloss used for TPC towards all UL RX nodes.

On the infrastructure side, the serving gNB is expected to have backhaul connections with the UL-only TRPs. Thus, the serving gNB can have timely channel state information and pathloss values of the UE to UL-only TRPs and UE to serving gNB links. The pathloss difference or the gap needs to be sent to UE for accurate TPC towards the UL-only TRPs. Obtaining the typical range of the pathloss gap value in different deployment scenarios would help to decide on the range of the pathloss gap indication by the gNB. Moreover, the pathloss gap indication may need to carry the quantized values of pathloss gap to reduce the control overhead. Thus, evaluating the performance with different pathloss gap quantization levels would be helpful to decide on the performance versus quantization level trade-off.

Proposal 5: Obtain the CDF curves of the pathloss gap (difference) for typical deployments to determine the range and the granularity of pathloss gap indication.

Proposal 6: Study and compare the performance with the different quantization levels of pathloss gap indication.

There could be multiple ways to convey the pathloss gap indication to the UEs from serving gNB. The power control command of the closed-loop power control over DCI may be leveraged. Or a new MAC control element could be defined to covey such indication.

Proposal 7: Study how to convey the pathloss gap indication to the UE, such as through power control command in DCI or MAC CE.

When the UL-Only TRPs are present in a cell, the serving gNB needs to inform the UE about it directly or indirectly. UL-Only TRPs are a special case of m-TRPs with no DL transmission capability, thus such indication would help the UE to better leverage the benefits of UL-only TRPs.

Proposal 8: Explore the ways to indicate to the UE of the presence of or the switching to an UL-Only TRP.

When the UL-only TRPs are introduced in the network, the transmit power calculation of the UE to UL-Only TRP link needs to use the pathloss of the UE to UL-Only TRP link. As the UE doesn’t have the means to estimate it, the serving gNB sends the pathloss gap (difference) to the UE. The UE estimates pathloss of the serving gNB to UE link and uses along with the pathloss gap indicated by the serving gNB to calculate the pathloss of the UE to UL-only TRP link. Thus, the power control formulation needs to be updated for the UL-Only TRP deployments.

According to section 7 of 3GPP TS 38.213, if a UE transmits PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index 𝑗 and PUSCH power control adjustment state with index 𝑙, the UE determines the PUSCH transmission PPUSCH,b,f,c(i,j,qs,l) in transmission occasion i as

PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ci,POPUSCH,b,f,cqs+10log102μ∙MRB, b,f,cPUSCHi+αb,f,cqs∙PLb,f,cqd-αb,f,cqs∙GPL,b,f,ci,l+∆TF,b,f,ci+fb,f,ci,l

(1)

A new term, the pathloss-gap,  GPL,b,f,ci,l is introduced in the expression (1). It may be defined as follows,

GPL,b,f,ci,l= (Pathloss of UE to serving gNB -Pathloss of UE to UL_only TRP ).

The UE calculates SRS transmission power in dBm for the UE to UL-only TRP link

PSRS,b,f,ci,q,l=minPCMAX,f,ci,POSRS,b,f,cqs+10log102μ∙MSRS,b,f,ci+αSRS,b,f,cqs∙PLb,f,cqd+hb,f,ci,l-αSRS,b,f,cqs∙GPL,b,f,ci,l

(2)

Where, active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, during transmission occasion 'i'. The UE to serving gNB link pathloss is applied to PLb,f,c(qd).

The UE determines its Tx power of PUCCH on UL BW part b of carrier f of serving cell c using PUCCH power control adjustment state with index l, using parameter set qu, path loss reference qd  in PUCCH transmission period i as shown in the equation below.

PPUCCHb,f,ci,qu,qd,l=minPCMAXf,ci,                                        P0PUCCHb,f,cqu+10log102μ*MRBb,f,cPUCCHi+PLb,f,cqd-GPL,b,f,ci,l+∆FPUCCHF+∆TF,b,f,ci+gb,f,c(i,l)

(3)

Proposal 9. For the transmit power calculation of a PUSCH/PUCCH/SRS towards an UL-only TRP, a new parameter is added to the transmission power formulas to account for the pathloss gap (or offset) corresponding to the UL-only TRP.

## Performance Evaluation

Figure 3 UL-only TRP deployment scenario

The UL-only TRP deployment is shown in Figure 3. The UL system simulations assumptions are listed in Table 4 in the Appendix. The node attachment to the UL-only TRP is based on RSRP_UL-only TRP + 3 dB > RSRP_gNB. The UL system-level performance with and without UL-only TRPs is presented in Table 2 to establish the performance gains due to the UL-only TRPs deployment in the cell. The genie-aided TPC is being assumed towards UL-only TRPs.

|  | UMi |  | UMa |  | RMa |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Legacy | UL-only TRP | Legacy | UL-only TRP | Legacy | UL-only TRP |
| Average Spectral efficiency [b/s/Hz] | 3.07 | 8.08 | 3.27 | 8.64 | 3.62 | 8.24 |
| Cell edge Spectral efficiency [b/s/Hz] | 0.006 | 0.053 | 0.0001 | 0.036 | 0.002 | 0.027 |

Table 1. The spectral efficiency performance comparison in the presence of UL-Only TRPs (genie aided)

Observation 1: There is a substantial average and cell-edge spectral efficiency gains when introducing UL-Only TRPs in a cell.

In Table 3 shows the performance comparison of two cases. They are gene-aided TPC for UL-only TRPs, and the serving gNB-to-UE pathloss is used for the TPC towards UL-Only TRP. The performance results show the performance degradation when the serving gNB-to-UE pathloss is used for the TPC towards UL-Only TRP. If the pathloss gap information is available to the UEs, the performance loss could be prevented.

|  | UMi |  | UMa |  | RMa |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Genie aided TPC towards UL-Only TRP | Serving gNB pathloss towards UL-only TRP | Genie aided TPC towards UL-Only TRP | Serving gNB pathloss towards UL-only TRP | Genie aided TPC towards UL-Only TRP | Serving gNB pathloss towards UL-only TRP |
| Average Spectral efficiency [b/s/Hz] | 8.08 | 4.65 | 8.64 | 6.36 | 8.24 | 6.59 |
| Cell edge Spectral efficiency [b/s/Hz] | 0.053 | 0.007 | 0.036 | 0.008 | 0.027 | 0.009 |

Table 2. The spectral efficiency performance comparison of genie aided UL-Only TRP PC and serving gNB pathloss based TPC.

Observation 2:  When the Serving gNB pathloss was used towards the UL-only TRP TPC there is a significant performance degradation.

The Table 4 depicts the performance comparison of gene-aided TPC towards UL-only TRPs and UL-Only TRPs with UE-to-gNB pathloss along with the pathloss gap or difference (sent from gNB) is used for UL-Only TRP power control. The pathloss gap is quantized into 4 levels, 8 levels and 16 levels as shown in Table 5.

|  | UMi |  |  |  | UMa |  |  |  | RMa |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Genie aided TPC towards UL-Only TRP | Serving gNB pathloss along with Pathloss gap |  |  | Genie aided TPC towards UL-Only TRP | Serving gNB pathloss along with Pathloss gap |  |  | Genie aided TPC towards UL-Only TRP | Serving gNB pathloss along with Pathloss gap |  |  |
|  |  | 2bits | 3bits | 4bits |  | 2bits | 3bits | 4bits |  | 2bits | 3bits | 4bits |
| Average Spectral efficiency [b/s/Hz] | 8.08 | 8.08 | 8.00 | 8.16 | 8.64 | 8.21 | 8.64 | 8.72 | 8.24 | 8.72 | 8.35 | 8.50 |
| Cell edge Spectral efficiency [b/s/Hz] | 0.053 | 0.028 | 0.044 | 0.047 | 0.036 | 0.029 | 0.033 | 0.033 | 0.027 | 0.026 | 0.028 | 0.026 |

Table 3.  The spectral efficiency performance comparison of genie aided UL-Only TRP PC and different granularity of the pathloss gap (difference) from gNB

Observation 3: Quantized pathloss gap values (2/3/4 bits) didn’t cause any significant performance loss.

Figure 4. Pathloss values and pathloss difference (gap) for Umi scenario

Figure 5. Pathloss difference (gap) for Umi, Uma and RMa scenarios

Observation 4: The pathloss gap distributions for Umi, Uma and RMa deployments show similar range.

## Related Issues

### UL transmissions within a RACH procedure

In the presence of UL-Only TRPs, the RACH process (both the CFRA and CBRA) may use a lower transmission power level to reach one of the closest UL-Only TRPs and improve the RACH performance in the cell. By the PDCCH order or other means, gNB may indicate the suitable UL-Only TRP for the RACH and the required pathloss gap values. It is beneficial to explore the impacts and benefits of enabling the RACH towards the UL-Only TRPs. In addition, the enhancements to the power control of the PUSCH and PUCCH transmissions after the successful transmission of the RACH preamble may be studied.

Proposal 10: Discuss whether UL transmissions of RACH procedures, such as PRACH, Msg3 (for 4-Step RACH procedure), and MsgA (for 2-step RACH procedure), are in the scope of the working item.

### Rel 17/18 unified TCI framework

Unified TCI framework has been standardized in Rel 17 and 18 for sTRP and mTRPs. The UL-Only TRPs are special case of mTRPs deployment with no DL transmission capability. Thus, the single DCI based separate UL/DL TCI framework may be suitable for UL-only TRPs deployment.

Proposal 11: Use s-DCI based separate DL/UL TCI framework for enabling UL-Only TRPs.

### Enhancement of timing advance procedure

The Rel. 18 MIMO WI included the two TA enhancements for UL multi-DCI for multi-TRP operation. UL-Only TRPs don’t have the capability to transmit DL control and data signals. Thus, it requires 2 TA enhancements for single DCI for multi TRP operation. It was discussed in the item 5 of the WID for the Rel. 19, asymmetric sTRP/mTRP (UL-Only TRP) but was not included. We propose to explore the possibility of including the limited scope 2TA enhancements for single DCI mode to enable UL-Only TRPs.

Proposal 12: we propose to explore the possibility of including the 2TA enhancements for single-DCI mode in the WID scope without extending the TU allocation in order to enable transmissions towards UL-only nodes/TRPs.

# Conclusion

In this contribution, we make following observations

Observation 1: There is a substantial average and cell-edge spectral efficiency gains when introducing UL-Only TRPs in a cell.

Observation 2:  When the serving gNB pathloss was used towards the UL-only TRP TPC there is a significant performance degradation.

Observation 3: Quantized pathloss gap values (2/3/4 bits) didn’t cause any significant performance loss.

Observation 4: The pathloss gap distributions for Umi, Uma and RMa deployments show similar range,

we make the following proposals:

Proposal 1: Agree on having two separate closed-loop power control adjustment states for SRS to facilitate UL-Only TRP deployments.

Proposal 2: Considering two power control adjustment states for SRS for scenarios including UL-only TRP(s), discuss how to enable the indication of a second TPC command in DCI format 2_3.

Proposal 3. Study the UL system-level performance with and without UL-only TRPs to establish the performance gains. Genie-aided TPC may be assumed towards UL-only TRPs.

Proposal 4. Study and compare the UL system-level performance in the presence of UL-only TRPs with a) genie aided TPC towards UL-only TRPs, and DL serving gNB to UE pathloss used for TPC towards serving gNB, and b) DL serving gNB to UE pathloss used for TPC towards all UL RX nodes.

Proposal 5: Obtain the CDF curves of the pathloss gap (difference) for typical deployments to determine the range and the granularity of pathloss gap indication.

Proposal 6: Study and compare the performance with the different quantization levels of pathloss gap indication.

Proposal 7: Study how to convey the pathloss gap indication to the UE, such as through power control command in DCI or MAC CE.

Proposal 8: Explore the ways to indicate to the UE of the presence of or the switching to an UL-Only TRP.

Proposal 9. For the transmit power calculation of a PUSCH/PUCCH/SRS towards an UL-only TRP, a new parameter is added to the transmission power formulas to account for the pathloss gap (or offset) corresponding to the UL-only TRP.

Proposal 10: Discuss whether UL transmissions of RACH procedures, such as PRACH, Msg3 (for 4-Step RACH procedure), and MsgA (for 2-step RACH procedure), are in the scope of the working item.

Proposal 11: Use s-DCI based separate DL/UL TCI framework for enabling UL-Only TRPs

Proposal 12: we propose to explore the possibility of including the 2TA enhancements for single-DCI mode in the WID scope without extending the TU allocation, in order to enable transmissions towards UL-only nodes/TRPs.

Please consider them in the discussions and adopt them.

# References

- RP-234007	New WID: NR MIMO Phase 5

# Appendix

For studying the benefits of introducing the UL-Only TRPs in a cell coverage area and deciding on the operational parameters, UL system simulations may be useful. In Table 4, our simulation assumptions are listed.

| Parameter | gNB | UL-only TRP |
| --- | --- | --- |
| Carrier frequency | 4 GHz |  |
| Bandwidth | 100 MHz |  |
| SCS | 30 kHz |  |
| Network layout | 3 cell | 3 UL-only TRPs placed in a sector |
| Channel model | Umi, Uma, RMa 3GPP TR 38.901 |  |
| BS antenna structure and TXRU | 128Rx = (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.5)λ. TXRU: 16TXRU=(Mp,Np,P,Mg,Ng) =(1,8,2,1,1) | 64Rx = (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.5)λ. TXRU: 8TXRU=(Mp,Np,P,Mg,Ng) =(1,4,2,1,1) |
| UE antenna structure and TXRU | 4Tx = (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, 0.5)λ TXRU: 4TXRU=(Mp,Np,P,Mg,Ng)=(1,2,2,1,1) |  |
| CSI-T | CB-based UL TX |  |
| Channel estimation | Real |  |
| Scheduling | PF |  |
| MIMO receiver (CSI/data) | MMSE |  |
| Traffic model | Full buffer |  |
| ISD | 200m, 500m, 1.732km |  |
| Number of average UEs per macro sector | 30 |  |
| UE power | 23dBm |  |
| UE mobility | Fixed 30 km/h, [mix:80% indoor (3 km/h), 20% outdoor (30 km/h)] |  |
| UL power control | Open Loop TPC | Closed loop TPC |
| Modulation | Upto 64QAM |  |
| UL RX node decision | Legacy | RSRP or Distance based with 3 dB offset |
| CSI feedback delay | 5 ms |  |
| Antenna height | 10m | 10m |
| receiver noise figure | 5 dB | 5dB |
| UE receiver noise figure | 9 dB |  |

TABLE 4. UL System Simulations parameter assumptions