3GPP TSG-RAN WG1 Meeting #116	R1-2400948

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to multi-CSI-PUCCH-ResourceList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN1#114, #114bis and #115 discussed what the UE behaviour should be when the UE is not provided with multi-CSI-PUCCH-ResourceList and PUCCH resources for transmissions of CSI reports overlap in a slot. It became evident that multiple interpretations exist and the safest way to operate the network is to always configure multi-CSI-PUCCH-ResourceList if the gNB operates the UE so that multiple CSI reports of different PUCCH resources may overlap. Summary of the RAN1#115 discussion recorded in R1-2312464, |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying that the UE behaviour is undefined if multi-CSI-PUCCH-ResourceList is not configured and PUCCH resources for transmissions of CSI reports overlap in a slot. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification appears to define the UE behaviour for the case where if multi-CSI-PUCCH-ResourceList is not configured and PUCCH resources for transmissions of CSI reports overlap in a slot, but as different interpretations exist this combination is unusable and should be obsoleted in the specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated impact analysis The impact of the CR is isolated to a case when multi-CSI-PUCCH-ResourceList is not configured and PUCCHs of multiple CSI reports overlap in a slot. - If the gNB is implemented according to the CR it will always configure the UE with multi-CSI-PUCCH-ResourceList if PUCCHs of multiple CSI reports may overlap in a slot and no interoperability issues arise. - If the gNB is not implemented according to the CR and it does not configure the UE with multi-CSI-PUCCH-ResourceList and PUCCHs of multiple CSI reports overlap in a slot the contents of the CSI report is undefined and the CSI report maybe lost. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR in this form. The first draft was submitted to RAN1#114 in R1-2308088 The second draft was submitted to RAN1#114bis in R1-2309901 The third draft was submitted to RAN1#115 in R1-2312117 |  |  |  |  |  |  |  |  |

< unchanged parts not copied >

#### 9.2.5.2	UE procedure for multiplexing HARQ-ACK/SR/CSI in a PUCCH

For a transmission occasion of a single CSI report, a PUCCH resource is provided by pucch-CSI-ResourceList. For a transmission occasion of multiple CSI reports, corresponding PUCCH resources is provided by multi-CSI-PUCCH-ResourceList. If a UE is provided first and second PUCCH-Config, multi-CSI-PUCCH-ResourceList is provided by the first PUCCH-Config, and PUCCH-ResourceId in pucch-CSI-ResourceList or multi-CSI-PUCCH-ResourceList indicates a corresponding PUCCH resource in PUCCH-Resource provided by the first PUCCH-Config. If multi-CSI-PUCCH-ResourceList is not provided, the CSI multiplexing for a transmission occasion of multiple CSI reports is up to the UE implementation.

< unchanged parts not copied >