3GPP TSG RAN WG1 #116			R1-2400964

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		9.6.3

Title:			LP-WUS operation in CONNECTED mode

Source:		Nokia, Nokia Shanghai Bell

Document for:		Discussion and Decision

# 1. Introduction

At the RAN#102 meeting, a new work item was agreed for the “Low-power wake-up signal and receiver for NR” [1].  The objectives for this WID are listed below.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103

In this document, we discuss several different aspects of CONNECTED mode LP-WUS design, including the monitoring occasion design.  From those discussions, we provide some initial observations and proposals.

# 2. Discussion

2.1	Basic Assumptions

The following note is included in the WID.

- Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

The note determines that for CONNECTED mode procedures, it is assumed that UE MR can either be in deep, light, or micro sleep. As the MR is assumed not to be in ultra-deep sleep, the MR frequency source (clocks) can be assumed to be operational. Furthermore, as we are considering CONNECTED mode operation, MR would need to be well synchronised so that it is able to receive PDSCH e.g. at the start of the Active time without the need for any further synchronisation (e.g. via reference signals). Therefore, it can be assumed that MR can provide synchronisation assistance to the LR, and/or LR can benefit from MR time and frequency sources. Hence, we believe that LP-SS is not needed for LR CONNECTED mode operation and that the LR time and frequency synchronisation is same as MRs.

Observation 1: 	For LP-WUS CONNECTED mode, LR time and frequency synchronisation can be provided by the MR.

Proposal 1: 	RAN1 confirms that LP-SS is not needed for CONNECTED mode operation with LP-WUR, and LP-SS is to support IDLE/Inactive mode devices.

It is also noted, that CONNECTED mode measurements (RRM/RLM/BFD/CSI) are performed by MR. As there is no explicit intent nor objective in the work item to change, offload or relax the MR based measurements in CONNECTED when LP-WUS is being monitored (contrary to IDLE/Inactive mode) the MR requirements can be kept unchanged.

Proposal 2: 	RAN1 confirms that no change is assumed for existing MR based measurements (RRM/RLM/BFD/CSI) due to LP-WUS monitoring in CONNECTED mode.

## 2.2	Connected Mode WUS Monitoring

In this sub-section, we discuss the relative pros and cons of different connected mode WUS monitoring schemes.  We split these schemes into 2 categories, depending on whether cDRX is configured or not.

### 2.2.1	LP-WUS schemes with cDRX

In this subsection, we describe 2 schemes which share the following characteristic:

- Conventional cDRX is configured and provides:

  - A mechanism to deactivate PDCCH monitoring e.g. via inactivity timer.

  - A fallback mechanism in the event WUS becomes unavailable. E.g. if LP-WUS is not monitored, UE can monitor PDCCH at least based on the cDRX configuration.

    - Relaxation for the RRM measurements (e.g. mobility and RLM) in RAN4 based on the applied DRX cycle. Noting also that CSI measurements are constrained to Active time in RAN1.

For deactivation or fallback to PDCCH monitoring, several mechanisms have been considered during study item in RAN2, e.g., using RSRP or traffic presence, which can be discussed later when basic schemes are decided. Thus, we will firstly focus on finding differences/challenges of each scheme.

#### 2.2.1.1    LP-WUS Scheme 1a:   LP-WUS MO positioned at a fixed offset from the OnDuration (like DCP)

Scheme 1a can be seen as a replacement of the Rel-16 DCP operation where the (LP-)WUS is used to trigger the start of the onDurationTimer. If the onDurationTimer is not triggered, the UE is not required to PDCCH during the timer, while maybe expected to carry out CSI related procedures (as configured, similarly as in Rel-16 DCP). The LP-WUS monitoring could in principle be continuous but would be more naturally periodic. In both cases there would need to be some minimum time offset for triggering the onDurationTimer to give MR time to wake up. The key characteristics of this scheme 1a can be considered to include:

- The LP-WUS monitoring occasions are defined within a LP-WUS monitoring window that precedes the cDRX configured onDurationTimer location i.e. like Rel-16 WUS.

  - This window can contain multiple LP-WUS channel occasions.

  - Within the monitoring window, whether the LR uses a duty cycle or continuous mode of monitoring can be discussed.

  - There’s a time offset between the LP-WUS monitoring window and the start of the onDurationTimer to provide the MR time to wake up from its sleep mode.

    - Per TR 38.840 [3], this offset can be expected to be up to 20ms for the deep sleep mode and 6ms for the light sleep mode.

cDRX cycle

Figure 1:    Scheme 1a: LP-WUS MO fixed offset from OnDuration (like DCP)

#### 2.2.1.2   LP-WUS Scheme 1b:   LP-WUS MO flexibly positioned outsideActive time

Scheme 1b can be seen as an extension of the afore DCP-like scheme, Scheme 1a, where the PDCCH monitoring can be more flexibly triggered outside the Active time. The monitoring outside Active time could be periodic or continuous.  The key characteristics of this scheme 1b include:

- LP-WUS positive detection triggers PDCCH monitoring, i.e. Aactive time

  - This Active time can be defined by the cDRX onDuration timer or by a different timer/procedure.

    - To ensure the use of legacy cDRX configuration as a fall-back

  - There’s a time offset between the WUS occasion and the start of an Active period to allow the MR to wake from it’s sleep mode.

    - The configured time offset implicitly decides the sleep mode of the MR, eg for latency sensitive traffic, a lower time offset maybe required, eg 6ms, limiting the MR to light sleep rather than deep sleep.

- LP-WUS monitoring occasions are defined anywhere outside ACTIVE time

  - Assuming also that if onDurationTimer (as set by cDRX configuration) is not triggered by LP-WUS, UE can monitor LP-WUSWhether this monitoring is duty cycled or continuous can be discussed.

Figure 2:  LP-WUS MO flexibly positioned outside Active Time

There is similarity to Scheme 1a with a shorter cDRX cycle, i.e. shortened cRX cycle resulting more frequent LP-WUS occasions. The key differences compared to Scheme 1a include:

- As cDRX is mostly used for fall-back, it can be configured in a more relaxed manner, resulting less power consumption due to more relaxed measurement activity (assuming that it is based to the applied DRX cycle)

- The allowed MR sleep state can be set by the configured offset independent of the configured cDRX cycle

### 2.2.2	LP-WUS schemes without cDRX

#### 2.2.1.2   LP-WUS Scheme 2:   LP-WUS MO during Active time to trigger PDCCH monitoring

In this scheme it is assumed that LP-WUS can be used to trigger PDCCH monitoring, and some additional methods are introduced to stop the PDCCH monitoring without configuration of cDRX.-, Effectively this results similar behaviour as considered above for Scheme 1b albeit there is uncertainty regarding the fall-back procedure and MR activity level. Hence, the benefits of this scheme over scheme 1b (with cDRX) are unclear.  The characteristics of Scheme 2 include:

- LP-WUS is monitored at least when PDCCH is not monitored

  - LP-WUS monitoring can be duty-cycled or continuous

  - PDCCH monitoring can be stopped based on timer or by DCI

- MR carries the measurements as normally in CONNECTED mode without cDRX

  - To relax these, complementary design to cDRX would be need thus not assumed

- As a fall-back UE would be expected to monitor PDCCH continuously.

To operate without any form of cDRX, can only be justified if there is:

- Near continuous incoming traffic, but by definition, such traffic has limited opportunities for powering down the MR

  - If there are natural gaps in the traffic, then there should be opportunities to exploit existing DRX configuration options to save power with or without LP-WUS enhancements.

Operating without cDRX also raises 3 significant technical questions.

Question 1:       A mechanism will be needed to end PDCCH monitoring.

- Defining a new inactivity timer, would potentially just be duplicating the existing cDRX inactivity timer functionality. DCI based approach would correspond to PDCCH skipping approach discussed in next section.

Question 2:    	How to account the MR mobility and CSI measurements?

- Mobility etc. measurement relaxations are currently only defined for C-DRX and based on the applied DRX cycle.

  - Without these relaxations UE should meet the normal measurement requirements, i.e. all the time.

- Carrying out measurements in normal mode, restricts the MR sleep state and the attainable power saving gain.

- Defining new, similar UE activity-based measurement relaxations as for C-DRX, would implicitly mean defining new C-DRX operation.

Question 3:	What fallback mechanism is applied in the event WUS coverage is lost?

- With high levels of traffic, one could create a new “no wus detected” timer to trigger continuous PDCCH monitoring.

  - If the traffic has low latency requirement, this timer will need to be short. With low levels of traffic, this would result in frequent unnecessary PDCCH monitoring, wasting power unnecessarily.

  - As the UE cannot autonomously stop PDCCH monitoring, a new network interaction maybe required to stop PDCCH monitoring and resume power saving. Any more sophisticated fallback mechanism will most likely, implicitly mean defining new C-DRX operation.

### 2.2.3	Discussion of LP-WUS monitoring schemes

Given the additional design and specification challenges associated with LP-WUS schemes not applying cDRX, i.e. the need for new mechanisms to:

- to end PDCCH monitoring,

- moderate/relax ongoing MR measurements which should be dependent on the activity like with cDRX

- provide a fallback method in the event of loss of LP-WUS

we propose deprioritising the specification of a LP-WUS scheme without c-DRX.

Proposal 3:     	RAN1 deprioritises the support of LP-WUS schemes that operate without cDRX.

Comparing the 2 schemes discussed with cDRX, both schemes have the advantage over the non-cDRX scheme of:

- Reusing of existing cDRX, which eliminates a lot of additional 3GPP work in defining:

  - new fallback mechanisms

  - relaxation for MR measurements to obtain power saving.

  - new mechanism to end PDCCH monitoring.

Scheme 1a additionally has the advantage of being easy to specify, given it can reuse much of the procedural text that already exists for the Release-16 DCP.

However, as observed in the SI, when the cDRX cycle is comparable in length to the wake-up time of the MR, the LP-WUS scheme can cause a noticeable degradation to the UPT when compared to the equivalent DCP configuration.

The Technical report TR38.396 [2] for the LP-WUS SI, makes the following observation for this scheme.

For the usage of LP-WUS as the wake-up mechanism of C-DRX similar to DCP - Significant UE power saving gain (up to more than 30%) and moderate degradation to the UPT (less than 10%) can be observed, especially when the C-DRX cycle is shorter (e.g. 40ms) .

Scheme 1b, whilst requiring more specification effort that scheme 1a, has the following advantages:

- Option to use longer cDRX cycles to reduce MR power consumption used for measurements further, yielding greater power savings, without negative impacts to latency/UPT.

    - Latency/UPT response can be maintained or even improved by support of more frequent LP-WUS monitoring occasions.

Proposal 4:     		RAN1 prioritises a RRC connected mode LP-WUS scheme with cDRX that through flexible positioning of LP-WUS monitoring occasions allows PDCCH monitoring to be triggered anywhere outside Active Time.

## 2.3	LP-WUS operation with Release-17 PDCCH Skipping

In this sub-section we describe a possible use of LP-WUS to enhance the existing PDCCH skipping procedure either independently of the aforementioned LP-WUS schemes or to support particular LP-WUS schemes.

### 2.3.1   Background

For Release-17, 2 power saving schemes were specified that enabled the network to use either 1 or 2 bits within existing DCI-formats 0_1/0_2/1_1/1_2, to signal to a UE to either:

PDCCH Skipping:

Skip a pre-configured defined duration in which the UE is not required to monitor PDCCH monitoring.

The 3 possible non-zero PDCCH skipping durations above, are preconfigured by RRC using the pdcch-SkippingDurationList-r17 IE

pdcch-SkippingDurationList Provides one or more values to derive the skipping duration in unit of slots, as specified in TS 38.213 [13], clause 10.4. The DCI which schedules data indicates which of the values is to be applied (see TS 38.213 [13], clause 10.4). For the 15kHz SCS, for each entry, only the first 26 values are valid and correspond to {1, 2, 3, …, 20, 30, 40, 50, 60, 80, 100}. For the 30kHz SCS, for each entry, only the first 46 values are valid and correspond to {1, 2, 3, …, 40, 60, 80, 100, 120, 160, 200}. For the 60kHz SCS, for each entry, only the first 86 values are valid and correspond to {1, 2, 3, …, 80, 120, 160, 200, 240, 320, 400}. For the 120kHz SCS, for each entry, the 166 values correspond to {1, 2, 3, …, 160, 240, 320, 400, 480, 640, 800}. For the 480kHz SCS, for each entry, the 166 values correspond to {4, 8, 12, …, 640, 960, 1280, 1600, 1920, 2560, 3200}. For the 960kHz SCS, for each entry, the 166 values correspond to {8, 16, 24, …, 1280, 1920, 2560, 3200, 3840, 5120, 6400}.

### 2.3.2   Early Termination of PDCCH Skipping using LP-WUS

If traffic has a low latency requirement, but bursty traffic rate, the use of skipping can provide some power saving opportunities. However, the skipping duration can be set too aggressively due to the possible latency requirement of the traffic. This can be overcome if the LP-WUS can be used to terminate the PDCCH skipping duration. Hence, once PDCCH skipping is triggered by DCI UE would start to monitor LP-WUS.  If the LP-WUS is detected within PDCCH skipping period, it could trigger the termination of skipping and UE would resume PDCCH monitoring.

Proposal 5:     	RAN1 study the use of LP-WUS to support early termination of PDCCH skipping.

## 2.4	Handling potential collisions between MR and LR

To minimise power consumption, the LR and MR should ideally never be on at the same time.  However, there could be times when the LR may be awake and monitoring for the WUS, when the MR is also awake performing ongoing measurements.  For these collision scenarios, we see at least 3 options:

Option 1:    Specifications mandate that the UE does not expect to be configured with a Connected mode WUS monitoring occasion that collides an occasion that the MR is awake for measurement purposes.

Option 2:     Specifications mandate the UE behaviour in the event of this type of LR and MR collision and forces either the WUS monitoring or the measurement procedures to be dropped.

Option 3:      Specifications support the collision and leave it to UE implementation to decide how to support simultaneous measurement and WUS monitoring.

Of the above options, we prefer option 3 for the following reasons:

- Maximises scheduling opportunities for WUS occasions and MR measurements.

- Minimises specification changes.

- Given the WUS signal configuration (is it on the same band as that being used for measurements?) and MR architecture, there is the possibility the UE can turn off the LR during the collisions and migrate the WUS detection functionality to the MR that is already on performing measurements.

Proposal 6:     	Simultaneous monitoring and detection of WUS whilst the MR is ACTIVE (eg performing measurements, measurement before SR), is supported.

FFS:   If this is a separate UE Capability

## 2.5	Supporting multiple UEs using one CONNECTED mode WUS

One feature of the existing Release 16 DCI based DCP WUS that we believe should be migrated to the new LP-WUS for connected mode, is the ability to use one WUS message to provide multiple UEs with their own UE-specific PDCCH monitoring indication bit.  For the release 16 version of this functionality,

- the DCP DCI contains a single PDCCH indication bit for each of the UEs, to indicate if to monitor or not the next PDCCH.

- RRC configuration is used to inform individual UEs which of the PDCCH indicator bits to use.

Whether this simple scheme can/should be copied across or whether a more sophisticated scheme is specified, can be studied further.

Proposal 7:    	A single LP-WUS message for connected mode, can support multiple UE specific PDCCH monitoring indicators for multiple UEs.

FFS:   How LP-WUS payload is defined to support this functionality.

## 2.6	Duty cycle or continuous operation?

As observed within the TR, for idle mode operation, duty-cycled LP-WUS monitoring offers higher power saving gains.

Compared with existing I-DRX operation, - Significant UE power saving gain (up to more than 90%) is observed by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. - For duty-cycled LP-WUS monitoring and same duty ratio, higher power saving gain is observed if the relative power consumption of LP-WUR ON for LP-WUS monitoring is lower. - For continuous LP-WUS monitoring, UE power saving gain is only observed if the relative power consumption of LP-WUR ON for LP-WUS monitoring is lower, e.g. no larger than 1 unit.

Unlike idle mode, for connected mode, there are some use-cases that are extremely delay sensitive and have a high jitter, such as, XR traffic.   For schemes addressing this type of operation, the LP-WUS ‘latency’ should be short. This may require some consideration on how to define the UE LP-WUS monitoring for such scenarios. When considering the difference between continuous monitoring and duty-cycled (period), the difference is effectively only how frequently the LP-WUS monitoring window starts. For continuous monitoring the assumption is that the LP-WUS monitoring window would start at every (NR) symbol, while for duty cycled, some less frequent periodicity could be considered, e.g. at slot border. For other type of traffic scenarios, ie.  with C-DRX, the duty-cycled (periodic) would be suited. In the Scheme 1a/b considered in previous sections, periodic LP-WUS monitoring would seem applicable.

Observation 2: 	Duty-cycled (periodic) LP-WUS would be applicable for CONNECTED mode with C-DRX configuration.

In CONNECTED mode operation, the MR power consumption is expected to dominate, and thus applying more frequent LP-WUS monitoring does not have significant impact. Effectively more frequent monitoring may cause more frequent wake-ups due to false alarms. In IDLE/Inactive mode these can have significant impact to the attainable power saving gain. However, in CONNECTED mode, as it can also be assumed that when more frequent LP-WUS monitoring is associated with low latency, thus the MR sleep could be assumed to be ‘high’ i.e. micro sleep, to meet the latency target, there would be no or very limited MR transition cost.

Observation 3: 	More frequent monitoring in CONNECTED mode has more limited cost in terms of power saving gains

A drawback of the continuous monitoring is that if it could limit the number of UEs that can be supported by LP-WUS as there is no possibility to TDM the LP-WUS for different UE (groups). I.e. depending how the LP-WUS message handless the UE multiplexing, continuous monitoring can either limit the number of individual UE to be supported or increase the false alarm of the UEs.

Observation 4: 	Continuous monitoring can limit the LP-WUS capacity in terms of concurrent simultaneous UEs

Hence, as duty-cycled monitoring (periodic) would be the baseline, adapting that to achieve to ‘almost continuous’ monitoring could be considered. For example, one could envision, for CONNECTED mode especially, that LP-WUS monitoring occasions are defined like PDCCH monitoring is configured today, i.e. a search space defined by multiple ‘starting’ symbols for LP-WUS monitoring occasions within a slot.

Proposal 8:     	RAN1 to further evaluate the monitoring frequency and occasion placement for LP-WUS in CONNECTED mode.

## 2.7	Network knowledge of WUS usage

In connected mode, LP-WUS is likely to be used more in a UE-specific mode, meaning that widespread usage will be costly in terms of over-the-air resources.  From a network perspective, it is preferable that the network knows if transmitted LP-WUS is being used as intended to avoid wasting resources.

Proposal 9:   	RAN1 specify a reporting mechanism to enable the Network to determine if the transmitted LP-WUS is being used as intended.

# 3.   Conclusions

# In this document, we have discussed several different aspects of CONNECTED mode LP-WUS design.  From those discussions, we have the following observations and proposals.

Observation 1: 	For LP-WUS CONNECTED mode, LR time and frequency synchronisation can be provided by the MR.

Observation 2: 	Duty-cycled (periodic) LP-WUS would be applicable for CONNECTED mode with C-DRX configuration.

Observation 3: 	More frequent monitoring in CONNECTED mode has more limited cost in terms of power saving gains

Observation 4: 	Continuous monitoring can limit the LP-WUS capacity in terms of concurrent simultaneous UEs

Proposal 1: 	RAN1 confirms that LP-SS is not needed for CONNECTED mode operation with LP-WUR, and LP-SS is to support IDLE/Inactive mode devices.

Proposal 2: 	RAN1 confirms that no change is assumed for existing MR based measurements (RRM/RLM/BFD/CSI) due to LP-WUS monitoring in CONNECTED mode.

Proposal 3:     	RAN1 deprioritises the support of LP-WUS schemes that operate without cDRX.

Proposal 4:     		RAN1 prioritises a RRC connected mode LP-WUS scheme with cDRX that through flexible positioning of LP-WUS monitoring occasions allows PDCCH monitoring to be triggered anywhere outside Active Time.

Proposal 5:     	RAN1 study the use of LP-WUS to support early termination of PDCCH skipping.

Proposal 6:     	Simultaneous monitoring and detection of WUS whilst the MR is ACTIVE (eg performing measurements, measurement before SR), is supported.

FFS:   If this is a separate UE Capability

Proposal 7:    	A single LP-WUS message for connected mode, can support multiple UE specific PDCCH monitoring indicators for multiple UEs.

FFS:   How LP-WUS payload is defined to support this functionality.

Proposal 8:     	RAN1 to further evaluate the monitoring frequency and occasion placement for LP-WUS in CONNECTED mode.

Proposal 9:   	RAN1 specify a reporting mechanism to enable the Network to determine if the transmitted LP-WUS is being used as intended.

# 4.   References

[1]     RP-234056 New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2]     TR28.869   Study on low-power wake-up signal and receiver for NR (Release 18)

[3]     TR38.840   Study on User Equipment (UE) power saving in NR