3GPP TSG RAN WG1 #116	R1-2400969

Athens, Greece, February 26 – March 1, 2024

Agenda item:		5

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations for satellite switch with resync for NR over NTN

Document for:		Discussion and Decision

# Introduction

For this meeting, RAN2 provided an LS on the topic of satellite switch with resync[1], which was earlier discussed in RAN1 under the topic of “Satellite switch with unchanged PCI”. In the LS RAN2 outlines the agreements that have been reached under this topic and asks whether RAN1 has any concerns.

In this contribution we will discuss some of the potential concerns that are seen from RAN1 point of view.

# Discussion

## Satellite switching with re-sync

In their LS RAN2 has indicated that they have specified a new feature (“satellite switch with re-sync”), which might be configured in two different types: soft and hard. According to the description of the feature functionality, they are exactly the same except for the differences highlighted in the box below:

|  | Soft satellite switch with re-sync | Hard satellite switch with re-sync |
| --- | --- | --- |
| Procedure start time | Between t-start and t-service (UE decision) | At t-service |
| SSB information of target cell | Mandatory | Optional |

RAN2 goes on to ask RAN1 whether based on such principles and on the procedural description presented in [2], whether there are concerns to the feasibility of UE performing downlink synchronization to the target satellite while the communication with the source satellite is going on.

The first feasibility concern for the acquisition of synchronization signals between source and target satellite depends on whether the SSB signals radiated by source and target satellites are not overlapping at UE reception. According to the description provided in [1] [2], the network will notify the UE about a transmit timing offset between source and target satellites:

Agreements according to [2]: - At least for soft switch, there needs to be an “SSB time offset” between the source and the target satellite. “SSB time offset” is specified as a new IE, with the same format as “offset” in SSB-MTC4 - Target satellite SSB tracking is handled autonomously by the UE based on the provided SSB time offset - The “SSB time offset” between the source and the target satellite should be provided in SIB19 - For soft satellite switch, as a baseline, it is sufficient to provide the “SSB time offset” of the target satellite in SIB19. (Can come back in the next meeting to check whether a different SSB index for the target satellite can optionally be provided)

The following analysis is provided assuming the “offset” is defined similarly to the “offset” used in SMTC4, i.e., it is a transmission time offset at the gNB:

From TS 38.331: smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

Following this, the feasibility will be determined based on whether or not it is possible to guarantee – via the offset in timing between the two nodes – that the SSB transmitted do not overlap at UE reception.

For clarity and brevity, the calculations used to derive the conclusions in this section are presented in Annex A of our corresponding RAN4 contribution [3]. From this contribution it is found that:

Observation 1: For NGSO, it is possible to guarantee that a proper offset can be set in any scenario to avoid overlap between SSBs.

Observation 2: For GSO, it is not always possible to guarantee non overlapping SSB at UE reception in every scenario simply based on offset.

Proposal 1: RAN1 should make an observation that soft satellite switch is only feasible for NGSO scenarios.

Another critical aspect of the soft-switch operation is that the current understanding is that during the “soft-switch duration”, i.e., the interval between t-start and t-service, both satellites will be broadcasting the same cell, while UEs are slowly moved from one anchor satellite to another, mainly driven by different UEs internal triggers. This means, that DL transmissions in the target satellite start earlier for some UEs, while other UEs might still be attached to the source satellite.

However, as it is demonstrated in the annex in [3], the DL delay differential observed by different UEs in the same NGSO cell can be as high as 6.28 ms. Meaning, the overlapping slots vary significantly depending on the observing UE.

In this case, source and target satellites will both be broadcasting the same cell, in the same geographical area but with different overlapping slots at UE reception. The DL payload data allocated to some UEs will necessarily constitute interference to the other UEs. In this case, depending on the difference in power between source and target satellite one or both SSBs (from source and target) might not be fully decodable.

Observation 3: Depending on the difference in power between source and target satellite, the destructive interference caused by the DL payload from either source or target satellite might make the SSB data from the other satellite not decodable at the UE side.

- Proposal 2: RAN1 should make an observation that soft satellite switch may create excessive inter-satellite interference during the switching window for both UL and DL.

# Conclusion

In this contribution we have analyzed the LS from RAN2 and have the following observations and proposals:

Observation 1: For NGSO, it is possible to guarantee that a proper offset can be set in any scenario to avoid overlap between SSBs.

Observation 2: For GSO, it is not always possible to guarantee non overlapping SSB at UE reception in every scenario simply based on offset.

Observation 3: Depending on the difference in power between source and target satellite, the destructive interference caused by the DL payload from either source or target satellite might make the SSB data from the other satellite not decodable at the UE side.

Proposal 1: RAN1 should make an observation that soft satellite switch is only feasible for NGSO scenarios.

- Proposal 2: RAN1 should make an observation that soft satellite switch may create excessive inter-satellite interference during the switching window for both UL and DL.

# References

- R1-2400009, ”LS on RAN2 agreements for satellite switch with resync”, RAN2 (R2-2314016)

- R2-2313877, “RACH-less satellite switch procedure”, Apple, RAN2#124bis, Chicago, USA, Nov 13 – Nov 17, 2023.

- R4-2402384, ”Considerations on Satellite Switching with re-sync”, RAN4, Nokia, Nokia Shanghai Bell