3GPP TSG RAN WG1 #116	R1-2400972

Athens, Greece, February 26 – March 1, 2024

Agenda item:		9.11.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RedCap support for NR over NTN while operating in FR1-NTN bands

Document for:		Discussion and Decision

# Introduction

In this contribution we address the release 19 objective on RedCap support for NR over NTN. The objective is captured in [1]:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In this contribution we will focus on the support of half duplex (HD) RedCap UEs, as this is the only subtopic which is targeted for RAN1 discussions.

# Discussion

The HD operation is defined in TS 38.213 and basically the UE is not capable of receiving and transmitting simultaneously.

## 17.2 Half-Duplex UE in paired spectrum A half-duplex UE (HD-UE) in paired spectrum is not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum. This clause is applicable for communication of a HD-UE on a serving cell with paired spectrum. Procedures for a HD-UE are same as described for a UE in all other clauses of this document unless stated otherwise. A HD-UE does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols.

The identification of a collision between uplink and downlink is relatively simple in Terrestrial Networks (TN) due to the limited propagation delay. The base station is in control of the Timing Advance and thus the base station is capable of calculating when an uplink transmission will be initiated on UE side, while at the same time also calculating when the UE will receive a certain downlink transmission.

Observation 1: In TN the base station can calculate when a UE will receive and transmit based on the Timing Advance controlled by the base station.

However, in Non-Terrestrial Networks (NTN) the propagation delay per UE varies and takes on values which greatly exceed the slot duration. For example, the TR 38.821 defines the cell differential delay for a 600 km LEO satellite which may exceed 3 ms. Furthermore, the UE is responsible for the autonomous time-domain pre-compensation of uplink transmissions covering the propagation delays of the service link and potentially the feeder link as well. This means it is challenging for the base station to determine when the UE will start an uplink transmission and whether such uplink transmission will collide with a downlink reception. The scenario is illustrated in Figure 1.

Observation 2: In NTN the propagation delay per UE is large and varying and the UE is controlling the Timing Advance via uplink pre-compensation, which makes it challenging for the base station to identify collisions.

Figure 1 Blocking depending on UE Timing Advance.

In IoT NTN release 17 the same issue existed because NB-IoT UEs and some eMTC UEs are HD devices. The issue was addressed by specifying the UE reports the applied Timing Advance in a “Timing Advance Report MAC Control Element” (see TS 36.321). The UE is triggered to report the Timing Advance as follows (TS 36.321):

### 5.4.9 Timing Advance Reporting … Timing Advance reporting shall be triggered if any of the following events occur: - if triggered by upper layers; - upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell; - if the variation between current information about Timing Advance and the last reported information about Timing Advance is equal to or larger than offsetThresholdTA, if configured.

According to TS 36.331 the offsetThresholdTA is in the range [0.5, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15] ms.

A similar solution could be adopted for RedCap because it enables the base station to estimate if and when there will be a collision.

Observation 3: The IoT NTN approach of UE reporting the applied Timing Advance may be used for collision prediction in RedCap NTN.

However, a key challenge for the IoT NTN solution is that even a stationary UE has to transmit Timing Advance reports (the number is depending on the offsetThresholdTA) due to the satellite movement while the UE is in RRC_CONNECTED mode. This results in signaling overhead and therefore shorter UE battery life.

Observation 4: Stationary UE Timing Advance reporting caused by satellite movement results in signaling overhead and therefore shorter UE battery life.

In our view, RAN1 should discuss whether the UE can report an indication of its location to the base station instead. By being aware of the approximate UE location the base station can estimate the current propagation delay and how it will change due to the satellite movement. It should be noted that the approximate UE location may be very coarse (in the range of a few km as provided with the already existing coarse location information in TS 38.331) since the gNB will only need to know the approximate impact to the UE experienced delay at any time through a satellite fly-over. With such an approach the UE will not need to provide regular updates on its timing advance information whenever this value changes beyond the configured threshold, as the gNB would automatically be able to evaluate the propagation delay and be aware of potential timing conflicts.

Observation 5: Using an approximate UE location the base station can estimate the Timing Advance.

Proposal 1: RAN1 to discuss if an approximate UE location can be reported by the UE instead of the current Timing Advance approach that is defined for IoT NTN.

The HD operation is described in detail in TS 38.213 based on the signal/channel the UE is receiving and transmitting.

For example, it is defined that the UE does not receive PDCCH, PDSCH, CSI-RS or DL PRS in a set of symbols if the UE is not scheduled via DCI to transmit PUSCH, PUCCH, PRACH or SRS in at least one symbol of the set. Likewise there are rules about uplink transmission colliding with the SSB reception.

In our view, RAN1 has to analyze further whether the current HD collision rules enable NTN operation or changes are needed. One key aspect here would be to take into account the much larger RTT that is observed for NTN operation compared to operation in regular terrestrial networks. The collision rules would need to be considered for both GSO and non-GSO, and in our understanding it is crucial that both scenarios are considered, as the non-GSO scenario will have high dynamics in the timing advance domain due to the satellite movement, while the GSO case would experience large values for the RTT, which may impact the collision rules (especially the prioritization between uplink and downlink). Further, considering that the UE may be experiencing quite large RTT, it may be that the existing HD collision rules from terrestrial networks need to be revisited to evaluate whether any changes are needed to accommodate NR-NTN operation.

Proposal 2: For the collision rule evaluations it is crucial that both GSO and non-GSO deployment scenarios are being considered.

Proposal 3: RAN1 to consider whether different HD collision rules should be applied for NTN compared to TN.

Proposal 4: RAN1 to analyze further whether the current half duplex collision enable NTN operation.

# Conclusion

In this contribution we have presented our brief analysis of the HD RedCap and eRedCap situation with respect to operation and overhead. Our observations and proposals are as follows:

Observation 1: In TN the base station can calculate when a UE will receive and transmit based on the Timing Advance controlled by the base station.

Observation 2: In NTN the propagation delay per UE is large and varying and the UE is controlling the Timing Advance via uplink pre-compensation, which makes it challenging for the base station to identify collisions.

Observation 3: The IoT NTN approach of UE reporting the applied Timing Advance may be used for collision prediction in RedCap NTN.

Observation 4: Stationary UE Timing Advance reporting caused by satellite movement results in signaling overhead and therefore shorter UE battery life.

Observation 5: Using an approximate UE location the base station can estimate the Timing Advance.

Proposal 1: RAN1 to discuss if an approximate UE location can be reported by the UE instead of the current Timing Advance approach that is defined for IoT NTN.

Proposal 2: For the collision rule evaluations it is crucial that both GSO and non-GSO deployment scenarios are being considered.

Proposal 3: RAN1 to consider whether different HD collision rules should be applied for NTN compared to TN.

Proposal 4: RAN1 to analyze further whether the current half duplex collision enable NTN operation.

# References

- RP-234078, ”New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”