3GPP TSG RAN WG1 #116	R1-2400973

Athens, Greece, February 26 – March 1, 2024

Agenda item:		9.11.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR-NTN uplink capacity/throughput enhancements

Document for:		Discussion and Decision

# Introduction

In RAN#102 a new work item on NR NTN (Non-Terrestrial Networks) enhancements was approved [1]. Among the objectives of the work item, there is uplink capacity/throughput enhancement for FR1-NTN, in particular using orthogonal cover codes (OCC). The objective description is as follows:

2. Uplink Capacity/Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Study then specify, if beneficial, DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC) - Determine the achievable capacity improvement to be targeted taking into account realistic impairments (e.g. Doppler, time variation, phase distortion, etc) - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed - Note: The study can consider orthogonal cover codes across OFDM symbols, across slots, and/or within an OFDM symbol. - Note: the study phase is targeted to be completed by RAN#104 - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

RAN1 should hence focus on discussing the applicability of OCC to NTN NR uplink data channel (i.e. PUSCH) and on identifying potential issues and enhancements if necessary, considering the NTN characteristics. Based on this guidance, this contribution provides our views on the above matters.

# Discussion

## Simulation setup

To validate the benefits of OCC-enabled PUSCH and identify corresponding limits, simulation studies are essential. Here, we outline our perspective on the simulation methodology, the steps to be undertaken, and the assumptions to be adopted in order to gain a comprehensive understanding of the potential performance degradation and capacity improvement in OCC-enabled PUSCH transmissions. Adding a feature like OCC on the data transmissions may introduce significant changes or modifications to the existing specifications, so adopting OCC should only happen if/when the feature shows to be both robust to impairments and efficient in terms of ability to multiplex UEs.

Proposal 1: The feature of OCC need to be evaluated thoroughly in terms of robustness and efficiency prior to determining to specify it.

### Simulation steps for feature evaluation

In order to identify the possible enhancements of uplink data capacity using OCC, first we need to evaluate the performance of OCC-enabled PUSCH and compare the results with scenarios without OCC. Considering non-ideal channel conditions, and therefore non ideal interference cancellation through OCC, it is expected that OCC application impacts the performance of single user PUSCH, and such degradation should be small enough to justify the introduction of OCC for capacity enhancements.

To understand the performance impact of applying OCC to PUSCH transmission, we suggest assessing the baseline by evaluating the BLER of a single UE PUSCH without OCC to establish a reference point and determine the SNR value for which the BLER is at 10%. Then BLER performance are evaluated for OCC enabled PUSCH for a number of UEs with overlapping PUSCH transmissions but different OCC codes, in the determined SNR range for single UE PUSCH. Finally any potential performance degradation of using OCC on PUSCH transmission is determined by evaluating the SNR gap at 10% BLER between the single UE PUSCH and OCC based PUSCH.

Proposal 2: Assess the performance impact of OCC-enabled PUSCH in comparison to PUSCH without OCC through link-level simulations. The evaluation involves the following steps:

- Step 1: Evaluate the single UE performance without OCC-enabled PUSCH and determine SNR for BLER at 10%

- Step 2: Evaluate BLER performance for OCC enabled PUSCH where there are other OCC-enabled PUSCH transmissions overlapping but with different OCC. The evaluation is performed within the determined SNR range for single UE PUSCH.

- Step 3. Determine SNR gap at 10% BLER between the single UE PUSCH and OCC enabled PUSCH to identify any potential performance degradation.

### Simulation assumptions

Inspired from the RAN1 #109-e agreements [2] for simulation assumptions concerning coverage evaluation of PUSCH in NR NTN, we propose to use the simulation assumptions in Table I for the performance evaluation of OCC-enabled PUSCH.

Table 1 Simulation assumptions for OCC evaluations.

| Assumptions | Value |
| --- | --- |
| Scenario | NTN, both GSO and Non-GSO to be considered |
| Frequency | 2 GHz |
| Waveform | DFT-s-OFDM |
| Subcarrier spacing | 15kHz |
| BLER Target | 10% |
| Channel model | NTN-TDL-C |
| Delay spread | 100ns |
| Sat. antenna configuration | Circular polarization |
| UE antenna configuration | (1,1,1) with omni-directional antenna element |
| Frequency offset | TBD, depending on deployment scenario |
| UE velocity | 3km/h |
| Number of code-division multiplexed users | Minimum 2 – Maximum to be reported by companies. |
| OCC characteristics | Companies to report OCC details such as length and type of OCC, mapping of OCCs, time or frequency domain |
| PRBs/MCS | PRB between 1 and 4 / MCS between 0 and 2 |
| PUSCH duration | 14 OSs |
| DMRS configuration | 2 DMRS symbols, no multiplexing with data. |
| Repetitions | k* OCC length <= 32; k = 1, 2, ... |
| Frequency hopping | No frequency hopping |

In Table 1 we propose simulation assumptions targeting a scenario where OCC is implemented across repetitions, specifically referring to inter-slot OCC. This involves multiplying part of or the entire set of repetitions by a single OCC sequence, ensuring orthogonality to OCC sequences employed by other concurrently multiplexed UEs. It is noteworthy that the size of the repetition, constrained by legacy considerations, may have varying relationships with the size of the OCC code. However, regardless of this relationship, the number of repetitions should adhere to the maximum allowed repetitions supported in Release 18.

Proposal 3: Incorporate the simulation assumptions outlined in Table 1 as the basis for conducting performance evaluations.

Proposal 4: The feature of OCC is considered only for cases with PUSCH repetitions configured.

Proposal 5: The length of the OCC being applied should match the number of repetitions used for PUSCH transmissions.

Furthermore, we think that it is very important that RAN1 conducts a thorough investigation and assessment of the maximum number of UEs that OCC can support and that can be multiplexed in the same time and frequency resources. It is crucial that the UEs supported by OCC do not experience significant BLER performance degradation compared to UEs without OCC. Additionally, companies should provide details regarding the OCC sequences adopted for their evaluations, including their size, elements, and specifications in the time or frequency domain, ensuring clarity in reporting.

Proposal 6: RAN1 to investigate and assess the maximum number of UEs that can be supported by OCC.

### Preliminary results[Mention was removed]

Our initial performance evaluation of OCC-enabled PUSCH follows the simulation methodology as follows. UEs transmit a number of PUSCH repetitions on the same time-frequency resources while also being subject to encoding by an OCC. The number of transmission repetitions depends on the number of OCC-enabled UEs, e.g. for 2 UEs multiplexed, 2 repetitions are simulated. All the received transmissions are placed in a buffer. Once all the transmissions have been received, the signals of the buffer are summed up and correlated with the OCC code of the UE that needs to be decoded. As a result, the OCC-decoded signal contains only the transmission data of one UE.

Applying the aforementioned methodology, we extend our assumption to the multiplexing of two UEs using OCC codes [1 1] and [1 -1] applied across two repetitions. The resulting BLER is then compared with the scenario in which OCC is not employed, as illustrated in Figure 1.

Figure 1 The BLER of UEs employing OCC across repetitions aligns well with that of UEs operating without OCC.

As expected, Figure 1 confirms that, under ideal channel conditions, two UEs utilizing inter-slot OCC across repetitions in the time domain do not undergo performance degradation when compared to a single UE without OCC. Consequently, inter-slot OCC demonstrates the potential for multiplexing gain, thereby enhancing UL capacity, particularly in scenarios characterized by repetitions.

Observation 1: The inter-slot time domain OCC across repetitions demonstrates the potential for boosting UL capacity.

## Discussion on OCC Spreading Schemes

In our understanding, three types of OCC spreading schemes can be envisioned: intra-symbol, inter-symbol and inter-slot OCC spreading.

The intra-symbol scheme refers to application of an OCC code within an OFDM symbol, either in time or frequency domain, with this latter resembling the OCC scheme used for DMRS multiplexing in MU-MIMO. Such a spreading scheme is based on a spreading of the data within one OFDM symbol either in time (in case of DFT-s-OFDM waveform, before DFT spreading) or in frequency (after DFT spreading), wherein the data to be transmitted are repeated in additional time or frequency location, respectively, of the same OFDM symbol. Although such a scheme might not suffer from channel uncorrelation in time or frequency for a limited number of UEs, it requires an expansion of the transmission bandwidth directly proportional to the number of UEs to multiplex, e.g. if two UEs are to be multiplexed both transmitting data occupying one PRB (12 REs), two PRBs are necessary for such multiplexing. However, considering that a gNB can schedule the two UEs separately and orthogonally in the two distinct PRBs, the motivation for introducing such a scheme needs to be further discussed.

The inter-symbol scheme refers to the application of an OCC code across OFDM symbols within a same UE PUSCH transmission in a slot. In this case, the spreading by the OCC is performed in the time domain, possibly by repetition of the coded bits within a same UE PUSCH transmissions. Differently than the intra-symbol scheme, the inter-symbol scheme does not require bandwidth expansion but at the same time might suffer of channel decorrelation in the time domain in cases of large residual frequency offset. Further discussion is also necessary for this scheme to understand how to realize it with possibly minimal specification impact. Correspondingly, it should be noted that applying OCC within a single slot will cause the supported payload size to be reduced since there would not be any “processing gain” to utilize through the inverse OCC processing in the receiver.

The inter-slot spreading scheme refers to the application of an OCC code across PUSCH repetitions. Although this is arguably the easiest scheme to implement from specification point of view since PUSCH repetitions are already well integrated in the 5G NR standard, the impact of channel impairments to such scheme needs to be carefully evaluated. In addition to the channel impairments, definition of such a spreading scheme would require definition of phase consistency requirements at the UE in a similar way as it was done for DMRS bundling in Rel-18 NTN, to ensure that the phase variations seen at gNB across the inter-slot repetitions (due to both RF impairments and timing drift) are contained within certain margins.

In Table 2 we have summarized some of the observed advantages and disadvantages of the three spreading schemes which has been described above.

Table 2 Comparing different OCC spreading schemes

| Spreading Scheme | FD intra-symbol OCC | TD intra-symbol OCC | Intra-slot OCC | Inter-slot OCC |
| --- | --- | --- | --- | --- |
| Advantages | Negligible impairment due to timing drift. | Negligible impairment due to timing drift. | Low impairment due to timing drift. | Enables multiplexing gain by spreading codes across repetitions in the time domain, potentially boosting capacity. Simple to implement since PUSCH repetitions are already standardized. |
| Challenges | Frequency selectivity, larger BW necessary decreasing PSD for power limited UEs. Transport block size will be scaled with OCC length. | Larger BW necessary decreasing PSD for power limited UEs. Transport block size will be scaled with OCC length. | Limited multiplexing gain compared to inter-slot OCC, potentially impacting capacity enhancement. Alignment of multiple UEs with different TBSs not straight forward. Transport block size will be scaled with OCC length. | Impairments due to timing drift need to be accounted for. |

Proposal 7: Examine and explore the benefits and challenges associated with various OCC spreading schemes.

When considering introducing a new feature such as OCC for PUSCH it is important that we take a number of aspects into account. These include: (a) the feature should introduce a gain, (b) the feature should be robust to impairments, (c) it would be desirable if the specification impacts are limited, and (d) many UEs would be supporting the feature. The latter aspect is especially crucial for a feature like OCC, since the gNB would only be able to apply the feature in case there are enough UEs in the system (that are having active traffic) for multiplexing. The last thing we would want is a feature which is not supported by devices in the network.

Proposal 8: If adopted, the feature of OCC should focus on mechanisms that are simple to implement and that ensure that multiple UEs would support the feature.

# Conclusion

In this contribution, we presented our view on the uplink performance enhancement of NR NTN via OCC. Our proposals and observations are summarized below.

Observation 1: The inter-slot time domain OCC across repetitions demonstrates the potential for boosting UL capacity.

Proposal 1: The feature of OCC need to be evaluated thoroughly in terms of robustness and efficiency prior to determining to specify it.

Proposal 2: Assess the performance impact of OCC-enabled PUSCH in comparison to PUSCH without OCC through link-level simulations. The evaluation involves the following steps:

- Step 1: Evaluate the single UE performance without OCC-enabled PUSCH and determine SNR for BLER at 10%

- Step 2: Evaluate BLER performance for OCC enabled PUSCH where there are other OCC-enabled PUSCH transmissions overlapping but with different OCC. The evaluation is performed within the determined SNR range for single UE PUSCH.

- Step 3. Determine SNR gap at 10% BLER between the single UE PUSCH and OCC enabled PUSCH to identify any potential performance degradation.

Proposal 3: Incorporate the simulation assumptions outlined in Table 1 as the basis for conducting performance evaluations.

Proposal 4: The feature of OCC is considered only for cases with PUSCH repetitions configured.

Proposal 5: The length of the OCC being applied should match the number of repetitions used for PUSCH transmissions.

Proposal 6: RAN1 to investigate and assess the maximum number of UEs that can be supported by OCC.

Proposal 7: Examine and explore the benefits and challenges associated with various OCC spreading schemes.

Proposal 8: If adopted, the feature of OCC should focus on mechanisms that are simple to implement and that ensure that multiple UEs would support the feature.

# References

- RP-234078, ”New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- R1-2205701, ”Final Report of 3GPP TSG RAN WG1 #109-e v1.0.0”