3GPP TSG-RAN WG1 Meeting #116	R1- 2400975

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on RAN4 LS on the system parameters for NTN above 10 GHz

Work Item:	NR_NTN_enh-Core

Document for:	Discussion, Decision

# 1	Introduction

At RAN1#113, a LS from RAN4 was received, discussing the potential support of NR system parameters over NTN for certain frequency bands in FR2-NTN [1]. At RAN1#114bis and RAN1#115, a number of agreements and conclusions were made while some topics were left for further discussion at RAN1#116. The discussion at RAN1#115 is summarized in a FL summary, see [2].

In this contribution, we discuss the remaining aspects and provide a preliminary analysis of the specification impact.

# 2	Discussion

## 2.1	PRACH configurations

At RAN1#115, the following agreement was made [3]:

Agreement Confirm the working assumption from RAN1#114-bis on the PRACH configuration. Working assumption For PRACH configuration for operation in FR2-NTN, Table 6.3.3.2-4 of TS 38.211 is used as baseline. FFS: Whether further modifications to the PRACH configuration Table would be needed

According to the agreement, the PRACH configuration table for operation in FR2-TDD, i.e., Table 6.3.3.2-4 of TS 38.211, should be used as baseline. Considering that the time scheduling of FDD is more flexible than TDD, the PRACH configuration for FR2-TDD can also be applied to FR2-FDD. If the table is reused without modification, it is up to the network to determine which PRACH configurations are suitable for FDD and choose a suitable one.

However, Table 6.3.3.2-4 is specifically designed for FR2-TDD operations. It is required to consider the impact of TDD DL-UL scheduling and SSBs periods, and hence not the optimal choice for FR2-FDD. Thus, Table 6.3.3.2-4 can be slightly modified to overcome those limitations.

Considering that the time scheduling of FDD is more flexible than TDD for FR2, the PRACH configurations for FR2-TDD in Table 6.3.3.2-4 can also be applied to FR2-FDD. If the table is reused without modification, it is up to the network to choose a suitable PRACH configuration.

On the other hand, Table 6.3.3.2-4 is designed for FR2-TDD operations, taking into account the impact of TDD DL-UL scheduling and SSBs periods. The entries designed for TDD impose unnecessary limitations for FR2-FDD operations.

In the following, we discuss a few optimizations to the table.

### 2.1.1	Using starting symbol zero

It should be considered to modify PRACH configurations in Table 6.3.3.2-4 that have a non-zero starting symbol. A non-zero starting symbol is useful to avoid TDD DL-UL interference, to allow PDCCH, and to have more freedom to schedule DL, which is only beneficial for TDD [4][2]. But for FDD, it unnecessarily limits the number of time-domain PRACH occasions within a PRACH slot. For example, as illustrated in Figure 1, PRACH configuration 12 of Table 6.3.3.2-4 has starting symbol 7 and 3 possible PRACH occasions in time domain  within a PRACH slot. This could be changed to starting symbol 0 and 6 possible PRACH occasions in time domain, as illustrated in Figure 2.

Figure 1: PRACH occasions within a PRACH slot for PRACH configuration 12 for FR2 and unpaired spectrum.

Figure 2: Proposed PRACH occasions within a PRACH slot for PRACH configuration 12 for FR2 and paired spectrum.

Corresponding changes should be made to other configurations that have non-zero starting symbol. However, for some configurations, this would make the configuration identical to another configuration. E.g., configuration 15 would be identical to configuration 17. For these configurations, it is proposed to replace them with new configurations, optimized for FR2-NTN FDD operation, as further discussed below.

Using a non-zero starting symbol for FDD unnecessarily limits the number of time-domain PRACH occasions within a PRACH slot.

### 2.1.2	Adding configurations with non-overlapping PRACH occasions

For most PRACH configurations for FR2 TDD, several slot numbers in a subframe are used for PRACH. E.g., Table 1 shows configuration 184 in which PRACH occasions are defined in 10 slots in a subframe. This is useful when many beams per cell are used, in which case the many PRACH occasions are divided between the beams in the cell. For NTN, however, there is no obvious reason to use many beams per cell. A one-to-one mapping between beam and cell is more likely.

Most PRACH configurations for FR2 TDD use several slot numbers in each PRACH subframe, which is useful when there are many beams per cell.

For NTN, a one beam per cell is more likely.

Table 1: Except from table 6.3.3.2-4 in TS 38.211.

| PRACH Config. Index | Preamble format | nf mod x=y |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 184 | C2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 |

For FR1 FDD, there are many PRACH configurations with a single subframe number. Further, there are many groups of PRACH configurations that are identical except for the single subframe number. One example is shown in Table 2, where the three configurations 243, 244 and 245 only differ in the subframe numbers. One advantage with this is that root sequence collisions can be avoided by applying a PRACH configuration reuse in the network, thereby avoiding simultaneous preamble transmissions in adjacent cells.

Table 2: Except from table 6.3.3.2-2 in TS 38.211.

| PRACH Configuration Index | Preamble format | nf mod x=y |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | x | y |  |  |  |  |  |
| 243 | C2 | 2 | 0 | 1 | 0 | 2 | 2 | 6 |
| 244 | C2 | 2 | 0 | 4 | 0 | 2 | 2 | 6 |
| 245 | C2 | 2 | 0 | 7 | 0 | 2 | 2 | 6 |

Similar configurations would be useful also for FR2-NTN FDD. It should therefore be considered to replace some PRACH configurations that are less useful in FR2-NTN FDD by new configurations with single, non-overlapping slot numbers, to enable non-colliding PRACH configuration reuse.

By defining new PRACH configurations with non-overlapping slot numbers, root sequence collisions can be avoided by applying a PRACH configuration reuse in the network.

As discussed in section 2.1.1, some PRACH configurations with non-zero starting symbols would become duplicates of other configurations if their starting symbol were changed to zero. These PRACH configurations could instead be replaced by new PRACH configurations supporting PRACH configuration reuse with non-overlapping slot numbers, as discussed above.

### 2.1.3	Proposed optimizations

The discussion above can be summarized in the following proposal:

For PRACH configurations for FDD operation in FR2-NTN:

- Introduce a new table Table 6.3.3.2-x in TS 38.211, which is a copy of Table 6.3.3.2-4, as a starting point for further optimizations

- For PRACH configurations with non-zero starting symbol for which changing their starting symbol to zero does not make them identical to another configurations, change the starting symbol to zero and increase (if possible) the number of time domain PRACH occasions.

- For PRACH configurations with a non-zero starting symbol for which changing the starting symbol to zero  makes them identical to another configuration:

  - Alt 1: Leave them unmodified.

  - Alt 2: Replace them with a new set of PRACH configurations with single, non-overlapping slot numbers, enabling PRACH configuration reuse.

There are in total 31 PRACH configurations with a non-zero starting symbol for which changing the starting symbol to zero would make it identical to another configuration. A proposed redefinition according to Alt 2 of Proposal 1 is shown in Table 3, where the new configurations are divided evenly between the 10 preamble formats, and where a periodicity of 2 frames has been used to provide high PRACH capacity.

Table 3: New PRACH configurations for FR2-NTN FDD.

| PRACH Configuration Index | Preamble format | nf mod x=y |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | number of time-domain PRACH occasions within a PRACH slot | PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | x | y |  |  |  |  |  |
| 15 | A1 | 2 | 1 | 4 | 0 | 2 | 6 | 2 |
| 19 | A1 | 2 | 1 | 14 | 0 | 2 | 6 | 2 |
| 21 | A1 | 2 | 1 | 24 | 0 | 2 | 6 | 2 |
| 25 | A2 | 2 | 1 | 4 | 0 | 2 | 3 | 4 |
| 44 | A2 | 2 | 1 | 14 | 0 | 2 | 3 | 4 |
| 48 | A2 | 2 | 1 | 24 | 0 | 2 | 3 | 4 |
| 50 | A3 | 2 | 1 | 4 | 0 | 2 | 2 | 6 |
| 52 | A3 | 2 | 1 | 14 | 0 | 2 | 2 | 6 |
| 55 | A3 | 2 | 1 | 24 | 0 | 2 | 2 | 6 |
| 74 | B1 | 2 | 1 | 4 | 0 | 2 | 6 | 2 |
| 78 | B1 | 2 | 1 | 14 | 0 | 2 | 6 | 2 |
| 81 | B1 | 2 | 1 | 24 | 0 | 2 | 6 | 2 |
| 83 | B4 | 2 | 1 | 4 | 0 | 2 | 1 | 12 |
| 85 | B4 | 2 | 1 | 14 | 0 | 2 | 1 | 12 |
| 98 | B4 | 2 | 1 | 24 | 0 | 2 | 1 | 12 |
| 103 | C0 | 2 | 1 | 4 | 0 | 2 | 7 | 2 |
| 106 | C0 | 2 | 1 | 14 | 0 | 2 | 7 | 2 |
| 107 | C0 | 2 | 1 | 24 | 0 | 2 | 7 | 2 |
| 127 | C2 | 2 | 1 | 4 | 0 | 2 | 2 | 6 |
| 139 | C2 | 2 | 1 | 14 | 0 | 2 | 2 | 6 |
| 159 | C2 | 2 | 1 | 24 | 0 | 2 | 2 | 6 |
| 162 | C2 | 2 | 1 | 34 | 0 | 2 | 2 | 6 |
| 166 | A1/B1 | 2 | 1 | 4 | 0 | 2 | 6 | 2 |
| 169 | A1/B1 | 2 | 1 | 14 | 0 | 2 | 6 | 2 |
| 190 | A1/B1 | 2 | 1 | 24 | 0 | 2 | 6 | 2 |
| 198 | A2/B2 | 2 | 1 | 4 | 0 | 2 | 3 | 4 |
| 215 | A2/B2 | 2 | 1 | 14 | 0 | 2 | 3 | 4 |
| 217 | A2/B2 | 2 | 1 | 24 | 0 | 2 | 3 | 4 |
| 230 | A3/B3 | 2 | 1 | 4 | 0 | 2 | 2 | 6 |
| 233 | A3/B3 | 2 | 1 | 14 | 0 | 2 | 2 | 6 |
| 235 | A3/B3 | 2 | 1 | 24 | 0 | 2 | 2 | 6 |

A complete table according to Proposal 1 is shown in Appendix A. The impact of the two alternatives of Proposal 1 is summarized in Table 4.

Table 4: Summary of impact of Proposal 1.

|  | Unmodified entries | Updated entries | Replaced entries |
| --- | --- | --- | --- |
| Alt 1 | 149 | 107 | - |
| Alt 2 | 118 | 107 | 31 |

## 2.2	UE autonomous timing advance aspects

It is well-known that the timing error budget for high SCS is challenging. Therefore, RAN1 should reconsider improvements that were already evaluated but not pursued in Rel-17, since these may be more relevant with a tighter timing error budget.

## 2.2.1	RAN4 timing accuracy requirements

In RAN4 #109, the following requirements on initial transmit timing accuracy were agreed [6]:

Agreement: - UL 60kHz SCS: 13 Ts for all cases with 120kHz/240kHz SSB - UL 120kHz SCS: - Case 1 and case 2: 7.5 Ts - FFS for the applicable side condition on case 2 - Case 3: Higher than 7.5 Ts, FFS for the exact value

Case 1, 2 and 3 are defined as shown in Table 5.

Table 5: RAN4 case definitions.

|  | GSO | LEO |
| --- | --- | --- |
| Stationary UE | Case 1 | Case 2 |
| Mobile UE (VSAT) | Case 3 | NA |

### 2.2.2	3rd order term of common TA

3rd order derivative of Common TA was evaluated in detail in Rel-17 e.g. in [5] and clear benefits were shown.

As described in section 2.2.1, the tightest timing error requirement for FR2-NTN, 7.5 Ts, applies for 120 kHz UL SCS and stationary UE. A breakdown of the different components of the timing error is performed in [7] and gives that most of this (6.5-6.7 Ts) corresponds to position estimation errors of the satellite and UE. No implementation margin in gNB is allowed, and in particular no margin for inaccuracy in Common TA.

The RAN4 timing accuracy requirements for FR2-NTN are very tight and leaves no margin for impairments in gNB, e.g., Common TA errors.

The black curve in Figure 3  (from [5]) shows the sum of UE-specific TA + Common TA estimation error with up to 2nd order term of common TA. It can be seen that the error exceeds the timing accuracy requirement of 7.5 Ts (244 µs) if SIB19 update period is approximately 17 seconds. Note that other sources of error (see e.g [7]) are disregarded in these simulations, so the actual update period would be even shorter.

The pink curve shows the sum of UE-specific TA + common TA estimation error with up to 3rd order term for Common TA. It can be seen that the timing accuracy is significantly increased and the timing error does not exceed the timing accuracy requirement of 7.5 Ts even at 30 seconds.

Figure 3: 99th percentile approximation error of Total TA (UE-specific  + Common TA).

The transmission timing accuracy can be significantly increased if a 3rd order term of common TA is introduced.

Introduce a 3rd order term of common TA for FR2-NTN.

### 2.2.3	Modified range for TAC in RAR

In Rel-17, it was discussed to modify the range of TAC in RAR to cover both negative and positive values. Since PRACH has longer CP than PUCCH/PUSCH, an initial timing error due to e.g. GNSS inaccuracy will have less impact if it can be corrected by a closed-loop TA command (TAC) before the UE transmits PUCCH or PUSCH. For this purpose, the TAC in Msg2 is useful. But the initial timing error can be both positive and negative, while the TAC in Msg2 only supports positive TA values. Therefore, it would be useful if the range of TAC in Msg2 is modified to include both positive and negative TA values. To avoid increasing the number of TAC bits, the range could simply be shifted to cover the maximum allowed negative timing offset.

The initial timing error can be both positive and negative, while the TAC in Msg2 only supports positive TA values.

Slightly shift the range of TAC in Msg2 (without increasing the number of bits) to cover also negative TA values corresponding to the maximum timing error Te_NTN.

## 2.4	Specification impact

The complete specification impact can be determined when all discussion points have been concluded. A preliminary and non-exhaustive analysis is given below:

### 2.4.1	PRACH configurations

A new table with PRACH configurations should be added to clause 6.3.3.2 of TS 38.211, e.g., the one shown in Table 6 in Appendix A.

### 2.4.2	3rd order term of common TA

The third order term, if agreed, should be added to clause 4.2 of TS 38.213, e.g. as shown in the TP below. Corresponding RRC signaling is also needed in TS 38.331.

## Draft TP to 38.213 ## 4.2 Transmission timing adjustments < Unchanged text omitted> Using higher-layer ephemeris parameters for a serving satellite, if provided, a UE pre-compensates the two-way transmission delay on the service link based on NTA,adjUE that the UE determines using the serving satellite position and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as: Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariant2×t-tepoch2+TACommonDriftThirdOrder2×t-tepoch3 where TACommon, TACommonDrift, and TACommonDriftVariant and TACommonDriftThirdOrder are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and ta-CommonDriftThirdOrder and tepoch is provided by epochTime which is the epoch time of ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and ta-CommonDriftThirdOrder [12, TS 38.331]. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset. < Unchanged text omitted>

### 2.4.3	Negative range of TAC in RAR

The use of negative range of TAC in RAR, if agreed, should be added to clause 4.2 of TS 38.213, e.g., as shown in the TP below. The amount of offset depends on timing accuracy requirements that will be agreed in RAN4. The value [16] in the TP should be seen as an example.

## Draft TP to 38.213 ## 4.2 Transmission timing adjustments < Unchanged text omitted > A timing advance command [11, TS 38.321] in case of random access response or in an absolute timing advance command MAC CE or in a cell switch command, TA, for a TAG indicates NTA values by index values of TA = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of 2μ∙15 kHz is NTA=TA∙16∙642μ, except for operation in FR2-NTN bands where NTA=(TA-[16])∙16∙642μ. NTA is defined in [4, TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response or absolute timing advance command MAC CE or the cell switch command. < Unchanged text omitted >

### 2.4.4	Reference subcarrier spacing

The reference subcarrier spacing for FR2-NTN should be specified in several clauses in TS 38.213 and TS 38.214, e.g., as shown in the TPs below.

## Draft TP to 38.213 # 9 UE procedure for reporting control information < Unchanged text omitted > For the remaining of this clause, if a UE is provided Kcell,offset by cellSpecificKoffset or KUE,offset by a MAC CE command, reference to a slot n+k for a PUCCH transmission or PUSCH transmission corresponds to a slot n+k+2μ-μKoffset∙Koffset for the PUSCH or the PUCCH transmission, and reference to a slot nU-K1,k corresponds to slot nU-K1,k-2μ-μKoffset∙Koffset, where μ is the SCS configuration for the PUCCH transmission or PUSCH transmission, Koffset is defined in clause 4.2, and μKoffset=0 in FR1 and FR2-NTN. If cellSpecificKoffset or if the MAC CE command is not provided, Kcell,offset=0 or KUE,offset=0, respectively. If the PUCCH or PUSCH transmission is scheduled by a DCI format, the value of KUE,offset is the one that is applicable at the slot overlapping with the last symbol of the PDCCH reception providing the DCI format. If the PUCCH transmission or the PUSCH transmission is scheduled by a DCI format with CRC scrambled by TC-RNTI, KUE,offset=0. If the UE is provided a KUE,offset value by a MAC CE command, the UE applies the MAC CE command in the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the MAC CE command, μ is the SCS configuration for the PUCCH transmission that is determined in the slot when the MAC CE command is applied. < Unchanged text omitted >

## Draft TP to 38.214 #### 5.1.4.2 PDSCH resource mapping with RE level granularity < Unchanged text omitted > For a UE configured with a list of semi-persistent ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList: - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. - when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. < Unchanged text omitted > ### 5.1.5 Antenna ports quasi co-location < Unchanged text omitted > When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. If tci-PresentInDCI is set to 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to qcl-Type set to 'typeA', and when applicable, also with respect to qcl-Type set to 'typeD'. < Unchanged text omitted > ##### 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology < Unchanged text omitted > A trigger state is initiated using the CSI request field in DCI. - When all the bits of CSI request field in DCI are set to zero, no CSI is requested. - When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to trigger states to the codepoints of the CSI request field in DCI. is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.. < Unchanged text omitted > ##### 5.2.1.5.2 Semi-persistent CSI/Semi-persistent CSI-RS < Unchanged text omitted > For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'. - when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. If a TCI-State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP. - when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ+2μ2μKmac∙kmac where  is the SCS configuration for the PUCCH and μKmac is the subcarrier spacing configuration for kmac with a value of 0 for frequency range 1 and FR2-NTN, and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided. < Unchanged text omitted > #### 5.2.2.5 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and FR2-NTN, < Unchanged text omitted > #### 6.1.2.1 Resource allocation in time domain < Unchanged text omitted > in CSI-ReportConfig for the triggered CSI Reporting Settings and is the (m+1)th entry of . - The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks=n⋅2μPUSCH2μPDCCH+K2+Koffset⋅2μPUSCH2μKoffset, otherwise, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and FR2-NTN, n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and the scheduling DCI is other than DCI format 0_0 with CRC scrambled by TC-RNTI. < Unchanged text omitted > ### 6.2.1 UE sounding procedure < Unchanged text omitted > - If the UE receives the DCI triggering aperiodic SRS in slot n and none of the resource sets is configured with parameter availableSlotOffsetList across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource - if the UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , - otherwise, the UE transmits aperiodic SRS in each of the triggered resource set(s) in slot Ks=n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where - k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively; - μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and FR2-NTN. - Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5. - If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter SRS-PosResource, the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks=n⋅2μSRS2μPDCCH+k+Koffset⋅2μSRS2μKoffset, otherwise, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6 TS 38.213], and where - k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively; - μKoffset is the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and FR2-NTN. - Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5. < Unchanged text omitted >

### 2.4.5	Cell search

FR2-NTN should be added to Case D and E of clause 4.1 in TS 38.213, e.g., as shown in the TP below.

## Draft TP to 38.213 ## 4.1 Cell search < Unchanged text omitted > - Case D - 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 4,8,16,20+28⋅n. For carrier frequencies within FR2 and FR2-NTN, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18. - Case E - 240 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 8,12,16,20,32,36,40,44+56⋅n. For carrier frequencies within FR2-1 and FR2-NTN, n=0, 1, 2, 3, 5, 6, 7, 8. < Unchanged text omitted >

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Considering that the time scheduling of FDD is more flexible than TDD for FR2, the PRACH configurations for FR2-TDD in Table 6.3.3.2-4 can also be applied to FR2-FDD. If the table is reused without modification, it is up to the network to choose a suitable PRACH configuration.

Observation 2	On the other hand, Table 6.3.3.2-4 is designed for FR2-TDD operations, taking into account the impact of TDD DL-UL scheduling and SSBs periods. The entries designed for TDD impose unnecessary limitations for FR2-FDD operations.

Observation 3	Using a non-zero starting symbol for FDD unnecessarily limits the number of time-domain PRACH occasions within a PRACH slot.

Observation 4	Most PRACH configurations for FR2 TDD use several slot numbers in each PRACH subframe, which is useful when there are many beams per cell.

Observation 5	For NTN, a one beam per cell is more likely.

Observation 6	By defining new PRACH configurations with non-overlapping slot numbers, root sequence collisions can be avoided by applying a PRACH configuration reuse in the network.

Observation 7	The RAN4 timing accuracy requirements for FR2-NTN are very tight and leaves no margin for impairments in gNB, e.g., Common TA errors.

Observation 8	The transmission timing accuracy can be significantly increased if a 3rd order term of common TA is introduced.

Observation 9	The initial timing error can be both positive and negative, while the TAC in Msg2 only supports positive TA values.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For PRACH configurations for FDD operation in FR2-NTN:

	Introduce a new table Table 6.3.3.2-x in TS 38.211, which is a copy of Table 6.3.3.2-4, as a starting point for further optimizations

	For PRACH configurations with non-zero starting symbol for which changing their starting symbol to zero does not make them identical to another configurations, change the starting symbol to zero and increase (if possible) the number of time domain PRACH occasions.

	For PRACH configurations with a non-zero starting symbol for which changing the starting symbol to zero  makes them identical to another configuration:

o	Alt 1: Leave them unmodified.

o	Alt 2: Replace them with a new set of PRACH configurations with single, non-overlapping slot numbers, enabling PRACH configuration reuse.

Proposal 2	Introduce a 3rd order term of common TA for FR2-NTN.

Proposal 3	Slightly shift the range of TAC in Msg2 (without increasing the number of bits) to cover also negative TA values corresponding to the maximum timing error Te_NTN.

# References

R4-2305926, “LS on the system parameters for NTN above 10 GHz”, RAN4, RAN4#106bis-e, April 2023

R1-2312142, “Summary #2 for FR2-NTN”, Moderator (Nokia), RAN1#115, November 2023

Draft Report of 3GPP TSG RAN WG1 #115 v0.3.0, MCC Support, RAN1#15, November 2023

R1-1805220 “Remaining details of RACH configurations”, Ericsson, RAN1#92bis, April 2018

R1-2111414, “On UL time and frequency synchronization enhancements for NTN”, Ericsson, RAN1#107-e, November 2021

R4-2321330, “Ad-hoc minutes for [109][226] NR_NTN_enh”, Samsung, RAN4#109, November 2023

R4-2318821, “RRM requirements in above 10 GHz bands”, Ericsson, RAN4#109, November 2023

# Appendix A	PRACH configuration table

The set of proposed PRACH configurations according to Proposal 1 is shown in Table 6.

In the comment column, “kept unmodified” corresponds to Alt 1 in Proposal 1 while “one beam per cell entry” corresponds to Alt 2.

Table 6: Random access configurations for FR2-NTN and paired spectrum.

| PRACH Config. Index | Preamble format | nf mod x=y |  | Slot number | Starting symbol | Number of PRACH slots within a 60 kHz slot | Number of time-domain PRACH occasions within a PRACH slot | PRACH duration | Ericsson Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | x | y |  |  |  |  |  |  |
| 0 | A1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 | Fully-reused |
| 1 | A1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 2 | A1 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 6 | 2 | Fully-reused |
| 3 | A1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 | Fully-reused |
| 4 | A1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 5 | A1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 6 | A1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 | Fully-reused |
| 7 | A1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 8 | A1 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 6 | 2 | Fully-reused |
| 9 | A1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 10 | A1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 6 | 2 | Fully-reused |
| 11 | A1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 12 | A1 | 1 | 0 | 19,39 | 7 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 13 | A1 | 1 | 0 | 3,5,7 | 0 | 1 | 6 | 2 | Fully-reused |
| 14 | A1 | 1 | 0 | 24,29,34,39 | 7 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 15 | A1 | 1 | 0 | 9,19,29,39 | 7 | 2 | 3 | 2 | Would become identical to configuration 17 kept unmodified or “one beam per cell entry” |
| 16 | A1 | 1 | 0 | 17,19,37,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 17 | A1 | 1 | 0 | 9,19,29,39 | 0 | 2 | 6 | 2 | Fully-reused |
| 18 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 19 | A1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 3 | 2 | Would become identical to configuration 18 kept unmodified or “one beam per cell entry” |
| 20 | A1 | 1 | 0 | 3,5,7,9,11,13 | 7 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 21 | A1 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 3 | 2 | Would become identical to configuration 23 kept unmodified or “one beam per cell entry” |
| 22 | A1 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 23 | A1 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 24 | A1 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 -> 0 | 2 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 25 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 3 | 2 | Would become identical to configuration 26 kept unmodified or “one beam per cell entry” |
| 26 | A1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 27 | A1 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 6 | 2 | Fully-reused |
| 28 | A1 | 1 | 0 | 0,1,2,…,39 | 7 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 29 | A2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 | Fully-reused |
| 30 | A2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 31 | A2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 | Fully-reused |
| 32 | A2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 33 | A2 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 3 | 4 | Fully-reused |
| 34 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 35 | A2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 | Fully-reused |
| 36 | A2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 37 | A2 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 3 | 4 | Fully-reused |
| 38 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 39 | A2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 3 | 4 | Fully-reused |
| 40 | A2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 41 | A2 | 1 | 0 | 19,39 | 5 -> 0 | 1 | 2 -> 3 | 4 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 42 | A2 | 1 | 0 | 3,5,7 | 0 | 1 | 3 | 4 | Fully-reused |
| 43 | A2 | 1 | 0 | 24,29,34,39 | 5 -> 0 | 1 | 2 -> 3 | 4 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 44 | A2 | 1 | 0 | 9,19,29,39 | 5 | 2 | 2 | 4 | Would become identical to configuration 33 kept unmodified or “one beam per cell entry” |
| 45 | A2 | 1 | 0 | 17,19,37,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 46 | A2 | 1 | 0 | 9, 19, 29, 39 | 0 | 2 | 3 | 4 | Fully-reused |
| 47 | A2 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 48 | A2 | 1 | 0 | 23,27,31,35,39 | 5 | 1 | 2 | 4 | Would become identical to configuration 49 kept unmodified or “one beam per cell entry” |
| 49 | A2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 50 | A2 | 1 | 0 | 3,5,7,9,11,13 | 5 | 1 | 2 | 4 | Would become identical to configuration 51 kept unmodified or “one beam per cell entry” |
| 51 | A2 | 1 | 0 | 3,5,7,9,11,13 | 0 | 1 | 3 | 4 | Fully-reused |
| 52 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 5 | 1 | 2 | 4 | Would become identical to configuration 53 kept unmodified or “one beam per cell entry” |
| 53 | A2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 54 | A2 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 5 -> 0 | 2 | 2 -> 3 | 4 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 55 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 5 | 1 | 2 | 4 | Would become identical to configuration 56 kept unmodified or “one beam per cell entry” |
| 56 | A2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 57 | A2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 3 | 4 | Fully-reused |
| 58 | A2 | 1 | 0 | 0,1,2,…,39 | 5 -> 0 | 1 | 2 -> 3 | 4 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 59 | A3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 60 | A3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 61 | A3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 62 | A3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 63 | A3 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 64 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 65 | A3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 66 | A3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 67 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 68 | A3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 69 | A3 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 70 | A3 | 1 | 0 | 19,39 | 7 -> 0 | 1 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 71 | A3 | 1 | 0 | 3,5,7 | 0 | 1 | 2 | 6 | Fully-reused |
| 72 | A3 | 1 | 0 | 9,11,13 | 2 -> 0 | 1 | 2 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 73 | A3 | 1 | 0 | 24,29,34,39 | 7 -> 0 | 1 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 74 | A3 | 1 | 0 | 9,19,29,39 | 7 | 2 | 1 | 6 | Would become identical to configuration 76 kept unmodified or “one beam per cell entry” |
| 75 | A3 | 1 | 0 | 17,19,37,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 76 | A3 | 1 | 0 | 9,19,29,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 77 | A3 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 78 | A3 | 1 | 0 | 23,27,31,35,39 | 7 | 1 | 1 | 6 | Would become identical to configuration 79 kept unmodified or “one beam per cell entry” |
| 79 | A3 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 80 | A3 | 1 | 0 | 3,5,7,9,11,13 | 0 | 1 | 2 | 6 | Fully-reused |
| 81 | A3 | 1 | 0 | 3,5,7,9,11,13 | 7 | 1 | 1 | 6 | Would become identical to configuration 80 kept unmodified or “one beam per cell entry” |
| 82 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 83 | A3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 | 1 | 1 | 6 | Would become identical to configuration 82 kept unmodified or “one beam per cell entry” |
| 84 | A3 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 -> 0 | 2 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 85 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 | 6 | Would become identical to configuration 86 kept unmodified or “one beam per cell entry” |
| 86 | A3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 87 | A3 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 88 | A3 | 1 | 0 | 0,1,2,…,39 | 7 -> 0 | 1 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 89 | B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 2 | 6 | 2 | starting symbol changed to zero |
| 90 | B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 2 | 6 | 2 | starting symbol changed to zero |
| 91 | B1 | 8 | 1,2 | 9,19,29,39 | 2 -> 0 | 2 | 6 | 2 | starting symbol changed to zero |
| 92 | B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 2 | 6 | 2 | starting symbol changed to zero |
| 93 | B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 2 | 6 | 2 | starting symbol changed to zero |
| 94 | B1 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |
| 95 | B1 | 1 | 0 | 19,39 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 96 | B1 | 1 | 0 | 3,5,7 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |
| 97 | B1 | 1 | 0 | 24,29,34,39 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 98 | B1 | 1 | 0 | 9,19,29,39 | 8 | 2 | 3 | 2 | Would become identical to configuration 100 kept unmodified or “one beam per cell entry” |
| 99 | B1 | 1 | 0 | 17,19,37,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |
| 100 | B1 | 1 | 0 | 9,19,29,39 | 2 -> 0 | 2 | 6 | 2 | starting symbol changed to zero |
| 101 | B1 | 1 | 0 | 7,15,23,31,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |
| 102 | B1 | 1 | 0 | 23,27,31,35,39 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 103 | B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 | 2 | Would become identical to configuration 102 kept unmodified or “one beam per cell entry” |
| 104 | B1 | 1 | 0 | 3,5,7,9,11,13 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 105 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 106 | B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 | Would become identical to configuration 105 kept unmodified or “one beam per cell entry” |
| 107 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 | 2 | Would become identical to configuration 109 kept unmodified or “one beam per cell entry” |
| 108 | B1 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 8 -> 0 | 2 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 109 | B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |
| 110 | B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |
| 111 | B1 | 1 | 0 | 0,1,2,…,39 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 112 | B4 | 16 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 | Fully-reused |
| 113 | B4 | 16 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 114 | B4 | 8 | 1,2 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 | Fully-reused |
| 115 | B4 | 8 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 116 | B4 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 1 | 12 | Fully-reused |
| 117 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 118 | B4 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 | Fully-reused |
| 119 | B4 | 4 | 1,2 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 120 | B4 | 2 | 1 | 7,15,23,31,39 | 2 -> 0 | 2 | 1 | 12 | starting symbol changed to zero |
| 121 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 122 | B4 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 1 | 12 | Fully-reused |
| 123 | B4 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 124 | B4 | 1 | 0 | 19, 39 | 2 -> 0 | 2 | 1 | 12 | starting symbol changed to zero |
| 125 | B4 | 1 | 0 | 17, 19, 37, 39 | 0 | 1 | 1 | 12 | Fully-reused |
| 126 | B4 | 1 | 0 | 24,29,34,39 | 2 -> 0 | 1 | 1 | 12 | starting symbol changed to zero |
| 127 | B4 | 1 | 0 | 9,19,29,39 | 2 | 2 | 1 | 12 | Would become identical to configuration 128 kept unmodified or “one beam per cell entry” |
| 128 | B4 | 1 | 0 | 9,19,29,39 | 0 | 2 | 1 | 12 | Fully-reused |
| 129 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 130 | B4 | 1 | 0 | 7,15,23,31,39 | 0 | 2 | 1 | 12 | Fully-reused |
| 131 | B4 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 132 | B4 | 1 | 0 | 23,27,31,35,39 | 2 -> 0 | 2 | 1 | 12 | starting symbol changed to zero |
| 133 | B4 | 1 | 0 | 9,11,13,15,17,19 | 0 | 1 | 1 | 12 | Fully-reused |
| 134 | B4 | 1 | 0 | 3,5,7,9,11,13 | 2 -> 0 | 1 | 1 | 12 | starting symbol changed to zero |
| 135 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 136 | B4 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 2 | 1 | 12 | starting symbol changed to zero |
| 137 | B4 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 2 -> 0 | 2 | 1 | 12 | starting symbol changed to zero |
| 138 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 139 | B4 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 | 1 | 1 | 12 | Would become identical to configuration 138 kept unmodified or “one beam per cell entry” |
| 140 | B4 | 1 | 0 | 3, 5, 7, …, 23,25 | 2 -> 0 | 1 | 1 | 12 | starting symbol changed to zero |
| 141 | B4 | 1 | 0 | 3, 5, 7, …, 23,25 | 0 | 2 | 1 | 12 | Fully-reused |
| 142 | B4 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 1 | 12 | Fully-reused |
| 143 | B4 | 1 | 0 | 0, 1, 2,…, 39 | 2 -> 0 | 1 | 1 | 12 | starting symbol changed to zero |
| 144 | C0 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 | Fully-reused |
| 145 | C0 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 146 | C0 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 147 | C0 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 148 | C0 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 7 | 2 | Fully-reused |
| 149 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 150 | C0 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 | Fully-reused |
| 151 | C0 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 152 | C0 | 2 | 1 | 7,15,23,31,39 | 0 | 2 | 7 | 2 | Fully-reused |
| 153 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 154 | C0 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 7 | 2 | Fully-reused |
| 155 | C0 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 156 | C0 | 1 | 0 | 19,39 | 8 -> 0 | 1 | 3 -> 7 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 157 | C0 | 1 | 0 | 3,5,7 | 0 | 1 | 7 | 2 | Fully-reused |
| 158 | C0 | 1 | 0 | 24,29,34,39 | 8 -> 0 | 1 | 3 -> 7 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 159 | C0 | 1 | 0 | 9,19,29,39 | 8 | 2 | 3 | 2 | Would become identical to configuration 161 kept unmodified or “one beam per cell entry” |
| 160 | C0 | 1 | 0 | 17,19,37,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 161 | C0 | 1 | 0 | 9,19,29,39 | 0 | 2 | 7 | 2 | Fully-reused |
| 162 | C0 | 1 | 0 | 23,27,31,35,39 | 8 | 1 | 3 | 2 | Would become identical to configuration 164 kept unmodified or “one beam per cell entry” |
| 163 | C0 | 1 | 0 | 7,15,23,31,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 164 | C0 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 165 | C0 | 1 | 0 | 3,5,7,9,11,13 | 8 -> 0 | 1 | 3 -> 7 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 166 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 | 1 | 3 | 2 | Would become identical to configuration 167 kept unmodified or “one beam per cell entry” |
| 167 | C0 | 1 | 0 | 4,9,14,19,24,29,34,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 168 | C0 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 8 -> 0 | 2 | 3 -> 7 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 169 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 8 | 1 | 3 | 2 | Would become identical to configuration 170 kept unmodified or “one beam per cell entry” |
| 170 | C0 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 171 | C0 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 7 | 2 | Fully-reused |
| 172 | C0 | 1 | 0 | 0,1,2,…,39 | 8 -> 0 | 1 | 3 -> 7 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 173 | C2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 174 | C2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 175 | C2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 176 | C2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 177 | C2 | 8 | 1,2 | 9,19,29,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 178 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 179 | C2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 180 | C2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 181 | C2 | 2 | 1 | 7,15,23,31,39 | 2 -> 0 | 2 | 2 | 6 | starting symbol changed to zero |
| 182 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 183 | C2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 0 | 2 | 2 | 6 | Fully-reused |
| 184 | C2 | 2 | 1 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 185 | C2 | 1 | 0 | 19,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 186 | C2 | 1 | 0 | 3,5,7 | 0 | 1 | 2 | 6 | Fully-reused |
| 187 | C2 | 1 | 0 | 24,29,34,39 | 7 -> 0 | 1 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 188 | C2 | 1 | 0 | 9,19,29,39 | 7 -> 0 | 2 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 189 | C2 | 1 | 0 | 17,19,37,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 190 | C2 | 1 | 0 | 9,19,29,39 | 2 | 2 | 2 | 6 | Would become identical to configuration 188 kept unmodified or “one beam per cell entry” |
| 191 | C2 | 1 | 0 | 7,15,23,31,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 192 | C2 | 1 | 0 | 3,5,7,9,11,13 | 7 -> 0 | 1 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 193 | C2 | 1 | 0 | 23,27,31,35,39 | 7 -> 0 | 2 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 194 | C2 | 1 | 0 | 23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 195 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 7 -> 0 | 2 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 196 | C2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 197 | C2 | 1 | 0 | 13,14,15, 29,30,31,37,38,39 | 7 -> 0 | 2 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 198 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 7 | 1 | 1 | 6 | Would become identical to configuration 199 kept unmodified or “one beam per cell entry” |
| 199 | C2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 200 | C2 | 1 | 0 | 1,3,5,7,…,37,39 | 0 | 1 | 2 | 6 | Fully-reused |
| 201 | C2 | 1 | 0 | 0,1,2,…,39 | 7 -> 0 | 1 | 1 -> 2 | 6 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 202 | A1/B1 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero | Note: Based on FR1 FDD in TS 38.211, the number of time domain PRACH occasions could be increased to “7. |
| 203 | A1/B1 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 204 | A1/B1 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 205 | A1/B1 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 206 | A1/B1 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 207 | A1/B1 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 208 | A1/B1 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 209 | A1/B1 | 1 | 0 | 19,39 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |  |
| 210 | A1/B1 | 1 | 0 | 9,19,29,39 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |  |
| 211 | A1/B1 | 1 | 0 | 17,19,37,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 212 | A1/B1 | 1 | 0 | 9,19,29,39 | 2 -> 0 | 2 | 6 | 2 | starting symbol changed to zero |  |
| 213 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |  |
| 214 | A1/B1 | 1 | 0 | 7,15,23,31,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 215 | A1/B1 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 6 | 2 | Would become identical to configuration 213 kept unmodified or “one beam per cell entry” |  |
| 216 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 8 -> 0 | 1 | 3 -> 6 | 2 | starting symbol changed to zero and increased number of time domain PRACH occasions |  |
| 217 | A1/B1 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 6 | 2 | Would become identical to configuration 216 kept unmodified or “one beam per cell entry” |  |
| 218 | A1/B1 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 219 | A1/B1 | 1 | 0 | 1,3,5,7,…,37,39 | 2 -> 0 | 1 | 6 | 2 | starting symbol changed to zero |  |
| 220 | A2/B2 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 221 | A2/B2 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 222 | A2/B2 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 223 | A2/B2 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 224 | A2/B2 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 225 | A2/B2 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 226 | A2/B2 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 227 | A2/B2 | 1 | 0 | 19,39 | 6 -> 0 | 1 | 2 -> 3 | 4 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 228 | A2/B2 | 1 | 0 | 9,19,29,39 | 6 -> 0 | 1 | 2 -> 3 | 4 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 229 | A2/B2 | 1 | 0 | 17,19,37,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 230 | A2/B2 | 1 | 0 | 9,19,29,39 | 2 | 2 | 3 | 4 | Would become identical to configuration 228 kept unmodified or “one beam per cell entry” |
| 231 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 6 -> 0 | 1 | 2 -> 3 | 4 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 232 | A2/B2 | 1 | 0 | 7,15,23,31,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 233 | A2/B2 | 1 | 0 | 23,27,31,35,39 | 2 | 1 | 3 | 4 | Would become identical to configuration 231 kept unmodified or “one beam per cell entry” |
| 234 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 6 -> 0 | 1 | 2 -> 3 | 4 | starting symbol changed to zero and increased number of time domain PRACH occasions |
| 235 | A2/B2 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 | 1 | 3 | 4 | Would become identical to configuration 234 kept unmodified or “one beam per cell entry” |
| 236 | A2/B2 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 237 | A2/B2 | 1 | 0 | 1,3,5,7,…,37,39 | 2 -> 0 | 1 | 3 | 4 | starting symbol changed to zero |
| 238 | A3/B3 | 16 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 239 | A3/B3 | 16 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 240 | A3/B3 | 8 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 241 | A3/B3 | 8 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 242 | A3/B3 | 4 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 243 | A3/B3 | 4 | 1 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 244 | A3/B3 | 2 | 1 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 245 | A3/B3 | 1 | 0 | 19,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 246 | A3/B3 | 1 | 0 | 9,19,29,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 247 | A3/B3 | 1 | 0 | 17,19,37,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 248 | A3/B3 | 1 | 0 | 9,19,29,39 | 2 -> 0 | 2 | 2 | 6 | starting symbol changed to zero |
| 249 | A3/B3 | 1 | 0 | 7,15,23,31,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 250 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 251 | A3/B3 | 1 | 0 | 23,27,31,35,39 | 2 -> 0 | 2 | 2 | 6 | starting symbol changed to zero |
| 252 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 253 | A3/B3 | 1 | 0 | 4,9,14,19,24,29,34,39 | 2 -> 0 | 2 | 2 | 6 | starting symbol changed to zero |
| 254 | A3/B3 | 1 | 0 | 3,7,11,15,19,23,27,31,35,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 255 | A3/B3 | 1 | 0 | 1,3,5,7,…,37,39 | 2 -> 0 | 1 | 2 | 6 | starting symbol changed to zero |
| 15 | A1 | 2 | 1 | 4 | 0 | 2 | 6 | 2 | new (“one beam per cell entry”) |
| 19 | A1 | 2 | 1 | 14 | 0 | 2 | 6 | 2 | new (“one beam per cell entry”) |
| 21 | A1 | 2 | 1 | 24 | 0 | 2 | 6 | 2 | new (“one beam per cell entry”) |
| 25 | A2 | 2 | 1 | 4 | 0 | 2 | 3 | 4 | new (“one beam per cell entry”) |
| 44 | A2 | 2 | 1 | 14 | 0 | 2 | 3 | 4 | new (“one beam per cell entry”) |
| 48 | A2 | 2 | 1 | 24 | 0 | 2 | 3 | 4 | new (“one beam per cell entry”) |
| 50 | A3 | 2 | 1 | 4 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |
| 52 | A3 | 2 | 1 | 14 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |
| 55 | A3 | 2 | 1 | 24 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |
| 74 | B1 | 2 | 1 | 4 | 0 | 2 | 6 | 2 | new (“one beam per cell entry”) |
| 78 | B1 | 2 | 1 | 14 | 0 | 2 | 6 | 2 | new (“one beam per cell entry”) |
| 81 | B1 | 2 | 1 | 24 | 0 | 2 | 6 | 2 | new (“one beam per cell entry”) |
| 83 | B4 | 2 | 1 | 4 | 0 | 2 | 1 | 12 | new (“one beam per cell entry”) |
| 85 | B4 | 2 | 1 | 14 | 0 | 2 | 1 | 12 | new (“one beam per cell entry”) |
| 98 | B4 | 2 | 1 | 24 | 0 | 2 | 1 | 12 | new (“one beam per cell entry”) |
| 103 | C0 | 2 | 1 | 4 | 0 | 2 | 7 | 2 | new (“one beam per cell entry”) |
| 106 | C0 | 2 | 1 | 14 | 0 | 2 | 7 | 2 | new (“one beam per cell entry”) |
| 107 | C0 | 2 | 1 | 24 | 0 | 2 | 7 | 2 | new (“one beam per cell entry”) |
| 127 | C2 | 2 | 1 | 4 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |
| 139 | C2 | 2 | 1 | 14 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |
| 159 | C2 | 2 | 1 | 24 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |
| 162 | C2 | 2 | 1 | 34 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |
| 166 | A1/B1 | 2 | 1 | 4 | 0 | 2 | 6 | 2 | new (“one beam per cell entry”) |
| 169 | A1/B1 | 2 | 1 | 14 | 0 | 2 | 6 | 2 | new (“one beam per cell entry”) |
| 190 | A1/B1 | 2 | 1 | 24 | 0 | 2 | 6 | 2 | new (“one beam per cell entry”) |
| 198 | A2/B2 | 2 | 1 | 4 | 0 | 2 | 3 | 4 | new (“one beam per cell entry”) |
| 215 | A2/B2 | 2 | 1 | 14 | 0 | 2 | 3 | 4 | new (“one beam per cell entry”) |
| 217 | A2/B2 | 2 | 1 | 24 | 0 | 2 | 3 | 4 | new (“one beam per cell entry”) |
| 230 | A3/B3 | 2 | 1 | 4 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |
| 233 | A3/B3 | 2 | 1 | 14 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |
| 235 | A3/B3 | 2 | 1 | 24 | 0 | 2 | 2 | 6 | new (“one beam per cell entry”) |