3GPP TSG-RAN WG1 Meeting #116	R1-2400976

Athens, Greece, February 26th – March 1st 2024

Agenda Item:	8.10

Source:	Ericsson

Title:	On maintenance of NR NTN enhancements

Document for:	Discussion

# 1	Introduction

A work item (WI) for enhancements of NR NTN was approved at RAN#94-e. The latest WID can be found in [1].

In this contribution, we discuss remaining issues of Rel-18 NR NTN maintenance.

# 2	Discussion

## 2.1	Coverage enhancement

### 2.1.1	TP to 38.214 for phase rotation pre-compensation

The following working assumption was made in RAN1#112bis-e:

Working assumption For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side. - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. - FFS: how to determine the actual TDW - FFS: specification impact - Send an LS to RAN4

The following agreement was made in RAN1#114:

Agreement For NTN-specific PUSCH DMRS bundling, - As UE capability report, - UE reports the max TDW size it can support by fulfilling the phase difference limit requirement. - Note: phase difference limit requirement is assumed to be at gNB receiver from RAN1 perspective. - Details, e.g., whether FG 30-4 is used without new FG or new FG is introduced, is discussed in UE feature session. - No consensus on whether to support Option 1d/1e/1f/1g.

At RAN1#115, this was captured in a TP to 38.300 that was endorsed and sent to RAN2 in an LS [2]. The TP describes the NTN-specific DMRS bundling enhancement as follows:

- NTN-specific PUSCH DMRS bundling enhancement that enables DMRS bundling in presence of timing drift, whereby UE maintains phase continuity considering effects of transmission delay variation between UE and uplink time synchronization reference point to enable improved channel estimation.

Two aspects of the NTN-specific DMRS bundling enhancement are currently not mentioned in the stage 3 specifications:

- That UE maintains phase continuity considering effects of transmission delay variations.

- That the reference point for phase continuity is the uplink time synchronization reference point.

Two aspects of the NTN-specific DMRS bundling enhancement are currently not mentioned in the stage 3 specifications:
1) That UE maintains phase continuity considering effects of transmission delay variations.
2) That the reference point for phase continuity is the uplink time synchronization reference point.

For comparison, details of transmission timing adjustments to compensate for transmission delay can be found in TS 38.213:

TS 38.213, clause 4.2: […] Using higher-layer ephemeris parameters for a serving satellite, if provided, a UE pre-compensates the two-way transmission delay on the service link based on NTA,adjUE that the UE determines using the serving satellite position and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as: Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariant2×t-tepoch2 where TACommon, TACommonDrift, and TACommonDriftVariant are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and tepoch is provided by epochTime which is the epoch time of ta-Common, ta-CommonDrift, and ta-CommonDriftVariant [12, TS 38.331]. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset. […]

Similar level of detail should be provided also for the compensation to maintain phase continuity for the NTN-specific DMRS bundling enhancement. Therefore, we propose to adopt the following TP to clause 6.1.7 of TS 38.214.

Adopt the TP to TS 38.214 in Table 1.

Table 1: TP for TS 38.214.

Reason for change: It is not specified that UE supporting NTN DMRS bundling enhancements for PUSCH shall maintain phase continuity at the uplink time synchronization reference point taking into account effects of transmission delay variation.

##### Summary of change: It is specified that UE supporting NTN DMRS bundling enhancements for PUSCH shall maintain phase continuity taking into account effects of transmission delay variation between the UE and the uplink time synchronization reference point.

##### Consequences if not approved: The meaning of maintaining phase continuity remains unclear for UE supporting NTN DMRS bundling enhancements.

### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged parts are omitted > The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or in response to the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to an event triggered by MAC-CE, subject to UE capability. of dmrs-BundlingRestart [13, TS 38.306] and when pusch-WindowRestart or pucch-WindowRestart is enabled. For a UE that supports NTN DMRS bundling enhancements for PUSCH [12, TS 38.331], phase continuity shall be maintained by compensating for effects of transmission delay variation (if any) between the UE and the uplink time synchronization reference point [6, TS 38.213]. < Unchanged parts are omitted >

## 2.2	Network-verified UE location

In RAN1#115, the issue of processing samples needed for PRS measurement was discussed. Specifically, it was proposed to make the existing UE capability “supportedDL-PRS-ProcessingSamples” a pre-requisite for network verification of UE location. We acknowledge that the UEs which do support this capability do not require a dense PRS configuration. However, the UEs that do not report this capability can still perform UE RX-TX measurement depending on the PRS resource configuration by the network.

The network may configure a dense PRS configuration to cater to the UEs that do not support “supportedDL-PRS-ProcessingSamples” UE capability.

The UE capability “supportedDL-PRS-ProcessingSamples” is not a pre-requisite for network verification of UE location.

# 3 	Conclusion

In the previous sections we made the following observations:

Observation 1	Two aspects of the NTN-specific DMRS bundling enhancement are currently not mentioned in the stage 3 specifications: 1) That UE maintains phase continuity considering effects of transmission delay variations. 2) That the reference point for phase continuity is the uplink time synchronization reference point.

Observation 2	The network may configure a dense PRS configuration to cater to the UEs that do not support “supportedDL-PRS-ProcessingSamples” UE capability.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the TP to TS 38.214 in Table 1.

Proposal 2	The UE capability “supportedDL-PRS-ProcessingSamples” is not a pre-requisite for network verification of UE location.

# References

RP-232669, "Revised WID: NR NTN (Non-Terrestrial Networks) enhancements", Thales, RAN#101, September 2023

R1-2312681, “LS on NR-NTN TP for TS 38.300”, RAN1, RAN1#115, November 2023