3GPP TSG RAN WG1 #116			R1-2400985

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	5

Source:	Apple

Title:	Discussion on RAN4 LS on SL-U RSSI Measurement

Document for:	Discussion/Decision

# Introduction

RAN4 sent an LS on SL-U RSSI measurement [1]. In the LS, RAN4 agreement on SL-U RSSI measurement was mentioned.

Agreement: - For congestion control purpose, SL-RSSI measurements use the OFDM symbols starting from the next symbol of the 2nd candidate starting symbol.

In this contribution, we discuss SL-U RSSI measurement.

# Discussion

In the current specification [2], Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol. This is because the first OFDM symbol is for AGC purpose.

In SL-U, to achieve a higher channel access chance, a maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission is supported. If a UE has successful LBT before the first candidate starting symbol, then it transmits from the first candidate starting symbol. If a UE has successful LBT before the second candidate starting symbol, then it transmits from the second candidate starting symbol.

Consider the case where two candidate starting symbols are configured in SL-U. For a slot with two candidate starting symbols, a UE performing SL RSSI measurements does not know beforehand whether a possible PSCCH/PSSCH transmission on a sub-channel starts from the first candidate starting symbol or starts from the second candidate starting symbol. Then, it is safe that SL RSSI measurements start from the next symbol of the second candidate starting symbol. That is the motivation of RAN4 agreement.

However, there is an exception on RAN1 physical structure design. By the following RAN1 agreement, we know that on a slot with PSFCH symbols, only one candidate starting symbol for PSCCH/PSSCH exists, even though two candidate starting symbols are configured in SL-U.

RAN1 #111 Agreement Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH.

By PSFCH configuration, a UE knows which slots have PSFCH symbols and which slots do not have PSFCH symbols. Hence, a UE knows beforehand which slots have only a single candidate starting symbol for PSCCH/PSSCH. For those slots, the SL RSSI measurements should start from the next symbol of the first candidate starting symbol. This is aligned with the existing definition of SL RSSI. Also, it achieves a better SL RSSI measurements performance since more OFDM symbols are used in the measurements.

On the other hand, for the slots without PSFCH symbols, SL RSSI measurements use the OFDM symbols starting from the next symbol of the second candidate starting symbol, as agreed in RAN4.

Hence, we have the following proposal.

Proposal 1: For congestion control measurements when the second candidate starting symbol is configured in SL-U,

- SL RSSI measurements use the OFDM symbols starting from the next symbol of the second  candidate starting symbol on slots without PSFCH symbols;

- SL RSSI measurements use the OFDM symbols starting from the next symbol of the first  candidate starting symbol on slots with PSFCH symbols.

Based on the above proposal, we have the following modifications of the specification:

Proposal 2: RAN1 to adopt the following text proposal:

- Reason for change: SL RSSI definition does not fit for SL-U when the second candidate starting symbol is configured.

- Summary of change: Specify SL RSSI measurement starts from the second OFDM symbol after the second candidate starting symbol if configured, on slots without PSFCH symbols.

- Consequences if not approved: RAN4’s agreement is not captured and SL RSSI measurement is not accurate.

### TS 38.215[2] ### 5.1.25 Sidelink received signal strength indicator (SL RSSI) Definition; Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol, or starting from the 2nd OFDM symbol after the second candidate starting symbol, if configured, on slots without PFSCH symbols. For frequency range 1, the reference point for the SL RSSI shall be the antenna connector of the UE. For frequency range 2, SL RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL RSSI value shall not be lower than the corresponding SL RSSI of any of the individual receiver branches.. Applicable for; Sidelink.

The corresponding draft reply LS on SL-U RSSI measurement is in [2].

# Conclusion

In this contribution, we provided our views on SL-U RSSI measurement. Our proposals are as follows:

Proposal 1: For congestion control measurements when the second candidate starting symbol is configured in SL-U,

- SL RSSI measurements use the OFDM symbols starting from the next symbol of the second  candidate starting symbol on slots without PSFCH symbols;

- SL RSSI measurements use the OFDM symbols starting from the next symbol of the first  candidate starting symbol on slots with PSFCH symbols.

Proposal 2: RAN1 to adopt the following text proposal:

- Reason for change: SL RSSI definition does not fit for SL-U when the second candidate starting symbol is configured.

- Summary of change: Specify SL RSSI measurement starts from the second OFDM symbol after the second candidate starting symbol if configured, on slots without PSFCH symbols.

- Consequences if not approved: RAN4’s agreement is not captured and SL RSSI measurement is not accurate.

### TS 38.215[2] ### 5.1.25 Sidelink received signal strength indicator (SL RSSI) Definition; Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol, or starting from the 2nd OFDM symbol after the second candidate starting symbol, if configured, on slots without PFSCH symbols. For frequency range 1, the reference point for the SL RSSI shall be the antenna connector of the UE. For frequency range 2, SL RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL RSSI value shall not be lower than the corresponding SL RSSI of any of the individual receiver branches.. Applicable for; Sidelink.

# References

- R1-2400019, “LS on SL-U RSSI measurement,” Feb. 2024.

- 3GPP TS 38.215, NR; Physical layer measurements, v18.1.0, Dec. 2023.

- R1-2400986, “Draft reply LS on SL-U RSSI measurement,” Feb. 2024.