3GPP TSG RAN WG1 #116			R1-2400986

Athens, Greece, February 26th – March 1st, 2024

Title:	Draft Reply LS to RAN4 on SL-U RSSI Measurement

Response to: 	R4-2321535

Release:	Release 18

Work Item:	NR_SL_enh2-Core

Source:	RAN1, Apple

To:	RAN4

Cc:

Contact Person:

Name:	   Chunxuan Ye

Email Address:  chunxuan_ye@apple.com

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# Overall description

RAN1 thanks RAN4 for the LS on SL-U RSSI measurement.

RAN1 would like to provide response to the following RAN4 agreement:

Agreement: - For congestion control purpose, SL-RSSI measurements use the OFDM symbols starting from the next symbol of the 2nd candidate starting symbol.

For congestion control measurements when the second candidate starting symbol is configured in SL-U, SL RSSI measurements use the OFDM symbols starting from the next symbol of the second candidate starting symbol on slots without PSFCH symbols; SL RSSI measurements use the OFDM symbols starting from the next symbol of the first candidate starting symbol on slots with PSFCH symbols.

# Actions

To RAN WG4

ACTION:  RAN1 kindly asks RAN4 to take above information into account for their future works.

# Dates of Next TSG-RAN WG1

TSG RAN WG1 Meeting #116bis			   15 – 19 April, 2024  		    Changsha, China

TSG RAN WG1 Meeting #117           20 – 24 May, 2022     		  Fukuoka City, Japan