3GPP TSG RAN WG1 #116		R1-2400991

Athens, Greece, February 26th – March 1st, 2024

Source: 	Apple Inc.

Title:                     Maintenance of further NR mobility enhancement

Agenda item:	8.5

Document for:	Discussion and Decision

# 1 Introduction

In this contribution, we provide our views on the remaining issues to complete the designs and enable the L1/L2-trigger mobility (LTM) in accordance with the latest progress made in RAN1 115 meeting [1].

# 2.  Discussions

## 2.1 Measurment reporting

### 2.1.1 Priority rules for LTM CSI reporting

In RAN1 115 meeting, the following rule was agreed for CSI reporting prioritization when the time occasions of LTM report and legacy CSI report overlapps at least one symbol and transmitted on a same carrier:

Agreement For the priority rule for CSI report, LTM CSI report is prioritized over legacy CSI report.

The remaining issue is how to define the prioritization rule when two LTM CSI reports collide. In the legacy, the priority rules for CSI reports are defined considiering the following factors:

- CSI report type (i.e., aperiodic or periodic or SP report).

- report quantity (i.e., L1-RSRP vs. others)

- serving cell index

- reporting ID

For LTM CSI report, it was agreed to support all of three report types. In addition, the colliding LTM CSI reports may associated different candidate cell ID and different reporting ID. In other words, except the report quantity, all the other factors still need to be considered for LTM CSI report priority rule. In our view, a simple solution is to reuse the legacy priority rule for LTM CSI report collision handling. Note that, although Rel-18 LTM CSI report only supports L1-RSRP, the legacy priority value equation still works as all collision LTM CSI reports set ‘k=0’. We therefore propose:

Proposal 1: The legacy priority rules of serving cell CSI reports are reused for LTM CSI reports.

### 2.1.2 SpCell Measurement

Another FFS aspect is how to identify the SpCell from the list of candidate cells configured with ‘LTM-CSI-SSB-ResourceSet’ to perform the measurement.

The following was agreed in RAN1 114 meeting to support SpCell measurement as part of LTM CSI report:

Agreement

- For the LTM L1 measurement report,

  - When a UE is configured is configured with SpCellInclusion, the SpCell measurements are the entries in the LTM-CSI-SSB-ResourceSet where the PCI and frequency information [SSB frequency/ARFCN] of the candidate cell is equal to the PCI and frequency information [SSB frequency/ARFCN] of the current SpCell.

It remains open regarding how to determine the corresponding entry in the LTM-CSI-SSB-Resource set in addition to the PCI information. This issue was debated in RAN1 115 meeting [3] without concensus being reached.

For each LTM candidate cell, the following was provided for the SSB configuration [4]:

LTM-SSB-Config-r18 ::= SEQUENCE { ssbFrequency-r18 ARFCN-ValueNR, subcarrierSpacing-r18 SubcarrierSpacing, ssb-Periodicity-r18 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL, -- Need R ssb-PositionsInBurst-r18 CHOICE { shortBitmap BIT STRING (SIZE (4)), mediumBitmap BIT STRING (SIZE (8)), longBitmap BIT STRING (SIZE (64)) } OPTIONAL, -- Need R ss-PBCH-BlockPower-r18 INTEGER (-60..50) OPTIONAL, -- Need R ... }

In addition, the SSB information is also explicitly indicated in a same way for each SpCell configuration:

FrequencyInfoDL ::= SEQUENCE { absoluteFrequencySSB ARFCN-ValueNR OPTIONAL, -- Cond SpCellAdd frequencyBandList MultiFrequencyBandListNR, absoluteFrequencyPointA ARFCN-ValueNR, scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier, ..., [[ referenceCell-r18 ServCellIndex OPTIONAL -- Cond SSBlessSCell ]] }

It seems nature to identify the SpCell based on the combination of SSB frequency (i.e., the corresponding ARFCN-ValueNR setting) and PCI from the list of candidate cells. It should be noted that the ‘ARFCN value’ is used to indicate the SSB frequency location in legacy and Rel-18 LTM operation, as shown in ASN.1 code above.

Based on the ASN.1 codes quoted above, the overall procedure is to identify SpCell by comparing the value of ‘ssbFrequency-r18’ in LTM-SSB-Config-r18 for a candidate cell and the ‘absoluteFrequencySSB’ value in SpCell configuration. In RAN1 115 meeting[3], different views are raised by companies on the need of capturing these exact RRC parameters in RAN1 specification. In general, we are supportive the practice to make the specification clear as much as possible to avoid IoDT issue later. Given the fact that RRC parameters names are widely quoted in many places in RAN1 specification and there is no explicit description in any spec regarding how to map the SpCell to a candidate cell when SpCellInclusion is configured, it is clearly beneficial to explicitly write the mapping into RAN1 specification. We therefore propose the following:

Proposal 2:  When SpCellInclusion is configured for LTM measurement, the ‘ssbFrequency-r18’ IE in ‘LTM-SSB-Config-r18’ is used to map a candidate cell to the SpCell in addition to PCI.

Proposal 3: Capturing the SpCell identification rule into TS 38.214.

## 2.2 Beam Indication

### 2.2.1 TCI-State application for LTM

In RAN1 115 meeting, the following was asgreed for beam indication:

Agreement After RACH procedure until a new TCI state is indicated by the target cell, a UE follows the indicated TCI-state in the cell switch command at least for CFRA triggered by cell switch command.

It remains open regarding how to determine the TCI-state before a new TCI-state is indicated by the target cell in case of CBRA procedure. Figure 2 depicted this open issue to faciliate the discussion.

Figure 2: CBRA-based LTM after cell-switch command MAC-CE

CBRA procedure is supported by L3 handover already, where the strongest SSB is selected by UE for RO determination and beam direction for UL/DL communication. After a CBRA procedure, the selected SSB is still followed until a new TCI state is indicated by the target cell. Given the gap between CBRA completion and the new TCI-state indication is a transition period and duration is under the control of the NW, it is sufficient for LTM to reuse the exsiting TCI-state determination rule defined in L3 handover.

Proposal 4: After RACH procedure triggered by the cell switch command until a new TCI state is indicated by the target cell, a UE follows the SSB identified during the CBRA procedure.

### 2.2.2 Beam application time

Another open issue related to DL sync procedure is beam application time for Rel-18 LTM. In reply LS [5], RAN2 provides the following feedback for the beam application time:

RAN2 understands that there is no impact on RAN2 specifications with regard to beam application time, and RAN2 understands further that a requirement, if any, would be specified by RAN4. RAN2 kindly ask RAN1 and RAN4 to decide the value of the beam application time.

In addition, LS reponse was received from RAN4 with the following reponse [6]:

RAN4 do not define beam application time. If the definition of beam application time is needed from RAN1’s perspective, RAN4 understands that it would be only specified by RAN1. Current cell switch delay defined in RAN4 can cover the TCI state application time, and there is no need to include additional beam application time component in the cell switch delay requirements.

As highlighted above, RAN4 concluded that the beam application time should be only specified in RAN1.

Observation 1: RAN4 has no intention to specify the beam application time and understand it is RAN1 task.

In Rel-17 unified TCI framework, two beam application times (BATs) are defined, one for TCI-state indicated by MAC-CE and the other is for TCI-state indicated by DCI format. For the first one, BAT is defined as 3ms after UE sends HARQ-ACK for the MAC-CE. On the other hand, BAT for the 2nd case is defined as ‘Y’ symbols after UE sends ACK for the DCI where Y={1,2,4,7,14,28,42,56,70,84,98,112,224,336}. The reason that different values were defined is mainly to consider that MAC-CE is transmitted over PDSCH and involves cross-layer interaction to parse the MAC-CE and deliver the decoded TCI-state back to Physical layers. For Rel-18 LTM, the beam information is indicated through MAC-CE, it is feasible to reuse the BAT defined for Rel-17 MAC-CE based signaling i.e., 3ms after UE sends a HARQ-ACK information for the CSC command.

In previous releases, the beam application time is always defined in RAN1 first and the corresonding requirement is defined later for different use cases in RAN4. We do not see any reason to handle it differently for LTM operation. Instead, the same handling should be followed to decouple with RAN4 work.

Proposal 5: The beam application time for beam information indicated in CSC command is defined as 3Nslotsubframe,µ after UE sends the HARQ-ACK information for the CSC MAC-CE.

# 3. Conclusion

In this contribution, we have presented our views on the remaining issues for further NR mobility enhancement.

We observed the following for beam application time used for LTM:

Observation 1: RAN4 has no intention to specify the beam application time and understand it is RAN1 task.

The following was proposed based on discussions:

Proposal 1: The legacy priority rules of serving cell CSI reports are reused for LTM CSI reports.

Proposal 2:  When SpCellInclusion is configured for LTM measurement, the ‘ssbFrequency-r18’ IE in ‘LTM-SSB-Config-r18’ is used to map a candidate cell to the SpCell in addition to PCI.

Proposal 3: Capturing the SpCell identification rule into TS 38.214.

Proposal 4: After RACH procedure triggered by the cell switch command until a new TCI state is indicated by the target cell, a UE follows the SSB identified during the CBRA procedure.

Proposal 5: The beam application time for beam information indicated in CSC command is defined as 3Nslotsubframe,µ after UE sends the HARQ-ACK information for the CSC MAC-CE.

# References

- RAN1 115 Meeting Chairman notes

- RAN1 114 Meeting Chairman notes

- R1-2312678	FL summary 4 on L1 enhancements for inter-cell beam management	Moderator (Fujitsu)

- 3GPP Technical specificaiton 38.331 v18.1.0

- R1-2308829	Reply LS on beam application time for LTM      RAN2

- R1-2310795	Reply LS on beam application time for LTM      RAN4