3GPP TSG RAN WG1 #116                                                               R1- 2401005

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.1.3.2

Source:	Apple Inc.

Title:	Discussion on CSI compression

Document for:	Discussion/Decision

# Introduction

CSI compression is identified as a sub-use case for further study in R19 [1].

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24):

- CSI feedback enhancement [RAN1]:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

  - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

In this paper, we discuss the evaluation and other aspects of CSI compression sub-use case.

# Discussion

## Summary of R18 CSI compression use case

In R18 study, frequency-spatial domain CSI compression using two-sided AI model have been evaluated. The traditional bench for evaluation was the DL throughput using e-type 2 codebook for PMI feedback. In RAN1 #115, the following summary and recommendation are agreed for CSI compression.

The performance benefit and potential specification impact were studied for AI/ML based CSI compression sub use case.

Evaluation has been performed to assess AI/ML based CSI compression from various aspects, including performance gain over non-AI/ML benchmark, model input/output type, CSI feedback quantization methods, ground-truth CSI format, monitoring, generalization, training collaboration types, etc. Some aspects were studied but not fully investigated, including the options of CQI/RI calculation, the options of rank>1 solution.

Performance gain over baseline and computational complexity in FLOPs are summarized in clause 6.2.2.8.

Potential specification impact on NW side/UE side data collection, dataset delivery, quantization alignment between CSI generation part at the UE and CSI reconstruction part at the NW, CSI report configuration, CSI report format, pairing information/procedure and monitoring approach were investigated but not all aspects were identified.

The pros and cons are analysed for each training collaboration types, and each training collaboration type has its own benefits and limitations in different aspects. The study has investigated the feasibility of the studied training collaboration types and necessity of corresponding potential RAN1 specification impact. However, not all aspects have been concluded.

Both NW side and UE side performance monitoring were studied, some but not all aspects were concluded.

From RAN1 perspective, there is no consensus on the recommendation of CSI compression for normative work.

At least the following aspects are the reasons for the lack of RAN1 consensus on the recommendation of CSI compression for normative work:

-	Trade-off between performance and complexity/overhead.

-	Issues related to inter-vendor training collaboration.

Other aspects that require further study/conclusion are captured in the summary above.

To improve performance and complexity trade-off, three examples are proposed. We discuss the evaluation methodology for each example.

## Evaluation methodology and observations

### Time-frequency-spatial domain CSI compression

## CSI compression using past CSI was discussed in R18. The overall architecture proposed is shown in Fig. 1. On high level concept, the frequency-spatial domain CSI compression treat the feedback content as an image, while the time-frequency-spatial domain CSI compression treat the feedback content as a video, and technology used in video compression can be used in this context.

Fig. 1. Time-frequency-spatial domain CSI compression

The evaluation methodology for CSI compression can reuse most of the EVM agreed for spatial-frequency domain CSI compression. One important difference is the time sequence length for each training and testing sample. Some example parameters we propose:

- CSI-RS periodicity of 5ms

- Feedback periodicity of 5ms.

- Time sequence length is 20 sample, 100ms long, for calculation purpose.

- SGCS is used as intermediate KPI per time instance.

Due to the strong correlation between CSI samples, with similar size of training dataset, the time-frequency-spatial domain CSI compression model might sample much less random channels, risking of over-fitting. Different examples of generating time sequence of 20 length are shown in Fig. 2. From SLS, per UE, a sequence of CSI-RS measurement is generated. In our experiment, a total of 100-time domain sample sequence with 5ms CSI-RS periodicity is used. With training input of 20-time sample sequence, there are multiple methods to segment the 100 sample time domain sequence. Figure 2-A shows that 5 training sequence is generated non-overlapping with each sequence is 20 CSI-RS samples for training/testing. Figure 2-B shows that each sequence is time shift by one sample, resulting 80 total sequences out of the 100 samples. This obviously dramatically increase the dataset size. Figure 2-C shows partial overlapping.

Fig. 2. Example of generating time sequence sample for training

In addition, partition of training dataset and testing dataset also play critical role. Preliminary testing shows that when training/testing dataset is partitioned among different time sequence of the same UE, better SGCS is observed.

Some preliminary results are shown for UMa, layer 1 in table I for parameter configuration 1, where sequence length is 20 in this preliminary evaluation.

Table I: preliminary result for layer 1 SGCS with 60 bits payload size

|  | e-Type 2 | Freq-spatial domain compression | Time-Freq-Spatial domain compression |
| --- | --- | --- | --- |
| SGCS of layer 1 | 0.69 | 0.72 | 0.75 |

Proposal 1: For time-spatial-frequency domain CSI compression, additional evaluation assumption should be aligned including:

- CSI-RS periodicity of 5ms

- Feedback periodicity of 5ms.

- Time sequence length of 20 samples (100ms) for training/testing

- Detailed methods to generate time sequences for training dataset and testing dataset.

### Cell specific model

For cell specific model, there are two methods to generate large number of training samples.

- Drop large number of UEs, and spatial consistency is assumed.

- Drop a limited number of UEs, for each UE, runs a long time to collect a large set of training data

With spatial consistency, adjacent UEs experience similar channel, therefore SGCS gains can be observed. However, when running such model in SLS, as running time pass, channel becomes uncorrelated in time domain, we observe SGCS drops over simulation time.

Observation 1: For cell specific model, SGCS loss is observed when testing dataset becomes uncorrelated in time domain comparing to training dataset.

Proposal 2: For cell specific model, generalization performance over different time samples should be evaluated.

### Joint CSI compression and prediction

R18 defined the codebook with Doppler basis, a few PMI are compressed together and feedback to gNB. The prediction algorithm is left for UE implementation.

Since there is a separate agenda for CSI prediction evaluation, we think it is better to focus only on the compression gain here. Therefore, the input to the AI encoder, assuming a layer common or layer specific model are Vji at the ith time instance and jth subband. The latent space information contain the UCI for all Vji, and the decoder output the reconstructed Vji.

Proposal 3: For joint CSI compression and prediction, focus on compression gain evaluation comparing to R18 codebook design with Doppler basis.

- Assuming perfect channel is known at the encoder side without actual prediction.

## Potential specification impact

For time frequency spatial domain CSI compression, semi-persisted CSI feedback can be used. The DCI can trigger the start of semi-persisted UCI report and can be used to reset the UCI reporting sequence when the state at the UE side and at the NW side are out of sync due to UCI loss. UCI loss can be caused by potential UL transmission loss, as well as UCI report dropping following UCI omission rule. In addition, mechanism to enable UCI retransmission in order to sync up the encoder and decoder state can be considered.

Proposal 4: For time-frequency-spatial domain CSI compression, the following potential specification impact are proposed:

- Enable semi-persistent CSI reporting for time-freq-spatial domain AI based CSI compression.

- Enable DCI based reset memory.

- Considering UCI retransmission in case of large amount of UCI drop or loss, to avoid the state at UE and gNB out of sync.

# Conclusion

In this contribution, we discussed evaluation methodology and potential specification impact of three additional CSI compression sub-use cases. Based on the discussion, the following proposals have been proposed.

Proposal 1: For time-spatial-frequency domain CSI compression, additional evaluation assumption should be aligned including:

- CSI-RS periodicity of 5ms

- Feedback periodicity of 5ms.

- Time sequence length of 20 samples (100ms) for training/testing

- Detailed methods to generate time sequences for training dataset and testing dataset.

Observation 1: For cell specific model, SGCS loss is observed when testing dataset becomes uncorrelated in time domain comparing to training dataset.

Proposal 2: For cell specific model, generalization performance over different time samples should be evaluated.

Proposal 3: For joint CSI compression and prediction, focus on compression gain evaluation comparing to R18 codebook design with Doppler basis.

- Assuming perfect channel is known at the encoder side without actual prediction.

Proposal 4: For time-frequency-spatial domain CSI compression, the following potential specification impact are proposed:

- Enable semi-persistent CSI reporting for time-freq-spatial domain AI based CSI compression.

- Enable DCI based reset memory.

- Considering UCI retransmission in case of large amount of UCI drop or loss, to avoid the state at UE and gNB out of sync.

# Reference

[1] RP-232745, Summary for RAN Rel-19 Package: RAN1/2/3-led, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023

[1] TR 38.843, Study on AI/ML for NR air interface, Nov 2023