3GPP TSG RAN WG1 #116                                                                     R1- 2401009

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.2.3

Source:	Apple Inc.

Title:	Views on R19 3Tx codebook based transmission

Document for:	Discussion/Decision

# Introduction

In RAN#102, a new work item was endorsed in RP-234007, to specify NR MIMO Phase 5 in Rel-19, where one of the objectives is related to a 3Tx UE as follows:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

In this contribution, we share our views on required specifications for 3Tx codebook-based UL transmission.

# Requirements for 3Tx UL Transmission

## Specifications to CB based SRS

In current specification, UE splits SRS power equally across the configured antenna ports (38.213, 7.3):

For SRS, - if a UE is provided tdm for an SRS resource with 8 ports in an SRS resource set with usage 'codebook' or 'antennaSwitching', the UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports on each symbol for SRS transmission. - else, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for SRS.

Given that in current spec, number of SRS antenna ports for a SRS resource with usage as ‘codebook’ cannot be 3, without enhancement on SRS resource, UE will be configured with 4 SRS antenna ports where for 3Tx UL transmission, one port has to be a dummy port. Obviously, in this case splitting power on 4 ports has no justification.

Proposal 1: For 3Tx PUSCH antenna ports, if UE is configured with SRS resource(s) with 4 antenna ports, UE splits SRS power equally across 3 antenna ports.

## Specifications to TPMI indication

In current specification (38.214, 6.1.1.1), the UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource(s) indicated by the DCI format 0_1, 0_2 or 0_3 or by configuredGrantConfig according to clause 6.1.2.3. Without enhancement on SRS resource, UE will be configured with 4 SRS antenna ports where for 3Tx UL transmission, one port has to be dummy.

Proposal 2: For 3Tx PUSCH antenna ports, if UE is configured with SRS resource(s) with 4 antenna ports, the 4th SRS antenna port (1003) is considered dummy in specification.

- UE does not expect the indicated precoding matrix by TPMI has a non-zero value on the 4th row

Based on current WID, 3Tx UL enhancement is limited to non-coherent UL codebook. Thus, precoding matrix will be a combination of these vectors, 100, 010, and 001. The following figure shows an example of TPMI indices for 3Tx antenna ports and different number of layers.

Fig. 1: Precoding matrix W for 3Tx antenna ports and different number of layers

Proposal 3: For 3Tx antenna ports if the higher layer parameter txConfig = codebook, number of bits to indicate TPMI is determined as follows

- 2 bits if transform precoder is enabled, or if transform precoder is disabled and maxRank = 1

- 3 bits if transform precoder is disabled and maxRank = 2 or 3

# Conclusion

In this contribution, we shared our views on 3Tx UL transmission for non-coherent UL codebook. Based on what we discussed, the following proposals are made:

Proposal 1: For 3Tx PUSCH antenna ports, if UE is configured with SRS resource(s) with 4 antenna ports, UE splits SRS power equally across 3 antenna ports.

Proposal 2: For 3Tx PUSCH antenna ports, if UE is configured with SRS resource(s) with 4 antenna ports, the 4th SRS antenna port (1003) is considered dummy in specification.

- UE does not expect the indicated precoding matrix by TPMI has a non-zero value on the 4th row

Proposal 3: For 3Tx antenna ports if the higher layer parameter txConfig = codebook, number of bits to indicate TPMI is determined as follows

- 2 bits if transform precoder is enabled, or if transform precoder is disabled and maxRank = 1

- 3 bits if transform precoder is disabled and maxRank = 2 or 3