3GPP TSG RAN WG1 #116 		                                    		   R1-2401042

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.1.2

Source:	InterDigital, Inc.

Title:	Discussion on support for AIML positioning

Document for:	Discussion and Decision

# Introduction

In RAN#104, WID for AI/ML for NR air interface was approved [1]. The WID contains the following items for AI/ML positioning.

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

In this contribution, specification support for AI/ML positioning is discussed.

# Support for AIML positioning

## Prioritization

In the AI/ML WID [1], it was proposed to discuss a framework for AIML positioning to support Case 1, Case 3b and Case 3a as the 1st priority while Case 2a and Case 2b will be treated at the 2nd priority. Given the limited amount of time units during Release 19 and amount of effort required to coordinate with the general framework, it is recommended that we will not discuss cases that have the 2nd priority.

Proposal 1: Postpone any discussion related to 2nd priority cases until framework to support the 1st priority items is stabilized

## New measurements

During the study item, effectiveness of new measurements for inputs or inference of an AI/ML model were analyzed and evaluated with numerical evaluation [2]. Some examples of propoesd new measurements are channel impulse response (CIR) and time of flight. While time of flight was discussed mainly for AIML assisted positioning, CIR was discussed as one of the candidates for inputs for an AIML model for direct AIML positioning. In this contribution, we focus on details related to the CIR.

There is a need to study how CIRs can be derived by the UE or LMF. A CIR can be defined as tapped delay model with amplitude, phase and relative time delay for each tap. In TS 38.215, phase measurement, specified for NR positioning, is specified only for the first path. Power profile can be derived based on RSRPP defined in TS 38.215. The delay profile of the CIR may be defined using RSTD measurements for multiple paths described in TS 37.355 [3]. The CIR, if specified, should also be accompanies by uncertainty so that the UE or LMF can use uncertainty information to determine quality of the CIR estimate. Thus the following proposals are made.

Proposal 2: Study how channel impulse response can be defined as a measurement based on the specified measurements for NR positioning

Proposal 3: If a CIR is to be specified, study how uncertainty related to CIR can be specified

It should be noted that delay profile (DP) and power delay profile (PDP) were also studied during the study item [2]. Once a CIR is defined, it should be straightforward to define DP or PDP since they retain characteristics such as delay profile of the CIR.

Although CIR was identified as the most effective AIML model input for AIML based positioning, complexity of CIR, such as the number of taps, can be large. To support AIML model inputs with lower complexity, existing measurements such as timing (e.g., RSTD) or power measurements (e.g., RSRP) should be considered.

The trade-off among horizontal positioning accuracy and measurement size, accuracy for different model input types(e.g., CIR, PDP, RSRP and RSRP+RSTD) and model input sizes are compared in Table 7 of [5]. Based on the numerical evaluation results, we make following proposal:

Proposal 4: For direct AI/ML based positioning, adopt RSRP and RSTD measurement as input for an AIML model for direct AIML positioning

## Definition of functionalities for AIML positioning

As shown below, WID [1] indicates support for functionality based model identification and LCM at least for one-sided model. Justification is needed for model based LCM, according to the WID.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

In TR 38.843 [2], the following definition is used for identification of functionalities.

Functionality identification: A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE. Note: Information regarding the AI/ML functionality may be shared during functionality identification. Where AI/ML functionality resides depends on the specific use cases and sub use cases.

Some examples of functionalities for AIML may be positioning and beam management, indicating the purpose an AIML model is used for. An AIML model may be trained differently based on the environment such as indoor, outdoor, LOS heavy or NLOS heavy environment. Thus, allocating one model per functionality may not be sufficient. There may be more than one models for a given functionality and identify for each model becomes important for LCM.

Thus, granularity of the functionalities need to be discussed. Granularity of a functionality can be defined according to some of PRS configurations. For example, an AIML model can be associated with a cell (cell-level functionality). If the UE changes a cell, the UE can decide whether to keep using the AIML model or use a different AIML model that is associated with a new cell. An AIML model can be associated with PRS frequency layers (frequency level functionality). For example, an AIML model can be trained with a set of PRS frequency layers as the performance of an AIML model may be dependent on configured PRS frequency layers. If the UE moves to a new area where the TRP uses a different or a subset of frequency layers from the ones the AIML model is trained with, the UE should decide whether to use a new AIML model that is associated with a different set of frequency layers or not.

Observation 1 : Granularity of functionality for AIML positioning needs to be discussed as there can be more than one model per functionality (e.g., positioning)

Based on the above analysis, the following proposal is made.

Proposal 5: Granularity of functionality for an AIML model for AIML positioning can be defined by PRS configurations (e.g., TRP ID, frequency layer ID, cell ID)

For UE-sided model, if the UE decides to use a different model or switch a model, the UE should inform the LMF about the new model so that the LMF can understand how the UE is determining its location.

Proposal 6: For UE-side model, the UE should notify the network if there are any changes to the model

Finally, for the UE-based positioning, the UE should decide whether AIML based positioning should be used or not. A mechanism for the UE to decide the fallback positioning method should be defined.

Proposal 7: The UE should decide whether to use an AIML based positioning or not and fallback positioning method should be configured by the network

## Training specific support

The following discussion is included in our companion contribution [6]. We recommend to refer to [6] for discussion on a general framework of data collection for different use cases.

Overview

For positioning sub-use case, training data for AIML based positioning consists of ground truth and inputs to an AIML model where inputs may consist of measurements made on received DL-PRS. Some examples of measurements are CIR, PDP, DP, timing or power measurements as identified during the study item phase [3]. For UE-based AIML based positioning with one-sided model, how the training data can be acquired needs to be discussed.

Quality of training data

In TR 38.853, the following agreement made during the study item phase is captured; PRU and/or UE can generate the ground truth label and label quality indicator can be generated by the UE which may use RAT dependent or RAT independent positioning method to estimate its location.

- The following options of entity and mechanisms to generate ground-truth label are identified: - UE with estimated/known location generates ground-truth label and corresponding label quality indicator - Based on non-NR and/or NR RAT-dependent and/or NR RAT-independent positioning methods - At least for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) - Network entity generates ground-truth label and corresponding label quality indicator - Based on non-NR and/or NR RAT-dependent and/or NR RAT-independent positioning methods - At least for UE-assisted/LMF-based positioning with LMF-side model (Case 2b), NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - At least PRU is identified to generate ground-truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) - At least LMF with known PRU location is identified to generate ground-truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - At least network entity with known PRU location is identified to generate ground-truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a) - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified: - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP

As quality of training data determines quality of the trained AIML model, integrity of training data becomes critical. For UE-sided model, the UE needs to quality of the training data based on the ground truth label quality indicator and there is a need to define a ground truth label quality indicator associated with a ground truth label.

Firstly, our proposal is to define a hard value (1 or 0) or soft value (0, 0.1, 0.2, …, 1.0) to indicate the quality of the ground truth label. A hard indicator may indicate that “1” is reliable for training (e.g., generated  by PRU) while “0” indicates the ground truth label is not reliable for training.  For a soft indicator, the value of “0.9” indicates relatively high confidence in using the associated ground truth for training an AIML model.

Proposal 8: Support both hard (1 or 0) and soft indicator (0, 0.1, 0.2, …, 1.0) for a ground truth label quality indicator

Delivery of training data to the UE

How training data can be delivered to the UE needs to be discussed. During Release 18, measurement forwarding functionality by the LMF for NRCP (NR Carrier Phase) positioning was supported [4]. Using the supported functionality, a UE can request for measurements made by PRU (Positioning Reference Unit) to the network. As a response for the request made by the UE, the LMF can send positioning timing, power and phase measurements (e.g., NRCP, RSTD, RSRP) made by the PRU to the UE who made the request.

The measurement forward functionality can be used to deliver training data to the UE. As the location of the PRU is known by the LMF, the measurements and location information, i.e., ground truth, can be  reliable training data. An additional benefit of the functionality, in the context of training data for AIML positioning, is that the LMF can check the content of the training data.

As the current specification supports only measurements, the content of assistance information needs to be improved. For example, ground truth label quality indicator needs to be associated with the ground truth, i.e., PRU location information.

Observation 2: Measurement forwarding by the LMF specified in Release 18 for carrier phase positioning can be used to deliver training data to the UE by the network

Proposal 9: Assistance information from the LMF which contains PRU measurements should also include the ground truth quality indicator associated with the PRU location information

In addition, as the number of deployed PRUs may be limited, or PRU may be stationary, the UE should obtain location information of non-PRUs from the LMF. Obtaining location information and associated measurements of non-PRUs, UE for example, can increase the volume of the training data at the UE. Thus the following proposal is made.

Proposal 10: Support LMF to forward location information of non-PRUs (e.g., UE), associated measurements and ground truth label quality indicator to a UE

Obtaining ground truth quality label

In positioning, uncertainty for UE location estimate is specified [5]. However uncertainty is determined by the UE if UE based positioning is implemented. Thus, in NLOS heavy environment assumed for the positioning use case, the UE could be locked on an inaccurate position estimate as illustrated in Figure 1.

Figure 1 An example of uncertainty related to UE location estimate

If the UE determines the ground truth label quality indicator based on the uncertainty derived at the UE, the generated ground truth label quality indicator may be unreliable as the estimate UE location may be inaccurate.

Observation 3: A ground truth label quality indicator generated by a UE may be unreliable as the estimate UE location may be inaccurate

To solve this issue, we propose to allow the UE to request the LMF to generate the ground truth label quality indicator, based on the measurements and UE location estimate made by the UE. It is assumed throughout NR positioning work that the LMF can collect a large amount of data from UEs and generate prior knowledge of the environment (e.g., LOS status between TRP and UE, or along PRS). Thus, the LMF should be able to generate a ground truth label quality indicator by comparing reported measurements and UE estimate against its database of collected measurements and UE location estimates. With this approach, the UE can obtain a reliable ground truth label quality indicator from the network. Thus, the following proposal is made:

Proposal 11: For UE-based positioning using AIML based positioning, support the UE to request the LMF for a ground truth label quality indicator based on measurement and UE location estimate made by the UE

Training status via RMSE

Different model architectures may require different amount of data to achieve certain positioning accuracy.  To select a trained model with highest accuracy for inference operation, a UE need a common metric to evaluate the AIML model. It is a common practice to use a performance metric of validation dataset to compare how different model architecture performs for a specific dataset.

We evaluate RMSE (root mean square error) of validation dataset as a measure of maturity of a trained AIML model. Details of simulation assumptions are described in Appendix A.  In the evaluation, different number of samples are used during training. We evaluate RMSE of validation dataset. A comparison between training data size and RMSE is shown in Table 1.

Table 1. Evaluation results for AI/ML model deployed on UE-side, differently trained models, without model generalization, UE distribution area = [120x60 m]

| Model input | Model output | (Percentage of training data set without) Label | Clutter param | Dataset size |  | AI/ML complexity |  | Performance metric for trained AIML model | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Model complexity | Comp. complexity | RMSE of validation dataset(meters) | AI/ML |
| CIR (18*256* 2) | UE position | 0% (default) | 60%, 6m, 2m | 12000 | 4000 | 37 M | 843 M | 1.33 | 1.20 |
| CIR (18*256* 2) | UE position | 0% (default) | 60%, 6m, 2m | 8000 | 4000 | 37 M | 843 M | 1.91 | 1.87 |
| CIR (18*256* 2) | UE position | 0% (default) | 60%, 6m, 2m | 4000 | 4000 | 37 M | 843 M | 3.94 | 4.00 |

Based on the results presented in Table 1, the following observation and proposal are made.

Observation 4: For direct AI/ML positioning, 90% horizontal positioning accuracy of test dataset deteriorates in proportion to RMSE of validation dataset.

Proposal 12: Support reporting of an AIML model training outcome, RMSE (Root Mean Square Error) result, to the network

## Inference generation specific support

The network may need to mask inputs for TRPs to reduce interference. If TRPs are masked, the UE may need to generate inferences using a subset of TRPs to generate inference. This means that the UE needs to generate inference with a number of inputs smaller than the required number of inputs for a configured AIML model.

In this subsection we analyse how the impact of limited number of TRPs (N’TRP<18) can impact the accuracy of AIML positioning. To evaluate the impact of different measurement(input) size on positioning accuracy, we sweep different number of TRPs (N’TRP) while CIR tap size (Nt) is constant. A unique model is trained for each input size configuration. The number of TRPs (N’TRP) that provides input to model varies and the remaining (NTRP  N’TRP) TRPs that do not provide measurements to model input is set to 0 (Approach 1). In these evaluations, we change the set of TRPs (N’TRP) that provide measurements dynamically based on the highest N’TRP RSRP values (Approach 1-B).

Table 2. Evaluation results for AI/ML model deployed on UE-side, different number of TRPs (N’TRP<18) with dynamic TRP selection (Approach 1-B), without model generalization, UE distribution area = [120x60 m]

| Model input (N’TRP * Nt *2) where 2 indicates the complex number factor | Model output | (Percentage of training data set without) Label | Clutter param | Dataset size |  | AI/ML complexity |  | Horizontal positioning accuracy at CDF=90% (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Train | Test | Model complexity | Computation complexity | AI/ML |
| CIR (18*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 37 M | 843 M | 0.98 |
| CIR (16*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 37 M | 843 M | 0.98 |
| CIR (14*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 37 M | 843 M | 1.05 |
| CIR (12*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 37 M | 843 M | 1.11 |
| CIR (10*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 37 M | 843 M | 1.11 |
| CIR (8*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 37 M | 843 M | 1.14 |
| CIR (6*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 37 M | 843 M | 1.70 |
| CIR (4*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 37 M | 843 M | 2.90 |
| CIR (2*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 37 M | 843 M | 9.16 |
| CIR (18*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 1.4 M | 38 M | 1.25 |
| CIR (16*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 1.4 M | 38 M | 1.25 |
| CIR (14*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 1.4 M | 38 M | 1.32 |
| CIR (12*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 1.4 M | 38 M | 1.32 |
| CIR (10*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 1.4 M | 38 M | 1.35 |
| CIR (8*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 1.4 M | 38 M | 1.65 |
| CIR (6*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 1.4 M | 38 M | 1.91 |
| CIR (4*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 1.4 M | 38 M | 3.34 |
| CIR (2*256*2) | UE position | 0% (default) | 60%, 6m, 2m | 16000 | 4000 | 1.4 M | 38 M | 10.32 |

Based on the results presented in Table 2, we make the following observations:

Observation 5: For direct AI/ML positioning, for different number of TRPs [N’TRP=16,14,12,10,8,6,4,2] when measurements are collected from TRPs with highest N’TRP RSRP values (Approach 1-B), horizontal positioning accuracy degrades as we reduce the number of TRPs (N’TRP) for 90% UEs.

Proposal 13: To introduce flexibility in configuration of AIML inputs, introduce a masking pattern to allow the network to mask some inputs for the configured AIML model at the UE

# Conclusion.

In this contribution, the following proposals and observations are made.

Observation 1 : Granularity of functionality for AIML positioning needs to be discussed as there can be more than one model per functionality (e.g., positioning)

Observation 2: Measurement forwarding by the LMF specified in Release 18 for carrier phase positioning can be used to deliver training data to the UE by the network

Observation 3: A ground truth label quality indicator generated by a UE may be unreliable as the estimate UE location may be inaccurate

Observation 4: For direct AI/ML positioning, 90% horizontal positioning accuracy of test dataset deteriorates in proportion to RMSE of validation dataset.

Observation 5: For direct AI/ML positioning, for different number of TRPs [N’TRP=16,14,12,10,8,6,4,2] when measurements are collected from TRPs with highest N’TRP RSRP values (Approach 1-B), horizontal positioning accuracy degrades as we reduce the number of TRPs (N’TRP) for 90% UEs.

Prioritization of specification items

Proposal 1: Postpone any discussion related to 2nd priority cases until framework to support the 1st priority items is stabilized

Measurements

Proposal 2: Study how channel impulse response can be defined as a measurement based on the specified measurements for NR positioning

Proposal 3: If a CIR is to be specified, study how uncertainty related to CIR can be specified

Proposal 4: For direct AI/ML based positioning, adopt RSRP and RSTD measurement as input for an AIML model for direct AIML positioning

Mode identification for LCM

Proposal 5: Granularity of functionality for an AIML model for AIML positioning can be defined by PRS configurations (e.g., TRP ID, frequency layer ID, cell ID)

Proposal 6: For UE-side model, the UE should notify the network if there are any changes to the model

Proposal 7: The UE should decide whether to use an AIML based positioning or not and fallback positioning method should be configured by the network

Ground truth and training data

Proposal 8: Support both hard (1 or 0) and soft indicator (0, 0.1, 0.2, …, 1.0) for a ground truth label quality indicator

Proposal 9: Assistance information from the LMF which contains PRU measurements should also include the ground truth quality indicator associated with the PRU location information

Proposal 10: Support LMF to forward location information of non-PRUs (e.g., UE), associated measurements and ground truth label quality indicator to a UE

Proposal 11: For UE-based positioning using AIML based positioning, support the UE to request the LMF for a ground truth label quality indicator based on measurement and UE location estimate made by the UE

Proposal 12: Support reporting of an AIML model training outcome, RMSE (Root Mean Square Error) result, to the network

Masking pattern

Proposal 13: To introduce flexibility in configuration of AIML inputs, introduce a masking pattern to allow the network to mask some inputs for the configured AIML model at the UE

# Reference

[1] RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” RAN#102, Edinburgh, Scotland, Dec., 2023.

[2] 3GPP, TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface,” v 2.0.1., Dec., 2023.

[3] 3GPP, TS 38.215, “Physical layer measurements, ” v 18.0.0, Sept. 2023.

[4] 3GPP, TS 37.355, “LTE Positioning Protocol (LPP), ” v 18.0.0, Dec. 2023.

[5] R1-2307582, “Evaluation on AI/ML for positioning accuracy enhancement”, InterDigital, RAN1#115 Aug. 2023.

[6] R1-2400190, “Discussion on other aspects of AI/ML model and data”, InterDigital, RAN1#116, Feb.,. 2024

# Appendix A -Simulation Assumptions

In this contribution, we evaluate the impact of following parameters on direct AIML positioning accuracy:

- Evaluation of input size reduction

- Performance metric for trained AIML model

Dataset has been generated by carrying out System Level Simulations for IIoT scenario. The InF-DH channel model is configured to simulate NLOS heavy environment. Furthermore, UEs are dropped in the entire deployment area including corners, which makes positioning even more challenging. It is expected that accuracy performance for the UEs located close to the corner of the factory floor using conventional methods (e.g., DL-TDOA) degrades considerably. Details of the IIoT scenario parameters are listed in Table A1. Furthermore, all the details related to model input/output and model structure are described in Table A2. Summary of the evaluation assumptions are as follows:

- Model Input: CIR measurements

- Model Output: UE Location information

- Model architecture: ResNet

- Model deployed on: UE side

Table A1: IIoT scenario system parameters

| Parameter | Values |
| --- | --- |
| Carrier frequency, GHz | 3.5GHz |
| Bandwidth, MHz | 100MHz |
| Subcarrier spacing, kHz | 30kHz |
| Channel model | InF-DH |
| Hall size | 120(L) x 60(W) m, D – 20 m |
| BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. |
| Room height | 10 m |
| Number of floors | 1 |
| Clutter parameters: {density , height ,size } | InF-DH - {60%, 6m, 2m} |
| UE model parameters |  |
| UE noise figure, dB | 9dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |
| Network synchronization | Fully synchronized |
| UE/gNB RX and TX timing error | T1= 0 ns |
| UE horizontal drop procedure | Uniformly distributed over entire factory floor |
| UE antenna height | 1.5 m |
| gNB model parameters |  |
| Total gNB TX power, dBm | 24 dBm |
| gNB noise figure, dB | 5dB |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 |
| gNB antenna height | 8 m |

Table A2: Model configuration for direct AI/ML positioning

| Parameter | Values |
| --- | --- |
| Training input measurements | CIR: CIR from multiple TRPs (4608 complex values for 18 TRPs, 256 complex values per TRP). |
| Output | UE position |
| Number of TRPs | 18 |
| BS locations | As specified in Table A1 |
| Size of total dataset | 20000 samples |
| ML model | ResNet (‘j’ Convolutional layer, ‘k’ residual layers, 1 fully connected layer) |