3GPP TSG RAN WG1 #116			R1-2401079

Athens, GR, February 26th – March 1st, 2024

Agenda item:	9.11.1 (NR-NTN downlink coverage enhancement)

Source: 	NICT

Title: 	Discussion on DL coverage enhancements for NR-NTN

Document for:	Discussion and Decision

- Introduction

In RAN#102, a new work item Non-Terrestrial Networks (NTN) for NR Phase 3 [1] was approved. One of the objectives is to study and specify beneficial downlink coverage enhancements covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN.  In this contribution, we present our views on downlink coverage enhancement for NTN.

- Discussion

The motivations to enhance NTN for NR in DL coverage perspective include:

- a support of handheld terminals that operates in FR1 bands

- a need of efficient beam management to cover large satellite footprint

Limited available power of the satellite stations is a major constraint to be considered in both motivations meanwhile antenna gain of handheld terminals and feeder link bandwidth are other limiting factors for the motivations (a) and (b) respectively. In the following subsections, we present our views on link level and system level enhancement when there are limitations in available power and radio resource usage and beam and cell management scenario that would be required when link level and system level enhancements are studied.

  - Link level enhancements

Technical Report [2] gives a calculation of CNR of a satellite wireless link as

The calculation uses parameters such as Tx EIRP, propagation path loss, ambient noise at the receiver, antenna gain and NF of the receiver. Link level enhancement improves decoding performance of a target physical channel under a certain CNR, i.e. considering that the parameters for the CNR calculation are given condition. What can be considered for the link level enhancements are:

- Increase power assignment of the target physical channel within the given EIRP

- Apply repetitions to increase available receiving energy for the decoding

- Apply low rate coding to reduce required Eb/No for the decoding

(a) decreases portion of the transmit power for other physical channels or other UEs. (b) and (c) require additional radio resource and/or reduce information rate. In any case, a penalty of a decrease of the radio resource usage efficiency accompanies with the link level enhancements under the given CNR.

Observation 1:

Link level enhancements have negative impact on the radio resource usage efficiency.

Proposal 1:

Radio resource usage efficiency must be considered when evaluating the effect of link level enhancements.

  - System level enhancements

As discussed in 2.1, link level enhancemnets has negative effect in radio resouce usage efficiency, the gain obtainable by the link level enhancements themselves would be limited if the decrease of the radio resouce usage efficiency have to be minimized. Therefore system level enhancements such as power sharing among satellite beams have also to be taken in order to achieve satisfiable performance gain. The power shareing can achieve maximum gain when each wireless link has different CNR requirement and proper resouce assignment and/or power control is performed, hence, the control system would be inherently complex.

On the other hand, diversity reception is one of the basic receiving performance improvement methods that has less or no impact on the control system required to apply the method. Among various modes of diversity, time or frequency diversity require additional transmissions in different time or frequency, which is actually equivalent to the repetition method of link level ehnancement. Site diversity reception using more than two receiving UEs require cordination of involving UEs but no additional radio resouces, which is a preferrable feature when enhancing the performance with limitations in transmitting power and/or radio resouces. Although further investigations are required, if main cause of receiving error is not interference or ambient noise, which undergo commonly to involving UEs, but receiver noise of each UE, the improvement of receiving performance can be expected with the site diversity reception.

Observation 2:

Performance improvement can be expected by site diversity reception.

Proposal 2:

Site diversity reception is to be considered as one of the system level enhancement solutions.

  - Beam management

In TR38.821[2], for 600km altitude LEOs, 10 degrees is designated as a minimum elevation angle in a table of reference scenario parameters, while 30 degrees is assumed in some link level analysis. In case that the minimum elevation angle is 10 degrees, the radius of a satellite footprint can be calculated as 1760km. As an easy first order estimation, the total number of beams that a satellite must handle is given by

total area of satellite footprintarea of a beam footprint.

It is about 5000 when beam footprint is 50km diameter (S-band), and 30000 when beam footprint is 20km diameter (Ka-band). In case of 30 degrees minimum elevation angle, in which the radius of the satellite footprint is 854km, they are about 1200 and 3700, respectively. Considering that supported SSB beams per cell (Lmax) is 4 or 8 in FR1 bands and 64 in FR2 bands in the current NR specification, the satellite must also operate numbers of cells unless Lmax is substantially increased or reuse of beams is supported.

On the other hand, considering that the satellite footprint, the radius of which is over 500 or 1000km, is very unlikely all populated city area but includes rural, mountainous areas as well as ocean, beams with minimum footprint size is required for only portion of the satellite footprint while other portion of the satellite footprint may be covered by beams with larger footprint size. Therefore, it is not practical to assume that a satellite must handle possible maximum number of beams that is calculated by satellite footprint area size and minimum beam footprint size, rather, there should be some practical beam and cell operation scenario with which further study will be carried out.

Observation 3:

It is impractical to assume all possible numbers of beams/cells that a satellite station must handle.

Proposal 3:

Establish some practical beam and cell operation scenario before carrying out the further study on link level or system level solutions.

- Conclusion

  - Observations

Observation 1:

Link level enhancements have negative impact on the radio resource usage efficiency.

Observation 2:

Performance improvement can be expected by site diversity reception.

Observation 3:

It is impractical to assume all possible numbers of beams/cells that a satellite station must handle.

  - Proposals

Proposal 1:

Radio resource usage efficiency must be considered when evaluating the effect of link level enhancements.

Proposal 2:

Site diversity reception is to be considered as one of system level enhancement solutions.

Proposal 3:

Establish some practical beam and cell operation scenario before carrying out the further study on link level or system level solutions.

References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- TR38.821, Solutions for NR to support non-terrestrial networks (NTN)