3GPP TSG RAN WG1 #116			R1-2401082

Athens, Greece, February 26th – March 1st, 2024

| Agenda item: | 9.1.2 |
| --- | --- |
| Source: | Baicells |
| Title: | Discussion on specification support for AI/ML-based positioning accuracy enhancement |
| Document for: | Discussion and Decision |

Introduction

Accurate positioning is an essential and integral component of many industrial use cases and verticals. In R18, a SI Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface investigated the benefits of using AI/ML to further enhance positioning accuracy as one of the target use cases. Spanning the entire duration of the release, the SI also studied the 3GPP general framework for AI/ML for air interface, evaluated the AI/ML model performance and assessed the potential specification impact considering use cases including CSI, beam management and positioning.

Under this background, a new WI [1] for R19 on AI/ML for NR Air Interface has been approved at RAN #102. This WI aims to provide normative support for general framework for AI/ML for air interface and use cases in the preceding R18 study. In addition, some study objectives will be continued to deal with outstanding issues identified in R18 study. The objective of the new WI for RAN1-lead AI/ML based positioning includes the following:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In this contribution, we present our views on positioning sub use cases and potential specification impact for positioning accuracy enhancement.

Discussion

Positioning sub-use cases

In RAN #102, the five sub-use cases have been assigned with different priorities as shown in Figure 2.1. In this contribution, we will focus on the three cases with highest priority.

Figure 2-1  Sub-use cases with different priorities

Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Table 2-1 shows the model input and model output related information for Case 3b.

Table 2-1  model input and model output (label) related information for Case 3b

|  | Measurements for model input | -Potential new measurement: CIR/PDP -Existing measurement: e.g., RSRP/RSRPP/RSTD | By TRP |
| --- | --- | --- | --- |
|  | Ground-truth label | UE position | By LMF with known PRU location |

Model inference

In our understanding, the key normative work for Case 3b model inference is specifying the measurements such as CIR/PDP, as these new measurements will be required by LMF to derive UE position. As for other aspects, we think no specification effort is needed for Case 3b model inference by reusing existing positioning procedures. Below we use UL-TDOA positioning procedure as an example in Figure 2-2 to explain our view.

Figure 2-2  UL-TDOA positioning procedure as an example to explain specification impact for Case 3b model inference

As shown in Figure 2-2, the model is deployed within LMF. In step 4 of the UL-TOOA positioning procedure, the serving gNB provides UE SRS configuration to LMF. Based on the given SRS configuration and other information LMF already has, LMF can select an appropriate model for positioning which can be considered as an implementation-based algorithm. Subsequently, in step 6, LMF requests specific gNBs (e.g., serving gNB and Neighbour gNBs) according to its algorithm to conduct measurements (e.g., CIR/PDP) based on the SRS configuration. After receiving these measurements, the LMF-side model can finally output UE position. An observation drawn from above is that, the procedure of UL-TDOA positioning can be reused, and new measurements such as CIR/PDP may need to be transferred from gNBs to LMF.

Model inference for Case 3b can almost reuse the existing positioning procedures except that new measurements such as CIR/PDP may need to be transferred from gNBs to LMF to support AI/ML-based positioning.

For Case 3b model inference, new measurements (if necessary) such as CIR/PDP may need to be specified to support AI/ML positioning while existing positioning procedures can be reused.

Model training

For Case 3b model training, the entity that collects the data and performs the training is considered as NW side. Therefore, new measurements such as CIR/PDP may need to be specified same as Case 3b model inferencing.

For Case 3b data collection for model training, new measurements (if necessary) such as CIR/PDP may need to be specified same as Case 3b model inferencing.

The existing traditional positioning framework and procedures are designed to derive UE position via UE-RAN-CN cooperation, which can be reused or provide designing reference for the model inference procedure. However, for data-driven AI/ML based positioning, the training data collection procedure becomes as important as inference procedure and may need to redesign such that it can meet various data collection requirements. For example, when a large scale of data is required for offline model training, it may have a lower real-time requirement compared to the existing positioning procedure while the number of types of the measurements in one transfer may increase significantly such that data with certain relationships can be collected in certain pattern for different training purpose.

For data-driven AI/ML based positioning, the training data collection procedure may need to redesign such that it can meet various data collection requirements which is different from the existing positioning procedure.

Based on the above analysis, there may be several potential signalling enhancements to facilitate training data collection in our understanding.

Firstly, in existing positioning procedures, the SRS configuration is decided by the serving gNB as illustrated in step 3 of Figure 2-2, then the specific SRS configuration will be provided to LMF in step 4. However, when LMF needs to collect data for Case 3b model training, LMF is expected to directly indicate the specific SRS configuration under which the data will be collected since it is the LMF or the NW side entity which has the full knowledge/control of the data and NW-side models. Therefore, for Case 3b data collection for model training, the LMF should indicate its desired SRS configuration in data collection request, and the serving gNB should provide the configuration result to LMF in the response message.

For Case 3b data collection for model training, the LMF should indicate its desired SRS configuration in data collection request, and the serving gNB should provide the configuration result to LMF in the response message.

Secondly, considering the number of types of the measurements in one transfer for Case 3b data collection may be large, and LMF may request multiple gNBs to transfer the data during the procedure, it would be beneficial that LMF is aware that whether all selected gNBs are able to provide all required measurements/data before the start of measurements reporting. Otherwise gNBs may provide different number of measurement types of the data that can only be aligned after further processing at NW side to form a dataset, which will cause heavy waste of the bandwidth transferring data that cannot be used, as well as extra NW-side computation resources when organizing the dataset.

For Case 3b data collection for model training, due to different gNB capabilities and other factors, gNBs may provide different number of measurement types of the data that can only be aligned after further processing at NW side to form a dataset, which will cause heavy waste of the bandwidth transferring data that cannot be used, as well as extra NW-side computation resources when organizing the dataset.

Therefore, we think it’s necessary to have a measurement request indicating the desired measurements for data collection from LMF, as well as a measurement response indicating the supported measurements by gNBs.

For Case 3b data collection for model training, a measurement request indicating the desired measurements for data collection from LMF, and a measurement response indicating the supported measurements by gNBs can be beneficial to better support data collection.

Model monitoring

As the model is deployed in LMF, model monitoring can be up to implementation, hence no specification impact is expected.

No specification impact is expected for Case 3b model monitoring since monitoring can be deployed within the NW and based on implementation.

Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Table 2-2 shows the model input and model output related information for Case 3a.

Table 2-2  model input and model output (label) related information for Case 3a

|  | Measurements for model input | CIR/PDP | By TRP |
| --- | --- | --- | --- |
|  | Ground-truth label | Intermediate measurement: -New measurement report: e.g., ToA, path phase -Existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP -Enhancement of existing measurement report: e.g., soft information/high resolution of RSTD | By network entity with known PRU location |

Model inference

For Case 3a model inference, intermediate measurements at gNB side are provided to LMF. While existing positioning procedures can be reused same as Case 3b, RAN1 can discuss whether normative effort is needed for new measurements such as ToA or enhanced existing measurements.

RAN1 can discuss whether normative effort is needed for new measurements for Case 3a model inference.

Model training

In preceding SI, the following consensus was reached.

Model training: - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM. - Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

As no conclusion was reached whether LMF can be used for gNB-side model training, in our understanding, the primary focus of normative work for Case 3a model training should be to explore the feasibility of training models at gNB side. In this logic, a start for RAN1 may be to discuss how labels are provided to gNB by NW entity (e.g., LMF).

For Case 3a model training, support to explore the feasibility of training models at gNB side as the primary focus of the normative work. Consider discussing how labels are provided to gNB by NW entity (e.g., LMF) as starting point.

Model monitoring

In preceding SI, the following agreement regarding monitoring was reached.

- Monitoring: - The UE monitors the performance of its UE-side model. - For monitoring at the gNB side, and if needed, calculated performance metrics or data required for performance metric calculation, can at least be generated by the gNB. - For monitoring at the LMF side, the gNB or UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination points for these metrics is the LMF.

In our understanding, the monitoring can be either at gNB side or LMF side. If the monitoring is at gNB side, minimum specification effort is expected as it can be up to implementation. If LMF can perform the monitoring, performance metrics or data required for performance metrics will be provided to LMF via NRPPa. However, given that monitoring may have a lower priority compared to model inference and training, and considering the limited time available during RAN1 meetings, we would prefer to discuss model monitoring after topics such as new measurements have been fully addressed. This is because monitoring may not have a significant impact on the specifications for Case 3a in the end.

Discussion on model monitoring for Case 3a can begin after topics such as new measurements for 1st priority sub-use cases have been fully addressed. This is because monitoring may not have a significant impact on the specifications for Case 3a.

Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

Table 2-3 shows the model input and model output related information for Case 1.

Table 2-3  model input and model output (label) related information for Case 1

|  | Measurements for model input | CIR/PDP | By PRU/UE |
| --- | --- | --- | --- |
|  | Ground-truth label | UE position | By UE with estimated/known location |

Model inference

For Case 1 model inference, the UE-side model performs the model function and directly outputs UE position information. Subsequently, this UE position is reported from UE to LMF server using existing LPP protocol. Input for the model can be channel observation e.g., CIR, PDP based on DL PRS measurement by UE. The measurements for model input usually have undergone a pre-processing procedure such as truncation, normalization.

As analyzed above, measurements such as CIR/PDP for model input are not required to be transferred to LMF. In addition, UE position can be delivered to LMF reusing existing LPP IE. Hence, there is no need to specify new measurements for Case 1 model inference.

There is no need to specify new measurements for Case 1 model inference.

Model training

Since no conclusion was reached in the preceding SI, CN/OAM/OTT collection of UE-sided model training data will be studied in a separate agenda of this WI. Hence, the specification impact for Case 1 data collection for model training depends on the study part of this WI.

The specification impact for Case 1 data collection for model training depends on the study part of this WI.

Model monitoring

- Monitoring: - The UE monitors the performance of its UE-side model.

As suggested in the preceding SI, the UE monitors the performance of its UE-side model. Therefore, no specification impact is expected for Case 1 model monitoring if UE doesn’t have to report the monitoring result to NW.

No specification impact is expected for Case 1 model monitoring if UE doesn’t have to report the monitoring result to NW.

Method to ensure consistency between training and inference regarding NW-side additional conditions

Generally, additional conditions are used to represent aspects (e.g., dynamic changing environment factors) related to model training but are not a part of the UE capability. Figure 2-3 shows the relation between UE capability, conditions, additional conditions, and models’ information. Although additional conditions show up in both functionality-based LCM and model-ID-based LCM, it is more often discussed in functionality-based LCM as functionality-based LCM needs an innovative mechanism to ensure the consistency of additional conditions between training and inference. Comparatively, for the model-ID-based LCM, the consistency between training and inference can be easily ensured via model ID after the mode identification process.

Due to the limitation of a model’s generalization ability, a model’s performance may face heavy degradation when there is mismatch between the model training environment and model deploying environment. Unfortunately, the environment in wireless communication is usually dynamic, e.g., changing scenarios/sites as the UE is moving. To overcome this issue, operations such as model switching are needed to ensure the most suitable model under specific scenario/site can be deployed. Assuming that there is one control entity that perform the management, and a model deployment entity which runs the model, there are three major steps to address the dynamic changing environment/additional conditions problem under the functionality-based LCM background. In the following example, we assume that UE is the model deployment entity and NW is the control entity such that they follow the 3GPP network interaction manner that UE behaves following the configuration of NW.

Step 1. The model deployment entity needs to be informed of the changing of the environment/additional conditions. In the case when UE is the model deployment entity, the NW-side additional conditions e.g., current cell information, SNR are provided to UE.

Step 2. The model deployment entity reports the applicable AI/ML capability under the specific environment.

Legacy UE capability reporting is used only for reporting static applicable conditions or RRC configurations. If a legacy-like UE capability report is used for functionality-based LCM, it may not handle dynamic scenarios/sites problem well. For example, in functionality-based LCM, a UE may only support AI/ML feature X or configuration X in site A but not in site B. Under this situation, the legacy-like UE capability reporting that the UE supports AI/ML feature X or configuration X may lead to serious problem when UE is in site B. Therefore, UE may need to dynamically report its applicable capability (e.g., UE supports AI/ML feature Y in site B) for supporting the functionality under certain scenarios/sites instead of legacy-like static UE capability report.

Step 3. The control entity further config the model deployment entity based on the above applicable capability. In this step, even if the control entity (NW) may not be aware of the model details at UE side, it can config the UE using traditional configuration mechanism such as RRC. After receiving the RRC configuration, UE as the model deployment entity can select the model which is best suitable to the RRC config, or to the current environment.

Figure 2-3  Relation between UE capability, conditions, additional conditions, and a model’s information

As discussed above, by providing information and/or indication on NW-side additional conditions to UE (Step 1), consistency between training and inference regarding NW-side additional conditions is ensured such that UE can select the best suitable UE-side models for model inference/deployment without NW being aware of the detail model information. In this manner, NW can achieve the control of UE behavior reusing the existing configuration methods.

Regarding specification impact, RAN1 and other WGs can start by discussing aspects which fall within or outside UE capabilities, or by firstly identifying the NW-side additional conditions for positioning. Then further specify the procedure how NW-side additional conditions are provided to UE.

To explore methods that ensure consistency between training and inference, RAN1 may need to firstly identify the NW-side additional conditions regarding positioning.

UE-side additional conditions

In our understanding, UE-side additional conditions can be referred to UE-side information which is not part of the UE capability. We believe that by providing UE-side information especially model information can be beneficial for the NW side control of UE behaviour to achieve better positioning performance. Therefore, we support that UE-side model information can be transferred to LMF by LPP protocol.

For Case 1, support the transfer of UE-side model information to LMF via LPP.

Conclusions

Observation 1.	Model inference for Case 3b can almost reuse the existing positioning procedures except that new measurements such as CIR/PDP may need to be transferred from gNBs to LMF to support AI/ML-based positioning.

Observation 2.	For data-driven AI/ML based positioning, the training data collection procedure may need to redesign such that it can meet various data collection requirements which is different from the existing positioning procedure.

Observation 3.	For Case 3b data collection for model training, due to different gNB capabilities and other factors, gNBs may provide different number of measurement types of the data that can only be aligned after further processing at NW side to form a dataset, which will cause heavy waste of the bandwidth transferring data that cannot be used, as well as extra NW-side computation resources when organizing the dataset.

Proposal 1.	For Case 3b model inference, new measurements (if necessary) such as CIR/PDP may need to be specified to support AI/ML positioning while existing positioning procedures can be reused.

Proposal 2.	For Case 3b data collection for model training, new measurements (if necessary) such as CIR/PDP may need to be specified same as Case 3b model inferencing.

Proposal 3.	For Case 3b data collection for model training, the LMF should indicate its desired SRS configuration in data collection request, and the serving gNB should provide the configuration result to LMF in the response message.

Proposal 4.	For Case 3b data collection for model training, a measurement request indicating the desired measurements for data collection from LMF, and a measurement response indicating the supported measurements by gNBs can be beneficial to better support data collction.

Proposal 5.	No specification impact is expected for Case 3b model monitoring since monitoring can be deployed within the NW and based on implementation.

Proposal 6.	RAN1 can discuss whether normative effort is needed for new measurements for Case 3a model inference.

Proposal 7.	For Case 3a model training, support to explore the feasibility of training models at gNB side as the primary focus of the normative work. Consider discussing how labels are provided to gNB by NW entity (e.g., LMF) as starting point.

Proposal 8.	Discussion on model monitoring for Case 3a can begin after topics such as new measurements for 1st priority sub-use cases have been fully addressed. This is because monitoring may not have a significant impact on the specifications for Case 3a.

Proposal 9.	There is no need to specify new measurements for Case 1 model inference.

Proposal 10.	The specification impact for Case 1 data collection for model training depends on the study part of this WI.

Proposal 11.	No specification impact is expected for Case 1 model monitoring if UE doesn’t have to report the monitoring result to NW.

Proposal 12.	To explore methods that ensure consistency between training and inference, RAN1 may need to firstly identify the NW-side additional conditions regarding positioning.

Proposal 13.	For Case 1, support the transfer of UE-side model information to LMF via LPP.

- References

[1] RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface RAN#102 December 2023.