3GPP TSG RAN WG1 #116	R1-2401092

Athens, Greece, February 26th – March 1st, 2024

Source:	NTT DOCOMO, INC.

Title:	Maintenance of NR SL evolution

Agenda Item:	8.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#115 meeting [1], there were discussion and agreements for maintenance on SL evolution. In this contribution, we share our further views on maintenance of R18 SL evolution.

- Discussion

  - SL-U channel access

    - SL HARQ feedback on UL in mode 1 RA

      - R16 SL discussion in maintenance phase

16.5 UE procedure for reporting HARQ-ACK on uplink … The UE generates a NACK when, due to prioritization, as described in clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 and the UE transmitted PSSCH in the resource or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information and the UE transmitted PSSCH in the resource. The priority value of the NACK is same as the priority value of the PSSCH transmission. The UE generates a NACK when, due to prioritization as described in clause 16.2.4, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to prioritization. For operation with shared spectrum channel access, the UE generates a NACK when, due to a failed channel access procedure [15, TS 37.213], the UE does not transmit a PSSCH with a single TB in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to the failed channel access procedure. …

At the previous meeting, the above correction was agreed for R16 SL (see R1-2310662/R1-2310663/R1-2310664). The intention is that NACK is reported when some resources are not used for PSSCH due to prioritization, the other resources are used for PSSCH, but any corresponding PSFCH is not received due to prioritization. Such a mixture case was not covered in the previous version of the spec while it is clear that the case should be included for the NACK report behavior, and thus the correction was introduced for clarification.

      - R18 SL-U discussion for mode 1

16.5 UE procedure for reporting HARQ-ACK on uplink … For operation with shared spectrum channel access, the UE generates a NACK when, due to a failed channel access procedure [15, TS 37.213], the UE does not transmit a PSSCH with a single TB in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to the failed channel access procedure. …

Now we have another paragraph for NACK report. In SL-U, when all scheduled resources are not used for PSSCH due to LBT failure, NACK is reported to gNB. This is specified as above. However, we believe that at least the following illustrated situation is not included in the current specification. Here, PSSCH transmissions on some resources are failed due to prioritization and PSSCH transmissions on the remaining resources are failed due to LBT failure. In this case, NACK should be reported to gNB.

Fig.1: Missed case for NACK report.

Proposal 1:

- For SL-U UE operates in Mode 1 resource allocation, when UE uses M PSSCH resource(s) provided by a DCI format 3_0 or, for a configured grant for a single TB,

  - NACK is reported when the UE does not use M1 resource(s) due to LBT failure, and the UE does not use (M − M1) resource(s) due to prioritization.

- Adopt the following TP for TS 38.213.

| Reason for change: | With respect to LBT failure for resources scheduled by gNB, NACK is reported only when all resources are involved with LBT failure. UE behavior has not been defined if some resources are not used due to LBT failure and the other resources are not used due to prioritization. |
| --- | --- |
| Summary of change: | NACK is reported to gNB if some resources scheduled by gNB are not used due to LBT failure and the other resources are not used due to prioritization. |
| Consequences if not approved: | What information should be reported in the above case is not unclear. |

16.5 UE procedure for reporting HARQ-ACK on uplink <Unchanged parts omitted> The UE generates a NACK when, due to prioritization, as described in clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 and the UE transmitted PSSCH in the resource or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information and the UE transmitted PSSCH in the resource. The priority value of the NACK is same as the priority value of the PSSCH transmission. The UE generates a NACK when, due to prioritization as described in clause 16.2.4, or due to a failed channel access procedure [15, TS 37.213] for operation with shared spectrum channel access, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to prioritization or due to the failed channel access procedure. For operation with shared spectrum channel access, the UE generates a NACK when, due to a failed channel access procedure [15, TS 37.213], the UE does not transmit a PSSCH with a single TB in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to the failed channel access procedure. <Unchanged parts omitted>

    - Resource allocation

      - Unavailable resources in SA

Basically, the existing mode 2 RA is reused for SL-U, but the mechanism may not align with channel access regulation. At least we found the following issue: starting timing of LBT for some resources in SA is earlier than resource selection timing.

In current mode 2 RA, resource selection is triggered at slot n and one or more resources are selected randomly from a window [n+T1, n+T2] after resource exclusion behavior based on received reservation information. However, a selected resource may not satisfy required LBT-sensing duration. For example, a resource at slot n+T1 can be selected by the random selection from an identified set based on sensing. Meanwhile, LBT duration for the TX, which is determined based on the including data, previous HARQ results, etc., may be larger than T1 as CWmax,p = 1023 and thereby the corresponding max LBT duration is 9.247 ms. This means, UE shall start LBT before the resource selection timing. This would be impossible for aperiodic transmissions; otherwise, UE shall perform LBT in any slot in preparation for potential aperiodic transmission. This issue is illustrated below. In our view, such an issue is almost the same as what we discussed for inter-UE blocking at the previous RAN1 meetings.

Fig.2: LBT starting timing prior to resource selection timing

Observation 1:

- There is a case where LBT starting timing for transmission at a resource selected at slot n is prior to slot n, e.g., max LBT duration with CWp = CWmax,p is 9.247 ms. Performing LBT before resource selection trigger is not desirable especially for aperiodic traffic.

To solve this issue, at least the following options should be considered. Our view is that Alt 1 would be a more reasonable way while Alt 2 would be controversial.

- Alt 1: For resource selection at slot n for a TX, LBT duration is determined before resource selection and then resource is selected such that the LBT-sensing starting timing for the TX at the selected resource is later than slot n

- Alt 2: Resource is selected firstly and then LBT duration is adjusted based on timing of the selected resource

Proposal 2:

- Support the following for Type 1 LBT.

  - For resource selection at slot n for a TX, LBT duration is determined before resource selection and then resource is selected such that the LBT-sensing starting timing for the TX at the selected resource is later than slot n.

    - The corresponding resources are excluded in MAC layer.

  - Send an LS to inform RAN2 of this mechanism.

      - Inter-UE blocking, Option 1 – Definition in MCSt case

In Option 1 for inter-UE blocking, N consecutive resource(s) and M consecutive resource(s) are excluded in MAC layer. For single-slot resource, naturally N consecutive resource(s) and M consecutive resource(s) mean resources of N consecutive slot(s) and M consecutive slot(s), respectively.

However, definition of N consecutive resource(s) and M consecutive resource(s) is unclear for MCSt case. When MCSt is applied, each resource is defined as multi-slot resource. Then e.g., if N_slot,MCSt = 2, whether 1) N = 2 means resources with 4 slots or 2) still resources with 2 slots is unclear. Example with N = 2, M = 4, and N_slot,MCSt = 2 is illustrated below. At the first one, N = 2 consecutive resource(s) and M = 4 consecutive resource(s) are resources of N = 2 consecutive slot(s) and M = 4 consecutive slot(s), regardless of whether the UE applies MCSt or not. At the second one, N = 2 consecutive resource(s) and M = 4 consecutive resource(s) are resources of 4 consecutive slot(s) and M = 8 consecutive slot(s), when the UE performs resource selection based on MCSt.

Fig.3: Ambiguity of N consecutive resource(s) and M consecutive resource(s) in MCSt case.

In our understanding, intention of exclusion of N consecutive resource(s) and M consecutive resource(s) is to avoid inter UE blocking due to type 1 LBT, which implies that the exclusion target (duration) is not relevant to MCSt. That is, exclusion duration for N consecutive resource(s) and M consecutive resource(s) should not be dependent on whether MCSt is used or not.

Proposal 3:

- Clarify that N consecutive resource(s) and M consecutive resource(s) in Option 1 for inter-UE blocking are referred to those in case of single-slot resource.

  - Send an LS to inform RAN2 of this clarification.

      - Inter-UE blocking, Option 2 – MCOT consideration

Another issue of Option 2 is that UE-X does not consider the number of UE-Y’s consecutive transmissions duration. If one or more of UE-Y’s transmissions is/are outside of the shared COT due to MCOT restriction as illustrated below, such a resource should not preferentially be selected by Option 2; otherwise, undesirable situation of transmission drop may occur more frequently than in a RP without Option 2. The additional restriction to apply Option 2 is essential not to unnecessary degrade UE-Y’s performance.

Fig.4: Issue on MCOT in Option 2.

Proposal 4:

- When UE-X detects UE-Y’s reservations for contiguous slots, UE-X can apply Option 2 for the UE-Y’s first reserved resource only when UE-Y’s all transmissions at the contiguous slots are included in the shared COT.

  - Send an LS to inform RAN2 of this mechanism.

    - Multi-channel access

      - Assumed scenarios

Although multi-channel access for SL channels was agreed, which is based on NR-U channel access procedure, the NR-U spec seems not to consider consecutive transmissions with different number of RB-sets by a single UE and transmissions in COT sharing case; whether type-1 or type-2 is applied for each channel is not relevant to whether COT has been obtained for each channel or not. In other words, even when COT has already been initiated/shared for a channel, type 1 LBT may be performed in some RB sets of a wideband operation. The following illustrated two cases would be typical situations.

Fig.5: Possible cases for multi-channel access
(Left: consecutive TXs with different no. of RB-sets by a single UE; Right: TXs in COT sharing case)

In NR-U, it may be assumed that gNB scheduler can handle efficient COT sharing with wideband operation to avoid such cases. However, this is not the case for SL system (mode 2 RA). In mode 2 RA, there is no scheduler. Then there is a case where for example, UE-A initiates a COT at a single RB set (called RB set X) for a transmission only within RB set X, the COT is shared to UE-B, and UE-B performs at a later slot a transmission across multiple RB sets including RB set X. UE-B can use the COT for RB set X by using type 2 LBT, but the existing mechanism seems to be not ‘always type 2 LBT’ for RB set X if the transmission is wideband operation. Then type 1 LBT may be applied for RB set X and as a result LBT failure could occur.

Observation 2:

- It seems that the existing multi-channel access does not cover the case where there has already been COT at a part of RB-sets.

Proposal 5:

- Discuss how to perform multi-channel access procedure in the following case.

  - Case: A UE performs a transmission in N1 RB sets where COT has already been initiated/shared in N2 (<N1) RB sets and not initiated/shared in N1-N2 RB-sets

      - Transmission with partial COT initiation/sharing

To avoid such an undesirable situation, we believe that how to perform LBT at each channel for wideband operation should be modified from that for NR-U DL/UL. The existing type-1 / type-2 determination for wideband operation is applied to each channel where there has not been initiated/shared COT, and type 2 LBT defined for COT sharing in single channel access is used for each channel where COT has been initiated/shared. This operation is illustrated below.

Fig.6: How to perform LBT at each channel
(Left: Current spec; Right: Modified procedure)

Proposal 6:

- For multi-channel access, support LBT type determination based on whether COT has been initiated/shared for each RB-set.

  - Type 2X LBT is performed at the already-initiated/shared N2 RB sets and the existing multi-channel access is performed at the non-initiated/shared N1-N2 RB-sets

  - Note

    - Case: A UE performs a transmission in N1 RB sets where COT has already been initiated/shared in N2 (<N1) RB sets and not initiated/shared in N1-N2 RB-sets

    - CW adjustment

      - When feedback is disabled

Agreement If UE performs SL transmission using Type 1 channel access procedures associated with the channel access priority class p on a channel and the SL transmission is not associated with explicit HARQ-ACK feedback by the corresponding UE(s), the following is adopted for the CW adjustment. - For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - If the same CWp≠CWmax,p value is consecutively used for X times for generation of Ninit, CWp is updated for every priority class p∈1,2,3,4 to the next higher allowed value. - FFS: whether this only applies to a resource pool without PSFCH configuration - FFS: value of X Agreement For the CWp autonomous update to the next higher allowed value when the same CWp≠CWmax,p value is consecutively used for X times for generation of Ninit, - The (pre-)configuration provides 1 value for X among a value range of {1, 8, 16, 32, ‘infinity’}. - This operation is restricted only to PSCCH/PSSCH transmission with HARQ feedback indicator in SCI-2 is set to disabled, regardless of PSFCH resources being configured in a resource pool.

At the last meeting, X value was introduced to avoid using small value for transmissions without HARQ feedback, which has negative impact to other systems. For this X-related behavior, it is not reasonable that UE shall use CWmax,p semi-permanently after several times of CWp increment. Some reset mechanism would be necessary.

On reset mechanism, CWp is reset after consecutively using the max value K times. K value is up to UE implementation from {1…8}. However, we do not think this reset behavior is reasonable for SL transmissions without HARQ feedback. The X value was agreed for fairness among systems. If the reset behavior is applicable after (not X but) only K=1 transmission, it would not be fair to other systems. Note that the existing reset mechanism is a copy from NR-U and thus not intended for transmissions without HARQ feedback. Our proposal is to use value X for reset behavior as well, rather than value K determined by UE implementation. The fairness is guaranteed in this rule.

Proposal 7:

- For CW adjustment when SL-HARQ feedback is disabled,

  - If the same CWp=CWmax,p value is consecutively used (not K but) X times for generation of Ninit, CWp is reset to CWmin,p.

- Adopt the following TP for TS 37.213.

| Reason for change: | For CW adjustment when SL-HARQ feedback is disabled, CWp increment is accumulatively applied after X times transmission. X is (pre-)configured. However, when CWp is the max value, CWp is initialized to the mix value after K times transmission. K is up to UE implementation. The (pre-)configuration of X is introduced for fairness to other system, but this K is not aligned with the intention. |
| --- | --- |
| Summary of change: | When CWp is the max value, CWp is initialized to the mix value after X times transmission. |
| Consequences if not approved: | Fairness b/w 3GPP sidelink and other system by (pre-)configuration of X is achieved due to CWp initialization by K value that is selected by UE implementation. |

### 4.5.4 Contention window adjustment procedures for SL transmissions <Unchanged parts omitted> If a UE transmits a SL transmission(s) using Type 1 channel access procedures associated with the channel access priority class p on a channel and the SL transmission(s) is not associated with explicit HARQ-ACK feedback(s) by the corresponding UE(s), the UE adjusts CWp before step 1 in the procedures described in clause 4.5.1, using the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. If the corresponding channel access priority class p has not been used for any SL transmissions on the channel, CWp=CWmin,p is used. If the latest CWp≠CWmax,p value is consecutively used for X times provided by higher layers parameter [sl-CWSforPsschWithoutHarqAck] for generation of Ninit as described in clause 4.5.1 for PSSCH transmission(s) without associated explicit HARQ-ACK feedback(s), the CWp is increased for every priority class p∈1,2,3,4 to the next higher allowed value. The following applies to the procedures described in this clause for contention window adjustment: - If CWp=CWmax,p, the next higher allowed value for adjusting CWp is CWmax,p. - If the CWp=CWmax,p is consecutively used K times for generation of Ninit, CWp is reset to CWmin,p only for that priority class p for which CWp=CWmax,p is consecutively used K times for generation of Ninit as described in clause 4.5.1 for SL transmission(s). K=X if the SL transmission(s) includes only PSSCH(s) without associated explicit HARQ-ACK feedback(s); otherwise, K is selected by UE from the set of values {1, 2, …,8} for each priority class p∈1,2,3,4. <Unchanged parts omitted>

  - SL-U channel design

    - PSCCH/PSSCH

      - RX UE behavior for the 2nd starting symbol

Although TX UE behavior has been specified in 8.1.2.1 of 38.214, RX UE behavior has not been clearly decided yet for two candidate starting symbols. In SL system, TX UE’s channel condition is different among UEs due to hidden-node issue and thus from RX UE perspective, it is feasible to find TX from the 2nd starting symbol in any slot without PSFCH. That is, RX UE behavior should not be different based on COT condition.

Proposal 8:

- RX UE monitors TX from both the 1st starting symbol and the 2nd starting symbol in any slot with two starting symbols.

  - Adopt the following TP for TS 38.214.

| Reason for change: | RX UE behavior has not been clearly specified for two candidate starting symbols. |
| --- | --- |
| Summary of change: | RX UE shall attempt to decode from the 2nd starting symbol in any slot without PSFCH, if configured. |
| Consequences if not approved: | RX UE may not perform receptions from the 2nd starting symbol. PSSCH TX becomes meaningless, and reservation signal in PSCCH transmitted with PSSCH starting from the 2nd starting symbol is not detected by other UEs. |

## 8.3 UE procedure for receiving the physical sidelink shared channel For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. A UE is required to decode neither the corresponding SCI formats 2-A, 2-B, 2-C nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support. In any slot without PSFCH symbols, the UE attempts to decode PSSCH transmission starting from the second candidate starting symbol provided by sl-startingSymbolSecond, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided.

      - Processing time constraints

When TX is started from the 2nd starting symbol, timing of completing decoding of each channel would change. For example, the ending timing of PSSCH decoding starting from the 2nd starting symbol may be later compared to that starting from the 1st starting symbol. Correspondingly, PSCCH/PSSCH to PSFCH slot offset should be larger than the existing sl-MinTimeGapPSFCH values. Besides, PSCCH location becomes later for TX starting from the 2nd starting symbol and then the ending timing of PSCCH decoding will be later. Timing of obtaining reservation information becomes later correspondingly; hence the ending timing of sensing window should be modified so that excessively small processing time is not required.

Fig.7: Issue - existing processing time constraints with 2nd starting symbol
(upper: PSSCH to PSFCH, lower: sensing to exclusion/selection)

Proposal 9:

- Update the following processing time constraints.

  - PSCCH/PSSCH to PSFCH slot offset is sl-MinTimeGapPSFCH + 1.

  - The last PSCCH monitoring occasion for a resource selection is the first PSCCH occasion at the last slot within the sensing window.

  - Adopt the following TPs for TS 38.213 and TS 38.214.

| Reason for change: | UE processing time in the current specification is insufficient for PSCCH/PSSCH reception starting from the 2nd starting symbol. |
| --- | --- |
| Summary of change: | For PSSCH decoding and PSFCH TX preparation, sl-MinTimeGapPSFCH is replaced to sl-MinTimeGapPSFCH + 1. For PSCCH decoding for UE autonomous resource allocation, PSCCH/PSSCH starting from the 2nd starting symbol at the last slot within a sensing window is excluded from the sensing target for resource allocation behavior. |
| Consequences if not approved: | Requirements of UE processing time are severer for feature of the 2nd starting symbol, which leads to undesirable UE complexity or increase of UEs not supporting this feature. |

38.213 ### 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged parts omitted> If a UE receives a PSSCH in a resource pool and the HARQ feedback enabled/disabled indicator field in an associated SCI format 2-A/2-B/2-C has value 1 [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. For operation without shared spectrum channel access, the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception. For operation with shared spectrum channel access, the UE can attempt to transmit the PSFCH over a number of first NoccasionPSFCH slots, provided by sl-candidatePSFCH-Occasions and indexed from 1 to NoccasionPSFCH in ascending order in time, that include PSFCH resources and are at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception. For operation with shared spectrum channel access, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided, sl-MinTimeGapPSFCH is replaced to sl-MinTimeGapPSFCH + 1. The UE attempts to transmit PSFCH in a slot only when the UE fails to transmit PSFCH associated with the PSSCH in all previous slots for PSFCH within the NoccasionPSFCH slots. <Unchanged parts omitted>

38.214 ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur, and except for the second PSCCH occasion in transmission starting from the second candidate starting symbol at the last slot within the sensing window, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7, except for the second PSCCH occasion in transmission starting from the second candidate starting symbol at the last slot earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ), if sl-startingSymbolFirst and sl-startingSymbolSecond are provided. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. The value of Preserve corresponds to sl-PBPS-OccasionReservePeriodList if (pre-)configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList. The UE monitors sensing occasion(s) determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS. When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB], except for the second PSCCH occasion in transmission starting from the second candidate starting symbol at the slot n+TB, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB], except for the second PSCCH occasion in transmission starting from the second candidate starting symbol at the slot n+TB, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when PrsvpTX=0, it is up to UE implementation to either continue with step 3) or perform random selection. Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter sl-PartialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given periodicity corresponding to Preserve, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots. <Unchanged parts omitted>

    - PSFCH

      - PSFCH prioritization order

Agreement When UE intends to transmit PSFCH, after performing PSFCH prioritization: - if LBT fails on all RB set(s) where the UE attempts to transmit PSFCH: - UE drops PSFCH transmission - No RAN1 specification impact - if TypeA and TypeB LBT fails on part of RB set(s) where the UE attempts to transmit PSFCH: - The UE may transmit PSFCH on RB set(s) where LBT was successful Agreement When a UE does not support PSFCH transmission over non-contiguous RB sets, the following belongs to PSFCH prioritization: - UE selects contiguous RB set(s) including PSFCH(s) with smallest SL priority value. If there are more than one contiguous RB set(s) including PSFCH(s) with the same smallest priority value, it is up to UE implementation to select one contiguous RB set among them.

Although the above agreements were reached at the last meeting, i.e., step 1) PSFCH prioritization; step 2) PSFCH selection based on LBT results, detailed order of PSFCH prioritization is unclear yet. PSFCH prioritization includes the following two types:

- X: Legacy prioritization. Prioritization among PSFCH transmissions to select NTx,PSFCH PSFCHs as defined in 16.2.4.2 of 38.213, and prioritization b/w PSFCH transmissions and UL transmissions as defined in 16.2.4.3.1 of 38.213.

- Y: The new prioritization. Prioritization of PSFCH transmissions to avoid transmissions over non-contiguous RB sets as defined in 16.2.4.2.

Which is applied earlier should be clarified since the outcome is different. The illustration below is an example situation. For the solution, our view is that X should be performed first. Although it may lead to unnecessary drop, repeat of Y is not required. If Y is applied first, one more Y behavior may be necessary after X prioritization. Simpler UE behavior would be better.

Fig.8: Application order of PSFCH prioritizations

Proposal 10:

- PSFCH prioritization for UE being incapable of PSFCH transmission in non-contiguous RB sets is applied after R16/17 PSFCH prioritization.

  - Adopt the following TP for TS 38.213.

| Reason for change: | Application order b/w R16/17 PSFCH prioritization and R18 PSFCH prioritization with respect to non-contiguous RB sets has not been decided yet. Prioritization outcome is dependent on the order. |
| --- | --- |
| Summary of change: | R18 PSFCH prioritization with respect to non-contiguous RB sets is applied after R16/17 PSFCH prioritization. |
| Consequences if not approved: | UE behavior of PSFCH prioritization is unclear in some cases. |

#### 16.2.4.2 Simultaneous PSFCH transmission/reception <Unchanged parts omitted> For operation with shared spectrum channel access, if a UE does not support PSFCH transmission in non-contiguous RB sets, the UE selects for PSFCH transmission any contiguous RB set(s) that include PSFCH with the smallest priority value. This PSFCH prioritization is applied after PSFCH prioritization for NTx,PSFCH PSFCHs determination described in this clause or prioritizations for sidelink and uplink transmissions in clause 16.2.4.3.1. <Unchanged parts omitted>

      - PSFCH prioritization for a single RB set

Discussion in the last section is done on top of assumption that UE is capable of multi-channel access. However, multi-channel access is an optional feature (FG 47-k2). Some UEs may not support channel access for PSFCH transmissions in multiple RB sets, but PSFCH transmissions in multiple RB sets at the same occasion could occur at the UEs. How to handle this case should also be discussed and specified.

The solution can be simple: when the situation occurs, the UE selects a single RB set including PSFCH with smallest priority value. We do not see reason to consider/adopt any other way.

Proposal 11:

- When PSFCH transmissions in multiple RB sets at the same PSFCH occasion occur at a UE, and if the UE is incapable of PSFCH transmission in multiple RB sets,

  - The UE selects a single RB set including PSFCH with smallest priority value.

- Adopt the following TP for TS 38.213.

| Reason for change: | UE may not support PSFCH transmissions in multiple RB sets at the same PSFCH occasion, but corresponding UE behavior has not been specified yet. |
| --- | --- |
| Summary of change: | A single RB set including PSFCH with smallest priority value is selected for transmission. |
| Consequences if not approved: | UE behavior of PSFCH prioritization is unclear in some cases. |

#### 16.2.4.2 Simultaneous PSFCH transmission/reception <Unchanged parts omitted> For operation with shared spectrum channel access, if a UE does not support PSFCH transmission in non-contiguous RB sets, the UE selects for PSFCH transmission any contiguous RB set(s) that include PSFCH with the smallest priority value. For operation with shared spectrum channel access, if a UE does not support multi-channel access for PSFCH transmission in multiple RB sets, the UE selects for PSFCH transmission a single RB set that includes PSFCH with the smallest priority value. <Unchanged parts omitted>

    - S-SSB

      - Condition of S-SSB transmission in a non-anchor RB set

Although a S-SSB TX can be mapped across multiple RB-sets, conditions to perform S-SSB TX in a non-anchor RB-set have not been clarified yet. In our view, UE should not perform S-SSB TX in a non-anchor RB-set when there is no necessity to obtain/keep COT in the RB-set. For example, a UE has no TX data provided from higher layer. In this case, what the UE should do is to transmit S-SSB in anchor RB-set only.

Proposal 12:

- For a S-SSB TX in multiple RB-sets, UE transmits S-SSB in each non-anchor RB-set only when one or more of the following are met.

  - A: there is an on-going COT in the RB-set that is available by the UE

  - B: the UE attempts to obtain a COT in the RB-set for its own transmission

- Adopt the following TP for TS 38.213.

| Reason for change: | There is no rule to transmit S-SSB in non-anchor RB set(s). RAN1 should specify some rule to avoid negative impact to other system. |
| --- | --- |
| Summary of change: | S-SSB transmissions in non-anchor RB set(s) are allowed only when conditions are met. |
| Consequences if not approved: | UE can transmit S-SSB in non-anchor RB set(s) on any occasions, which have negative impacts to other system. |

## 16.1 Synchronization procedures <Unchanged parts omitted> For operation with shared spectrum channel access, a UE attempts to transmit at least S-SS/PSBCH blocks in the slots including S-SS/PSBCH blocks in the anchor RB set. A UE can attempt to transmit S-SS/PSBCH blocks in non-anchor RB set(s) only when there has been a COT in the RB set that is available by the UE or the UE attempts to initiate a COT in the RB set for its own transmission at the next slot of the S-SS/PSBCH transmission occasion. The UE applies CP extension to the first symbol of an S-SS/PSBCH block and within the first one or two symbols before the first symbol of the S-SS/PSBCH block according to an index [4, TS 38.211] provided by sl-CP-Extension-SSB. The UE assumes PRB(s) in an intra-cell guard band [6, TS 38.214] are not used for transmission of S-SS/PSBCH blocks. <Unchanged parts omitted>

- Conclusion

In this contribution, we discussed R18 SL maintenance. Proposals are summarized as following:

Proposal 1:

- For SL-U UE operates in Mode 1 resource allocation, when UE uses M PSSCH resource(s) provided by a DCI format 3_0 or, for a configured grant for a single TB,

  - NACK is reported when the UE does not use M1 resource(s) due to LBT failure, and the UE does not use (M − M1) resource(s) due to prioritization.

- Adopt the following TP for TS 38.213.

| Reason for change: | With respect to LBT failure for resources scheduled by gNB, NACK is reported only when all resources are involved with LBT failure. UE behavior has not been defined if some resources are not used due to LBT failure and the other resources are not used due to prioritization. |
| --- | --- |
| Summary of change: | NACK is reported to gNB if some resources scheduled by gNB are not used due to LBT failure and the other resources are not used due to prioritization. |
| Consequences if not approved: | What information should be reported in the above case is not unclear. |

16.5 UE procedure for reporting HARQ-ACK on uplink <Unchanged parts omitted> The UE generates a NACK when, due to prioritization, as described in clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 and the UE transmitted PSSCH in the resource or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information and the UE transmitted PSSCH in the resource. The priority value of the NACK is same as the priority value of the PSSCH transmission. The UE generates a NACK when, due to prioritization as described in clause 16.2.4, or due to a failed channel access procedure [15, TS 37.213] for operation with shared spectrum channel access, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to prioritization or due to the failed channel access procedure. For operation with shared spectrum channel access, the UE generates a NACK when, due to a failed channel access procedure [15, TS 37.213], the UE does not transmit a PSSCH with a single TB in any of the resources provided by a DCI format 3_0 or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The priority value of the NACK is same as the priority value of the PSSCH that was not transmitted due to the failed channel access procedure. <Unchanged parts omitted>

Observation 1:

- There is a case where LBT starting timing for transmission at a resource selected at slot n is prior to slot n, e.g., max LBT duration with CWp = CWmax,p is 9.247 ms. Performing LBT before resource selection trigger is not desirable especially for aperiodic traffic.

Proposal 2:

- Support the following for Type 1 LBT.

  - For resource selection at slot n for a TX, LBT duration is determined before resource selection and then resource is selected such that the LBT-sensing starting timing for the TX at the selected resource is later than slot n.

    - The corresponding resources are excluded in MAC layer.

  - Send an LS to inform RAN2 of this mechanism.

Proposal 3:

- Clarify that N consecutive resource(s) and M consecutive resource(s) in Option 1 for inter-UE blocking are referred to those in case of single-slot resource.

  - Send an LS to inform RAN2 of this clarification.

Proposal 4:

- When UE-X detects UE-Y’s reservations for contiguous slots, UE-X can apply Option 2 for the UE-Y’s first reserved resource only when UE-Y’s all transmissions at the contiguous slots are included in the shared COT.

  - Send an LS to inform RAN2 of this mechanism.

Observation 2:

- It seems that the existing multi-channel access does not cover the case where there has already been COT at a part of RB-sets.

Proposal 5:

- Discuss how to perform multi-channel access procedure in the following case.

  - Case: A UE performs a transmission in N1 RB sets where COT has already been initiated/shared in N2 (<N1) RB sets and not initiated/shared in N1-N2 RB-sets

Proposal 6:

- For multi-channel access, support LBT type determination based on whether COT has been initiated/shared for each RB-set.

  - Type 2X LBT is performed at the already-initiated/shared N2 RB sets and the existing multi-channel access is performed at the non-initiated/shared N1-N2 RB-sets

  - Note

    - Case: A UE performs a transmission in N1 RB sets where COT has already been initiated/shared in N2 (<N1) RB sets and not initiated/shared in N1-N2 RB-sets

Proposal 7:

- For CW adjustment when SL-HARQ feedback is disabled,

  - If the same CWp=CWmax,p value is consecutively used (not K but) X times for generation of Ninit, CWp is reset to CWmin,p.

- Adopt the following TP for TS 37.213.

| Reason for change: | For CW adjustment when SL-HARQ feedback is disabled, CWp increment is accumulatively applied after X times transmission. X is (pre-)configured. However, when CWp is the max value, CWp is initialized to the mix value after K times transmission. K is up to UE implementation. The (pre-)configuration of X is introduced for fairness to other system, but this K is not aligned with the intention. |
| --- | --- |
| Summary of change: | When CWp is the max value, CWp is initialized to the mix value after X times transmission. |
| Consequences if not approved: | Fairness b/w 3GPP sidelink and other system by (pre-)configuration of X is achieved due to CWp initialization by K value that is selected by UE implementation. |

### 4.5.4 Contention window adjustment procedures for SL transmissions <Unchanged parts omitted> If a UE transmits a SL transmission(s) using Type 1 channel access procedures associated with the channel access priority class p on a channel and the SL transmission(s) is not associated with explicit HARQ-ACK feedback(s) by the corresponding UE(s), the UE adjusts CWp before step 1 in the procedures described in clause 4.5.1, using the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. If the corresponding channel access priority class p has not been used for any SL transmissions on the channel, CWp=CWmin,p is used. If the latest CWp≠CWmax,p value is consecutively used for X times provided by higher layers parameter [sl-CWSforPsschWithoutHarqAck] for generation of Ninit as described in clause 4.5.1 for PSSCH transmission(s) without associated explicit HARQ-ACK feedback(s), the CWp is increased for every priority class p∈1,2,3,4 to the next higher allowed value. The following applies to the procedures described in this clause for contention window adjustment: - If CWp=CWmax,p, the next higher allowed value for adjusting CWp is CWmax,p. - If the CWp=CWmax,p is consecutively used K times for generation of Ninit, CWp is reset to CWmin,p only for that priority class p for which CWp=CWmax,p is consecutively used K times for generation of Ninit as described in clause 4.5.1 for SL transmission(s). K=X if the SL transmission(s) includes only PSSCH(s) without associated explicit HARQ-ACK feedback(s); otherwise, K is selected by UE from the set of values {1, 2, …,8} for each priority class p∈1,2,3,4. <Unchanged parts omitted>

Proposal 8:

- RX UE monitors TX from both the 1st starting symbol and the 2nd starting symbol in any slot with two starting symbols.

  - Adopt the following TP for TS 38.214.

| Reason for change: | RX UE behavior has not been clearly specified for two candidate starting symbols. |
| --- | --- |
| Summary of change: | RX UE shall attempt to decode from the 2nd starting symbol in any slot without PSFCH, if configured. |
| Consequences if not approved: | RX UE may not perform receptions from the 2nd starting symbol. PSSCH TX becomes meaningless, and reservation signal in PSCCH transmitted with PSSCH starting from the 2nd starting symbol is not detected by other UEs. |

## 8.3 UE procedure for receiving the physical sidelink shared channel For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. A UE is required to decode neither the corresponding SCI formats 2-A, 2-B, 2-C nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support. In any slot without PSFCH symbols, the UE attempts to decode PSSCH transmission starting from the second candidate starting symbol provided by sl-startingSymbolSecond, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided.

Proposal 9:

- Update the following processing time constraints.

  - PSCCH/PSSCH to PSFCH slot offset is sl-MinTimeGapPSFCH + 1.

  - The last PSCCH monitoring occasion for a resource selection is the first PSCCH occasion at the last slot within the sensing window.

  - Adopt the following TPs for TS 38.213 and TS 38.214.

| Reason for change: | UE processing time in the current specification is insufficient for PSCCH/PSSCH reception starting from the 2nd starting symbol. |
| --- | --- |
| Summary of change: | For PSSCH decoding and PSFCH TX preparation, sl-MinTimeGapPSFCH is replaced to sl-MinTimeGapPSFCH + 1. For PSCCH decoding for UE autonomous resource allocation, PSCCH/PSSCH starting from the 2nd starting symbol at the last slot within a sensing window is excluded from the sensing target for resource allocation behavior. |
| Consequences if not approved: | Requirements of UE processing time are severer for feature of the 2nd starting symbol, which leads to undesirable UE complexity or increase of UEs not supporting this feature. |

38.213 ### 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged parts omitted> If a UE receives a PSSCH in a resource pool and the HARQ feedback enabled/disabled indicator field in an associated SCI format 2-A/2-B/2-C has value 1 [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. For operation without shared spectrum channel access, the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception. For operation with shared spectrum channel access, the UE can attempt to transmit the PSFCH over a number of first NoccasionPSFCH slots, provided by sl-candidatePSFCH-Occasions and indexed from 1 to NoccasionPSFCH in ascending order in time, that include PSFCH resources and are at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception. For operation with shared spectrum channel access, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided, sl-MinTimeGapPSFCH is replaced to sl-MinTimeGapPSFCH + 1. The UE attempts to transmit PSFCH in a slot only when the UE fails to transmit PSFCH associated with the PSSCH in all previous slots for PSFCH within the NoccasionPSFCH slots. <Unchanged parts omitted>

38.214 ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur, and except for the second PSCCH occasion in transmission starting from the second candidate starting symbol at the last slot within the sensing window, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7, except for the second PSCCH occasion in transmission starting from the second candidate starting symbol at the last slot earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ), if sl-startingSymbolFirst and sl-startingSymbolSecond are provided. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. The value of Preserve corresponds to sl-PBPS-OccasionReservePeriodList if (pre-)configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList. The UE monitors sensing occasion(s) determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS. When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB], except for the second PSCCH occasion in transmission starting from the second candidate starting symbol at the slot n+TB, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31. When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB], except for the second PSCCH occasion in transmission starting from the second candidate starting symbol at the slot n+TB, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when PrsvpTX=0, it is up to UE implementation to either continue with step 3) or perform random selection. Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter sl-PartialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given periodicity corresponding to Preserve, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots. <Unchanged parts omitted>

Proposal 10:

- PSFCH prioritization for UE being incapable of PSFCH transmission in non-contiguous RB sets is applied after R16/17 PSFCH prioritization.

  - Adopt the following TP for TS 38.213.

| Reason for change: | Application order b/w R16/17 PSFCH prioritization and R18 PSFCH prioritization with respect to non-contiguous RB sets has not been decided yet. Prioritization outcome is dependent on the order. |
| --- | --- |
| Summary of change: | R18 PSFCH prioritization with respect to non-contiguous RB sets is applied after R16/17 PSFCH prioritization. |
| Consequences if not approved: | UE behavior of PSFCH prioritization is unclear in some cases. |

#### 16.2.4.2 Simultaneous PSFCH transmission/reception <Unchanged parts omitted> For operation with shared spectrum channel access, if a UE does not support PSFCH transmission in non-contiguous RB sets, the UE selects for PSFCH transmission any contiguous RB set(s) that include PSFCH with the smallest priority value. This PSFCH prioritization is applied after PSFCH prioritization for NTx,PSFCH PSFCHs determination described in this clause or prioritizations for sidelink and uplink transmissions in clause 16.2.4.3.1. <Unchanged parts omitted>

Proposal 11:

- When PSFCH transmissions in multiple RB sets at the same PSFCH occasion occur at a UE, and if the UE is incapable of PSFCH transmission in multiple RB sets,

  - The UE selects a single RB set including PSFCH with smallest priority value.

- Adopt the following TP for TS 38.213.

| Reason for change: | UE may not support PSFCH transmissions in multiple RB sets at the same PSFCH occasion, but corresponding UE behavior has not been specified yet. |
| --- | --- |
| Summary of change: | A single RB set including PSFCH with smallest priority value is selected for transmission. |
| Consequences if not approved: | UE behavior of PSFCH prioritization is unclear in some cases. |

#### 16.2.4.2 Simultaneous PSFCH transmission/reception <Unchanged parts omitted> For operation with shared spectrum channel access, if a UE does not support PSFCH transmission in non-contiguous RB sets, the UE selects for PSFCH transmission any contiguous RB set(s) that include PSFCH with the smallest priority value. For operation with shared spectrum channel access, if a UE does not support multi-channel access for PSFCH transmission in multiple RB sets, the UE selects for PSFCH transmission a single RB set that includes PSFCH with the smallest priority value. <Unchanged parts omitted>

Proposal 12:

- For a S-SSB TX in multiple RB-sets, UE transmits S-SSB in each non-anchor RB-set only when one or more of the following are met.

  - A: there is an on-going COT in the RB-set that is available by the UE

  - B: the UE attempts to obtain a COT in the RB-set for its own transmission

- Adopt the following TP for TS 38.213.

| Reason for change: | There is no rule to transmit S-SSB in non-anchor RB set(s). RAN1 should specify some rule to avoid negative impact to other system. |
| --- | --- |
| Summary of change: | S-SSB transmissions in non-anchor RB set(s) are allowed only when conditions are met. |
| Consequences if not approved: | UE can transmit S-SSB in non-anchor RB set(s) on any occasions, which have negative impacts to other system. |

## 16.1 Synchronization procedures <Unchanged parts omitted> For operation with shared spectrum channel access, a UE attempts to transmit at least S-SS/PSBCH blocks in the slots including S-SS/PSBCH blocks in the anchor RB set. A UE can attempt to transmit S-SS/PSBCH blocks in non-anchor RB set(s) only when there has been a COT in the RB set that is available by the UE or the UE attempts to initiate a COT in the RB set for its own transmission at the next slot of the S-SS/PSBCH transmission occasion. The UE applies CP extension to the first symbol of an S-SS/PSBCH block and within the first one or two symbols before the first symbol of the S-SS/PSBCH block according to an index [4, TS 38.211] provided by sl-CP-Extension-SSB. The UE assumes PRB(s) in an intra-cell guard band [6, TS 38.214] are not used for transmission of S-SS/PSBCH blocks. <Unchanged parts omitted>

References

- 3GPP RAN1#115 meeting,	chair’s notes