3GPP TSG RAN WG1 #116			R1-2401093

Athens, Greece, February 26th – March 1st, 2024

Source:	NTT DOCOMO, INC.

Title:	Maintenance on Further NR Mobility Enhancements

Agenda Item:	8.5

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID of ‘Further NR mobility enhancements’ was approved and the WID was further updated in RAN#97-e meeting [1]. In this contribution, we mainly discuss the remaining issued on the enhancements for further NR mobility enhancements.

- Measurement reporting

Priority of LTM CSI report

Agreement

For the priority rule for CSI report, LTM CSI report is prioritized over legacy CSI report.

In the last meeting, we made following agreement. To solve this issue, we need to discuss in more details.

Following options can be considered.

- Option 1: introduce a new factor in legacy CSI priority formula, for CSI-ReportConfig for LTM.

- Option 2: No revision of legacy CSI priority formula, which is applied to LTM CSI reports only or legacy CSI reports only. Introduce the new rule that LTM CSI report always has higher priority than legacy CSI report, regardless of calculated priority value.

- Option 3: No revision of legacy CSI priority formula, which is applied to LTM CSI reports and legacy CSI reports when the calculated priority values are different. For a LTM CSI report and a legacy CSI report with the same calculated priority value, LTM CSI report is prioritized.

From the three options above, Option 1 has larger specification impact, which is less preferred. The difference between Option 2 and Option 3 is that whether we should always allow LTM CSI report to have higher priority than legacy CSI report. To give gNB more flexibility on configuration, and considering periodic LTM CSI report may not need to always have higher priority than legacy aperiodic CSI report, Option 3 is slightly preferred.

Proposal 1

- Legacy CSI priority formular is applied to LTM CSI reports and legacy CSI reports when the calculated priority values are different. For a LTM CSI report and a legacy CSI report with the same calculated priority value, LTM CSI report is prioritized.

- Adopt following TP for TS 38.214.

### 5.2.5	Priority rules for CSI reports

[…]

A first CSI report is said to have priority over second CSI report if the associated  value is lower for the first report than for the second report, or if the associated  value is the same for the two reports but first CSI report is configured with LTM-CSI-ReportConfig-r18 and second CSI report is configured with CSI-ReportConfig .

Two CSI reports are said to collide if the time occupancy of the physical channels scheduled to carry the CSI reports overlap in at least one OFDM symbol and are transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports,

-	if y values are different between the two CSI reports, the following rules apply except for the case when one of the y value is 2 and the other y value is 3 (for CSI reports transmitted on PUSCH, as described in Clause 5.2.3; for CSI reports transmitted on PUCCH, as described in Clause 5.2.4):

-	The CSI report with higher  value shall not be sent by the UE.

-	otherwise, the two CSI reports are multiplexed or either is dropped based on the priority values, as described in Clause 9.2.5.2 in [6, TS 38.213].

A CSI report configured with LTM-CSI-ReportConfig has a higher priority in case of collision with CSI report(s) configured with CSI-ReportConfig.

Identification of SpCell

In the RAN1#114bis meeting, we made following agreement. However, the exact parameters to be used for frequency information had not yet been determined.

Agreement

- For the LTM L1 measurement report,

  - When a UE is configured is configured with SpCellInclusion, the SpCell measurements are the entries in the LTM-CSI-SSB-ResourceSet where the PCI and frequency information [SSB frequency/ARFCN] of the candidate cell is equal to the PCI and frequency information [SSB frequency/ARFCN] of the current SpCell.

In current RRC parameter, the frequency information of SSB for a LTM candidate cell is provided in the field of ssbFrequency-r18 in the IE of LTM-SSB-Config-r18. For the serving cell, the frequency information of SSB configured by CSI-SSB-ResourceSet is provided by the field of absoluteFrequencySSB of FrequencyInfoDL of serving cell. Thus, we propose

Proposal 2

- When a UE is configured is configured with SpCellInclusion, the SpCell measurements are the entries in the LTM-CSI-SSB-ResourceSet where the PCI and frequency information ssbFrequency-r18 of the candidate cell is equal to the PCI and frequency information absoluteFrequencySSB of the current SpCell.

- Adopt following TP for TS 38.214.

##### 5.2.1.4.2	Report Quantity Configurations

[…]

If the UE is configured with the higher layer parameter SSB-MTC-AdditionalPCI, the UE is allowed to report in a single reporting instance up to four SSBRIs for each report setting, where SSB resources are associated with PCI indices referring to the PCI of the serving cell and PCI(s) different from the PCI of the serving cell within the set of PCIs configured.

If a UE is configured with a [LTM-CSI-ReportConfig-r18],

-	if the UE is configured with spCellInclusion, the UE shall report in a single reporting instance [noOfReportedRS-PerCell-r18] different SSBRI for the current SpCell and each of the [noOfReportedCells-r18] -1 candidate cells. Otherwise, the UE shall report in a single reporting instance [noOfReportedRS-PerCell-r18] different SSBRI for each of the [noOfReportedCell-r18] candidate cells.

- where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated [ltm-CSI-SSB-ResourceList-r18] in the corresponding [LTM-CSI-SSB-ResourceSet-r18].

if spCellInclusion is configured, SSB resources in [ltm-CSI-SSB-ResourceList-r18] associated with the current SpCell are the entries where PCI [given by ltm-CandidatePCI-r18] and frequency information [given by ssbFrequency-r18SSB frequency/ ARFCN-ValueNR] of the associated candidate cell (given in [ltm-CandidateIdList-r18]) is equal to the PCI and frequency information [given by absoluteFrequencySSBSSB frequency/ ARFCN-ValueNR] of the current SpCell.

TCI state indication and activation

TCI state configuration

Agreement

In R18 LTM, on the QCL source of the TCI state before/during the cell switch command,

- SSB or TRS can be configured in a TCI state for the candidate cell(s) before/during cell switch command

  - Whether the TRS can be used for the candidate cell(s) before/during cell switch command is up to UE capability

Agreement

The TCI states in the candidate Cell TCI activation/deactivation command is associated with LTM TCI state pool of the target cell, i.e. configured under LTM-Candidate-r18.

Agreement

The TCI state indicated in the cell switch command is associated with LTM TCI state pool of the target cell, i.e. configured under LTM-Candidate-r18.

It has been agreed to configure SSB or TRS in a TCI state for candidate cells. Moreover, it has been agreed to associate between TCI state(s) in cell switch command MAC CE/candidate cell TCI state activation/deactivation MAC CE and LTM TCI state pool of target cell.

For a TCI state with SSB configured as resource RS for DMRS of PDCCH/PDSCH from candidate cell, new QCL rule needs to be introduced from RAN1 perspective. On the other hand, if SSB from candidate cell is configured as QCL type C resource RS for TCI state for PDCCH/PDSCH, whether the performance can be ensured or not needs RAN4 study. An LS to RAN4 is needed. Moreover, when TRS from candidate cell is configured as resource RS for TCI state, new QCL rule is also needed from RAN1 perspective. In addition, its association configuration with SSB is also needed.

Proposal 3

- New QCL rule needs to be introduced for configuration of SSB /TRS from candidate cell as resource RS for TCI state for PDCCH/PDSCH from RAN1 perspective.

  - Send LS to RAN4.

- When TRS from candidate cell is configured as resource RS for TCI state for PDCCH/PDSCH, its association with SSB from candidate cell is also configured.

- Adopt following TP for TS 38.214.

#### 5.1.5 Antenna ports quasi co-location […] For the DM-RS of PDCCH, if the UE is configured with dl-OrJointTCI-StateList, the UE shall expect that an indicated TCI-State indicates one of the following quasi co-location type(s): - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, the reference RS may additionally be a CSI-RS having a PCI different from the PCI of the serving cell if LTM-Config is configured, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block having a PCI different from the PCI of the serving cell, if LTM-Config is configured. For the DM-RS of PDSCH, if the UE is configured with dl-OrJointTCI-StateList, the UE shall expect that an indicated TCI-State indicates one of the following quasi co-location type(s): - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, the reference RS may additionally be a CSI-RS having a PCI different from the PCI of the serving cell if LTM-Config is configured, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block having a PCI different from the PCI of the serving cell, if LTM-Config is configured.

TCI state activation

In RAN1#114bis meeting, following conclusion was made on TCI state activation and there is an FFS on UE assumption on the active TCI states after the reception of the cell switch command.

Conclusion

For R18 LTM, in order to activate multiple joint TCI state or/and pair of (DL/UL) TCI states for candidate cell case, do not support TCI state activation together with beam indication of the candidate cell in the same MAC-CE message.

- FFS: UE assumption on the active TCI states other than the indicated TCI state after the reception of the cell switch command.

Based on discussion in last meeting, there are generally following three options.

- Option 1: UE retains the activated LTM TCI states only for the target cell.

- Option 2: UE retains the activated LTM TCI states for all candidate cells.

- Option 3: UE deactivates all activated LTM TCI states other than indicated TCI state.

The purpose of Option 1 is to reuse those LTM MAC CE activated TCI states for following beam management on the new target cell. Based on legacy specification, only legacy MAC CE activated TCI states can be used for beam management. In addition, considering the case of SSB as source RS of LTM MAC CE activated TCI states, it seems not so useful to use those TCI states for beam management. On the other hand, the LTM MAC CE activated TCI states are beneficial for potential subsequent LTM if UE retains the activated LTM TCI states for all candidate cells. Thus, after the reception of cell switch command, UE retains the active TCI states until new signaling is received from new serving cell to update the candidate cells for LTM or active TCI states for candidate cells for LTM. Some companies have concerns on UE complexity to retain all activate TCI states. Thus, we’re fine to introduce UE capability for this feature. If no consensus could be reached, we have to go to Option 3 or let RAN2 solve the issue.

Proposal 4

- After the reception of cell switch command, subject to UE capability, UE can be configured to retain all the active TCI states for all candidate cells; otherwise, UE deactivates the active TCI states for candidate cells other than the indicated TCI state.

  - Send an LS to RAN2.

Regarding UE behavior on tracking for candidate cells before cell switch command, if UE does not support TRS tracking before cell switch command, UE could measure/track SSBs only for candidate cells before cell switch command. But after receiving cell switch command, UE could measure/track TRS for target cell if TRS is provided in the indicated TCI state. In addition, if UE supports TRS tracking before cell switch command, UE could measure/track CSI-RSs before cell switch command.

Proposal 5

- If UE does not support TRS tracking for candidate cells before cell switch, UE measures/tracks SSBs for candidate cells before cell switch command. After cell switch command, UE measures/tracks TRS for target cell if TRS is provided in the indicated TCI state.

- If UE supports TRS tracking for candidate cells before cell switch, UE measures/tracks TRS for candidate cells before cell switch command if TRS is provided in the activated TCI states.

Agreement

After RACH procedure until a new TCI state is indicated by the target cell, a UE follows the indicated TCI-state in the cell switch command at least for CFRA triggered by cell switch command.

For beam indication after cells switch, we discussed UE assumption of beam for RACH-based handover in last meeting and made the following agreement for CFRA. However, we have not yet made agreement for CBRA.

In legacy RACH procedure, the beam for PRACH is selected by UE itself. But when beam has been indicated together with cell switch command, for the scenario where UE needs to perform RACH-based handover after receiving cell switch command, UE could follow the indicated beam in cell switch command for PRACH selection and the whole RACH procedure until a new TCI state is indicated by new serving cell. In discussion in last meeting, we think the main controversial part is whether to use SSB or indicated TCI state for different channels/RSs. Thus, we suggest following proposal.

Proposal 6

- When beam is indicated in cell switch command and UE needs to perform CBRA RACH-based handover after receiving cell switch command,

  - During RACH procedure, UE follows the SSB associated with the indicated TCI state in the cell switch command.

- After RACH procedure, UE follows the indicated TCI state in cell switch command until a new TCI state is indicated by the new serving cell.

- Adopt following TP for TS 38.213.

## 21	L1/L2-triggered mobility procedures

[…]

A UE can be provided by a MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI-State and/or TCI-UL-State in LTM-dl-OrJointTCI-StateToAddModList and/or LTM-ul-TCI-ToAddModList indicating a unified TCI state [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells. The UE applies the TCI-State and/or TCI-UL-State, if indicated by the MAC CE, from a first slot that is TBD after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, and μ is the SCS configuration for the TBD. If the MAC CE triggers a PRACH transmission [11, TS 38.321]PRACH transmission on the candidate cell after the reportion of the MAC CE is performed, the UE applies the TCI-State for receptions on the candidate cell, and applies a spatial domain filter corresponding to the TCI-State or the TCI-UL-State for transmissions on the candidate cell, that are after the completion of the random access procedure associated with the PRACH transmission on the candidate cell and before a new TCI state is indicated for the candidate cell.

Beam application time

On beam application time, following agreement was made in RAN1#113 meeting.

Agreement

- For the beam application time for Rel-18 LTM,

  - Beam application time is supported, and starts after the last symbol of the PUCCH or PUSCH carrying the HARQ-ACK for the PDSCH which carries MAC-CE containing cell switch command with the beam indication for the target cell(s)

    - FFS: reference SCS, i.e. serving cell and/or target cell

  - At least the following components are further studied to define the beam application time

    - Whether TCI state activation is received before/together with cell switch command

    - Legacy values, i.e. 3Nslotsubframe,µ and BeamAppTime-r17

    - RF retuning time when inter-frequency switch is performed, which is up to RAN4

    - Whether the target cell is one of the current serving cells

- Cell switching time, which is defined by RAN2 and RAN4, may or may not include the potential components of beam application time above.

- Send an LS to RAN2 and RAN4 to ask their feedback

Based on current agreement, the beam application time may have some overlap with the cell switching time somehow. But the definition of beam application time and cell switching time are discussed separately in different WGs. Based on RAN4 reply [2], current cell switch delay defined in RAN4 can cover the TCI state application time, thus, there seems no need to further discuss beam application time in RAN1. On the other hand, even if RAN1 defines beam application time additionally, the actual cell and beam application time should be MAX {beam application time, cell switching time}.

Proposal 7

- No need to define beam application time in RAN1.

- UE-based TA measurement

In RAN1#113 meeting, UE-based TA measurement was supported as below.

Agreement Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism Working Assumption From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

And there is the reply below from RAN4 on the feasibility of UE-based TA measurement.

Due to TAE, DL timing estimation error (of both serving and target cell), serving cell TA resolution error and TA adjustment error, though UE may be able to derive the TA, the actual UL Rx timing error at the target gNB may be larger than CP and may cause performance degradation at the gNB. However, in some specific scenarios, for example, in FR1, where the TAE between serving cell and candidate cell is within 260ns, UE may be able to derive the TA based on UE based TA measurement and may meet the UL transmit timing requirements under good SNR condition and may not cause any performance degradation at the gNB. Note: TAE of 260ns corresponds to Intra-band contiguous carrier aggregation, where the CC are collocated.

Based on the situation above, we should focus on the synchronized case between current serving cell and candidate cells to make UE-based TA measurement work for all frequency bands.

Observation 1

- For UE-based TA measurement, it is beneficial to focus on the synchronized case between current serving cell and candidate cells.

On the other hand, to make UE based TA measurement work for synchronized case, we should further discuss details of it. Thus, we analyse it below. Procedure of UE-based TA measurement in our assumption is shown in Figure 1.

Figure 1: Assumed procedures of UE-based TA measurement.

Firstly, for UE-based TA measurement, it needs to be considered when to perform it. NW needs to know whether UE-based TA measurement is performed or not for a candidate cell (i.e., NW needs to know whether UE has available TA for a candidate cell.).  If NW cannot know it, there are two problems below.

- Even if UE already obtains TA of target cell by UE-based TA measurement, NW may trigger PDCCH ordered RACH without RAR to acquire TA of target cell. It causes a duplication of TA acquisition of target cell which means there is no benefit of UE-based TA measurement.

- According to current spec., if cell switch command MAC CE does not contain TA of target cell, RACH-based LTM is triggered which means there is no benefit of UE-based TA measurement because RACH for target cell is needed after reception of cell switch command MAC CE.

Thus, NW needs to know at least whether UE-based TA measurement is performed or not for a candidate cell. To solve each problem, we can consider some methods below for each problem.

For (1),

(1-A) If ltm-ServingCellUE-MeasuredTA-ID is provided, UE-based TA measurement is periodically performed for configured candidate cells.

It is not efficient if UE-based TA measurement is periodically performed for non-target cells. In addition, it requires UE to maintain TA for each candidate cell, which increases UE complexity. Thus, this method is not preferred.

(1-B) If UE-based TA measurement is performed, UE reports related information (e.g., candidate cell ID, TA value) to NW.

The benefit of UE-based TA measurement is to avoid signalling exchange between NW and UE for TA acquisition and to avoid the latency brought by signalling exchange. However, this method mitigates the benefits of UE-based TA measurement which is also not preferred.

(1-C) NW triggers UE-based TA measurement by a signal including related information (e.g., candidate cell ID)

This method is more efficient, and the key point is which signalling should be used for this purpose, e.g., to define new signalling or to reuse legacy signalling.

Observation 2

- It is beneficial to consider NW triggers UE-based TA measurement by a signal including related information (e.g., candidate cell ID).

Regarding the detailed signaling to trigger UE based TA measurement, it is preferred to reuse legacy signaling. For this signal, the requirement is to trigger UE-based TA measurement for the potential target cell and contain the indicator of a candidate cell for which UE-based TA measurement should be performed. To meet the requirement, we can consider following alternatives.

Alt1: Candidate TCI state activation/deactivation MAC CE,

Alt2: Absolute TAC MAC CE on serving cell.

Alt3: LTM Cell Switch Command MAC CE

Regarding Alt1, no further enhancement is needed, and the MAC CE can be used for both purposes simultaneously. On the other hand, regarding Alt2, it can work if “R” bit is used to indicate candidate cell indicator. However, Absolute TAC MAC CE on serving cell may be not efficient to trigger UE-based TA measurement because it is not clear why these two purposes are needed at same time. Regarding Alt3, it can be used to handle the case that UE does not receive Candidate TCI state activation/deactivation MAC CE before cell switch and UE does not have TA of target cell before cell switch. Thus, we think both Alt1 and Alt3 can be supported as triggering UE-based TA measurement.

Proposal 8

- UE-based TA measurement for indicated candidate cells is triggered by MAC CE for TCI state activation for candidate cells.

- UE-based TA measurement for indicated target cell is triggered by cell switch command MAC CE, if not triggered by MAC CE for TCI state activation for candidate cells.

- Adopt the following TP for TS 38.213.

## 21 L1/L2-triggered mobility procedures […] If ltm-UE-MeasuredTA-ID of a candidate cell and ltm-UE-MeasuredTA-ID of the serving cell are provided to a UE and have same value, the UE can be triggered estimation of a timing advance for candidate cell(s) by TCI state activation MAC CE or cell switch command MAC CE estimates based on the UE implementation a timing advance to apply from a first transmission on the candidate cell that is after the reception of a cell switch command for the candidate cell [11, TS 38.321].A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. If the serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause 8.1, the UE

For (2),

(2-A) RACH-less LTM is triggered even when the TA field in the cell switch command MAC CE is FFF if UE has TA of target cell derived by UE-based TA measurement.

This method is flexible to handle the two cases differently, including UE has obtained TA of target cell or UE has not obtained TA of target cell. However, for the case that UE does not have TA of target cell, if RACH-based LTM is applied, longer interruption time may occur. If UE-based TA measurement is used for TA acquisition of target cell, it is better to always apply RACH-less LTM to shorten interruption time. Thus, this method is not preferred.

(2-B) If ueMeasuredTA is provided, RACH-less LTM is always performed for target cell.

As discussed above, ueMeasuredTA does not ensure to obtain TA of target cell before the reception of cell switch command. If UE-based TA measurement is triggered by cell switch command MAC CE, UE can still obtain the TA of target cell instead of triggering RACH, which may have shorter interruption time than RACH-based LTM.

Based on the above analysis, the combination of (1-C) and (2-B) could be beneficial. So we proposed following

Proposal 9

- If ueMeasuredTA is provided, RACH-less LTM is always performed for target cell.

- Conclusion

In this contribution, we discussed enhancements on the remaining issued on the enhancements for further NR mobility enhancements. Based on the discussion, we made following observations and proposals.

Proposal 1

- Legacy CSI priority formular is applied to LTM CSI reports and legacy CSI reports when the calculated priority values are different. For a LTM CSI report and a legacy CSI report with the same calculated priority value, LTM CSI report is prioritized.

Adopt following TP for TS 38.214.

### 5.2.5	Priority rules for CSI reports

[…]

A first CSI report is said to have priority over second CSI report if the associated  value is lower for the first report than for the second report, or if the associated  value is the same for the two reports but first CSI report is configured with LTM-CSI-ReportConfig-r18 and second CSI report is configured with CSI-ReportConfig .

Two CSI reports are said to collide if the time occupancy of the physical channels scheduled to carry the CSI reports overlap in at least one OFDM symbol and are transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports,

-	if y values are different between the two CSI reports, the following rules apply except for the case when one of the y value is 2 and the other y value is 3 (for CSI reports transmitted on PUSCH, as described in Clause 5.2.3; for CSI reports transmitted on PUCCH, as described in Clause 5.2.4):

-	The CSI report with higher  value shall not be sent by the UE.

-	otherwise, the two CSI reports are multiplexed or either is dropped based on the priority values, as described in Clause 9.2.5.2 in [6, TS 38.213].

A CSI report configured with LTM-CSI-ReportConfig has a higher priority in case of collision with CSI report(s) configured with CSI-ReportConfig.

Proposal 2

- When a UE is configured is configured with SpCellInclusion, the SpCell measurements are the entries in the LTM-CSI-SSB-ResourceSet where the PCI and frequency information ssbFrequency-r18 of the candidate cell is equal to the PCI and frequency information absoluteFrequencySSB of the current SpCell.

- Adopt following TP for TS 38.214.

##### 5.2.1.4.2	Report Quantity Configurations

[…]

If the UE is configured with the higher layer parameter SSB-MTC-AdditionalPCI, the UE is allowed to report in a single reporting instance up to four SSBRIs for each report setting, where SSB resources are associated with PCI indices referring to the PCI of the serving cell and PCI(s) different from the PCI of the serving cell within the set of PCIs configured.

If a UE is configured with a [LTM-CSI-ReportConfig-r18],

-	if the UE is configured with spCellInclusion, the UE shall report in a single reporting instance [noOfReportedRS-PerCell-r18] different SSBRI for the current SpCell and each of the [noOfReportedCells-r18] -1 candidate cells. Otherwise, the UE shall report in a single reporting instance [noOfReportedRS-PerCell-r18] different SSBRI for each of the [noOfReportedCell-r18] candidate cells.

- where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated [ltm-CSI-SSB-ResourceList-r18] in the corresponding [LTM-CSI-SSB-ResourceSet-r18].

if spCellInclusion is configured, SSB resources in [ltm-CSI-SSB-ResourceList-r18] associated with the current SpCell are the entries where PCI [given by ltm-CandidatePCI-r18] and frequency information [given by ssbFrequency-r18SSB frequency/ ARFCN-ValueNR] of the associated candidate cell (given in [ltm-CandidateIdList-r18]) is equal to the PCI and frequency information [given by absoluteFrequencySSBSSB frequency/ ARFCN-ValueNR] of the current SpCell.

Proposal 3

- New QCL rule needs to be introduced for configuration of SSB /TRS from candidate cell as resource RS for TCI state for PDCCH/PDSCH from RAN1 perspective.

  - Send LS to RAN4.

- When TRS from candidate cell is configured as resource RS for TCI state for PDCCH/PDSCH, its association with SSB from candidate cell is also configured.

- Adopt following TP for TS 38.214.

#### 5.1.5 Antenna ports quasi co-location […] For the DM-RS of PDCCH, if the UE is configured with dl-OrJointTCI-StateList, the UE shall expect that an indicated TCI-State indicates one of the following quasi co-location type(s): - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, the reference RS may additionally be a CSI-RS having a PCI different from the PCI of the serving cell if LTM-Config is configured, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block having a PCI different from the PCI of the serving cell, if LTM-Config is configured. For the DM-RS of PDSCH, if the UE is configured with dl-OrJointTCI-StateList, the UE shall expect that an indicated TCI-State indicates one of the following quasi co-location type(s): - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, the reference RS may additionally be a CSI-RS having a PCI different from the PCI of the serving cell if LTM-Config is configured, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - 'typeC' with an SS/PBCH block and, when applicable, 'typeD' with the same SS/PBCH block having a PCI different from the PCI of the serving cell, if LTM-Config is configured.

Proposal 4

- After the reception of cell switch command, subject to UE capability, UE can be configured to retain all the active TCI states for all candidate cells; otherwise, UE deactivates the active TCI states for candidate cells other than the indicated TCI state.

  - Send an LS to RAN2.

Proposal 5

- If UE does not support TRS tracking for candidate cells before cell switch, UE measures/tracks SSBs for candidate cells before cell switch command. After cell switch command, UE measures/tracks TRS for target cell if TRS is provided in the indicated TCI state.

- If UE supports TRS tracking for candidate cells before cell switch, UE measures/tracks TRS for candidate cells before cell switch command if TRS is provided in the activated TCI states.

Proposal 6

- When beam is indicated in cell switch command and UE needs to perform CBRA RACH-based handover after receiving cell switch command,

  - During RACH procedure, UE follows the SSB associated with the indicated TCI state in the cell switch command.

- After RACH procedure, UE follows the indicated TCI state in cell switch command until a new TCI state is indicated by the new serving cell.

## 21	L1/L2-triggered mobility procedures

[…]

A UE can be provided by a MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI-State and/or TCI-UL-State in LTM-dl-OrJointTCI-StateToAddModList and/or LTM-ul-TCI-ToAddModList indicating a unified TCI state [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells. The UE applies the TCI-State and/or TCI-UL-State, if indicated by the MAC CE, from a first slot that is TBD after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, and μ is the SCS configuration for the TBD. If the MAC CE triggers a PRACH transmission [11, TS 38.321]PRACH transmission on the candidate cell after the reportion of the MAC CE is performed, the UE applies the TCI-State for receptions on the candidate cell, and applies a spatial domain filter corresponding to the TCI-State or the TCI-UL-State for transmissions on the candidate cell, that are after the completion of the random access procedure associated with the PRACH transmission on the candidate cell and before a new TCI state is indicated for the candidate cell.

Adopt following TP for TS 38.213.

Proposal 7

- No need to define beam application time in RAN1.

Observation 1

- For UE-based TA measurement, it is beneficial to focus on the synchronized case between current serving cell and candidate cells.

Observation 2

- It is beneficial to consider NW triggers UE-based TA measurement by a signal including related information (e.g., candidate cell ID).

Proposal 8

- UE-based TA measurement for indicated candidate cells is triggered by MAC CE for TCI state activation for candidate cells.

- UE-based TA measurement for indicated target cell is triggered by cell switch command MAC CE, if not triggered by MAC CE for TCI state activation for candidate cells.

- Adopt the following TP for TS 38.213.

## 21 L1/L2-triggered mobility procedures […] If ltm-UE-MeasuredTA-ID of a candidate cell and ltm-UE-MeasuredTA-ID of the serving cell are provided to a UE and have same value, the UE can be triggered estimation of a timing advance for candidate cell(s) by TCI state activation MAC CE or cell switch command MAC CE estimates based on the UE implementation a timing advance to apply from a first transmission on the candidate cell that is after the reception of a cell switch command for the candidate cell [11, TS 38.321].A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. If the serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause 8.1, the UE

Proposal 9

- If ueMeasuredTA is provided, RACH-less LTM is always performed for target cell.

References

- 3GPP RP-222332, “Revised WID on Further NR mobility enhancements”, RAN#97e, Sep. 2022.