3GPP TSG RAN WG1 Meeting #116															R1-2401094

Athens, Greece, February 26th – March 1st, 2024

Source: 	NTT DOCOMO, INC.

Title:	Maintenance on Further NR Coverage Enhancements

Agenda item:	8.6

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss several issues regarding maintenance on Further NR Coverage Enhancements WI in Rel-18.

- Discussion on PRACH coverage enhancement

Power ramping counter

At RAN1#115 meeting, whether/when L1 notifies higher layers to suspend power ramping counter was discussed for multiple PRACH transmissions. Possible options were listed, and down-selection would be decided at RAN1#116 meeting.

Agreement For multiple PRACH transmissions, down-select one of the following options at RAN1#116: Option 1: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped or with reduced transmit power. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in any of PRACH occasions are dropped or with reduced transmit power. Option 1a: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission on part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power. Option 2: - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped or with reduced transmit power. Note: this implies it’s up to UE implementation. Option 2a: - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is with reduced transmit power. - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped. Option 3: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are with reduced transmit power. Note: whether any of the above options have specification impact is a separate discussion.

When PRACH transmission in all of PRACH occasions are dropped, preamble is not transmitted in this attempt. In legacy, when UE doesn’t transmit preamble in the attempt, layer 1 notifies higher layer to suspend the power ramping counter. Therefore, the power ramping counter should be suspended for this case similarly. In other words, Layer 1 notifies higher layer to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped. In option 2 and option 2a, the power ramping counter may or may not be suspended. Therefore, option 2 and option 2a are not preferred.

TS 38.213, v18.1.0, section 7.4 <*************************************omitted*************************************> If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, or due to slot format determination as described in clause 11.1, or due to the PUSCH/PUCCH/PRACH/SRS transmission occasions are in the same slot or the gap between a PRACH transmission and PUSCH/PUCCH/SRS transmission is small as described in clause 8.1, or due to DAPS operation as described in clause 15, or due to HD-UE operation in paired spectrum as described in clause 17.2, the UE does not transmit a PRACH in a transmission occasion, Layer 1 notifies higher layers to suspend the corresponding power ramping counter. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, the UE transmits a PRACH with reduced power in a transmission occasion, Layer 1 may notify higher layers to suspend the corresponding power ramping counter.

When part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power, there are opportunities for preamble transmission but the effect of repetitions or power ramping are not fully utilized in this attempt. In legacy, when UE transmits PRACH with reduced power, UE may suspend the power ramping counter. Therefore, the power ramping counter may or may not be suspended for this case similarly. In other words, Layer 1 may notify higher layer to suspend the corresponding power ramping counter when part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power. Therefore, option 1a is preferred.

Proposal 1: For multiple PRACH transmissions, prefer option 1a.

- Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped.

- Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission on part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power.

RO group and PRACH mask index

At RAN1#115 meeting, RO group determination with indication of PRACH mask index for multiple PRACH transmission was discussed. Possible options were listed, and down-selection would be decided at RAN1#116 meeting.

Agreement For multiple PRACH transmissions with indication of PRACH mask index, down-select one of the following options at RAN1#116 - Option 1: UE applies PRACH mask prior to RO group determination. RO group is determined based on the ROs indicated by the PRACH mask index. - Option 2: UE applies PRACH mask after RO group determination. UE transmits PRACH with Npreamblerep preamble repetitions only on a RO group with all the ROs indicated by the mask. - Option 3: UE applies PRACH mask after RO group determination. UE transmits PRACH with Npreamblerep preamble repetitions only on a RO group where at least one the first RO of this RO group is indicated by the mask - Option 4: UE applies PRACH mask after RO group determination. The PRACH mask index indicates one or multiple RO groups for multiple PRACH transmission. - Note: this implies the PRACH mask index indicates the RO group index(es) instead of RO index(es).

For option 1, UE determines RO groups based on the determined ROs indicated by the PRACH mask index. It is aligned with the principle that only ROs indicated by the PRACH mask index can be used for PRACH transmission.

For option 2, UE determines RO groups first without considering PRACH mask index. Then UE selects a RO group with all the ROs indicated by the mask. The possible worst case is that UE may not be able to select such a RO group (e.g., no RO group including all ROs satisfying mask index configuration). Then PRACH repetitions can’t be performed by the UE.

For option 3, UE determines RO groups first without considering PRACH mask index. Then UE selects a RO group with at least one RO indicated by the mask. If preamble is transmitted in each RO of the selected RO group, ROs not indicated by the mask index may be used for PRACH transmission, which contradicts with the principle of PRACH mask value indication in legacy.

For option 4, the PRACH mask indexing definition is different from legacy. How to define the mask indexing and when to reset needs to be defined. It may be not good to have such large spec impact at this late stage.

Therefore, option 1 is preferred.

Proposal 2: For multiple PRACH transmissions with indication of PRACH mask index, option 1 is preferred.

- UE applies PRACH mask prior to RO group determination. RO group is determined based on the ROs indicated by the PRACH mask index.

- Discussion on dynamic waveform switching

At RAN1#115, there was a discussion on how to capture enhanced PHR (i.e., reporting of PCMAX for assumed PUSCH) in 38.213. However, the discussion was not converged between the following two options:

TP 4-2r6-Opt1

Reason for change: Introduce power headroom information for assumed PUSCH in TS38.213. Summary of changes: Description of the provision of Pcmax for an assumed PUSCH and associated conditions. Consequences if not approved: Specification does not support reporting of power headroom information for assumed PUSCH. 7.7.1 Type 1 PH report <<< Start changes >>> - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State. If a UE provides a Type 1 power headroom report for an activated serving cell based on an actual PUSCH transmission, is provided assumedPUSCHInfo, and dynamicTransformPrecoderIndicationDCI-0-1 or dynamicTransformPrecoderIndicationDCI-0-2 is set to enabled for the active bandwidth part of the serving cell: the UE provides PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder enabled, if supported, if transform precoder is disabled for the actual PUSCH; or PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder disabled, if supported, if the transform precoder is enabled for the actual PUSCH. All other parameters used for the calculation of PCMAX,f,c(i) of the assumed PUSCH are the same as the actual PUSCH. <<< End changes >>>

TP 4-2r6-Opt2

Reason for change: Introduce power headroom information for assumed PUSCH in TS38.213. Summary of changes: Description of the provision of Pcmax for an assumed PUSCH and associated conditions. Consequences if not approved: Specification does not support reporting of power headroom information for assumed PUSCH. 7.7.1 Type 1 PH report <<< Start changes >>> - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State. If a UE provides a Type 1 power headroom report for an activated serving cell based on an actual PUSCH transmission scheduled by a PDCCH indicating if transform precoder is enabled or disabled, is provided assumedPUSCHInfo, and dynamicTransformPrecoderIndicationDCI-0-1 or dynamicTransformPrecoderIndicationDCI-0-2 is set to enabled for the active bandwidth part of the serving cell: the UE provides PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder enabled, if supported, if transform precoder is disabled for the actual PUSCH; or PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder disabled, if supported, if the transform precoder is enabled for the actual PUSCH. All other parameters used for the calculation of PCMAX,f,c(i) of the assumed PUSCH are the same as the actual PUSCH. <<< End changes >>>

Although this is not a strong opinion, we prefer to go with TP 4-2r6-Opt1 (option 1) compared with TP 4-2r6-Opt2 (option 2), based on the following reasons:

- Procedurally, option 1 is something that captures the relevant agreements in this agenda item. The cyan part in option 2 does not come from any agreement. In this very late stage of WI, we rather prefer to stick to the agreements we made only.

- Technically, the cyan part means UL MAC CE content will be different depending on how the PUSCH is scheduled. We expect this implies some changes on implementation for both gNB and UE (which we assume is why there are option 1 proponents from both gNB vendor and UE vendor in RAN1#115). Meanwhile there seems to be no technical benefit of this cyan part. Therefore, we do not think the cyan part makes much technical sense.

Note that, while we prefer to go with only either way, if the discussion is not converged again in RAN1#116, the only way that we can see for now would be to define new UE capability (or adding component in basic FG for the enhanced PHR) such that UE can select its own implementation (i.e., whether to implement option 1 or option 2).

Proposal 3: Regarding the specification impact for PHR for assumed PUSCH, prefer TP 4-2r6-Opt1 in [1]

Reason for change: Introduce power headroom information for assumed PUSCH in TS38.213. Summary of changes: Description of the provision of Pcmax for an assumed PUSCH and associated conditions. Consequences if not approved: Specification does not support reporting of power headroom information for assumed PUSCH. 7.7.1 Type 1 PH report <<< Start changes >>> - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State. If a UE provides a Type 1 power headroom report for an activated serving cell based on an actual PUSCH transmission, is provided assumedPUSCHInfo, and dynamicTransformPrecoderIndicationDCI-0-1 or dynamicTransformPrecoderIndicationDCI-0-2 is set to enabled for the active bandwidth part of the serving cell: the UE provides PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder enabled, if supported, if transform precoder is disabled for the actual PUSCH; or PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder disabled, if supported, if the transform precoder is enabled for the actual PUSCH. All other parameters used for the calculation of PCMAX,f,c(i) of the assumed PUSCH are the same as the actual PUSCH. <<< End changes >>>

- Conclusion

Proposal 1: For multiple PRACH transmissions, prefer option 1a.

- Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped.

- Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission on part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power.

Proposal 2: For multiple PRACH transmissions with indication of PRACH mask index, option 1 is preferred.

- UE applies PRACH mask prior to RO group determination. RO group is determined based on the ROs indicated by the PRACH mask index.

Proposal 3: Regarding the specification impact for PHR for assumed PUSCH, prefer TP 4-2r6-Opt1 in [1]

Reason for change: Introduce power headroom information for assumed PUSCH in TS38.213. Summary of changes: Description of the provision of Pcmax for an assumed PUSCH and associated conditions. Consequences if not approved: Specification does not support reporting of power headroom information for assumed PUSCH. 7.7.1 Type 1 PH report <<< Start changes >>> - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State. If a UE provides a Type 1 power headroom report for an activated serving cell based on an actual PUSCH transmission, is provided assumedPUSCHInfo, and dynamicTransformPrecoderIndicationDCI-0-1 or dynamicTransformPrecoderIndicationDCI-0-2 is set to enabled for the active bandwidth part of the serving cell: the UE provides PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder enabled, if supported, if transform precoder is disabled for the actual PUSCH; or PCMAX,f,c(i) based on any applicable maximum output power reduction for an assumed PUSCH with transform precoder disabled, if supported, if the transform precoder is enabled for the actual PUSCH. All other parameters used for the calculation of PCMAX,f,c(i) of the assumed PUSCH are the same as the actual PUSCH. <<< End changes >>>

Reference

- R1-2312623	Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM	Moderator (InterDigital, Inc.)