3GPP TSG RAN WG1 #116			R1-2401123

Athens, Greece, February 26th – March 1st, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SSB SCell operation

Agenda Item:	9.5.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#102 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. The work item scope is showed as follows. In this contribution, we provide our views on the first scope on on-demand SSB Scell operation.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

- Discussion

  - Legacy SCell operation

Firstly, we consider the legacy SCell operation and corresponding UE behaviors regarding SSB on SCell to clarify the purpose and target of this objective.

SSB-less SCell operation

From Rel-15, SSB-less Scell operation is defined only in FR1 intra-band. In Rel-18, SSB-less SCell operation was extended to FR1 inter-band with some restrictions, such as, SSB-less SCell and reference cell need to be co-located, TRS or A-TRS needs to be configured on SCell, and restrictions on RTD, power difference, QCL relationship between reference cell and SCell are defined. These operations are subject to UE capabilities that are separated from the CA capabilities.

As described above, on the current SSB-less SCell operation, it requires several constraints on SCell and reference cell, and hence the available scenarios in practice would be limited.

Normal SCell operation with SSB

As normal SCell operation with SSB, the typical procedure is described in Fig.1. SSB on SCell is configured in SCell addition and optionally configured in L3 measurement before SCell addition. The initial state of the SCell can be “activated” or “deactivated”. According to the SCell state, SCell can be activated or deactivated afterwards. The SCell activation delay is defined depending on the SCell SSB periodicity and whether the SCell is already known or not to the UE. During SCell is activated, UE shall perform normal SCell operation such as monitoring PDCCH on SCell for data TX/RX and CSI reporting. In order to ensure such normal operation, during SCell is activated, a UE would expect SSB on SCell to be transmitted periodically according to the configurations which can be updated by RRC reconfiguration. In addition, L3 measurement on SCell can be performed during SCell is activated or deactivated, which would also require periodic SSB transmission according to RRC configuration.

RAN4 requirements on UE behavior about SCell should be taken into account for effective NES operation. In the current spec., UE on active SCell may assume that SSB is transmitted with a configured SSB periodicity (at longest 160ms) to meet the RRM/Demod requirements while UE on deactivated SCell may not assume the SSB transmission (unless deactivated SCell measurement is configured).

Fig.1 Typical procedure of normal SCell operation

Observation 1:

- SSB-less SCell operation requires several constraints on SCell and reference cell, and hence the available scenarios in practice would be limited.

- In normal SCell operation, a UE expects that SSB is periodically transmitted on active SCell according to RRC configurations, which inhibits NES.

- UE on deactivated SCell does not require SSB on the SCell at all unless deactivated SCell measurement is configured.

  - Target scenarios

From NES perspective, at any scenario and any timing, it is preferable that SSB transmission on SCell is minimized, e.g., only when it is essential for the UE to meet minimum requirements, so that gNB can achieve longer sleep time. Therefore, on-demand SCell operation should cover any cases where SSB-less SCell operation does not cover and/or UE does not support the capabilities for SSB-less SCell operation.

Proposal 1:

- Rel-19 on-demand SSB SCell operation should target any cases where the legacy SSB-less SCell operation is not applicable.

- For Rel-19 on-demand SSB SCell operation, constraints on SCell should be minimized.

  - E.g., it would be preferable to avoid/minimize requiring reference cell with specific relationship of RTD, power and QCL.

As shown in Fig.1, there can be some cases where a UE requires SSB in normal SCell operation. We think at this stage, all procedures where a UE assume SSB is periodically transmitted according to (RRC-)configurations should be considered in this objective. This is because gNB normally controls multiple UEs where each UE may perform different procedures and deals with the same SSB differently, so on-demand SSB should be applicable for any cases regarding the normal SCell operation to achieve effective NES. For example, at a SSB on SCell, (case.1) some UEs may perform L3 measurement using the SSB and (case.2) other UEs may use the same SSB for time/freq synchronization to monitor SCell PDCCH during SCell is activated. In this case, if on-demand SSB can be applied for either case (either set of UEs), gNB anyway has to transmit legacy/periodic SSB for the case which on-demand SSB is not applicable for. Therefore, we’d like to consider on-demand SSB for any cases regarding normal SCell operation.

Proposal 2:

- Consider the following cases to use on-demand SSB on SCell in Rel-19

  - To perform L1/L3 measurements on activated SCell.

  - To perform PDCCH monitoring on activated SCell.

  - To perform SCell activation from deactivated SCell.

  - To perform L3 measurements on deactivated SCell.

  - Properties of on-demand SSB

Since it is preferable for NW to transmit only “truly required minimum SSB” as on-demand, the SSB properties, such as SSB position, periodicity and time domain behaviour should be flexible/changeable according to each of on-demand transmission scenarios.

Moreover, it would be necessary to consider the relaxed properties of SSB (such as longer periodicity, e.g., 320, 640 ms, aperiodic transmission) for Rel-19 NES capable UEs on active SCell in order to make NES effective in SCell operation with on-demand SSB. Otherwise, once SCell is activated for a UE, NW has to transmit SSB at least every 160ms, which is not much different from the possible operation based on current specifications, i.e., NES gain is limited.

Proposal 3:

- Consider relaxation of minimum necessary SSB transmissions on active SCell, such as aperiodic or longer maximum periodicity of SSB transmission on the SCell.

Especially, always-ON periodic SSB transmission is not suitable for NES purpose, and time domain behaviour of SSB TX should be enhanced from periodic-based to aperiodic-based SSB TX which can achieve more flexible and only essential SSB TX for higher NES gain. Therefore, we believe that “no SSB TX on Scell should be a default state”, and “SSB is TXed on-demand with only essential properties for each on-demand purpose” in Rel-19 NES.

Proposal 4:

- Consider the following options as a default state of SSB transmission on SCell in Rel-19 NES

  - Opt.1: No SSB transmission

  - Opt.2: periodical SSB transmission with longer periodicity than the maximum of the legacy

- Consider Opt.1 as baseline.

In addition to on-demand “aperiodic” SSB TX, in some scenarios, “periodic-based” SSB transmission may be more suitable for efficient operation, as it avoids potential large overhead of indication for aperiodic TX. For example, it enables UEs to monitor and track the performance of the cell/beam offering on-demand SSB in real-time (e.g., FR2 small cell for on-demand SSB), that may achieve more NES gain in some scenarios. In this sense, enhancement for “periodic-based” SSB transmission can also be a target of on-demand transmission. The state (i.e., on/off) and properties (e.g., periodicity, position) of “periodic-based” SSB should be changeable according to overall NW management compared to legacy operations. Thus, “semi-persistent” SSB TX which means periodic SSB transmission can be turned on/off via a faster method over the legacy RRC-configuration in this context.

Based on the above discussion, we can consider the SSB state on Scell as shown in Fig.2. On the other hand, transition between state#2 and state#3 such as changing properties of periodic SSB TX is to be studied/covered in NES objective 3 (adaptation of common channel/signal transmissions).

Proposal 5:

- Support aperiodic SSB transmission on SCell for on-demand SSB SCell operation in Rel-19.

- Consider semi-persistence SSB transmission on SCell for on-demand SSB SCell operation in Rel-19, which means less signalling overhead to change the SSB properties (e.g., periodicity, position, time-domain behaviour) or to turn off the SSB transmission.

  - FFS: Change in SSB properties on SCell to be studied in objective2 or 3 in Rel-19 NES.

Proposal 6:

- The following SSB properties should be changeable at every on-demand SSB transmission:

  - SSB index / position within a SSB burst.

  - Time-domain behaviour:

    - For aperiodic TX, the number of SSB bursts (how many SSB bursts are transmitted at one triggering)

    - For periodic/semi-persistent SSB TX, periodicity

Fig.2 Consideration on SSB states on SCell and transitions b/w each state in Rel-19 NES

  - Triggering mechanism for on-demand SSB TX

RAN1 should study how to trigger and potentially how to indicate on-demand SSB transmission, with considering two types of triggering methods i.e., UE triggering and gNB triggering. We think the first discussion point should be whether the initiation should be done by either UE or NW side.

For UE triggering method, we have some concerns. In general, NW controls multiple UEs that have different properties about when/how many/which index of (on-demand) SSB on SCell is truly required (although UE knows them). As a result, in case of UE triggering method, there could be many requests from different UEs and NW is forced to transmit (on-demand) SSB frequently, which is not suitable for NES. Even if a final decision on whether to transmit SSB is up to NW with considering request from UE, specifying UE triggering method may allow a UE not behave for a proper normal SCell operation if requested SSB is not correctly received. In other words, with the UE triggering method in any specifications, a UE can argue that the UE cannot monitor PDCCH or perform RRM on SCell because the UE is not given the SSB requested by the UE. This is not preferable situation for NW operation. Moreover, required SSB properties such as when/how frequent/how many SSB is required by a UE could be statically determined e.g., as UE capability. Thus, we think UE triggering based on-demand transmission is not appropriate and is not necessary.

Observation 2:

- For UE triggering method, gNB may fall into transmitting SSB frequently on SCell to meet all UE’s request and requirements on SCell, which is not desirable for NES operation.

- The required SSB properties such as when/how frequent/how many SSB is required by the UE could be statically determined e.g., as UE capability, and hence a mechanism for dynamic request of SSB transmission from UE would be unnecessary.

For gNB triggering, in the legacy operation, NW controls when/whether to perform L3 measurement, SCell addition, SCell activation, etc., where UE is required to receive SSB. Therefore, NW should be able to control when/how many/which index of (on-demand) SSB on SCell is transmitted, so that NW can concentrate Tx timing to achieve longer sleep time.

Proposal 7:

- Support gNB triggering on-demand SSB transmission on Scell

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SSB SCell operation for network energy saving.

Observation 1:

- SSB-less SCell operation requires several constraints on SCell and reference cell, and hence the available scenarios in practice would be limited.

- In normal SCell operation, a UE expects that SSB is periodically transmitted on active SCell according to RRC configurations, which inhibits NES.

- UE on deactivated SCell does not require SSB on the SCell at all unless deactivated SCell measurement is configured.

Proposal 1:

- Rel-19 on-demand SSB SCell operation should target any cases where the legacy SSB-less SCell operation is not applicable.

- For Rel-19 on-demand SSB SCell operation, constraints on SCell should be minimized.

  - E.g., it would be preferable to avoid/minimize requiring reference cell with specific relationship of RTD, power and QCL.

Proposal 2:

- Consider the following cases to use on-demand SSB on SCell in Rel-19

  - To perform L1/L3 measurements on activated SCell.

  - To perform PDCCH monitoring on activated SCell.

  - To perform SCell activation from deactivated SCell.

  - To perform L3 measurements on deactivated SCell.

Proposal 3:

- Consider relaxation of minimum necessary SSB transmissions on active SCell, such as aperiodic or longer maximum periodicity of SSB transmission on the SCell.

Proposal 4:

- Consider the following options as a default state of SSB transmission on SCell in Rel-19 NES

  - Opt.1: No SSB transmission

  - Opt.2: periodical SSB transmission with longer periodicity than the maximum of the legacy

- Consider Opt.1 as baseline.

Proposal 5:

- Support aperiodic SSB transmission on SCell for on-demand SSB SCell operation in Rel-19.

- Consider semi-persistence SSB transmission on SCell for on-demand SSB SCell operation in Rel-19, which means less signalling overhead to change the SSB properties (e.g., periodicity, position, time-domain behaviour) or to turn off the SSB transmission.

  - FFS: Change in SSB properties on SCell to be studied in objective2 or 3 in Rel-19 NES.

Proposal 6:

- The following SSB properties should be changeable at every on-demand SSB transmission:

  - SSB index / position within a SSB burst.

  - Time-domain behaviour:

    - For aperiodic TX, the number of SSB bursts (how many SSB bursts are transmitted at one triggering)

    - For periodic/semi-persistent SSB TX, periodicity

Observation 2:

- For UE triggering method, gNB may fall into transmitting SSB frequently on SCell to meet all UE’s request and requirements on SCell, which is not desirable for NES operation.

- The required SSB properties such as when/how frequent/how many SSB is required by the UE could be statically determined e.g., as UE capability, and hence a mechanism for dynamic request of SSB transmission from UE would be unnecessary.

Proposal 7:

- Support gNB triggering on-demand SSB transmission on Scell

References

- RP-234065, New WID: Enhancements of network energy savings for NR, RAN Meeting #102, December 11th-15th, 2023.