3GPP TSG RAN WG1 #116	R1-2401131

Athens, Greece, February 26th – March 1st, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on support of RedCap and eRedCap UEs in FR1-NTN

Agenda Item:	9.11.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#102 meeting [1], the new WI for NR-NTN was endorsed for R19. In the WID, RedCap/eRedCap UE support is included as below and RAN1 is tasked with checking whether enhancements to support HD-FDD operation is necessary or not. In this contribution, we share our further views for support of RedCap/eRedCap UEs in FR1-NTN.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

- Discussions

  - HD-FDD operation in TN

HD-FDD UE was defined in R17 RedCap WI. The corresponding UE behavior can be found in 17.2 of 38.213. The specified rule can be summarized as below:

Fig. 1: HD-FDD UE behavior in the existing specification

All possible patterns of overlap b/w DL channel/signal reception behavior and UL channel/signal transmission behavior, including insufficient processing time for TX/RX switching, are considered carefully and prioritization rule is defined for most cases. Meanwhile, the following two cases are treated as error cases, which means that NW scheduler should handle these cases.

- Case 1: Dynamic PDSCH/CSI-RS RX vs Dynamic PUCCH/PUSCH/PRACH/SRS TX

- Case 2: Configured PDCCH (in CSS)/PDSCH/CSI-RS/DL-PRS RX vs Configured PUCCH/PUSCH/SRS TX

There is no valid case where gNB schedules TX/RX as in Case 1 or Case 2 and thus it is straightforward to avoid the cases by gNB scheduler.

Observation 1:

- There are cases in TN where RedCap/eRedCap UEs do not assume TX/RX overlap and thereby NW schedular avoids the situations.

  - HD-FDD operation in NTN

As discussed above, in TN, RedCap/eRedCap UEs perform TX/RX prioritization for some cases and NW avoids overlap b/w TX and RX by implementation for the other cases. Then for NTN, at least the former cases will not have any problem. If TX/RX overlap occurs at RedCap/eRedCap UEs in NTN, the same prioritization behavior as in R17/18 specs is applied. No critical issue can be found. On the other hand, the latter cases, i.e., Case 1 and Case 2 in the previous section, are questionable. If NW can be aware of potential overlap b/w TX and RX at each RedCap/eRedCap UE, appropriate NW scheduler should handle the issue as assumed in TN and then no spec change is necessary. In other words, the question for this agenda in the WID is equivalent to feasibility of the NW awareness.

Observation 2:

- If NW can be aware of TX/RX overlap at each RedCap/eRedCap UE, NW scheduler should avoid the overlap and thereby no specification change is necessary.

- Otherwise, new rule for cases where the existing spec does not define any prioritization rule is newly necessary for FR1-NTN.

Then, for the question, our current view is that NW cannot be aware of TX/RX overlap at each RedCap/eRedCap UE accurately since TA value is determined by UE based on its location information and satellite ephemeris. Although TA value can be controlled by TA command from gNB, it does not mean the exact TX timing is known at gNB side. This can be illustrated as below.

Fig. 2: PUCCH corresponding to a SPS PDSCH vs another SPS PDSCH

The following arguments may be assumed, but it seems that the counterargument for each as below is reasonable.

- 1) gNB can provide appropriate scheduling without the TX/RX overlap. TA value report defined in R17 NTN can be used for the purpose.
 It does not mean that gNB knows exact TA value anytime in the R17 TA report.

- 2) Frequent TA report in the existing specification is applicable so that gNB can provide appropriate scheduling without the TX/RX overlap.
 RedCap/eRedCap UEs will perform repetitions in typical situations and thus frequent TA report is not preferable from perspectives of resource efficiency and UE energy saving.

- 3) Rough TA information can be used in gNB scheduler with a certain time margin in consideration of gap b/w a reported TA value and the actual TA value.
 Although this may be a possible way, resource efficiency/flexibility will degrade, and much more frequent RRC reconfiguration may be required for configured TX/RX case.

Therefore, gNB may not know when DL/UL overlap at UE side will occur and will not occur.

Observation 3:

- New rule for cases defined as error cases in the existing spec is necessary for HD-FDD RedCap/eRedCap UEs in FR1-NTN.

Proposal 1:

- Support new for cases defined as error cases in the existing spec.

For the solution, the following three mechanisms may be possible options. Further discussion is necessary, while the first one may be more reasonable.

Proposal 2:

- For HD-FDD RedCap/eRedCap UEs in FR1-NTN, discuss at least the following for cases defined as error cases in the existing spec.

  - Opt. 1: Define TX/RX prioritization rule.

  - Opt. 2: Define report of overlap occurrence.

  - Opt. 3: Enhance TA report.

  - Other issues for RedCap/eRedCap UEs in NTN

Although HD-FDD is the only topic in the current R19 NR NTN WID, it may be valid that the following two issues are important aspects to support practically RedCap/eRedCap UEs in NTN.

- Repetition-related parameters

Repetition-related parameter commonly configured b/w both non-RedCap UE and RedCap/eRedCap UE may not be desirable. For example, PUCCH repetition-related parameters (repetition factor, RSRP threshold) introduced in R18. For another example, PDSCH aggregation-related parameters that may be introduced in R19.

Observation 4:

- It seems that it is better to discuss whether/how to define repetition-related parameters separately b/w non-RedCap UE and RedCap/eRedCap UE.

  - Whether enh is necessary or not is also dependent on R18/19 spec for repetition related parameter.

- Capacity of common PUCCH resources

Capacity of common PUCCH for RedCap/eRedCap UE may be insufficient. Each PUCCH TX would be performed with repetition due to coverage issue as discussed in R18 NR NTN WI. Meanwhile, in the current specification, only 16 UEs as max can use common PUCCH resources simultaneously in several slots. This issue would be valid for normal handheld UEs, but more critical for RedCap/eRedCap UEs since much more UEs will exist in an NTN-cell.

Observation 5:

- It seems that it is better to discuss whether/how to enhance capacity of common PUCCH for RedCap/eRedCap UE.

- Conclusion

In this contribution, we discussed support of RedCap/eRedCap UEs in FR1-NTN. Observations/Proposals are summarized as following:

Observation 1:

- There are cases in TN where RedCap/eRedCap UEs do not assume TX/RX overlap and thereby NW schedular avoids the situations.

Observation 2:

- If NW can be aware of TX/RX overlap at each RedCap/eRedCap UE, NW scheduler should avoid the overlap and thereby no specification change is necessary.

- Otherwise, new rule for cases where the existing spec does not define any prioritization rule is newly necessary for FR1-NTN.

Observation 3:

- New rule for cases defined as error cases in the existing spec is necessary for HD-FDD RedCap/eRedCap UEs in FR1-NTN.

Proposal 1:

- Support new for cases defined as error cases in the existing spec.

Proposal 2:

- For HD-FDD RedCap/eRedCap UEs in FR1-NTN, discuss at least the following for cases defined as error cases in the existing spec.

  - Opt. 1: Define TX/RX prioritization rule.

  - Opt. 2: Define report of overlap occurrence.

  - Opt. 3: Enhance TA report.

Observation 4:

- It seems that it is better to discuss whether/how to define repetition-related parameters separately b/w non-RedCap UE and RedCap/eRedCap UE.

  - Whether enh is necessary or not is also dependent on R18/19 spec for repetition related parameter.

Observation 5:

- It seems that it is better to discuss whether/how to enhance capacity of common PUCCH for RedCap/eRedCap UE.

References

- RP-234078		New WID: Non-Terrestrial Networks (NTN) for NR Phase 3	Huawei (Moderator, RAN1 Vice-Chair)