3GPP TSG RAN WG1 #116			R1-2401149

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.5.3

Source:	KT Corp.

Title:	Views on adaptation of SS/PBCH blocks

Document for:	Discussion and decision

1. Introduction

Many techniques to achieve energy savings for NR BS is discovered during the study period [1], and some of them are specified with the Rel. 18 specification. Since there were many other remained techniques showing significant energy saving gain, adopting and specifying them became the objective correspondingly. The objectives of the Rel. 19 enhanced NES is as follows [2]:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3]

  - Triggering method by uplink wake-up-signal using an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

    - Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Checkpoint for normative work in RAN#105

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial

    - This study is to be done in 2Q’2024 only

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

- Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we focus on adaptation of common signal/channel transmissions, especially about the adaptation of SSB in time domain.

2. Design principle and possible issues on SSB adaptation

The study [1] showed that transmitting SSB requires huge portion of energy consumption during low UE traffic, so dramatic enhancement for energy saving can be expected with some appropriate SSB transmission reductions. With these assumptions, the SSB-less SCell operation is specified in Rel. 18 as the first approach to reduce SSB transmissions. To reduce SSB transmissions more than the possible configuration on the legacy specification in PCell, it is required to relax the condition of 20ms period of SSB with the preserved quasi-co-location.

The pattern of SSB transmission in a frame has been determined at RRC message ssb-PositionInBurst in the SIB1 for the current cell and CellGroupConfig for the neighbor cells, and legacy UEs will expect the configuration will not be changed ever. That is, any adaptation relaxing the condition of 20ms period of SSB can be configured only to Rel. 19 applied UEs.

However, the relaxation may increase UE processing complexity of synchronization. For example, the relaxation with doubled period may result doubled complexity for the cell search of Rel. 19 NES supporting UEs. The total energy consumption, that is the sum of BS’s and UEs’, still may decrease. However, increased consumption is often far more critical for the mobile devices depending on the battery in commercial service aspect.

Proposal 1: The increasement of complexity of the synchronization procedures on UEs is minimized by the possible SSB adaptations.

One approach to minimize the complexity may be limiting the frequency variation of SSB, i.e., GSCN. We believe the limitation may be efficient at least during the migration step to eNES applied BSs.

Proposal 2: At least for the complexity of synchronization procedures on UEs, the adaptation is applied only for the SSBs transmitted at frequencies limited by the specification.

3. Potential Issues

There may be two approaches for the SSB adaptation. One is the periodicity adaptation, and another is the pattern adaptation. Since the pattern adaptation is more general operation and the periodicity adaptation is one of the specific configurations with periodical application of all-zero pattern, it is needed to decide the range of possible adaptation.

Figure 1. An example of the periodicity adaptation of SSB

Figure 2. An example of the pattern adaptation of SSB

Although the configuration and process are simpler with the periodicity adaptation, we believe that the pattern adaptation may be more efficient, since in many practical environments the probability of UEs access for each beam can be different. That is, it is often true that some SSB indices having a lot of involved UEs are more important while others are not.

Proposal 3: For the adaptation of SSB in time domain, the pattern adaptation is applied.

On the other hand, the radio link monitoring and CSI feedback performed by UE uses reference signal like CSI-RS and SSB. Some issues may be raised when some adaptation is applied to SSBs used as reference signal, like false triggering and report.

Proposal 4: Discuss how to handle RLM and CSI related operations using SSBs with the possibility of adaptation.

4. Conclusion

We give our proposals in this contribution as follows:

Proposal 1: The increasement of complexity of the synchronization procedures on UEs is minimized by the possible SSB adaptations.

Proposal 2: At least for the complexity of synchronization procedures on UEs, the adaptation is applied only for the SSBs transmitted at frequencies limited by the specification.

Proposal 3: For the adaptation of SSB in time domain, the pattern adaptation is applied.

Proposal 4: Discuss how to handle RLM and CSI related operations using SSBs with the possibility of adaptation.

4. References

[1] 3GPP TR 38.864 V18.0.0, Study on network energy savings for NR.

[2] RP-234065, New WID: Enhancements of network energy savings for NR.