3GPP TSG RAN WG1 #116						                      R1-2401152

Athens, Greece, February 26 – March 01, 2024

Agenda Item:		9.1.3.2

Source:		IIT KANPUR, Indian Institute of TechNOLOGY MADRAS (IITM)

Title:		Discussion on study of AIML for CSI compression

Document for:		Discussion/Decision

# Introduction

In RAN#102, the new WID for AI/ML for NR air interface was finalized [1]. The WID describes additional study on CSI compression with the following objectives:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

Up to RAN1#115, various agreements/conclusions related to study of AI/ML for CSI compression were reached and the detailed analysis and description of the study is summarized in TR 38.843 [2].

In this contribution, we provide our views and analyses on aspects related to AI/ML-based CSI compression including considering spatial-frequency-temporal compression and study for specification support.

# Spatial-frequency-temporal compression for AI/ML based CSI compression

For Rel-19, CSI compression using temporal correlation is a new direction for study and a significant effort is needed study the performance and complexity gains and for its specification effort.

For the evaluation methodology, modelling of UE movement, spatial consistency, temporal window (in time unit), number of time-domain CSI instances, range of UE speeds, should be considered, as these will directly impact dataset size, AI/ML model complexity and its performance.

Proposal 1: Consider the following parameters as part of evaluation assumptions for spatial-frequency-temporal domain compression:

- different UE movement profile modelling

- temporal window (in time unit)

- number of time-domain CSI instances, for training and inference

- range of UE speeds

- spatial consistency

For comparison with a baseline scheme, the Rel-18 codebook for high/medium velocities can be considered. The CSI-RS resource configurations, CSI report configurations, and subsequently the feedback overhead size for this Rel-18 Doppler codebook should be taken as a baseline for comparison.

Further, for the study of specification impacts, the potential enhancements on CSI-RS resource configurations of Rel-18 Doppler codebook which consider burst CSI-RS resources can be taken up.

For the reporting of CSI, enhancements on reporting configuration of Rel-18 codebook and format of output-CSI-UE and input-CSI-NW considering temporal domain CSI should be studied.

Proposal 2: Consider CSI-RS resource configurations, CSI report configurations, and subsequently the feedback overhead size of Rel-18 codebook for high/medium UE velocities for comparison with spatial-frequency-temporal domain CSI compression.

Proposal 3: Study the specification impact considering potential enhancements on CSI-RS resource configurations of Rel-18 codebook for high/medium UE velocities.

Proposal 4: Study specification impact considering Rel-18 codebook reporting configuration and format of output-CSI-UE and input-CSI-NW for including temporal domain CSI.

# Conclusions

In this contribution, we have provided our views on the study of AI/ML for CSI compression. We have the following proposals:

Proposal 1: Consider the following parameters as part of evaluation assumptions for spatial-frequency-temporal domain compression:

- different UE movement profile modelling

- temporal window (in time unit)

- number of time-domain CSI instances, for training and inference

- range of UE speeds

- spatial consistency

Proposal 2: Consider CSI-RS resource configurations, CSI report configurations, and subsequently the feedback overhead size of Rel-18 codebook for high/medium UE velocities for comparison with spatial-frequency-temporal domain CSI compression.

Proposal 3: Study the specification impact considering potential enhancements on CSI-RS resource configurations of Rel-18 codebook for high/medium UE velocities.

Proposal 4: Study specification impact considering Rel-18 codebook reporting configuration and format of output-CSI-UE and input-CSI-NW for including temporal domain CSI.

# References

RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023

3GPP TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), V2.0.0 (2023-12)