3GPP TSG RAN WG1 #116		R1-2401158

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.4.2.4

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	Discussion on Waveform Characteristics of External Carrier Wave in Ambient IoT

Document for:	Discussion

# Introduction

This document includes our discussions, observations and proposals on the scenario in which a carrier wave is provided externally to an ambient IoT device for Release 19. Specifically, we propose to study the feasibility of frequency switching of the ambient IoT device. In this section we reproduce the relevant agreements and proposals from the RAN Plenary study report [1] and Ambient IoT SID [2].

It has been agreed to study 2 topologies in the Release 19 study item [1] [2]. In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a basestation. The communication between the basestation and the ambient IoT device includes Ambient IoT data and/or signalling. This topology includes the possibility that the BS transmitting to the Ambient IoT device is a different from the BS receiving from the Ambient IoT device. In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and base station. In this topology, the intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT. The intermediate node transfers Ambient IoT data and/or signalling between BS and the Ambient IoT device.

# Discussions on Providing an External Carrier Wave

  - Capability of the Ambient IoT device to perform frequency switching

Consider a scenario, when a base station or an intermediate node is connected to an Ambient IoT device. The device requires a carrier signal for the uplink transmission of signal. This proposal discusses the scenario where the carrier signal is provided externally to the device. In this scenario, the Ambient IoT device receives the carrier signal from an external source. The device then encodes information on the received carrier signal and backscatters to the receiver (gNB / intermediate node).

The device can adopt two configurations for the uplink transmission

- Transmission with Frequency Shift (FS):

In this case, the carrier signal received at the Ambient IoT device is different from the one used by the Ambient IoT device for its uplink transmission. Hence the device has to perform a frequency shift on the carrier signal received from the external node, and then encode its uplink information on the shifted carrier.

- Transmission without Frequency Shift (NFS):

The carrier signal received at the Ambient IoT device is the same frequency that it uses for its uplink transmission. Hence no frequency shifting is needed.

If the Ambient IoT device supports both configurations, RAN 1 should explore the possibility of transmitting the carrier wave from the base station or intermediate node itself. In such a scenario, the carrier wave could be transmitted in the downlink band and the Ambient IoT device could still backscatter at its desired uplink band (different from the carrier received externally) by performing frequency switching.

Proposal 1: Study the feasibility of an Ambient IoT device to perform frequency switching of a received carrier wave to the desired frequency and subsequently encode uplink data on the shifted carrier.

  - Management of Interference between external carrier transmission and existing 5G NR signaling

Proposal 2: Study the impact and management of potential interference arising from joint transmission of external carrier wave and existing 5G NR signaling.

# Conclusion

In this contribution, we have provided a discussion on the scenario in which a carrier wave is provided externally to an Ambient IoT device for Release 19. The discussion yields the following proposals:

Proposal 1: Study the feasibility of an Ambient IoT device to perform frequency switching of a received carrier wave to the desired frequency and subsequently encode uplink data on the shifted carrier.

Proposal 2: Study the impact and management of potential interference arising from joint transmission of external carrier wave and existing 5G NR signaling.

# References

[1] 3GPP TR 38.848 Technical Specification Group Radio Access Network; Study on Ambient IoT in RAN (Release 18)

[2] RP-234058 RAN #102 Edinburgh December 2023 Study on solutions for Ambient IoT (Internet of Things) in NR