3GPP TSG RAN WG1 #116	R1-2401166

Athens, Greece, February 26th – March 1st, 2024

Source:	Sharp

Title:	Remaining issues on Further NR Coverage Enhancements

Agenda Item:	8.6

Document for:	Discussion and Decision

Introduction

In RAN1#114, the agenda of CovEnh - PRACH enhancement has been completed and mainly TS38.213 capture the previous agreements on the CovEnh - PRACH. In this contribution, we share the view of remaining issues on CovEnh - PRACH for Rel-18 coverage enhancement.

Discussions

Mask for multiple PRACH transmissions

In RAN1#115, how to apply RO masking for preamble repetitions case has been discussed, and following has been agreed for further down-selection [1]:

Agreement For multiple PRACH transmissions with indication of PRACH mask index, down-select one of the following options at RAN1#116 - Option 1: UE applies PRACH mask prior to RO group determination. RO group is determined based on the ROs indicated by the PRACH mask index. - Option 2: UE applies PRACH mask after RO group determination. UE transmits PRACH with preamble repetitions only on a RO group with all the ROs indicated by the mask. - Option 3: UE applies PRACH mask after RO group determination. UE transmits PRACH with preamble repetitions only on a RO group where at least one RO of this RO group is indicated by the mask - Option 4: UE applies PRACH mask after RO group determination. The PRACH mask index indicates one or multiple RO groups for multiple PRACH transmission. - Note: this implies the PRACH mask index indicates the RO group instead of RO index(es).

To compare above 4 options, the difference of applicability of RO masking may occurs when RO mask index indicates the availability of ROs “per time instance” (i.e. RO mask index does not indicate a part of ROs in time domain). For example, when current RO mask index is applied for preamble repetitions, following 3 typical cases can be considered.

Figure 1: example of RO masking (msg1-FDM:8, ssb-PerRACH-Occasion: 1/4, mask index: 1)

Figure 2: example of RO masking (msg1-FDM:2, ssb-PerRACH-Occasion: 1/8, mask index: 10)

Figure 3: example of RO masking (msg1-FDM:2, ssb-PerRACH-Occasion: 1/8, mask index: 1)

Figure 1 shows the case one RO mapping cycle is confined in one time instance. In this case, masked/unmasked ROs are same between different time instance and there is no case to mask a part of ROs for a RO group. Figure 2 shows the case on RO mapping cycle is across multiple time instance, and mask index indicates every even ROs or every odd ROs. Also in this case, if msg1-FDM > 2, there is no case to mask a part of ROs for a RO group. Therefore, in these 2 cases, the RO masking works for preamble repetitions without partial masking, and all listed 4 options works well without any operation difference.

Observation 1: As long as the RO mask index indicates the availability of ROs “per time instance” (i.e. RO mask index does not indicate a part of ROs in time domain), there are no operation difference between listed 4 options.

On the other hand, as shown in figure 3, if the mask index indicates a specific RO index in the same case with figure 2, the masked RO and unmasked RO is different between different time instance and most of ROs for a RO group are indicated as unavailable (in case that RO masking is applied after RO group determination), then listed 4 options have different approaches.

Option 1: RO groups are determined (mapped) on valid ROs which are available after RO masking

- Pros: RO masking does not impact the availability of determined RO group

- Cons:

  - When RO masking indicates a part of ROs in time domain, the time duration of a RO group may be long which cause latency issue

  - RO group determination depends on RO masking which may cause large impact

Option 2: RO groups are determined (mapped) on valid ROs before RO masking and each RO group is available if all ROs in the RO group are available by RO masking

- Pros: It may not require any spec change

- Cons: There is restriction on application of RO mask index (If RO mask index indicates a part of ROs in time domain, all RO groups would not be unavailable (i.e. UE does not expect RO masking in time domain))

Option 3: RO groups are determined (mapped) on valid ROs before RO masking and each RO group is available if at least one RO in the RO group is available by RO masking

- Pros: There is no restriction on application of current RO mask index for RO groups

- Cons: Small spec changes are required

Option 4: RO groups are determined (mapped) on valid ROs before RO masking and each RO group is available if it is available by RO group masking

- Pros: Full flexibility on application of mask index for RO groups

- Cons:

  - RO group masking may not align with legacy RO masking

  - Large spec changes including 38.321 are required for RO group masking

Firstly, considering the phase of maintenance, large spec modification should be avoided as much as possible. In our view, Option 1 and Option 4 requires modification of description relating to the RO masking. Even considering the scheduling flexibility, we don’t find strong necessity to apply these options.

Observation 2: Considering it is maintenance phase, the specification impact should be minimized as long as the legacy RO mask index can be applied for preamble repetition case.

Proposal 1: For RO masking for PRACH repetition case, Option 1 and Option 4 should NOT be applied.

Comparing Option 2 and Option 3, the difference is the scheduling flexibility with RO masking and the spec impact.

Regarding the scheduling flexibility, for separation of RACH resources for PRACH repetition, there are several schemes:

- Dedicated RACH configuration (AdditionalRACH-Config) for PRACH repetition

- Preamble partitioning

- RO masking

  - Frequency domain (the number of ROs in a RO mapping cycle <= msg1-FDM)

  - Time domain (the number of ROs in a RO mapping cycle > msg1-FDM)

From our view, the concept of Option 2 is that the separation of PRACH repetitions is achieved by a combination of above schemes other than “Time domain RO masking” and time domain RO masking can be applied for the separation between different feature combinations (FCs) without PRACH repetitions. On the other hand, the concept of Option 3 is full acceptance of above schemes with small spec changes.

Since we assume there are not big flexibility impact on Option 2, less spec impact is more preferred with Option 2. However, if majority supports more flexibility with the spec modification, we are open to go to Option 3.

Observation 3: Based on the consideration of balance between the necessity of more scheduling flexibility and spec impact, Option 2 or Option 3 should be selected.

Power ramping counter

In RAN1#115, how to apply the procedure of the suspension of power ramping counter has been discussed, and following has been agreed for further down-selection [1]:

Agreement For multiple PRACH transmissions, down-select one of the following options at RAN1#116: Option 1: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped or with reduced transmit power. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in any of PRACH occasions are dropped or with reduced transmit power. Option 1a: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission on part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power. Option 2: - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped or with reduced transmit power. Note: this implies it’s up to UE implementation. Option 2a: - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is with reduced transmit power. - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped. Option 3: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are with reduced transmit power. Note: whether any of the above options have specification impact is a separate discussion.

Regarding the case of dropping of PRACH transmission and PRACH transmission with reduced power, following is described in 38.213 [2].

38.213 7.4 Physical random access channel If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, or due to slot format determination as described in clause 11.1, or due to the PUSCH/PUCCH/PRACH/SRS transmission occasions are in the same slot or the gap between a PRACH transmission and PUSCH/PUCCH/SRS transmission is small as described in clause 8.1, or due to HD-UE operation in paired spectrum as described in clause 17.2, the UE does not transmit a PRACH in a transmission occasion, Layer 1 notifies higher layers to suspend the corresponding power ramping counter. If due to power allocation to PUSCH/PUCCH/PRACH/SRS transmissions as described in clause 7.5, or due to power allocation in EN-DC or NE-DC or NR-DC operation, the UE transmits a PRACH with reduced power in a transmission occasion, Layer 1 may notify higher layers to suspend the corresponding power ramping counter.

In legacy procedure (i.e. for single PRACH transmission), the notification to suspend the power ramping counter is sent to higher layer when the single PRACH transmission is failed and the notification may be sent to higher layer when the single PRACH transmission is performed with reduced power.

For PRACH transmission with preamble repetitions, comparing the case of single preamble transmission, following aspects should be considered:

- Alignment with legacy procedure

  - When all preamble repetitions are dropped, whether the notification of suspending power ramping counter should be mandated (as same as legacy procedure) or not

  - When all preamble repetitions are transmitted with reduced power, whether the notification of suspending power ramping counter can be applied (as same as legacy procedure) or not (or should be mandated)

- Suspension for partial dropping/reduced power

  - When a part of preamble repetitions is dropped, whether the notification of suspending power ramping counter should be applied or not (or should be mandated)

  - When a part of preamble repetitions is transmitted with reduced power, whether the notification of suspending power ramping counter can be applied or not (or should be mandated)

- Determination of suspension notification based on a part of preamble repetitions

  - Whether the UE can determine to suspend/not suspend by checking one or a part of preamble repetitions

Observation 4: To down-select from the listed options, following aspects are considered:

- Alignment with legacy procedure

- Suspension for partial dropping/reduced power

- Determination of suspension notification based on a part of preamble repetitions

Regarding 3 aspects above, the comparison of listed 5 options is summarized as Table 1.

Table 1. Comparison of listed options for the aspects

|  | Option 1 | Option 1a | Option 2 | Option 2a | Option 3 |
| --- | --- | --- | --- | --- | --- |
| Alignment with legacy procedure in case of dropping of (all) preamble repetitions | Yes | Yes | No (can align by UE implementation) | Yes | Yes |
| Alignment with legacy procedure in case of reduced power for (all) preamble repetitions | No (Suspension is mandated) | Yes | Yes | Yes | Yes |
| Ramping counter suspension with partial dropping of preamble repetitions | Possible to suspend | Possible to suspend | Possible to suspend | Always suspended | Always ramped up |
| Ramping counter suspension with partial reduced power of preamble repetitions | Possible to suspend | Possible to suspend | Possible to suspend | Possible to suspend | Always ramped up |
| Availability of determination of the suspension with one of the PRACH occasions | No | No | Yes | No | No |

Aspect 1 (Alignment with legacy procedure)

Aspect 1-1: When all of preamble repetitions are dropped, it means the failure of Msg1 attempt and power ramping is unnecessary approach as same as legacy procedure. In this perspective, the suspension of power ramping should be baseline and the options other than Option 2 aligns with this approach. For option 2, whether to suspend power ramping can be up to UE implementation.

Aspect 1-2: When all of preamble repetitions are transmitted with reduced power, it means the Msg1 attempt is performed without enough transmission power for target received power in any preamble transmission, and the power ramping may not be unnecessary as same as legacy procedure. Assuming the case follows the principle, the options other than Option 1 should be prioritized and the mandatory suspension in Option 1 is unnecessary restriction comparing with option 1a.

Observation 5: Mandatory suspension for the case that all of preamble repetitions are transmitted with reduced power in Option 1 is unnecessary restriction comparing with option 1a.

Aspect 2 (Suspension for partial dropping/reduced power)

When a part of preamble repetitions is dropped or reduced, it means the Msg1 attempt is performed but not performed with fully desirable performance of preamble repetitions, and the case is similar with the case of legacy procedure with reduced power. From this perspective, we prefer to put it as up to UE implementation (i.e. possible to suspend) and mandatory suspension for the case that a part of preamble repetitions is dropped in Option 2a seems unnecessary restriction.

Observation 6: Mandatory suspension for the case that a part of preamble repetitions is dropped in Option 2a is unnecessary restriction comparing with Option 1a and Option 2.

Observation 7: Suspension for the case that a part of preamble repetitions is dropped or is transmitted by reduced power should be able to be applied.

Aspect 3 (Determination of suspension notification based on a part of preamble repetitions)

If suspension notification of power ramping counter is mandated based on dropping of all of preamble repetitions, the UE is required to check whether every preamble of the preamble repetitions is dropped or not, and it is not desirable from UE implementation perspective. In other words, to enable to determine to notify the suspension based on one or a part of preamble repetitions, it is impossible to mandate the notification.

Observation 8: Determination to notify the suspension based on one or a part of preamble repetitions is desirable from UE implementation point of view.

Observation 9: To enable to determine to notify the suspension based on one or a part of preamble repetitions, it is impossible to mandate the notification for all dropping case.

In that point, only Option 2 allows UE to determine the necessity of notification based on one (or a part) of preamble repetitions (e.g. last repetition). Although Option 2 may allow power ramping even if all of the preamble repetitions is dropped (and it does not align with legacy procedure), we assume the impact is quite limited since the case of all dropping may be a corner case.

Proposal 2 Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped or with reduced transmit power.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: As long as the RO mask index indicates the availability of ROs “per time instance” (i.e. RO mask index does not indicate a part of ROs in time domain), there are no operation difference between listed 4 options.

Observation 2: Considering it is maintenance phase, the specification impact should be minimized as long as the legacy RO mask index can be applied for preamble repetition case.

Proposal 1: For RO masking for PRACH repetition case, Option 1 and Option 4 should NOT be applied.

Observation 3: Based on the consideration of balance between the necessity of more scheduling flexibility and spec impact, Option 2 or Option 3 should be selected.

Observation 4: To down-select from the listed options, following aspects are considered:

- Alignment with legacy procedure

- Suspension for partial dropping/reduced power

- Determination of suspension notification based on a part of preamble repetitions

Observation 5: Mandatory suspension for the case that all of preamble repetitions are transmitted with reduced power in Option 1 is unnecessary restriction comparing with option 1a.

Observation 6: Mandatory suspension for the case that a part of preamble repetitions is dropped in Option 2a is unnecessary restriction comparing with Option 1a and Option 2.

Observation 7: Suspension for the case that a part of preamble repetitions is dropped or is transmitted by reduced power should be able to be applied.

Observation 8: Determination to notify the suspension based on one or a part of preamble repetitions is desirable from UE implementation point of view.

Observation 9: To enable to determine to notify the suspension based on one or a part of preamble repetitions, it is impossible to mandate the notification for all dropping case.

Proposal 2 Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped or with reduced transmit power.

References

“RAN1 Chairman’s Notes,” RAN1 #115 November 2023.

TS 38.213 (V18.1.0), December 2023.