3GPP TSG RAN WG1 #116		   		 	         	    	         R1-2401187

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:  9.2.1

Source:	ITRI

Title:	Discussion on UE-initiated/event-driven beam management

Document for:	Discussion and Decision

Introduction

In RAN meeting #102, the new working item on NR MIMO Phase 5 was approved for Release 19. One of the key objectives in WID [1] is:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

According to the above description, the UE-initiated beam reporting will be one of the major issues in the following meetings in Rel-19. Thus, in this contribution, we provide our views on the UE-initiated beam reporting.

UE-initiated beam reporting

For establishing the best DL beam pair link between Tx beam at gNB side and Rx beam at UE side, the UE could report beam index SSBRI/CRI and corresponding beam quality L1-RSRP/L1-SINR via CSI reporting configuration. The timing behaviour of the beam reporting could be periodic, semi-persistent, or aperiodic according to the configuration. However, the reporting periodicity is determined by the gNB. When beam reporting is configured with long periodicity, gNB may acquire the outdated beam information, but when beam reporting is configured with short periodicity, UE may report similar beam information in consecutive reporting instances reducing UL resource utilization and wasting UE’s UL power consumption. Therefore, the UE-initiated beam reporting would be a alternative option, because the UE is more aware of the beam’s variation than gNB. To perform the procedure of UE initiated beam reporting, we consider triggering condition(s), some reporting contents, a reporting medium, and the impact on current CSI framework.

Triggering condition

To reduce redundant beam information reporting, the beam report should focus on the variation of quality of measured RS(s), because it would be helpful for gNB to determine whether to apply new TCI state for channel(s)/RS(s) reception. Therefore, the UE-initiated beam reporting can be triggered if the best beam is changed or the variation of quality of the best beam is larger than a threshold.

Proposal 1: If the variation of quality of the best beam is larger than a threshold, the UE-initiated beam reporting is triggered.

Reporting medium

Since it is a UE-initiated beam reporting, it’s not necessary to pre-reserve a UL resource for beam reporting. To save UL resource overhead, beam information can be reported via MAC CE through PUSCH. If there is no UL resource for transmitting the beam information, UE could send a SR for UE-initiated beam reporting. Note that in current specification, the procedure of UE-initiated beam reporting was introduced but for SCell BFR, where UE may report new beam information through BFR MAC CE and if there is no PUSCH for reporting BFR MAC CE, UE can send a SR for BFR.

Proposal 2: Support MAC CE based reporting medium. If there is no UL resource for transmitting the beam information, UE can send a SR for UE-initiated beam reporting.

Reporting contents

UE may not be configured with only one UE-initiated beam reporting, e.g., reporting L1-RSRP for monitoring receiving power of the serving beam, and reporting L1-SINR for determining whether to co-schedule a pair of UEs. In this case, gNB needs to know the reporting is associated with which measurement object and corresponding measurement quantity. Therefore, CSI report configuration ID can also be reported in a reporting instance in addition to the current reporting quantity i.e., SSBRI/CRI and corresponding L1-RSRP/L1-SINR.

Proposal 3: Support reporting CSI report configuration ID in a reporting instance in addition to the current reporting quantity i.e., SSBRI/CRI and corresponding L1-RSRP/L1-SINR.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: If the variation of quality of the best beam is larger than a threshold, the UE-initiated beam reporting is triggered.

Proposal 2: Support MAC CE based reporting medium. If there is no UL resource for transmitting the beam information, UE can send a SR for UE-initiated beam reporting.

Proposal 3: Support reporting CSI report configuration ID in a reporting instance in addition to the current reporting quantity i.e., SSBRI/CRI and corresponding L1-RSRP/L1-SINR.

References

[1]    RP-234007, “New WID: NR MIMO Phase 5”