3GPP TSG RAN WG1 #116	R1-2401200

# Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	8.5

Source: 	ASUSTeK

Title:  	Discussion on early PRACH

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss remaining issues for early PRACH.

- Discussion

Intra-band CA scenario

RAN1 #114 Agreement When the UE does not support simultaneous/parallel transmissions of PRACH in candidate cell and UL channels and signals in serving cell, support - serving cell UL TX is dropped. TS 38.213, i10 21 L1/L2-triggered mobility procedures < omitted > A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. If the serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause 8.1, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX < omitted >

As agreed in RAN1 114e, when the UE does not support simultaneous UL transmission between candidate cell and serving cell, the UE drops UL transmission on the serving cell. However, it’s not clear whether UL transmission on another serving cell, that is different than PDCCH order reception cell, in same frequency range would be dropped due to the PRACH transmission. Thus, for intra-band CA, it’s proposed to clarify whether UL transmission on another serving cell in same frequency band as candidate cell is dropped or not. There are two options to clarify UE’s behaviour for this issue that option 1 is that UE only drops UL transmission on serving cell, in same frequency range as candidate cell, that the UE receives PDCCH order triggering PRACH while option 2 is that UE drops UL transmission(s) on serving cell(s), in same frequency range as candidate cell.

Observation 1: According to section 21 in TS 38.213, it’s not clear whether UL transmission on another serving cell, that is different than “the serving cell”, in same frequency range would be dropped due to the PRACH transmission.

Proposal 1:

- For intra-band carrier aggregation, when the UE does not support simultaneous/parallel transmissions of PRACH in candidate cell and UL transmissions on serving cell, RAN1 down-select one of following option:

- Opt1: UE drops UL transmission on serving cell only, in same frequency range as candidate cell, that the UE receives PDCCH order triggering PRACH.

- Opt2: UE drops UL transmission(s) on serving cell(s), in same frequency range as candidate cell. (i.e., including serving cell(s) where the UE does not receive PDCCH order triggering PRACH)

- Applies Text proposal 1, if Opt1 is selected, or Text proposal 2, if Opt2 is selected.

Reason for change: UL transmission dropping considering intra-band carrier aggregation is missing

Summary for change: To specify the serving cell where the UE receives the PDCCH order.

Consequences if not approved: For intra-band carrier aggregation, impacted serving cell due to PRACH on candidate cell is not clear

Text proposal 1 21 L1/L2-triggered mobility procedures < omitted > A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. If the serving cell where the UE receives the PDCCH order and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause 8.1, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX < omitted >

Reason for change: UL transmission dropping considering intra-band carrier aggregation is missing.

Summary for change: To not bundle impacted serving cell due to PRACH on candidate cell as only the serving cell where the UE receives the PDCCH order.

Consequences if not approved: For intra-band carrier aggregation, impacted serving cell due to PRACH on candidate cell is not clear

Text proposal 2 21 L1/L2-triggered mobility procedures < omitted > A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. If a serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause 8.1, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX < omitted >

Cell Indicator field in DCI

TS 38.331, i00 #### 5.3.5.18 LTM configuration and execution ##### 5.3.5.18.1 LTM configuration The network configures the UE with one or more LTM candidate configurations within the LTM-Config IE. In NR-DC, the UE may receive two independent ltm-Config: - an ltm-Config associated with the MCG that is included within an RRCReconfiguration message received via SRB1; and - an ltm-Config associated with the SCG that is included within an RRCReconfiguration message either received via SRB3, or, alternatively, embedded in an RRCReconfiguration message received via SRB1. TS38.212. i10 - 7.3.1.2 DCI formats for scheduling of PDSCH - 7.3.1.2.1 Format 1_0 < omitted > - Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUlSyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity. < omitted >

According to section 5.3.5.18.1 in TS 38.331, UE operating in NR-DC may receive two independent LTM-Config, which corresponds to MCG and SCG, respectively. Besides, one LTM-Config could provide up to 8 LTM-Candidate, which each LTM-Candidate could be configured with EarlyUL-SyncConfig. That is, size of Cell indicator field in DCI format 1_0 is based on the number of candidate cells configured with higher layer parameter EarlyUL-SyncConfig (e.g., C). Also, order of each code-point mapping to which LTM-Candidate is based on ascending order of LTM-Candidate ID. However, if NR-DC is configured, current description in TS 38.212 seems to describe up to 16 candidate cells configured with higher layer parameter EarlyUL-SyncConfig being used to determine size of Cell indicator field resulting same set of LTM-Candidate ID for both serving cell groups. Thus, we think RAN1 should clarify whether PDCCH order indicating a candidate cell supports cross-CG indication or not. If cross-CG indication is not supported, text proposal 3 is proposed to apply.

Observation 2: According to section 5.3.5.18.1 in TS 38.331, UE operating in NR-DC may receive two independent ltm-Config, and the UE could be configured up to 16 LTM-Candidate with EarlyUL-SyncConfig.

Proposal 2: Regarding size determination of Cell indicator field in DCI format 1_0, RAN1 to clarify whether PDCCH order indicating a candidate cell supports cross-CG indication or not. If not supported, text proposal 3 is proposed to apply

Reason for change: For NR-DC, it’s not clear whether size of cell indicator field in DCI format 1_0 is based on one serving cell group or both serving cell groups.

Summary for change: To not bundle impacted serving cell due to PRACH on candidate cell as only the serving cell where the UE receives the PDCCH order.

Consequences if not approved: For NR-DC, ascending order of a candidate identity cannot work for determining each bit field index due to same candidate identity set for MCG and SCG.

Text proposal 3 - 7.3.1.2 DCI formats for scheduling of PDSCH - 7.3.1.2.1 Format 1_0 < omitted > - Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig and associated with same serving cell group as the serving cell for receiving DCI format 1_0; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUlSyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity. < omitted >

- Conclusion

In this contribution, we have following proposals:

Observation 1: According to section 21 in TS 38.213, it’s not clear whether UL transmission on another serving cell, that is different than “the serving cell”, in same frequency range would be dropped due to the PRACH transmission.

Proposal 1:

- For intra-band carrier aggregation, when the UE does not support simultaneous/parallel transmissions of PRACH in candidate cell and UL transmissions on serving cell, RAN1 down-select one of following option:

- Opt1: UE drops UL transmission on serving cell only, in same frequency range as candidate cell, that the UE receives PDCCH order triggering PRACH.

- Opt2: UE drops UL transmission(s) on serving cell(s), in same frequency range as candidate cell. (i.e., including serving cell(s) where the UE does not receive PDCCH order triggering PRACH)

- Applies Text proposal 1, if Opt1 is selected, or Text proposal 2, if Opt2 is selected.

Reason for change: UL transmission dropping considering intra-band carrier aggregation is missing

Summary for change: To specify the serving cell where the UE receives the PDCCH order.

Consequences if not approved: For intra-band carrier aggregation, impacted serving cell due to PRACH on candidate cell is not clear

Text proposal 1 21 L1/L2-triggered mobility procedures < omitted > A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that the UE receives on a serving cell and includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. If the serving cell where the UE receives the PDCCH order and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause 8.1, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX < omitted >

Reason for change: UL transmission dropping considering intra-band carrier aggregation is missing

Summary for change: To not bundle impacted serving cell due to PRACH on candidate cell as only the serving cell where the UE receives the PDCCH order.

Consequences if not approved: For intra-band carrier aggregation, impacted serving cell due to PRACH on candidate cell is not clear

Text proposal 2 21 L1/L2-triggered mobility procedures < omitted > A UE can be provided configurations, by EarlyUlSyncConfig, for PRACH transmission parameters for each of the candidate cells. The UE can be triggered a PRACH transmission on a candidate cell by a PDCCH order that includes an indication of the candidate cell for the PRACH transmission [4, TS 38.212]. If a serving cell and the candidate cell operate in a same frequency range and the UE would have transmissions that overlap in time, or when a gap between a first or last symbol of a PRACH transmission to the candidate cell is less than 𝑁 symbols from a last or first symbol, respectively, of an UL transmission to the serving cell, where N is defined in Clause 8.1, the UE - drops the transmissions on the serving cell when the UE does not support transmissions that overlap in time or are separated by less than the gap on the serving cell and the candidate cell - prioritizes power allocation to the PRACH transmission on the candidate cell in clause 7.5 when the UE supports transmissions that overlap in time or are separated by less than the gap, and a total UE transmit power in the frequency range would exceed PCMAX < omitted >

Observation 2: According to section 5.3.5.18.1 in TS 38.331, UE operating in NR-DC may receive two independent ltm-Config, and the UE could be configured up to 16 LTM-Candidate with EarlyUL-SyncConfig.

Proposal 2: Regarding size determination of Cell indicator field in DCI format 1_0, RAN1 to clarify whether PDCCH order indicating a candidate cell supports cross-CG indication or not. If not supported, text proposal 3 is proposed to apply

Reason for change: For NR-DC, it’s not clear whether size of cell indicator field in DCI format 1_0 is based on one serving cell group or both serving cell groups.

Summary for change: To not bundle impacted serving cell due to PRACH on candidate cell as only the serving cell where the UE receives the PDCCH order.

Consequences if not approved: For NR-DC, ascending order of a candidate identity cannot work for determining each bit field index due to same candidate identity set for MCG and SCG.

Text proposal 3 - 7.3.1.2 DCI formats for scheduling of PDSCH - 7.3.1.2.1 Format 1_0 < omitted > - Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig and associated with same serving cell group as the serving cell for receiving DCI format 1_0; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUlSyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity. < omitted >

- Reference