g3GPP TSG RAN WG1 Meeting #116	R1-2401210

Athens, Greece, February 26th – March 1st, 2024

Source:	Fujitsu

Title:	Discussion on SBFD random access operation

Agenda Item:	9.3.2

Document for:	Discussion

# Introduction

In previous RAN plenary meeting, the WID related with SBFD operation in both RRC_CONNECTED and RRC_IDLE/INACTIVE state is decided as shown in below.

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2] - Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN#104 to check whether to proceed normative work

In study item phase, there was no chance to discuss about random access operation in RRC_CONNECTED state as well as RRC_IDLE/INACTIVE state. Therefore, we need to discuss and investigate its pros and cons per use case. In this contribution, we discuss on SBFD random access operation in both RRC_IDLE/INACTIVE and RRC_CONNECTED.

# Random access configuration in SBFD operation

This section is common for RRC_IDLE/INACTIVE and RRC_CONNECTED. One of the most important issues to achieve random access configuration in SBFD operation is the method how to configure the RACH occasion on UL subband. Since the bandwidth of UL subband is different from that of UL BWP, we need to carefully configure RACH occasion.

By making use of SBFD symbols, RACH occasions (ROs) can be configured onto an UL subband. However, the current PRACH configuration does not fit well to configure RO on UL subband due to different bandwidth between UL subband and UL BWP. Hence, we firstly need to determine how to configure ROs on SBFD symbols. One may say that the common PRACH configuration can be applied to both legacy UE and SBFD aware UE. In this case, there is an issue that multiple SSBs are mapped in one RO. Figure 1 shows an example of the common PRACH configuration. Assuming that three ROs, say RO#1, RO#2 and RO#3, are configured on both UL subband and UL only symbols with one PRACH configuration, each RO is corresponding to an SSB. It is noteworthy that an SBFD-aware UE can recognize all of three ROs, but a legacy UE can recognize only RO#3 configured on UL only symbol.

Figure 1. RO configuration on UL subband and UL only symbol

Each UE performs SSB-to-RO mapping onto its own ROs, that is, the SBFD-aware UE will map as [(SSB0-RO#1), (SSB1-RO#2), (SSB2-RO#3)] and the legacy UE will map as [(SSB0-RO#3)]. In this case, there is a critical issue that the RO#3 is mapped with different SSB per UE, that is, SBFD-aware UE maps SSB2 while the legacy UE maps SSB0 on RO#3, respectively. From the viewpoint of the gNB, it is difficult to distinguish the PRACH preamble received in RO#3. Furthermore, the gNB suffers from the reception beam setting.

Observation1: The common PRACH configuration for both SBFD aware UE and the legacy UE makes gNB to have difficulty to configure PRACH, properly.

On the other hand, the separate PRACH configuration can be used between SBFD-aware UE and the legacy UE, respectively. In this case, since the configuration is independent from each other, the issue raised in the common PRACH configuration will not occur. Moreover, if SBFD operation in RRC_IDLE/INACTIVE states is supported, the gNB needs to recognize the UE capability of SBFD as soon as the PRACH preamble is received. This behaviour is important because the gNB can determine whether the subsequent scheduling (e.g., msg.3) is indicated on UL subband or UL only symbol. This configuration can achieve by using feature combination framework defined in Rel-16.

Based on our analysis above, we propose that the separate PRACH configuration for the SBFD-aware UE is beneficial if RACH occasion is configured on UL subband.

Proposal 1: The separate PRACH configuration for the SBFD-aware UE is beneficial if RACH occasion is configured on UL subband.

# SBFD operation in RRC_IDLE / INACTIVE state

In study item phase, the topic about SBFD operation in RRC_IDLE / INACTIVE state was controversial. As a result, it is determined at RAN plenary that the study is kept continuing and will be decided whether it is accepted to discuss as WID at RAN#104. As merits of SBFD operation in RRC_IDLE / INACTIVE, firstly, the latency of random access can be improved. Also, because of the concatenated uplink resources, long preamble of PRACH format can be used, and thus we can enhance the PRACH coverage. But the demerit is a severe cross link interference.

## SBFD configuration on RRC_IDLE / INACTIVE state

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

In RAN1#110b meeting, we made an agreement that both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs at least for RRC_CONNECTED state. However, we need to discuss whether such locations for SBFD operation are known to SBFD aware UEs for RRC_IDLE/INACTIVE state. For the time locations of subbands, it is required to determine the valid RO. For example, assuming that the slot format is [DDDFU] and the UL subband is configured on the slots from second to fourth slot (i.e., DXXXU). If the ROs are configured in every slot, then the ROs configured in X and U slots are valid while the ROs configured in D are invalid based on the time locations of subbands.

On the other hand, one may say that the frequency location is not needed for the SBFD-aware UE in RRC_IDLE state. This is because the gNB can implicitly guarantee the frequency resources of uplink transmission using UL subband is located within the UL subband such as RACH occasion or Msg.3 FDRA. Having said that, such gNB’s guarantee requires additional standardization efforts. For instance, if the frequency location of subband is not known to SBFD-aware UE, RO is configured on the nominal DL slot which is indicated as SBFD symbol by time location. The UE behaviour on the nominal DL slot should be defined additionally. Alternatively, the gNB simply indicates the frequency location of subbands to the SBFD-aware UE. This direct indication is more efficient way rather than the implicit indication.

Proposal 2: Both time and frequency locations of subbands for SBFD operation are also known to the SBFD aware UE in RRC_IDLE/INACTIVE state.

Based on our analysis, now the further question is that how to indicate the time and frequency locations to SBFD-aware UE in RRC_IDLE/INACTIVE state. We have only a few choices for the indication such as MIB or SIB. As well-known fact, the MIB has no room to carry the indication. Then, rest options are SIB1 or other system information rather than SIB1. As the SBFD configuration is strongly related with the slot format indication carried by TDD-UL-DL-ConfigCommon in SIB1, it is beneficial to indicate the time and frequency locations of subbands for operation by SIB1.

Proposal 3: The SBFD configuration for time and frequency resources can be provided by SIB1.

In RRC_CONNECTED state, the gNB can indicate the transmission direction to the UE with several methods such as direct signalling with scheduling/non-scheduling DCI or semi-static indication. Similarly, the transmission direction for msg1/2/3/4 in RRC_IDLE/INACTIVE state can be designated by same manner with RRC_CONNECTED. At this point, the random access procedure of msg2/3/4 is directly controlled by the gNB, and the msg1 should be transmitted only on the UL resources. It means, the transmission direction indication is not required in RRC_IDLE/INACTIVE state, but it can be determined according to the random access procedure, naturally.

Observation 2: The transmission direction of the UE in RRC_IDLE state is deterministic that Msg1 by valid RO and Msg2/3/4 by gNB’s scheduling.

Proposal 4: For SBFD aware UE in RRC_IDLE/INACTIVE state, gNB does not have to separately indicate the transmission direction of Msg1/2/3/4 on SBFD symbols.

## CORESET0 overlapping with UL subband

In study item phase, there were several discussions about channel overlapping with UL subband. If SBFD operation in RRC_IDLE/INACTIVE is supported, then the overlapping CORESET0 with UL subband in time and frequency domain should be considered. We can consider three cases for the overlapping as shown in Fig.2.

Figure 2. CORESET0 overlapping with UL subband. (a) The RBs of CORESET0 are excluded, (b) The RBs of UL subband are excluded, (c) UL subband is configured in the resources not overlapping with CORESET0.

Figure 2 (a) shows that the UL subband is prioritized, and as a result, the usable PRBs of CORESET0 are reduced. As defined in TS38.213, the CORESET0 bandwidth can be set with three kinds of bandwidth such as 24PRBs (4CCEs), 48PRBs (8CCEs) and 96PRBs (16CCEs). Moreover, the search space in random access procedure is common search space (CSS) where the aggregation level is set from AL4. Based on this fact, if the frequency resources of CORESET0 overlapping with UL subband are skipped/punctured/excluded as shown in Fig.2 (a), the performance of PDCCH transmitted on CSS will significantly decrease. For the worst case, the PDCCH cannot be mapped to the CORESET0. Hence, it is beneficial to protect the frequency resources of CORESET0 from the overlapping case. As a solution, we can consider two cases of UL subband configuration as shown in Fig.2 (b) and (c). In Fig.2 (b), the UL subband is not configured in the resources overlapping with CORESET0. Consequently, the CORESET0 performance is guaranteed, but the number of transition point increases. We can consider another alternative that the UL subband is not overlapped with CORESET0 in frequency domain as shown in Fig2 (c). In this case, the position and the resources of UL subband in frequency domain are restricted.

Alternatively, the gNB can configure the CORESET0 which is not overlapping with UL subband in time and frequency domain. In this case, however, it affects severe configuration restriction, and thus we prefer to preclude such case.

Proposal 5: Study further the case that the UL subband is overlapping with CORESET0 in time and frequency domain.

# Conclusion

In this contribution, we provide our view as below.

Observation1: The common PRACH configuration for both SBFD aware UE and the legacy UE makes gNB to have difficulty to configure PRACH, properly.

Proposal 1: The separate PRACH configuration for the SBFD-aware UE is beneficial if RACH occasion is configured on UL subband.

Proposal 2: Both time and frequency locations of subbands for SBFD operation are also known to the SBFD aware UE in RRC_IDLE/INACTIVE state.

Proposal 3: The SBFD configuration for time and frequency resources can be provided by SIB1.

Observation 2: The transmission direction of the UE in RRC_IDLE state is deterministic that Msg1 by valid RO and Msg2/3/4 by gNB’s scheduling.

Proposal 4: For SBFD aware UE in RRC_IDLE/INACTIVE state, gNB does not have to separately indicate the transmission direction of Msg1/2/3/4 on SBFD symbols.

Proposal 5: Study further the case that the UL subband is overlapping with CORESET0 in time and frequency domain.

# References

- RP-234035, New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.

- 3GPP TR 38.858 V1.1.0 (2023-11), Study on Evolution of NR Duplex Operation (Release 18)