3GPP TSG RAN1 WG1 Meeting #116 	R1-2401239

Athens, Feb 26 – Mar 01 2024

Source:	ETRI

Title:	Discussion on NR-NTN uplink capacity/throughput enhancement

Agenda Item:	9.11.3	NR-NTN uplink capacity/throughput enhancement

Document for:	Discussion/Decision

1	Introduction

In RAN#102, the new WID (NTN for NR Phase 3) [1] was approved with the following justification.

In Release 17, a work item was carried out to define solutions enabling New Radio and NG-RAN to support Non-Terrestrial Networks (NTN). Then, Release 18 introduced enhancements for NR NTN. As part of Release 19, a new work item is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks in order to: - … - Offer optimized capacity performance on uplink through multiplexing techniques, motivated by: - The coverage of NTN satellites is very wide, and considering device density, it is expected that a large number of UEs will be within a satellite’s coverage. Especially for LEO, a large number of UEs in coverage must succeed in transmitting desired data during a satellite coverage which means that rapid access to and release of satellite resources is required. - The total spectrum resources available to the network will be limited especially in the early phases of NR NTN deployments. - Some users will require higher resources than others, depending on their traffic patterns. Therefore, further granularity of resource multiplexing can significantly improve system capacity efficiency. - Possibly to allocate higher per-UE resources to better support VoNR/VoIP services in coverage-limited scenarios. - …

According to new WID [1], the following objectives may be discussed with the following assumptions in Rel-19 RAN1.

The objectives of the work item are the following: - … - Uplink Capacity/Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Study then specify, if beneficial, DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC) - Determine the achievable capacity improvement to be targeted taking into account realistic impairments (e.g. Doppler, time variation, phase distortion, etc) - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed - Note: The study can consider orthogonal cover codes across OFDM symbols, across slots, and/or within an OFDM symbol. - Note: the study phase is targeted to be completed by RAN#104 - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design - … The work item aims at specifying further enhancements for NG-RAN based NTN (Non-Terrestrial Networks) with the following assumptions: - GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS (Global Navigation Satellite Systems) capabilities - In frequency band above 10 GHz, both Terminal Type 1 (Electronic steering antenna) and Type 2 (Mechanical steering antenna) to be considered for GSO and NGSO - Implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios, where relevant Note 1: In Rel-19 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion).

2	Discussion

According to NTN WID for NR phase 3 [1], the study phase of DFT-s-OFDM PUSCH enhancements via OCC might be targeted by RAN#104. For efficient evaluation in study phase, the discussion and decision on the simulation parameters for evaluating capacity improvements might be necessary in this meeting.

In RAN1#109 [2], the simulation parameters of NTN coverage enhancement were agreed. Although those parameters were used for evaluating coverage enhancement, it seems that most of the simulation parameters might be re-usable again for evaluating capacity improvement with below modifications. The detail of modifications would be described as followings.

RAN1#109 Agreement For NR NTN coverage enhancement, evaluate the following cases. Case; Satellite orbit; Satellite parameter set; Elevation angle (deg); Terminal; Frequency band; Service type. 1; GEO; 1; 12.5; Handset; S-band; Low-data rate service. 2; GEO; 2; 20; Handset; S-band; Low-data rate service. 3 (Optional); LEO-1200; 1; 30; Handset; S-band; VoIP. 4; LEO-1200; 2; 30; Handset; S-band; VoIP. 5; LEO-1200; 2; 30; Handset; S-band; Low-data rate service. 6 (Optional); LEO-600; 1; 30; Handset; S-band; VoIP. 7; LEO-600; 2; 30; Handset; S-band; VoIP. 8 (Optional); LEO-600; 2; 30; Handset; S-band; Low-data rate service. 9 (Optional, with higher priority than case 10); MEO; 1; 30; Handset; S-band; Low-data rate service. 10 (Optional); MEO; 2; 30; Handset; S-band; Low-data rate service. Agreement For coverage performance evaluation, the following are assumed for all channels/signals - Channel model/Delay spread - Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS) - Evaluation scenario - Rural (LOS/NLOS) - Sub-urban (LOS/NLOS) (optional) - Channel estimation: Realistic estimation - Companies are encouraged to report channel estimation method. - SCS - 15 kHz only - UE speed: 3 km/h - Frequency drift: Not assumed - Frequency offset: 0.1 ppm Agreement For coverage evaluation of PUSCH in NR NTN, the following table is assumed. Parameter; Value. Frequency hopping; w/ or w/o frequency hopping. BLER; For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER.. Number of UE transmit chains; 1, 2 (optional). DMRS configuration; For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies.. Waveform; DFT-s-OFDM, CP-OFDM (optional). PUSCH duration; 14 OS. Repetitions; w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies.. HARQ configuration; Whether/How HARQ is adopted is reported by companies.. PRBs/TBS/MCS for low data rate service; Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead.. PRBs/MCS for VoIP; Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional). Other parameters; Reported by companies. RAN1#110 Conclusion For Rel-18 coverage enhancement in NTN, NLOS environment is deprioritized.

First of all, the modifications might be necessary on some mismatches between 38.821 and the parameters in RAN1#109. According to 38.821, the minimum elevation angle is given as 10 (deg) and the UE velocity is considered as 1200 km/h. However, the parameters in RAN1#109 specifies that that the values of minimum elevation angle and UE velocity are 30 (deg) and 3 km/h, respectively. Furthermore, for efficient evaluation, some extremely challenging scenarios like (GEO, MEO, satellite parameter set 2) might be deprioritized because CNR values for those scenarios are very poor in link budget analysis. The CNR values for all Rel-18 NTN UL coverage enhancement (CE) scenarios are given in Table 1. Considering that the link level performance of non-OCC transmission might be better than (or equal to) that of OCC transmission, the (GEO, MEO, satellite parameter set 2) scenarios might be more challenging with OCC and those might be deprioritized for evaluation. Moreover, for easy alignment of evaluation, the number of the scenarios might be reduced by deleting optional parameters, which are related on the number of UE transmit chains, channel model, evaluation scenario. Especially, NLOS environment was included in RAN1#109, but the de-prioritization of NLOS was concluded in RAN1#110. NLOS environment might be also deprioritized again for evaluating capacity improvement.

Table 1 CNR for Rel-18 NTN UL CE scenarios

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type | CNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | 12.5 | Handset | S-band | Low-data rate service | -19.391138 |
| 2 | GEO | 2 | 20 | Handset | S-band | Low-data rate service | -24.166476 |
| 3 | LEO-1200 | 1 | 30 | Handset | S-band | VoIP | -11.107511 |
| 4 | LEO-1200 | 2 | 30 | Handset | S-band | VoIP | -17.107511 |
| 5 | LEO-1200 | 2 | 30 | Handset | S-band | Low-data rate service | -17.107511 |
| 6 | LEO-600 | 1 | 30 | Handset | S-band | VoIP | -5.7206512 |
| 7 | LEO-600 | 2 | 30 | Handset | S-band | VoIP | -11.720651 |
| 8 | LEO-600 | 2 | 30 | Handset | S-band | Low-data rate service | -11.720651 |
| 9 | MEO | 1 | 30 | Handset | S-band | Low-data rate service | -14.938682 |
| 10 | MEO | 2 | 30 | Handset | S-band | Low-data rate service | -24.138682 |

Observation 1. Among the scenarios defined for UL link budget analysis in Rel-18 NTN coverage enhancement, It is beneficial to focus on LEO scenarios rather than the others (i.e., MEO and GEO) to evaluate OCC-based UL capacity enhancement.

Observation 2. (GEO, MEO, sat. param. set-2) might be more challenging with OCC, and those might be deprioritized.

- It is noted that noise limited or power limited scenarios are not the major environments to apply OCC.

Proposal 1. For efficient evaluation, RAN1 to consider re-use of simulation parameters in Rel-18 NTN UL coverage enhancement with below modifications.

- service type : {low rate (3 kbps), VoIP (AMR 4.75 kbps)}  {VoIP (AMR 4.75 kbps)}

- satellite orbit : {GEO, MEO, LEO-1200, LEO-600}  {LEO-600, LEO-1200}

- satellite parameter : {set-1, set-2}  {set-1}

- elevation angle : 𝛼=30°  𝛼=10°

- number of UE transmit chains : {1,2 (optional)}  {1}

- waveform : {DFT-s-OFDM, CP-OFDM}  {DFT-s-OFDM}

- channel model : {NTN-TDL-A(NLOS), NTN-TDL-C(LOS)}   {NTN-TDL-C(LOS)}

- evaluation scenario : {Rural(NLOS/LOS), Sub-urban (LOS/NLOS) (optional)}  {Rural (LOS)}

- UE speed: {vUE=3𝑘𝑚/ℎ}  {vUE=1200𝑘𝑚/ℎ}

According to the objective of WID [1], the discussion and decision on range of 3 impairments (doppler, phase distortion, time variation) for realistic evaluation might be necessary. Since doppler seems to have same meaning as frequency error (after pre-compensation), the range of doppler (frequency error) can be described as ±0.1ppm according to 38.101-5 Section 6.4.1. Since phase distortion seems to have same meaning as phase noise, phase distortion (phase noise) might be considered as optional (FR1)/38.803 phase noise profile (FR2) according to 38.821. Since the time variation seems to have the same meaning as TA variation, the range of time variation (TA variation) can be calculated by a function of (elevation, satellite altitude, UE velocity) as specified in R1-2103719. The TA variation of {GEO,LEO-1200, LEO-600} are given in Figure 1 with respect to {vUE=3𝑘𝑚/ℎ, vUE=1200𝑘𝑚/ℎ }. As depicted in Figure 1, the higher TA variation might be derived from (lower elevation angle, lower altitude, higher UE velocity). TA variation has the largest value (~93.0 us/s) when {LEO-600, 𝛼=10°, vUE=1200𝑘𝑚/ℎ}.

Figure 1: TA variation vs. elevation

Observation 3. Doppler (frequency error) might be considered as ±0.1 PPM according to 38.101-5.

Observation 4. Phase distortion (phase noise) might be considered as optional (FR1)/38.803 phase noise profile (FR2) according to 38.821

Observation 5.TA variation has the largest value (~93.0 us/s) when {LEO-600, 𝛼=10°, vUE=1200𝑘𝑚/ℎ}

- (lower altitude, lower 𝛼, higher UE speed)  (higher  TA variation)

Proposal 2. RAN1 to assume the following set of impairments for uplink capacity evaluations in Rel-19:

- Doppler (frequency error) : ±0.1 ppm

- Phase Distortion (phase noise) : optional (FR1)/38.803 (FR2)

- time variation (TA variation): max. 93.0 us/s or select some values in below

  - TA variation [us/s] @ some scenarios ~ 
(LEO-600, 1200km/h)   : 93.0 (@ 10°), 81.7 (@ 30°),
(LEO-1200, 1200km/h) : 82.4 (@ 10°), 72.4 (@ 30°) 
(GEO,1200km/h) 	    :   2.2 (@ 10°),    1.9 (@ 30°) or negligible (0) (@ all angles)

(LEO-600, 3km/h)	 : 90.8   (@ 10°), 79.8   (@ 30°)
(LEO-1200, 3km/h)   : 80.2   (@ 10°), 70.5   (@ 30°)
(GEO, 3km/h)	: 5.4e-3 (@ 10°),4.8e-3 (@ 30°) or negligible (0) (@ all angles)

3	Conclusion

In this contribution, the following observations and proposals are made.

Observation 1. Among the scenarios defined for UL link budget analysis in Rel-18 NTN coverage enhancement, It is beneficial to focus on LEO scenarios rather than the others (i.e., MEO and GEO) to evaluate OCC-based UL capacity enhancement.

Observation 2. (GEO, MEO, sat. param. set-2) might be more challenging with OCC, and those might be deprioritized.

- It is noted that noise limited or power limited scenarios are not the major environments to apply OCC.

Observation 3. Doppler (frequency error) might be considered as ±0.1 PPM according to 38.101-5.

Observation 4. Phase distortion (phase noise) might be considered as optional (FR1)/38.803 phase noise profile (FR2) according to 38.821

Observation 5.Time variation has the largest value (~93.0 us/s) when {LEO-600, 𝛼=10°, vUE=1200𝑘𝑚/ℎ}

- (lower altitude, lower 𝛼, higher UE speed)  (higher  TA variation)

Proposal 1. For efficient evaluation, RAN1 to consider re-use of simulation parameters in Rel-18 NTN UL coverage enhancement with below modifications.

- service type : {low rate (3 kbps), VoIP (AMR 4.75 kbps)}  {VoIP (AMR 4.75 kbps)}

- satellite orbit : {GEO, MEO, LEO-1200, LEO-600}  {LEO-600, LEO-1200}

- satellite parameter : {set-1, set-2}  {set-1}

- elevation angle : 𝛼=30°  𝛼=10°

- number of UE transmit chains : {1,2 (optional)}  {1}

- waveform : {DFT-s-OFDM, CP-OFDM}  {DFT-s-OFDM}

- channel model : {NTN-TDL-A(NLOS), NTN-TDL-C(LOS)}   {NTN-TDL-C(LOS)}

- evaluation scenario : {Rural(NLOS/LOS), Sub-urban (LOS/NLOS) (optional)}  {Rural (LOS)}

- UE speed: {vUE=3𝑘𝑚/ℎ}  {vUE=1200𝑘𝑚/ℎ}

Proposal 2. RAN1 to assume the following set of impairments for uplink capacity evaluations in Rel-19:

- Doppler (frequency error) : ±0.1 ppm

- Phase Distortion (phase noise) : optional (FR1)/38.803 (FR2)

- time variation (TA variation): max. 93.0 us/s or select some values in below

  - TA variation [us/s] @ some scenarios ~ 
(LEO-600, 1200km/h)   : 93.0 (@ 10°), 81.7 (@ 30°),
(LEO-1200, 1200km/h) : 82.4 (@ 10°), 72.4 (@ 30°) 
(GEO,1200km/h) 	    :   2.2 (@ 10°),    1.9 (@ 30°), negligible (0) (@ all angles)

(LEO-600, 3km/h)	 : 90.8   (@ 10°), 79.8   (@ 30°)
(LEO-1200, 3km/h)   : 80.2   (@ 10°), 70.5   (@ 30°)
(GEO, 3km/h)	: 5.4e-3 (@ 10°),4.8e-3 (@ 30°), negligible (0) (@ all angles)

4	Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#102

- Chair’s note RAN1#109