3GPP TSG RAN WG1 #116			R1-2401255

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	8.5

Source:	Google

Title:	Discussion on maintenance on further NR mobility enhancements

Document for: 	Discussion

- Introduction

In this contribution, we discuss some issues related to beam management in Rel-18 LTM, including beam indication, beam measurement and beam report. In Rel-18 mobility WID [1], RAN Plenary has agreed to specify a L1/L2 triggered mobility procedure, aiming to reduce latency in mobility/handover. In previous RAN1 meetings [2] [3] [4], RAN1 has made some agreements with respect to LTM, as shown in the followings. In subsequent sections, we discuss and provide our opinions on these related issues.

RAN1 #114 Agreement - TCI state activation by MAC CE before cell switch command for one or more than one candidate cells is allowed RAN1 #114b Conclusion - For the necessity of Padding bit in the L1 measurement report for LTM in the case where the report size is less than 12-bits, no enhancements are specified in the spec Conclusion - No specific specification change in RAN1 is pursued for scenario 3 for LTM (i.e. Beam indication after cell switch command) Agreement - For the LTM L1 measurement report, - When a UE is configured is configured with SpCellInclusion, the SpCell measurements are the entries in the LTM-CSI-SSB-ResourceSet where the PCI and frequency information [SSB frequency/ARFCN] of the candidate cell is equal to the PCI and frequency information [SSB frequency/ARFCN] of the current SpCell. RAN1 #115 Agreement UE may expect that: - For a candidate cell, the configuration of an LTM TCI state in ltm-DL-OrJointTCI-StateToAddModList-r18 and ltm-ul-TCI-ToAddModList-r18 is same as its counterpart in dl-OrJointTCI-StateList-r17 and ul-TCI-ToAddModList-r17 of the first active BWP in ServingCellConfig, at least in terms of TCI state ID, the corresponding qcl-Type1 and qcl-Type2 for the DL or joint TCI state or referenceSignal for the UL TCI state. - The LTM TCI state(s) in ltm-DL-OrJointTCI-StateToAddModList-r18 and ltm-ul-TCI-ToAddModList-r18 of a candidate cell is a subset of serving cell TCI state(s) in dl-OrJointTCI-StateList-r17 and ul-TCI-ToAddModList-r17 of the same cell. Agreement The TCI states in the candidate Cell TCI activation/deactivation command is associated with LTM TCI state pool of the target cell, i.e. configured under LTM-Candidate-r18. Agreement After RACH procedure until a new TCI state is indicated by the target cell, a UE follows the indicated TCI-state in the cell switch command at least for CFRA triggered by cell switch command.

- Discussion

- Beam indication

RAN1 #114 Agreement In R18 LTM, on the QCL source of the TCI state before/during the cell switch command, - SSB or TRS can be configured in a TCI state for the candidate cell(s) before/during cell switch command - Whether the TRS can be used for the candidate cell(s) before/during cell switch command is up to UE capability

In previous meetings, RAN1 had some progress on TCI state indication conveyed for LTM. However, there is still one left issue on QCL type information. More specifically, we should also discuss which QCL type information can be provided by TRS and SSB as QCL source for TCI state for LTM. Otherwise, paragraphs proving QCL information in TS 38.214 would be incorrect.

For TRS as QCL source RS, it is clear we can use the legacy design as below:

- TRS as QCL source RS for QCL-TypeA and QCL-TypeD

However, for SSB as QCL source RS, QCL-TypeA may not be able to be provided by SSB. Instead, we suggest SSB only provide QCL-TypeC and QCL-TypeD for a TCI state for LTM.

Proposal 1: If a SSB is QCL source RS of TCI state for a LTM candidate cell, QCL-TypeC and QCL-TypeD are provided by the SSB.

Another one left issue in RAN1#113 is about whether to deactivate TCI states in serving cell after reception of TCI activation for candidate cell(s). The following two alternatives were on the table:

- Alt 1: TCI state activation for LTM does not deactivate the activated TCI states for legacy beam management and vice versa.

- Alt 2: TCI state activation for LTM may deactivate the activated TCI states for legacy beam management and vice versa.

We believe it actually mainly depends on UE capability of supported active TCI states. If the number of activated TCI states in serving cell and the number of activated TCI state(s) for candidate cell(s) is larger than UE capability, it is impossible to go with Alt 1. For example, when UE only supports one active TCI state, UE cannot perform Alt 1 to maintain activated TCI states in serving cell. Hence, we suggest we should first check whether the total number of activated TCI states across serving cell and candidate cell(s), and then discuss which Alts to adopt.

Proposal 2: Upon receiving TCI state activation for LTM, if the number of activated TCI state(s) in serving cell and activated TCI state(s) for candidate cell(s) is larger than UE capability, UE deactivates activated TCI states in serving cell.

- FFS when to deactivate activated TCI state(s) in serving cell if the number of activated TCI state(s) in serving cell and activated TCI state(s) for candidate cell(s) is less than or equal to UE capability.

RAN4 would like to thank RAN1 and RAN2 for the LS in R1-2306259 and R2-2309250. RAN4 further discussed the beam application time and kindly asks RAN1 and RAN2 to consider following agreements for their future work on specification. RAN4 do not define beam application time. If the definition of beam application time is needed from RAN1’s perspective, RAN4 understands that it would be only specified by RAN1. Current cell switch delay defined in RAN4 can cover the TCI state application time, and there is no need to include additional beam application time component in the cell switch delay requirements.

According to RAN4’s LS reply (R1-2310795) [5] shown above. RAN4 does not see the need to further define a TCI state application time (beam application time) for LTM beam indication. The cell switch delay defined in RAN4 has accommodated the beam application time. Hence, we should reflect RAN4’s decision in RAN1 SPEC on when to apply the TCI state indicated in LTM cell switch command.

Proposal 3: Make SPEC change to reflect RAN4’s LS that TCI state indicated in LTM cell switch command is applied from a first slot after the cell switch delay.

- Beam measurement and report

So far, RAN1 has completed the main structure of CSI measurement and report for LTM. Only L1-RSRP is reported by UE in a CSI report for LTM, i.e., only beam report is supported for LTM (no L1-SINR or channel measurement related report). In addition, such beam report for LTM is also transmitted via a PUCCH or PUSCH in the serving cell. Different from legacy beam report, the beam report for LTM can comprise more than one candidate cells, each of which is reported with one or more beams.

RAN1 #115 Agreement For the priority rule for CSI report, LTM CSI report is prioritized over legacy CSI report.

In RAN1 #115, RAN1 agreed that priority of LTM CSI report is higher than legacy CSI report as shown above. However, the details are still missing. In legacy, there is a CSI report priority value calculation equation, which is to calculate a CSI priority value of a CSI report. This CSI priority value can be used when one or more CSI reports are to be multiplexed or overlapped. A CSI report with higher priority actually has a lower CSI priority value. Since we have introduced a beam report for LTM, which is way different from legacy CSI report in serving cell, a modification to existing CSI report priority value calculation equation is necessary.

Current CSI priority value calculation equation is actually built based on the following principles

      - CSI priority is firstly determined by its time domain behaviour: Aperiodic CSI report > SP CSI report on PUSCH > SP CSI report on PUCCH > periodic CSI report;

      - CSI priority is then determined by its report quantity: CSI report with L1-RSRP/SINR > CSI report without L1-RSRP/SINR;

      - CSI priority is then determined by serving cell ID: CSI report for a serving cell with lower cell ID > CSI report for a serving cell with larger cell ID;

      - CSI priority is then determined by its report configuration ID: CSI report with lower report configuration ID > CSI report with larger report configuration ID.

From our perspective, beam report for LTM should be more important than CSI report for serving cell, since beam report for LTM can also carry information of serving cell, if configured. The network also relies on this beam report for LTM to judge whether to trigger a LTM. For example, if the LTM beam report shows the tendency that cell quality of serving cell is decreasing, the network may trigger a LTM procedure via the CSC. With that said, we suggest before comparing time domain behaviour of CSI report(s), UE should first determine CSI report priority based on whether a CSI report is for LTM or for serving cell. A CSI report for LTM has higher priority (i.e., lower CSI priority value) than a CSI report for serving cell.

From above analysis, we propose to modify current CSI priority value calculation rules as below, where the major differences between legacy rules are marked in red.

- Compare whether it is LTM CSI report or legacy CSI report: LTM CSI report > legacy CSI report;

- CSI priority is firstly determined by its time domain behaviour: Aperiodic CSI report > SP CSI report on PUSCH > SP CSI report on PUCCH > periodic CSI report;

- CSI priority is then determined by its report quantity: CSI report with L1-RSRP/SINR > CSI report without L1-RSRP/SINR;

- CSI priority is then determined by serving cell ID: CSI report for a serving cell with lower cell ID > CSI report for a serving cell with larger cell ID, where for LTM CSI reports, the candidate cell with lowest candidate cell ID included in the LTM CSI report is used to compare;

- CSI priority is then determined by its report configuration ID: CSI report with lower report configuration ID > CSI report with larger report configuration ID, where for LTM CSI reports, LTM-CSI-ReportConfigId is used to compare.

Proposal 4: When calculating CSI priority value, adopt the following principles:

- Step 0: Compare whether it is LTM CSI report or legacy CSI report: LTM CSI report > legacy CSI report

- Step 1: CSI priority is firstly determined by its time domain behaviour: Aperiodic CSI report > SP CSI report on PUSCH > SP CSI report on PUCCH > periodic CSI report

- Step 2: CSI priority is then determined by its report quantity: CSI report with L1-RSRP/SINR > CSI report without L1-RSRP/SINR

- Step 3: CSI priority is then determined by serving cell ID: CSI report for a serving cell with lower cell ID > CSI report for a serving cell with larger cell ID, where for LTM CSI reports, the candidate cell with lowest candidate cell ID included in the LTM CSI report is used to compare

- Step 4: CSI priority is then determined by its report configuration ID: CSI report with lower report configuration ID > CSI report with larger report configuration ID, where for LTM CSI reports, LTM-CSI-ReportConfigId is used to compare

- Supplementary Uplink for LTM

- UL/SUL indicator – 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved

In current version of TS 38.212 for Rel-18, we have the above description for the UL/SUL indicator field in PDCCH order. In legacy, the field is present if the PDCCH order is for CFRA and the UE is configured with supplementaryUplink in ServingCellConfig in the cell. However, the PDCCH order for LTM is to acquire TA value for candidate cell. There should be no relation with serving cell configuration.

In addition, in the latest TS 38.331, RAN2 has specified ltm-EarlyUL-SyncConfigSUL for SUL in LTM candidate cell. To support SUL in early TA acquisition in LTM and enable to PDCCH order to indicate which UL carrier for performing RA in a LTM candidate cell, we suggest the following TP.

- UL/SUL indicator – 1 bit. If the Cell indicator field is absent or the Cell indicator field indicates serving cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; If the Cell indicator field indicates a candidate cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with ltm-EarlyUL-SyncConfigSUL in LTM-Candidate for the candidate cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved

In conclusion, we propose the following proposals.

Proposal 5: Endorse the following TP for supporting SUL for early TA acquisition for LTM.

TS 38.212 Reason for change: UL/SUL indicator in PDCCH order is not applicable for early TA acquisition for LTM, since no bit value can be pointed to UL/SUL in LTM candidate cell. However, in the latest TS 38.331, early TA acquisition performed in UL and SUL in LTM candidate cell are both supported. The UL/SUL indicator in PDCCH order should be able to indicate whether it is UL or SUL to perform RA for early TA acquisition in LTM candidate cell. Summary of change: Capture that UL/SUL indicator in PDCCH order can indicate whether it is UL or SUL to perform RA for early TA acquisition in LTM candidate cell. Consequences if not approved: Incorrect function in TS 38.212 and misalignment with TS 38.331. ## 8.1 Random access preamble ## *** Unchanged parts are omitted *** - UL/SUL indicator – 1 bit. If the Cell indicator field is absent or the Cell indicator field indicates serving cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; If the Cell indicator field indicates a candidate cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with ltm-EarlyUL-SyncConfigSUL in LTM-Candidate for the candidate cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved ## *** Unchanged parts are omitted ***.

- CFRA triggered by cell switch command

Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

RAN2 has introduced a feature that a CFRA can be triggered by LTM CSC MAC-CE and has specified it in the latest RAN2 SPEC. The CFRA triggered by LTM CSC MAC-CE is used for RACH-based LTM. From above capture, we notice Oct 6 and Oct 7 are almost the same as the fields included in PDCCH order. However, RAN2 SPEC still describes CFRA triggered by PDCCH order and CFRA triggered by LTM CSC MAC-CE separately. In RAN1 SPEC, this has not been captured. We propose to capture the CFRA triggered by LTM CSC MAC-CE as below. Otherwise, it is ambiguous since it is difficult to connect PDCCH order with LTM CSC command.

Proposal 6: Endorse the following TP for CFRA triggered by cell switch command

TS 38.213 Reason for change: CFRA triggered by a LTM Cell Switch Command MAC CE is not well reflected in TS 38.213. Summary of change: Capture that a CFRA can be triggered by LTM Cell Switch Command MAC CE. Consequences if not approved: Misalignment with TS 38.321 ## 8.1 Random access preamble ## *** Unchanged parts are omitted *** For a PRACH transmission by a UE triggered by a PDCCH order or a LTM Cell Switch Command MAC CE, the PRACH mask index field, if the value of the random access preamble index field is not zero, indicates the PRACH occasion for the PRACH transmission where the PRACH occasions are associated with the SS/PBCH block index indicated by the SS/PBCH block index field of the PDCCH order or the LTM Cell Switch Command MAC CE and, if any, a cell indicator field indicates a cell for the PRACH transmission [5, TS 38.212]. If the UE is provided Kcell,offset by cellSpecificKoffset, the PRACH occasion is after slot n+2μ∙Kcell,offset where n is the slot of the UL BWP for the PRACH transmission that overlaps with the end of the PDCCH order reception assuming TTA=0, and μ is the SCS configuration for the PRACH transmission. If the PDCCH reception for the PDCCH order includes two PDCCH candidates from two linked search space sets based on searchSpaceLinkingId, as described in clause 10.1, the last symbol of the PDCCH reception is the last symbol of the PDCCH candidate that ends later. The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, 11.1.1 and 17.2. ## *** Unchanged parts are omitted ***.

- Conclusion

According to the above discussion(s), we have the following observation(s) and proposal(s).

Proposal 1: If a SSB is QCL source RS of TCI state for a LTM candidate cell, QCL-TypeC and QCL-TypeD are provided by the SSB.

Proposal 2: Upon receiving TCI state activation for LTM, if the number of activated TCI state(s) in serving cell and activated TCI state(s) for candidate cell(s) is larger than UE capability, UE deactivates activated TCI states in serving cell.

- FFS when to deactivate activated TCI state(s) in serving cell if the number of activated TCI state(s) in serving cell and activated TCI state(s) for candidate cell(s) is less than or equal to UE capability.

Proposal 3: Make SPEC change to reflect RAN4’s LS that TCI state indicated in LTM cell switch command is applied from a first slot after the cell switch delay.

Proposal 4: When calculating CSI priority value, adopt the following principles:

- Step 0: Compare whether it is LTM CSI report or legacy CSI report: LTM CSI report > legacy CSI report

- Step 1: CSI priority is firstly determined by its time domain behaviour: Aperiodic CSI report > SP CSI report on PUSCH > SP CSI report on PUCCH > periodic CSI report

- Step 2: CSI priority is then determined by its report quantity: CSI report with L1-RSRP/SINR > CSI report without L1-RSRP/SINR

- Step 3: CSI priority is then determined by serving cell ID: CSI report for a serving cell with lower cell ID > CSI report for a serving cell with larger cell ID, where for LTM CSI reports, the candidate cell with lowest candidate cell ID included in the LTM CSI report is used to compare

- Step 4: CSI priority is then determined by its report configuration ID: CSI report with lower report configuration ID > CSI report with larger report configuration ID, where for LTM CSI reports, LTM-CSI-ReportConfigId is used to compare

Proposal 5: Endorse the following TP for supporting SUL for early TA acquisition for LTM.

TS 38.212 Reason for change: UL/SUL indicator in PDCCH order is not applicable for early TA acquisition for LTM, since no bit value can be pointed to UL/SUL in LTM candidate cell. However, in the latest TS 38.331, early TA acquisition performed in UL and SUL in LTM candidate cell are both supported. The UL/SUL indicator in PDCCH order should be able to indicate whether it is UL or SUL to perform RA for early TA acquisition in LTM candidate cell. Summary of change: Capture that UL/SUL indicator in PDCCH order can indicate whether it is UL or SUL to perform RA for early TA acquisition in LTM candidate cell. Consequences if not approved: Incorrect function in TS 38.212 and misalignment with TS 38.331. ## 8.1 Random access preamble ## *** Unchanged parts are omitted *** - UL/SUL indicator – 1 bit. If the Cell indicator field is absent or the Cell indicator field indicates serving cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; If the Cell indicator field indicates a candidate cell, if the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with ltm-EarlyUL-SyncConfigSUL in LTM-Candidate for the candidate cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved ## *** Unchanged parts are omitted ***.

Proposal 6: Endorse the following TP for CFRA triggered by cell switch command

TS 38.213 Reason for change: CFRA triggered by a LTM Cell Switch Command MAC CE is not well reflected in TS 38.213. Summary of change: Capture that a CFRA can be triggered by LTM Cell Switch Command MAC CE. Consequences if not approved: Misalignment with TS 38.321 ## 8.1 Random access preamble ## *** Unchanged parts are omitted *** For a PRACH transmission by a UE triggered by a PDCCH order or a LTM Cell Switch Command MAC CE, the PRACH mask index field, if the value of the random access preamble index field is not zero, indicates the PRACH occasion for the PRACH transmission where the PRACH occasions are associated with the SS/PBCH block index indicated by the SS/PBCH block index field of the PDCCH order or the LTM Cell Switch Command MAC CE and, if any, a cell indicator field indicates a cell for the PRACH transmission [5, TS 38.212]. If the UE is provided Kcell,offset by cellSpecificKoffset, the PRACH occasion is after slot n+2μ∙Kcell,offset where n is the slot of the UL BWP for the PRACH transmission that overlaps with the end of the PDCCH order reception assuming TTA=0, and μ is the SCS configuration for the PRACH transmission. If the PDCCH reception for the PDCCH order includes two PDCCH candidates from two linked search space sets based on searchSpaceLinkingId, as described in clause 10.1, the last symbol of the PDCCH reception is the last symbol of the PDCCH candidate that ends later. The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, 11.1.1 and 17.2. ## *** Unchanged parts are omitted ***.

# Reference

- RP-222332 Revised WID on Further NR mobility enhancements, MediaTek

- Chair notes of 3GPP TSG RAN WG1 #114

- Chair notes of 3GPP TSG RAN WG1 #114b

- Chair notes of 3GPP TSG RAN WG1 #115

- R1-2310795 Reply LS on beam application time for LTM, RAN4