3GPP TSG RAN WG1 #116                                         R1-2401276

Athens, Greece, February 26th - March 1st, 2024

Source:	CEWiT

Title:	Discussion on General aspects of physical layer design

Agenda Item:	9.4.2.1

Document for:	Discussion and Decision

- Introduction

In recent years, IoT has attracted much attention in the wireless communication world. More ‘things’ are expected to be interconnected for improving productivity efficiency and increasing comforts of life. Further reduction of size, complexity, and power consumption of IoT devices can enable the deployment of tens or even hundreds of billion IoT devices for various applications and provide added value across the entire value chain. It is impossible to power all the IoT devices by battery that needs to be replaced or recharged manually, which leads to high maintenance cost, serious environmental issues, and even safety hazards for some use cases (e.g., wireless sensor in electric power and petroleum industry). Since existing technologies cannot meet all the requirements of target use cases, a new IoT technology is recommended to open new markets within 3GPP systems, whose number of connections and/or device density can be orders of magnitude higher than existing 3GPP IoT technologies. The new IoT technology shall provide complexity and power consumption orders of magnitude lower than the existing 3GPP LPWA technologies (e.g. NB-IoT and eMTC) and shall address use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technologies. The new IoT technology is termed as Ambient IoT. The Ambient IoT is suitable for deployment in a 3GPP system, relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications.

The detailed study on the design targets for relevant uses cases and different deployment scenarios for Ambient IoT 3GPP technology was conducted by 3GPP RAN during Rel.18. The result of the detailed study is captured in the technical report [1]. In the RAN#102 meeting, RAN WG level study on Ambient -IoT was agreed and the SID on Study on solutions for Ambient IoT (Internet of Things) was approved [2]. In this document, waveform, and modulation schemes for various operations at Ambient IoT device are discussed. Further, the document describes different solutions for multiplexing the Ambient-IoT devices.

- Discussion

The Ambient IoT device can be categorized into three categories based on their capabilities. The three categories are following below:

Category 1: Backscattering device

The category 1 Ambient IoT device is capable of only backscattering the carrier wave and it is not capable of receiving any control signal from the Ambient IoT reader.

Category 2: Backscattering Device with Control Unit

The category 2 Ambient IoT device can receive control signal from the Ambient IoT reader and backscatter the modulated carrier wave.

Category 3: Ambient IoT device capable of signal generation and transmission

The category 3 Ambient IoT device has active RF components, and the capability of generating and transmitting the signal. The details for the above categories are provided in the companion contribution [3].

# 2.1 Waveform and modulation of Ambient IoT device

Ambient IoT devices are simple devices with ultra-low power consumption. However, the existing NR transmitter and receiver, based on OFDM modulator, involves quite a few processing, and are complex with high-power consumption. Hence, OFDM based waveform in NR is not a suitable candidate for Ambient IoT devices.

Observation 1: OFDM based transmitter/receiver in NR is complex and cannot achieve ultra-low power consumption.

The Ambient IoT device can perform two operations according to the capability, first the reception of control signal and second modulation of the carrier wave and backscatter the modulated carrier wave to the Ambient IoT reader. The waveform and modulation for control signal should be chosen such that the receiver is simple and power consumption should be very low, whereas the waveform and modulation for backscattered signal should be such that the complexity and power consumption of the transmitter unit is minimum. The OOK based modulation along with envelope detector can be a starting point for backscattering signal. Similarly, the Ambient IoT device with control unit receives control signal from the Ambient IoT reader. Therefore, the waveform and modulation for control signal should be chosen such that the receiver is simple with minimum power consumption. OFDM based OOK, considered in Rel. 18 LP-WUS, can be the starting point, as it requires a simple envelop detector for receiving. Also, it can be implemented at the Ambient IoT reader using existing OFDM based modulator with minimum changes required at the co-existed Ambient IoT reader site.

The Ambient IoT device with the capability of generating its own transmission signal involves the active RF components. Because of the active RF components, the Ambient IoT device can handle quite a few processing at the transmitter unit. The waveform and the modulation for the transmission signal can be decided with the few hundred µW peak power consumption. The OFDM based OOK can be considered the potential candidate for the transmission signal for the independently signal generated Ambient IoT device, as it maintains compatibility with the receiver/transmitter of Ambient IoT reader, in case of co-existence with NR Base station (BS).

Observation 2: The waveform and modulation for Ambient IoT devices should consider the following

- Waveform and modulation for control signal should ensure simple receiver structure with ultra-low power consumption and compatibility with NR BS

- Waveform and modulation for backscattering signal should ensure complexity and power consumption of the transmitter unit is minimum.

- Waveform and modulation for the independently generated transmission signal should have similar characteristics as the control signal.

Proposal 1: OOK based modulation along with envelope detector can be a starting point for backscattering signal.

Proposal 2: OFDM based OOK, considered in Rel. 18 LP-WUS, can be the starting point for control signal.

Proposal 3: OFDM based OOK, considered in Rel. 18 LP-WUS can be the starting point for the independently generated transmission signal.

# 2.2 Multiple Access techniques

In practice, a large number of Ambient IoT devices are deployed in industrial or inventory settings. All these Ambient IoT devices, after acquiring enough energy, start modulating and backscattering the carrier wave. Therefore, the Ambient IoT reader simultaneously receives backscattered signal from multiple Ambient IoT devices and should be able to differentiate each of them to decode the information correctly. To address this challenge, one solution involves using distinct orthogonal sequences for every Ambient IoT device. Therefore, the backscattered signal is modulated with the orthogonal sequence, i.e., device ID of every Ambient IoT device and hence it can be separated out at the Ambient IoT reader. The other solution can be based on the control signal, where the backscattering from Ambient IoT devices can be multiplexed in time or frequency domain using the control signal. The control signal can carry the information of the time and frequency resource for backscattering for each Ambient IoT device or for group of Ambient IoT devices. In another solution, the Ambient IoT reader can multiplex the backscattering signals from the Ambient IoT devices by controlling the transmission of carrier waves from the carrier wave node.

Proposal 4: Study following multiple access options for Ambient IoT device

- Device ID based

- Control signal based

- Carrier wave based

# Conclusion

# In this document different options for waveform and modulation and multiple access techniques for the Ambient IoT device are discussed and the following proposals are made:

Observation 1: OFDM based transmitter/receiver in NR is complex and cannot achieve ultra-low power consumption.

Observation 2: The waveform and modulation for Ambient IoT devices should consider the following

- Waveform and modulation for control signal should ensure simple receiver structure with ultra-low power consumption and compatibility with NR BS

- Waveform and modulation for backscattering signal should ensure complexity and power consumption of the transmitter unit is minimum.

- Waveform and modulation for the independently generated transmission signal should have similar characteristics as the control signal.

Proposal 1: OOK based modulation along with envelope detector can be a starting point for backscattering signal

Proposal 2: OFDM based OOK, considered in Rel. 18 LP-WUS, can be the starting point for control signal

Proposal 3: OFDM based OOK, considered in Rel. 18 LP-WUS can be the starting point for the independently generated transmission signal

Proposal 4: Study following multiple access options for Ambient IoT device

- Device ID based

- Control signal based

- Carrier wave based

# Reference

- TR 38.848, V18.0.0, Study on Ambient IoT (Internet of Things) in RAN.

- RP-234058, SID, Study on solutions for Ambient IoT (Internet of Things), Moderator (Huawei).

- R1-2401275, Discussion on Ambient IoT device architectures, CEWiT.