3GPP TSG RAN WG1 Meeting #116	R1-2401280

Greece, February 26th – March 1st, 2024

Agenda Item:	9.5.2

Source:	CEWiT

Title:	Discussion on on-demand SIB1.

Document for:	Discussion

- Introduction

A Rel. 18 study item on “Study on network energy savings for NR” was approved in RAN#94-e [1]. During the study in the SI phase, the network energy saving gains for potential techniques in various domains along with their impacts such as access delay, UE power consumption, etc. were studied and the summary of the potential techniques along with brief description of each technique was captured in TR 38.864 [2]. The Rel-18 work item on network energy savings focused on adapting operations for RRC connected UEs and user specific signals and channels. RAN#102 agreed to extend work on techniques for NES to common signals/channels and approved WID Enhancements of network energy savings for NR [3]. The objective on on-demand SIB1 transmissions in the WID is given below.

The objectives of the work item are the following: Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. Checkpoint for normative work in RAN#105

This contribution discusses some of the techniques to support on-demand SIB1 and required configurations for the Wake-up-signal to request SIB1.

- Techniques to support on-demand SIB1

In NR, the gNB perform transmission of SIB1 periodically to help a new UE trying to connect to the network. The periodic transmissions of SIB1 consume energy at the gNB. However, the transmitted SIB1 is used when a UE is trying to connect to the gNB. Further, the frequent transmissions of SIB1 by the gNB results in smaller inactive period for gNB. Therefore, transmission of SIB1 based on a request from a node in the network helps in power saving at the gNB. A WUS can be used by the UE for requesting the transmission of the SIB1.

2.1. Design of WUS

Currently, the SIB1 is transmitted periodically, where the periodicity depends on multiplexing pattern of SSB and CORESET0. In the case of on-demand SIB1, the gNB transmits SIB1 only upon reception of a WUS. Further, the transmission of SIB1 can be on all beams of SSB burst or on certain SSB beam(s). In the former case the WUS is common indication whereas in latter the WUS should indicate the SSB(s) in which SIB1 transmission is needed. The design and configuration of the WUS can vary depending on the applicable scenarios. Following cases can be considered for the design of the WUS,

- Common WUS for the entire network: UE trying to access the cell will transmit the WUS. The gNB(s) receiving the WUS, transmits SIB1 in its active SSB beams.

- Cell specific WUS: The WUS contains an identifier of the cell in which the SIB1 is requested. The gNB receiving the WUS check the cell in which SIB1 is requested and transmit SIB1 in active SSB beams of the requested cell.

- SSB(s) specific WUS: The WUS contains an identifier of the beam(s) in which the SIB1 is requested. Upon receiving the WUS, the gNB will identify the beam(s) in which SIB1 transmission is requested and transmit SIB1 only in those beam(s).

Observation1: Design and configuration of the WUS vary depending on the scenario of on-demand SIB1.

Proposal 1: Following alternatives can be considered for the design of WUS

Alt.1. Common WUS for the network.

Alt.2. Cell specific WUS.

Alt.3. SSB(s) specific WUS.

2.2  Signal/channel for the WUS

Currently, during the initial access procedure, a gNB transmits an SSB containing the PSS, SSS, and MIB. A UE utilizes this SSB for achieving DL synchronization and getting SIB1. The SIB1 contains information about the network and the configuration for RACH. The UE initiates the RACH procedure to establish a connection with the network. Typically, the RACH procedure begins with the UE transmitting a random-access preamble to the gNB. Upon receiving the preamble, the gNB employs it for UL synchronization and schedules subsequent communications with the UE.

In case of on demand SIB1, a signal contained within the SSB, such as PSS or SSS, can serve as a WUS. This simplifies the WUS configuration. However, using the method, the WUS cannot be beam specific. Alternatively, the RACH procedure can be considered as a baseline for WUS to request transmission of SIB1 for e.g., a RACH preamble can be utilized as the WUS.

Observation 2: PSS or SSS as a WUS simplifies the WUS configuration but is not applicable to SSB specific WUS.

Proposal 2: Support RACH as a baseline for WUS for on demand SIB1.

2.3 Configuration of WUS

The WUS configuration comprises the configuration to generate a WUS, the time and frequency resources to transmit the WUS, etc. The configuration can be configured to the UE by the gNB in DL. To minimize the configurations in DL some of the parameters of WUS can be predefined in specification.

Proposal 3: Following alternatives can be considered to provide resources of the trigger.

Alt.1. Configured by gNB.

Alt.2. Predefined in specification.

- Conclusion

In this contribution, we discussed the techniques required for NES. According to the technical analysis, the following proposals are made,

Observation1: Design and configuration of the WUS vary depending on the scenario of on-demand SIB1.

Proposal 1: Following alternatives can be considered for the design of WUS

Alt.1. Common WUS for the network.

Alt.2. Cell specific WUS.

Alt.3. SSB(s) specific WUS.

Observation 2: PSS or SSS as a WUS simplifies the WUS configuration but is not applicable to SSB specific WUS.

Proposal 2: Support RACH as a baseline for WUS for on demand SIB1.

Proposal 3: Following alternatives can be considered to provide resources of the trigger.

Alt.1. Configured by gNB.

Alt.2. Predefined in specification.

- References

- RP-213554, New SI: Study on network energy savings for NR, RAN#94-e, Dec. 2021.

- 3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

- RP-234065, New WID: Enhancements of network energy savings for NR. RAN#102, Dec. 2023.