3GPP TSG RAN WG1 #116			           R1-2401293

February 26th – March 1st, 2024

Agenda Item: 9.5.3

Source: MediaTek Inc.

Title: Adaptation of common signal/channel transmissions

Document for: Discussion and Decision

Introduction

3GPP TR 38.864 [1] shows that relaxing the periodicity of SSB, Paging Occasion (PO), and Random-Access Occasion (RO) is beneficial for network energy saving. The current 5G specs allow time-domain adaptations of common signals and channels via the SIB1 message, including adaptation to SSB periodicity. However, this method is not sufficiently agile for the dynamic nature of wireless networks. A faster adaptation mechanism is essential for enhancing network efficiency and traffic adaptability.

At the same time, the network must ensure that the fast adaptation does not adversely affect legacy UEs in idle/inactive mode. This contribution will address potential issues for legacy UEs in Section 2 and discuss the adaptation mechanism in Section 3.

Potential Issues for Legacy UE in Idle/Inactive Mode

For legacy UEs operating in idle/inactive mode, ensuring the reliability of synchronization is vital. These UEs depend on the periodic SSBs for maintaining time and frequency synchronization before PO reception. As a safeguard, the network should guarantee the availability of a minimum number of SSB bursts—for example, at least three—prior to each paging occasion to ensure synchronization performance for legacy UEs.

When SSB periodicity is dynamically changed, there is a risk that legacy UEs cannot receive SSB properly, leading to poor synchronization results. As a result, UEs might miss critical network communications or experience degraded service quality. The implications of such changes and the importance of preserving SSB bursts for legacy UEs are depicted in Figure 1, highlighting the potential impact on synchronization stability.

Figure 1. Fast adaptation of SSB transmission will impact legacy idle mode UE

Observation 1: As illustrated in Figure 1, dynamic adaptation to SSB periodicity can lead to poor synchronization result to legacy UEs in idle/inactive mode. It's crucial to ensure that legacy idle/inactive mode UEs can receive at least [3] SSB bursts prior to each paging occasion to maintain time and frequency synchronization, especially during periods of signal adaptation.

Proposal 1: Network should ensure that dynamic adaptation to SSB periodicity has no negative impact to legacy UEs in idle/inactive mode.

Dynamic SSB Periodicity/Position Adaptation

Non-uniform SSB transmission adaptation

As explained in Section 2, legacy UEs in idle/inactive mode rely on a predictable periodicity and pattern of SSB bursts to maintain synchronization with the network. To allow the flexibility in SSB periodicity adaptation while maintaining the synchronization performance for legacy UEs, the non-uniform SSB transmission can be considered. For new UEs that support dynamic adaptation of SSB periodicity, they can understand SSB periodicity change through fast adaptation mechanism, e.g., L1 signaling. For legacy UEs in idle/inactive mode, the SSB periodicity change is still signaled via SIB1. During the modification period, non-uniform SSB transmission can be applied to allow a reduction in SSB transmission while ensuring there are sufficient SSBs for synchronization for legacy UEs. After the modification period, the updated SSB periodicity is valid for both legacy and new UEs. The information regarding the non-uniform SSB transmission can be signaled to new UEs; therefore, they can also benefit from the non-uniform SSB transmission during the modification period.

One approach for signaling non-uniform SSB transmission signaling is to provide new UEs with a bitmap to indicate whether an SSB burst is to be transmitted during each specific occasion within the updated SSB period. The length of the bitmap is proportional to the ratio of the updated SSB periodicity over the SSB periodicity initially indicated in the SIB.

One example is provided: if the SSB periodicity defined in the SIB is 20ms, and it has been updated to 160ms, the bitmap is represented by an 8-bit string. Each bit corresponds to one of the 20ms-SSB transmission occasions.

- A bitmap [1 1 1 1 1 1 1 1] means that the gNB will transmit an SSB burst in every 20ms interval within 160ms period during the modification period. This results in a total of 8 SSB bursts in the 160ms period, ensuring that legacy UEs have ample opportunities for synchronization.

- A bitmap [0 0 0 0 0 1 1 1] indicates that the gNB will only transmit SSB bursts in the last three 20ms intervals within 160ms period during the modification period. This leads to a total of 3 SSB bursts in the 160ms period, which should still be sufficient for legacy UEs to maintain synchronization, as illustrated in Figure 2.

Figure 2. Non-uniform SSB transmission adaptation

Proposal 2: Non-uniform SSB transmission during modification period can be applied to ensure synchronization performance for legacy UEs.

Proposal 3: The information regarding non-uniform SSB transmission can be signaled to UEs supporting dynamic adaptation to SSB periodicity. A bitmap that indicates whether an SSB burst will be transmitted during each interval of the updated SSB periodicity can be introduced. The length of the bitmap is determined by the ratio of the updated SSB periodicity to the SSB periodicity indicated in the SIB.

SSB periodicity and position adaptation

In current spec, network can update SSB periodicity through SIB1, particularly within the ‘ServingCellConfigCommonSIB’ field. This field includes information such as the ‘ssb-PositionInBurst’ and ‘ssb-PeriodicityServingCell’. To enhance the flexibility and responsiveness of these updates, a similar bit string for conveying SSB information can be indicated through DCI or MAC-CE.

Figure 3. Field that includes the information of SSB periodicity and position in burst

For SSB periodicity adaptation, an indicator with variable bit lengths can be introduced. The indicator can indicate the selected SSB periodicity from the SSB periodicity array in SIB. Specifically, a 3-bit indicator can be employed to select from several pre-established SSB periodicity choices. For instance, an indicator value of '001' might be designated to represent an update to the SSB period, changing it to a 5 ms periodicity, referred to as ‘ms5’.

Proposal 4: A 3-bit indicator is used for updating the SSB period. This indicator allows for the selection of multiple predefined SSB periodicity options in SIB.

- For example, an indicator value of ‘001’ could correspond to an update of the SSB period (ssb-periodicityServingCell) to a periodicity of 5ms (ms5).

For SSB position adaptation, it is carrier frequency (Fc) dependent:

- For Fc ≤ 3GHz, a 4-bit indicator can be utilized for "inOneGroup" updates.

- For 3GHz < Fc ≤ 6GHz, an 8-bit indicator can be implemented for "inOneGroup" updates.

- For Fc > 6GHz, an 8-bit indicator for "GroupPresence" and another 8-bit indicator for "inOneGroup" updates can be utilized.

Proposal 5: For SSB position adaptation, the signaling depends on the carrier frequency range (Fc):

- For Fc ≤ 3GHz, a 4-bit indicator is utilized for "inOneGroup" updates.

- For 3GHz < Fc ≤ 6GHz, an 8-bit indicator is utilized for "inOneGroup" updates.

- For Fc > 6GHz, an 8-bit indicator is utilized for "GroupPresence" updates, and another 8-bit indicator is utilized for "inOneGroup" updates.

SSB information update through DCI

For fast SSB configuration adaptation, the indication can be signaled by existing DCI formats, e.g., DCI format 0_1 for paging, DCI format 2_7 for PEI indication, and DCI format 2_9 for NES indication. Additionally, the introduction of a new DCI format can also be considered for this purpose.

Within this section, we will delve into the availability of reserved bits within each DCI format that could potentially be repurposed for conveying SSB information updates:

- DCI format 2_7 (PEI): The number of available bits in this format is influenced by factors such as the number of subgroups, the PO per PEI, and the TRS indication. A minimum of 5 bits can be used for SSB information updates.

- Paging DCI: The number of available bits is upon whether the TRS availability indication is enabled, with a range of 0 to 6 bits potentially available.

- DCI format 2_9 (NES): The number of available bits in this format is yet to be determined. (The size of DCI format 2_9 has not determined)

Given these considerations, the following strategies can be applied:

- Adaptation of SSB periodicity only: due to the limited number of available DCI bits, only SSB periodicity is adjusted to ensure that the bit length of information fits within the bit number constraints of the existing DCI formats.

- Adaptation of both SSB periodicity and position: due to the limited number of available DCI bits, only certain combinations of SSB period and position are selectable for adaptation.

To allow for maximum flexibility in adaptation, a new dedicated DCI format for carrying SSB update information can be considered. However, this requires a greater effort in spec implementation.

Proposal 6: To enable fast adaptation to SSB configuration, the updated information of SSB period and/or SSB position can be indicated through DCI.

- FFS existing DCI formats or new DCI format

In summary, the adaptation mechanism should aim to provide dynamic and efficient control over SSB configurations, accommodating the varying demands of network conditions while ensuring no negative impact to legacy idle/inactive mode UE.

Conclusion

In this contribution, the following observations and proposals are provided:

Observation 1: As illustrated in Figure 1, dynamic adaptation to SSB periodicity can lead to poor synchronization result to legacy UEs in idle/inactive mode. It's crucial to ensure that legacy idle/inactive mode UEs can receive at least [3] SSB bursts prior to each paging occasion to maintain time and frequency synchronization, especially during periods of signal adaptation.

Figure 1. Fast adaptation of SSB transmission will impact legacy idle mode UE

Proposal 1: Network should ensure that dynamic adaptation to SSB periodicity has no negative impact to legacy UEs in idle/inactive mode.

Proposal 2: Non-uniform SSB transmission during modification period can be applied to ensure synchronization performance for legacy UEs.

Proposal 3: The information regarding non-uniform SSB transmission can be signaled to UEs supporting dynamic adaptation to SSB periodicity. A bitmap that indicates whether an SSB burst will be transmitted during each interval of the updated SSB periodicity can be introduced. The length of the bitmap is determined by the ratio of the updated SSB periodicity to the SSB periodicity indicated in the SIB.

Figure 2. Non-uniform SSB transmission adaptation

Proposal 4: A 3-bit indicator is used for updating the SSB period. This indicator allows for the selection of multiple predefined SSB periodicity options in SIB.

- For example, an indicator value of ‘001’ could correspond to an update of the SSB period (ssb-periodicityServingCell) to a periodicity of 5ms (ms5).

Proposal 5: For SSB position adaptation, the signaling depends on the carrier frequency range (Fc):

- For Fc ≤ 3GHz, a 4-bit indicator is utilized for "inOneGroup" updates.

- For 3GHz < Fc ≤ 6GHz, an 8-bit indicator is utilized for "inOneGroup" updates.

- For Fc > 6GHz, an 8-bit indicator is utilized for "GroupPresence" updates, and another 8-bit indicator is utilized for "inOneGroup" updates.

Proposal 6: To enable fast adaptation to SSB configuration, the updated information of SSB period and/or SSB position can be indicated through DCI.

- FFS existing DCI formats or new DCI format

Reference

TR 38.864, “Study on network energy savings for NR(Release 18)”