3GPP TSG RAN WG1 Meeting #116                                               R1-2401294

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		9.3.1

Source:	MediaTek Inc.

Title:	Discussion on SBFD TX/RX/measurement procedures

Document for:		Discussion and Decision

# Introduction

The following is agreed as part of the scope of the Rel-19 WI on Evolution of NR Duplex Operation [1].

- Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - Enhancement on resource allocation in frequency domain in SBFD symbols - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

In the contribution, we discuss semi-static indication of time/freqeuncy location of SBFD subbands. In addition, we discuss transmission/reception behaviour of SBFD-aware UE in SBFD symbols, including enhancements to frequency domain resource allocation for various PHY channels/signals.

# Discussion

## Semi-static Indication of SBFD Resources

In this section, we provide our views on semi-static indication of time and frequency location of SBFD subbands in RRC_CONNECTED mode.

### Indication of Time Location of SBFD Subbands

In TDD operation, the network configures the UE with the higher layer parameter, tdd-UL-DL-ConfigurationCommon, which indicates the time location of downlink, flexible and uplink slots. This parameter is configured in ServingCellConfigCommonSIB for the UE’s primary serving cell and in ServingCellConfigCommon for the UE’s other serving cells.

For SBFD operation, the parameter tdd-UL-DL-ConfigurationCommon can still be used to indicate time location of downlink/flexible/uplink resource. Moreover, an additional higher layer parameter (e.g., SBFD-ConfigCommon) can be provided to indicate the time location of SBFD subbands (i.e., the location of SBFD slots/symbols) to a SBFD-aware UE. The new higher layer parameter will provide additional information on which slots – configured by TDD-UL-DL-ConfigCommon – are SBFD slots and non-SBFD slots. Like the TDD case, the new parameter (e.g., SBFD-ConfigCommon) can be provided within ServingCellConfigCommonSIB and/or ServingCellConfigCommon.

Moreover, the new parameter can be configured as a slot-level bitmap. The length of the bitmap can be set equal to the total number of slots configured as “downlink” and “flexible” by tdd-UL-DL-ConfigurationCommon – since “uplink” slots are not expected to be partitioned. As shown in Figure 1, one bit value (e.g., 1) is used to indicates that a slot is a SBFD slot whereas another bit value (e.g., 0) is used to indicate that a slot is a non-SBFD slot.

Figure 1: Indication of time location of SBFD subbands

Proposal 1: Provide a new higher layer parament to indicate time location of SBFD subbands to SBFD-aware UE;

- The new parameter can be provided within ServingCellConfigCommonSIB and/or ServingCellConfigCommon information element(s).

- The new parameter can be configured as a slot-level bitmap which indicates whether a slot is SBFD or not.

### Special slots: RRC Common vs RRC Dedicated Signaling

For slot-level indication of SBFD subband time location, the SBFD-aware UE requires a separate interpretation of slot partitioning when a “special” slot is indicated as a SBFD slot. A “special” slot in this case is any slot configured with a mix of symbol types (downlink, uplink and/or flexible) by RRC common signalling (using tdd-UL-DL-ConfigurationCommon) or by RRC dedicated signalling (using tdd-UL-DL-ConfigurationDedicated).

In the case of tdd-UL-DL-ConfigurationCommon, whenever a TDD pattern configures some slots as “flexible” slots, there are two “special” slots. The first special slot, immediately following the downlink-only slots, consist of downlink and flexible symbols. The other special slot, immediately preceding the uplink-only slots, consist of flexible and uplink symbols. Since these two slots contain a mix of symbol types, a different interpretation is required when these slots are indicated as SBFD slots – compared to the case when a slot containing only one symbol type (e.g., downlink-only symbols) is indicated as a SBFD slot.

Additionally, some slots configured as flexible by tdd-UL-DL-ConfigurationCommon may be reconfigured as “special” slots using tdd-UL-DL-ConfigurationDedicated. However, this may be a less frequent configuration compared the configuration of special slots by RRC common signalling. Therefore, a new rule can be defined to restrict reconfiguration of a “flexible” slot as a “special” by dedicated signalling when the slot is indicated as a SBFD slot. This simplifies the signaling requirements and reduces signaling overhead.

Observation 1: A different interpretation is required when a special slot is indicated as a SBFD slot, since the slot contains a mix of symbol types.

Proposal 2: When a slot configured as a “special” slot by TDD-UL-DL-ConfigCommon and also indicated as a SBFD slot, the UE considers only downlink and/or flexible symbols of the “special” slot as SBFD symbols – uplink symbols a special slot are not considered as partitioned.

Proposal 3: If a “flexible” slot is indicated as a SBFD slot, the UE does not expect the slot to be reconfigured as a “special” slot using tdd-UL-DL-ConfigurationDedicated.

### Indication of Frequency Location of SBFD Subbands

In the current specification, the network configures the UE with the higher layer parameter SCS-SpecificCarrier, which indicates the location and width of the uplink and/or downlink carrier. For the primary serving cell this parameter is configured as part of DownlinkConfigSIB and UplinkConfigSIB (provided within ServingCellConfigCommonSIB) for the downlink and uplink carriers, respectively. For example, considering the case of downlink, DownlinkConfigSIB contains the parameter FrequencyInfoDL-SIB, which provides an instance of SCS-SpecificCarrier for each configured subcarrier spacing. In our view SBFD subbands can be configured for each downlink/uplink carrier.

For SBFD operation it is expected that each configured carrier, i.e., each instance of SCS-SpecificCarrier within FrequencyInfoDL-SIB, is partitioned into subband(s) – during SBFD symbols. As indicated in the scope of the WID [1], knowledge of the frequency location of subbands must be provided to a SBFD-aware UE. One way to achieve this is to provide a new higher layer parameter (e.g., downlinkSubband and uplinkSubband) within FrequencyInfoDL-SIB that explicitly indicates the frequency location DL and UL subband. The new higher layer parameter, for each subband, can be configured as a single integer value that represents the location and bandwidth of the subband (SLIV approach), similar to the existing configuration for bandwidth part (BWP) indication.

Proposal 4: The SBFD subbands are configured per carrier.

Proposal 5: Provide new higher layer parameter within FrequencyInfoDL-SIB to indicate frequency location of SBFD subbands.

### Subband Indication for “Flexible” Slots

As mentioned above, tdd-UL-DL-ConfigurationCommon configures some slots/symbols as a “flexible”, i.e., slots/symbols that are not configured as either downlink or uplink. In the scenario where a flexible slot/symbol is indicated as a SBFD slot, and the UE is provided the frequency location of subbands – the UE needs an instruction/rule on how to interpret the slot.

Following legacy operation, the UE interprets the slot as flexible and would therefore not assign a transmission direction to the configured subbands. However, the SBFD subband configuration indicates which subbands represents downlink, uplink and/or guardband resources. In our view, if the network indicates a flexible slot as a SBFD slot, the slot partition indication can be considered to override the configuration of a slot/symbol as “flexible”, i.e., the UE no longer considers a slot/symbol as “flexible” if it is indicated as a SBFD slot/symbol. This implies that dedicated signaling (using tdd-UL-DL-ConfigurationDedicated) cannot be used to reconfigure/change the transmission direction of the configured subbands – as is the case for slots indicated as downlink or uplink by tdd-UL-DL-ConfigurationCommon.

Proposal 6: Time/frequency indication of SBFD subbands overrides flexible indication by TDD-UL-DL-ConfigCommon, i.e., a flexible symbol that has been partitioned is not considered as flexible anymore.

## TX/RX/Measurement Behaviour for SBFD-aware UE

In this section we discuss transmission, reception and measurement behavior and procedures for SBFD-aware UE

### Transmission/Reception Behaviour on SBFD Subbands

As stated in the scope of the WID [1], for SBFD operation in a slot defined as downlink and/or flexible by tdd-UL-DL-ConfigurationCommon, uplink transmission is within uplink subband only whereas downlink reception is within downlink subband(s) only. The only exception is CLI measurement, which can be allowed outside downlink subbands. For dynamic/aperiodic resource allocation, gNB has the flexibility to ensure that any scheduled uplink or downlink resources is confined within the uplink or downlink subbands, respectively. However, for configured/periodic/semi-persistent resource allocation, the transmission periodicity can be such that allocated resources fall outside the corresponding subband.

To ensure that uplink transmissions and downlink receptions are always confined within uplink and downlink subbands, new frequency domain validation rules must be defined to restrict valid uplink/downlink resource allocation to be within uplink/downlink subbands. In other words, the UE considers uplink/downlink resource allocation that fall outside uplink/downlink subbands as invalid allocation.

In the case where a downlink (uplink) resource allocation overlaps fully with uplink (downlink) subband, the UE drops the entire resource allocation (e.g., UE considers this as an error case). In the case where a downlink (uplink) resource allocation overlaps partially with uplink (downlink) subband, the UE can be configured to:

- Transmit/receive only on the portion of the allocated resources that fall within the uplink/downlink subbands, i.e., partial allocation, or

- Drop the entire resource allocation

By following the above rules, uplink transmission and downlink reception of a SBFD-aware UE can be confined within uplink and downlink subbands.

Proposal 7: Define new frequency domain validation rules to ensure that UL transmissions and DL receptions are confined within uplink and downlink subbands, respectively.

- Rule 1: The part of UL resources allocation on a SBFD slot/symbol that falls outside outside the UL subband is considered as invalid by the UE

  - FFS for SRS tranmissions on guardbands

- Rule 2: The part of DL resources allocation on a SBFD slot/symbol that falls outside outside the DL subband is considered as invalid by the UE.

  - Resource allocation for CLI measurement is valid even if it falls outside the downlink subbands.

### FDRA Enhancements in SBFD Symbols

#### PDSCH Resource Allocation

SBFD symbols may consist of non-contiguous downlink subbands. FDRA Type-0 readily supports non-contiguous RB allocations and can directly be used for resource allocation across non-contiguous downlink subbands. The only issue that can arise when using FDRA Type-0 for resource allocation on SBFD symbols is the potential misalignment between SBFD subbands and RBGs. However, it has been agreed by RAN1 that when such a scenario occurs, the part of the RBG inside the downlink subband(s) can be used. Therefore, existing FDRA Type-0 can be directly reused in SBFD operation with no specification impact.

Meanwhile, FDRA Type-1 only supports contiguous resource allocation, based on indication of a starting RB and RB length. Also, FDRA Type-1 supports VRB-to-PRB interleaving, where the allocated VRBs are mapped to PRBs using an interleaving function (even VRBs are mapped to the lower segment and odd VRBs to the higher segment). Following the existing specification, FDRA Type-1 (with or without interleaving) can only be used for RB allocation within a single downlink subband, which limits the resource allocation flexibility.

Non-contiguous resource allocation for FDRA Type-1 can be achieved by configuring a single contiguous resource allocation (as in existing FDRA Type-1) and excluding any portion of allocated resources that fall outside the downlink subband(s). In this case, the VRBs (or PRBs) can be renumbered/reindexed to exclude VRBs that fall outside the downlink subbands, as shown in Figure 2. In this case, the gNB can flexibly allocate resources across the downlink subbands since the allocation remains contiguous from the UE’s perspective due to the renumbered VRB/PRBs. In addition, any potential issues with VRB-to-PRB interleaving can be avoided since RBs outside the downlink subbands will not be considered in the interleaving procedure after VRB/PRB reindexing.

Observation 2: FDRA Type-0 can directly be reused for SBFD operation with no specification impact.

Observation 3: Non-contiguous resource allocation using FDRA Type-1 can be achieved by configuring a single contiguous resource and excluding any portion of allocated resources that fall outside the downlink subband(s).

Observation 4: If non-contiguous resource allocation is supported for FDRA Type-1, any potential issues with VRB-to-PRB interleaving can be avoided by adopting RB reindexing.

Proposal 8: Support non-contiguous resource allocation for Type-1 FDRA.

Proposal 9: Support VRB/PRB reindexing for non-contiguous resource allocation for Type-1 FDRA, excluding RBs that fall outside the downlink subbands.

Figure 2: VRB/PRB reindexing for non-contiguous resource allocation for FDRA Type-1

#### PDSCH Precoding

In the current specification, the network can configure the UE to apply a precoding resource block group (PRG) of size 2 RBs (default value), 4 RBs or “wideband”. The case of 2 RBs and 4 RBs can be directly applied on SBFD symbols – the only potential issue being the possible misalignment boundaries between PRG and the uplink subband. Like the RBG case, the part of the PRG inside the downlink subbands can be used, which implies that the UE may perform channel estimation on fewer RB at the edge of downlink subbands.

If PRG size is determined as “wideband” it only applies to contiguous RB allocations. To enable flexible resource allocation in SBFD operation, indication of “wideband” PRG can be allowed to cover non-contiguous RB allocation across downlink subbands – with contiguous resources within each downlink subband. In this case, the UE can assume that the allocated RBs within each downlink subband experience similar channel conditions. Therefore, the gNB applies the same precoding in each downlink subband.

Proposal 10: Further study the performance gain and UE impact of allowing “wideband” support non-contiguous frequency resources across downlink subbands.

#### CSI-RS Resource

Agreement RAN1#112 Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

In the current specification, CSI-RS resource allocation in frequency domain is across contiguous RBs. For SBFD operation, two options are considered to enable non-contiguous CSI-RS resource allocation across non-contiguous downlink subbands.

The first option is to configure two contiguous CSI-RS resources – one for each downlink subband. In some configurations up to 64 CSI-RS resources can be configured within a CSI-RS Resource Set – in which case Option 1 can readily be supported using the existing specification. For this option, additional signaling is needed to link the two CSI-RS resources in the two downlink subbands. However, in other configurations, the number of CSI-RS resources per Resource Set is limited which can impact the number of CSI-RS resources that can be configured for each downlink subband.

The second option, on the other hand, is to configure a single non-contiguous CSI-RS resource to cover the two downlink subbands. A variant of this option (Option 2-2) involves configuring one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside downlink subbands. This can be achieved using the existing specification – assuming that the UE follows Rule 2 in Proposal 7 above. In our view, Option 2-2 is similar to Option 1 in practice – since they both result in contiguous CSI-RS resources in each downlink subband. The main difference between the two Options is RRC signalling – which is minimal for Option 2-2. At this stage, the impact of non-contiguous CSI-RS allocation (using either Option 1 or Option 2-2) on measurement accuracy is still unclear.

Observation 5: The impact of non-contiguous CSI-RS allocation (using either Option 1 or Option 2-2) on measurement accuracy is still unclear.

Proposal 11: RAN1 to further study the impact of Option#1 and Option#2 on UE implementation and measurement accuracy.

### PHY Channels/Signals/Procedures across SBFD and non-SBFD Symbols in Different Slots

#### CSI Reporting Configuration

Agreement RAN1#112bis-e For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols: - Option 1: two CSI-ReportConfigs, where one is associated with SBFD symbols and the other is associated with non-SBFD symbols - Option 1-1: One CSI-ReportConfig is associated with a CSI-RS restricted to SBFD symbols only and the second CSI-ReportConfig is associated with a second CSI-RS restricted to non-SBFD symbols only; - Option 1-2: Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only. - Option 2: one CSI-ReportConfig associated with both SBFD symbols and non-SBFD symbols - Option 2-1: One CSI-ReportConfig is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived based on the first and second CSI-RSs respectively. - Option 2-2: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances.

Two main options been agreed in RAN1 for achieving slot CSI reporting in SBFD operation. The need for separate CSI reporting arises from the distinct interference patterns observed on SBFD and non-SBFD slots. In the most part, slot-specific CSI reporting can readily be supported by current specification. For instance, the higher layer parameter timeRestrictionForChannelMeasurements (or timeRestrictionForInterferenceMeasurements) defines a restriction on the time domain resources used for channel (or interference) measurements. When this parameter is set to “configured”, channel/interference measurement is restricted to resources within a single slot (the “reference resource”), which can be either a SBFD or a non-SBFD slot.

When this parameter is set to not “not configured”, channel/interference measurement can be across multiple slots. In this case, one the agreed options for CSI reporting configuration can be adopted to achieve slot-specific CSI reporting. Among these options, we prefer Option 1-1 since it can be achieved by the current specification with minimum enhancements. In the case of periodic CSI reporting and semi-persistent CSI reporting on PUSCH, UE can be configured with multiple CSI reporting Configurations (CSI-ReportConfig). One CSI reporting configuration can be assigned to each SBFD slot type. A new higher layer parameter (e.g., slotType) can be provided CSI-ResourceConfig to indicate the slot type for which the CSI-RS resource configuration is applied. Thus, CSI-ReportConfig associated CSI-ResourceConfig is used to provide CSI reports based on measurement performed on the sets of slots indicated by slotType.

In the case of semi-persistent CSI Reporting on PUCCH, up to 4 CSI reporting configurations are provided by RRC. A MAC CE activation command can be used to activate one reporting configuration for use by the UE on PUCCH. This restriction limits the ability to configure one CSI-ReportConfig for each slot type. Therefore, this restriction can be relaxed to enable the MAC CE activation command to activate multiple (at least 2) CSI reporting configurations for SBFD operation.

Observation 6: For periodic/semi-persistent CSI reporting PUSCH, Option 1-1 for CSI report associated with periodic/semi-persistent CSI-RS can be achieved by the existing specification.

Observation 7: To support slot-specific semi-persistent CSI reporting PUCCH, multiple CSI report configuration must be activated by MAC CE.

Proposal 12: Option 1-1 for CSI reporting, as specified in current NR specs, is considered the baseline for SBFD operation. Other options can be considered if they show considerable performance gains.

Proposal 13: For semi-persistent CSI reporting PUCCH, allow MAC CE to activate at least 2 CSI reporting configurations, one for each slot type.

#### Resource Allocation for Multi-slot Transmissions

Agreement RAN1#112bis-e For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, study at least the following frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE: - Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots. - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication - Option 1-3: single FDRA configuration/indication and RB offset(s) - Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals. - Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed. Note: Different options can be studied for different signals/channels.

Three frequency resource allocation options were agreed for uplink transmissions and downlink receptions across SBFD symbols and non-SBFD symbols in different slots. These options have been proposed to enable flexibility in resource allocation – considering the different resource availability SBFD and non-SBFD slots/symbols. As an example, FDRA for Type-1 CG PUSCH the FDRA is across multiple slots within a configured periodicity – which may overlap with downlink subbands in SBFD operation, as shown in Figure 3.

Option 1 of the above agreement can be supported for periodic/semi-persistent resource allocation (e.g., SPS, CG-PUSCH, PUCCH, SRS) since it can enable flexible and more efficient resource allocation in SBFD operation. In the case of CG-PUSCH, as an example, a new higher layer parameter (e.g., frequencyDomainAllocation2) can be provided, in addition to the existing parameter, frequencyDomainAllocation (following Option 1-1). In this case, one of the two parameters provide FDRA for SBFD symbols, whereas the other parameter provides FDRA for non-SBFD symbols. A similar configuration can be used SPS, PUCCH and SRS FDRA.

Option 3 can also be supported, especially for repetitions. In this case the current specification can be reused in addition to a new rule that instructs UEs to transmit/receive repetitions on non-SBFD slots and skip (drop or postpone) transmission/reception on SBFD slots.

Figure 3: CG-PUSCH transmission in SBFD.

Observation 8: For multi-slot transmissions/reception, a single FDRA is configured for all slots. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths.

Proposal 14: Support separate FDRA for SBFD and non-SBFD symbols for multi-slot transmissions/reception.

Proposal 15: Support skipping for multi-slot transmissions when allocated UL/DL RBs overlap with RBs outside DL/UL subband in SBFD symbols.

#### Frequency Hopping

On SBFD symbols, resource allocation with frequency hopping is likely to overlap with downlink subband since the bandwidth of the uplink subband is less than the BWP size, as shown in Figure 4. To avoid this issue separate frequency hopping offsets can be configured for SBFD and non-SBFD symbols. For PUSCH and SRS, the frequency hopping procedure needs to be modified to enable frequency hopping that is confined within the uplink subband in the SBFD slots/symbols.

Moreover, the main advantage of frequency hopping is to provide frequency diversity and robustness to poor channel conditions. On SBFD symbols, the diversity gain obtained from frequency hopping is reduced compared to non-SBFD slots because of the smaller uplink frequency resources. Therefore, a mechanism/rule to disable frequency hopping on SBFD symbols could help in reduce signalling overhead.

Figure 4: PUSCH frequency hopping in SBFD.

Proposal 16: Support separate FDRA (including separate frequency hopping offsets) for SBFD and non-SBFD symbols when frequency hopping enabled.

Proposal 17: Support frequency hopping only on non-SBFD symbols for inter-slot frequency hopping.

### SBFD Operation in SSB Symbols

For SBFD operation, RAN1 agreed that uplink subband can be configured in SSB symbols. However, it is yet to be determined if a SBFD-aware UE is allowed to transmit in SSB symbols or not. In our opinion, all UEs in a given serving cell are expected perform SSB measurements during SSB occasions. Some SSB occasions may be skipped, as shown in Figure 5 – but usually for other measurements (e.g., neighbour cell measurements).

Figure 5: Skipping of SSB occasions overlapping with SMTC and/or measurement gaps

Since UEs remain half duplex nodes, uplink transmission cannot be expected on SSB symbols if all UEs in a cell are meant to perform SSB measurement. Therefore, if SSB occurs on SBFD slots/symbols the UE can be configured to prioritize the SSB measurement. To avoid any wastage of resources, the occurrence of SSB on SBFD slots/symbols can be considered to override the SBFD slot partitioning. That is, the UE no longer considers a slot/symbol as a SBFD slot/symbol if it overlaps in time and/or frequency domain with SSB symbols.

The cost of overriding of SBFD slots/symbols may not be too detrimental to uplink performance. Figure 6 shows an example considering a typical SSB configuration for operation bandwidth between 3GHz and 6GHz (with a maximum of 8 SSB candidates on 4 slots), with SSB periodicity of 20ms, DXXXU SBFD pattern, and 30kHz SCS. As shown in the figure, even with overriding of SBFD slots that overlap with SSB, there remains a significant number of uplink resources within the SSB periodicity where the UE can perform uplink transmissions. The cost is even less if SSB configuration for operation bandwidth less than 3GHz (with a maximum of 4 SSB candidates on 2 slots) is considered.

Figure 6: Potential cost of overriding on uplink opportunities

Observation 9: UEs in a cell are expected to perform measurement during SSB occasions. Since UEs are half duplex, uplink transmission can therefore not be expected during SSB occasions.

Observation 10: The cost of overriding SBFD partitioning on uplink is manageable.

Proposal 18: Occurrence of SSBs in a SBFD slot override slot partitioning.

# Conclusion

In this contribution, we discussed indication of time/frequency location of SBFD subbands. We also provided our views on SBFD-aware UE behaviour in SBFD symbols.

Observation 1: A different interpretation is required when a special slot is indicated as a SBFD slot, since the slot contains a mix of symbol types.

Observation 2: FDRA Type-0 can directly be reused for SBFD operation with no specification impact

Observation 3: Non-contiguous resource allocation using FDRA Type-1 can be achieved by configuring a single contiguous resource and excluding any portion of allocated resources that fall outside the downlink subband(s)

Observation 4: If non-contiguous resource allocation is supported for FDRA Type-1, any potential issues with VRB-to-PRB interleaving can be avoided by adopting RB reindexing.

Observation 5: The impact of non-contiguous CSI-RS allocation (using either Option 1 or Option 2-2) on measurement accuracy is still unclear.

Observation 6: For periodic/semi-persistent CSI reporting PUSCH, Option 1-1 for CSI report associated with periodic/semi-persistent CSI-RS can be achieved by the existing specification

Observation 7: To support slot-specific semi-persistent CSI reporting PUCCH, multiple CSI report configuration must be activated by MAC CE

Observation 8: For multi-slot transmissions/reception, a single FDRA is configured for all slots. This may impact flexibility in resource allocation in SBFD since it has two types of slots with different bandwidths.

Observation 9: UEs in a cell are expected to perform measurement during SSB occasions. Since UEs are half duplex, uplink transmission can therefore not be expected during SSB occasions.

Observation 10: The cost of overriding SBFD partitioning on uplink is manageable.

Proposal 1: Provide a new higher layer parament to indicate time location of SBFD subbands to SBFD-aware UE

Proposal 2: When a slot configured as a “special” slot by TDD-UL-DL-ConfigCommon and also indicated as a SBFD slot, the UE considers only downlink and/or flexible symbols of the “special” slot as SBFD symbols – uplink symbols a special slot are not considered as partitioned.

Proposal 3: If a “flexible” slot is indicated as a SBFD slot, the UE does not expect the slot to be reconfigured as a “special” slot using tdd-UL-DL-ConfigurationDedicated.

Proposal 4: The SBFD subbands are configured per carrier.

Proposal 5: Provide new higher layer parameter within FrequencyInfoDL-SIB to indicate frequency location of SBFD subbands.

Proposal 6: Time/frequency indication of SBFD subbands overrides flexible indication by TDD-UL-DL-ConfigCommon, i.e., a flexible symbol that has been partitioned is not considered as flexible anymore.

Proposal 7: Define new frequency domain validation rules to ensure that UL transmissions and DL receptions are confined within uplink and downlink subbands, respectively:

- Rule 1: The part of UL resources allocation on a SBFD slot/symbol that falls outside outside the UL subband is considered as invalid by the UE

  - FFS for SRS tranmissions on guardbands

- Rule 2: The part of DL resources allocation on a SBFD slot/symbol that falls outside outside the DL subband is considered as invalid by the UE.

Proposal 8: Support non-contiguous resource allocation for Type-1 FDRA.

Proposal 9: Support VRB/PRB reindexing for non-contiguous resource allocation for Type-1 FDRA, excluding RBs that fall outside the downlink subbands.

Proposal 10: Further study the performance gain and UE impact of allowing “wideband” support non-contiguous frequency resources across downlink subbands.

Proposal 11: RAN1 to further study the impact of Option#1 and Option#2 on UE implementation and measurement accuracy.

Proposal 12: Option 1-1 for CSI reporting, as specified in current NR specs, is considered the baseline for SBFD operation. Other options can be considered if they show considerable performance gains.

Proposal 13: For semi-persistent CSI reporting PUCCH, allow MAC CE to activate at least 2 CSI reporting configurations, one for each slot type.

Proposal 14: Support separate FDRA for SBFD and non-SBFD symbols for multi-slot transmissions/reception.

Proposal 15: Support skipping for multi-slot transmissions when allocated UL/DL RBs overlap with RBs outside DL/UL subband in SBFD symbols.

Proposal 16: Support separate FDRA (including separate frequency hopping offsets) for SBFD and non-SBFD symbols when frequency hopping enabled.

Proposal 17: Support frequency hopping only on non-SBFD symbols for inter-slot frequency hopping.

Proposal 18: Occurrence of SSBs in a SBFD slot override slot partitioning.

# References

- RP-234035, “New WID: Evolution of NR Duplex Operation: Sub-band Full Duplex (SBFD)”, RAN#102, December 2023.