3GPP TSG-RAN WG1 Meeting #116	R1-2401302

Athens, Greece, 26th February – 1st March 2024

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN uplink capacity and throughput enhancement

Agenda item:	9.11.4

Document for:	Discussion

# Introduction

In RAN#102 [1], a new WID on IoT NTN enhancements was endorsed for Release 19. This contribution aims to discuss Uplink Capacity/Throughput Enhancement as shown below. We’ll outline enhancements for IoT-NTN uplink capacity and throughput, focusing on code optimization, single-tone prioritization, NPUSCH performance, OCC mapping, and UE pairing.

  - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2]

    - Multi-tone support for 15 kHz SCS should also be considered

Note: Impact of impairment shall be taken into account

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]:

    - Msg3 transmission without msg1/RAR

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

# Discussion on Orthogonal Cover Code length

Delay drift in IoT NTN influences the code size and repetition requirements, which are important aspects that need to be considered for reliability. It's necessary to tailor the code length and repetition frequency according to the delay drift experienced and the UE's ability to pre-compensate for timing errors. Optimizing these parameters will be necessary to preserve frame efficiency and minimize SNR loss, also taking into account the delay drift in satellite communications.

For example, at lowest elevation angle over the service link and feeder link considered in Release 17 Study Phase in TR 36.763, a maximum delay drift in the order of 100 us/s could be experienced. This would result in maximum delay drift per UL transmission times that could exceed the cyclic prefix within 32 ms as shown in Table 1. The most significant delay drift occurs between a UE at the beam center and one at the edge, while the greatest Doppler drift is between UEs at opposite edges of the beam. This suggests that for effective UE multiplexing, elevation angle, beam size and edge of the beam aspects should be considered when mapping OCC to PRB resources.

| UL transmission time (repetitions) | Max Delay drift per UE (calculated) | Max Delay drift among UEs | Max Doppler drift among UEs | CP [%] |
| --- | --- | --- | --- | --- |
| 2 ms (x2) | 0.2 us | 0.0074 us | 0.001 ppm | 4.27% |
| 4 ms (x4) | 0.4 us | 0.0155 us | 0.002 ppm | 8.55% |
| 8 ms (x8) | 0.8 us | 0.0305 us | 0.005 ppm | 17.1% |
| 16 ms (x16) | 1.6 us | 0.0634 us | 0.01 ppm | 34.2% |
| 32 ms (x32) | 3.2 us | 0.12 us | 0.02 ppm | 68.3% |
| Note: Lowest elevation angle over the service link and feeder link considered in Release 17 Study Phase in TR 36.763 is assumed |  |  |  |  |

Table 1: Illustration of delay drift and doppler drift with UL transmission time (repetitions).

The evaluation assumes the satellite moves on the z-axis with an ideal circular orbit, and both the UE and the gateway are co-located at the satellite beam center. The satellite beam is fixed on the Earth and maintains a perfect circle during the satellite's movement. This idealized scenario helps to simplify the analysis but may not capture all the complexities of a real-world system. More details in  Appendix.

RAN1 to study solutions to adjust code length and number of repetitions based on delay drift and UE pre-compensation capabilities.

# Discussion on Single Tone

Single-tone transmissions with 15 kHz and 3.75 kHz frequencies are key to improving NB-IoT NTN capacity in low SNR conditions. RAN1 should prioritize single tone over multitone. This would allow system optimization by fully leveraging single tone transmission robustness and gains under challenging signal conditions. It is known since release 13 of cellular NB-IoT that single-tone transmission at low SNR levels translates into substantial capacity gains. If not prioritized, the network risks forgoing these improvements, which could lead to suboptimal performance and unmet capacity potential.

Prioritize single tone 15 kHz and 3.75 kHz for NB-IoT NTN.

# Discussion on NPUSCH Performance with OCC

The use of repetitions for NPUSCH performance is essential in NB-IoT NTN deployment scenarios. RAN1 can discuss how to efficiently use NPUSCH repetitions with OCC in NB-IoT NTN systems that experience typically very low SNR scenarios. To enhance NPUSCH performance, RAN1 can consider how to optimize both the code lengths and the scheduling of UE transmissions. Without such optimization, large code lengths and concurrent transmissions can severely degrade performance, leading to diminished capacity gains and poorer link-level quality. Addressing these issues will be necessary to maintain a robust and efficient network.

RAN1 to study solutions to optimize Orthogonal Cover Code length and UE transmission scheduling to improve NPUSCH performance when repetitions is used in low SNR conditions.

# OCC Mapping to Allocated Resources

Assessing the impact of OCC mapping is critical for optimizing system specifications and performance. By mapping OCC to NPUSCH Format 1 repetition units within the time domain, and considering UE pre-compensation capabilities, we believe the system can achieve the objective with minimal impact on specifications. Time domain mapping will also minimize impact on device complexity by leveraging legacy UE pre-compensation capability.

RAN1 to study solutions to map OCC to NPUSCH Format 1 repetition units in the time domain, considering UE pre-compensation capabilities.

# Pairing of UEs with OCC for Simultaneous Transmission

Simultaneous transmission of paired UEs where each UE uses a specific OCC can cause interference, potentially disrupting the orthogonality of OCC. To counter this, UEs should be paired thoughtfully, using appropriate OCC code sizes and MCS. Nested OCC structure for efficient multiplexing could also be considered.

Pairing of UEs via OCC should be designed to boost spectral efficiency and may allow to support devices with different UE pre-compensation capabilities. By considering OCC code sizes and MCS during pairing, the system can better handle channel imperfections and maintain orthogonality, ensuring that simultaneous transmissions do not compromise network capacity and efficiency.

RAN1 to study pairing of UEs using same/different OCC code sizes, MCS, and UE pre-compensation capabilities to optimize UE multiplexing for capacity and throughput enhancements at system level.

# Conclusion

In this contribution, we have the following proposals:

- RAN1 to study solutions to adjust code length and number of repetitions based on delay drift and UE pre-compensation capabilities.

- Prioritize single tone 15 kHz and 3.75 kHz for NB-IoT NTN.

RAN1 to study solutions to optimize Orthogonal Cover Code length and UE transmission scheduling to improve NPUSCH performance when repetitions is used in low SNR conditions.

- RAN1 to study solutions to map OCC to NPUSCH Format 1 repetition units in the time domain, considering UE pre-compensation capabilities.

- RAN1 to study pairing of  UEs using same/different OCC code sizes, MCS , and UE pre-compensation capabilities to optimize UE multiplexing for capacity and throughput enhancements at system level.

# References

- RP-234078, Moderator (Deutsche Telekom, RAN VC), New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#102, Edinburgh, 11-15 December, 2023

# Appendix

The appendix outlines maximum Doppler and delay drifts among UEs in a LEO-600km satellite system, detailing their variation with gateway elevation angles and uplink transmission times.

Settings

| Setting/Assumption | Description |
| --- | --- |
| Satellite Altitude | LEO 600 km above Earth's surface |
| Satellite Speed | 7565.95 m/s |
| Carrier Frequency | 2 GHz |
| Beam Radius | 25 km (50 km diameter) |
| Minimum Elevation Angle | 10 degrees for service and feeder links |
| Number of UEs | 100 randomly distributed within the satellite beam |
| UE Altitude | Assumed to be at sea level (0 meters) |
| RTT and Doppler Errors | Calculated for each UE and compared to find maximum differences |

Gateway Elevation Angle = 10

| UL transmission time (repetitions) | 2 ms (x2) | 4 ms (x4) | 8 ms (x8) | 16 ms (x16) | 32 ms (x32) |
| --- | --- | --- | --- | --- | --- |
| Max Delay drift gap among UEs | 0.0002 us | 0.0003 us | 0.0006 us | 0.0014 us | 0.0027 us |
| Max Delay drift per UE | 0.1815 us | 0.3630 us | 0.7260 us | 1.4521 us | 2.9040 us |
| Min Delay drift per UE | 0.1813 us | 0.3627 us | 0.7254 us | 1.4507 us | 2.9013 us |
| Mean of Max Delay drift | 0.1814 us | 0.3628 us | 0.7257 us | 1.4514 us | 2.9028 us |
| Variance of Max Delay drift | 2e-15 us | 7e-15 us | 3e-14 us | 1e-13 us | 5e-13 us |
| Max Doppler drift gap among UEs | 1e-6 ppm | 3e-6 ppm | 5e-6 ppm | 1e-5 ppm | 2e-5 ppm |

Gateway Elevation Angle = 30

| UL transmission time (repetitions) | 2 ms (x2) | 4 ms (x4) | 8 ms (x8) | 16 ms (x16) | 32 ms (x32) |
| --- | --- | --- | --- | --- | --- |
| Max Delay drift gap among UEs | 0.0013 us | 0.0028 us | 0.0052 us | 0.0112 us | 0.0225 us |
| Max Delay drift per UE | 0.1597 us | 0.3194 us | 0.6386 us | 1.2780 us | 2.5556 us |
| Min Delay drift per UE | 0.1584 us | 0.3166 us | 0.6334 us | 1.2668 us | 2.5331 us |
| Mean of Max Delay drift | 0.1590 us | 0.3181 us | 0.6359 us | 1.2718 us | 2.5440 us |
| Variance of Max Delay drift | 1e-13 us | 6e-13 us | 2e-12 us | 9e-12 us | 4e-11 us |
| Max Doppler drift gap among UEs | 1e-5 ppm | 3e-5 ppm | 5e-5 ppm | 1e-4 ppm | 2e-4 ppm |

Gateway Elevation Angle = 60

| UL transmission time (repetitions) | 2 ms (x2) | 4 ms (x4) | 8 ms (x8) | 16 ms (x16) | 32 ms (x32) |
| --- | --- | --- | --- | --- | --- |
| Max Delay drift gap among UEs | 0.0057 us | 0.0115 us | 0.0224 us | 0.0453 us | 0.0769 us |
| Max Delay drift per UE | 0.0895 us | 0.1793 us | 0.3585 us | 0.7157 us | 1.4285 us |
| Min Delay drift per UE | 0.0837 us | 0.1678 us | 0.3362 us | 0.6704 us | 1.3416 us |
| Mean of Max Delay drift | 0.0867 us | 0.1736 us | 0.3469 us | 0.6948 us | 1.3858 us |
| Variance of Max Delay drift | 2e-12 us | 9e-12 us | 4e-11 us | 2e-10 us | 5e-10 us |
| Max Doppler drift gap among UEs | 4e-5ppm | 9e-5ppm | 2e-4 ppm | 3e-4 ppm | 7e-4 ppm |

Gateway Elevation Angle = 90

| UL transmission time (repetitions) | 2 ms (x2) | 4 ms (x4) | 8 ms (x8) | 16 ms (x16) | 32 ms (x32) |
| --- | --- | --- | --- | --- | --- |
| Max Delay drift gap among UEs | 0.0074 us | 0.0155 us | 0.0305 us | 0.0634 us | 0.1211 us |
| Max Delay drift per UE | 8.2e-3 us | 0.0161 us | 0.0328 us | 0.065137 us | 0.1259 us |
| Min Delay drift per UE | 7.5e-4 us | 5.5e-4 us | 2.3e-3 us | 1.7e-3 us | 4.8e-3 us |
| Mean of Max Delay drift | 0.0867 us | 8.6e-3 us | 0.0177 us | 0.03372 us | 0.069 us |
| Variance of Max Delay drift | 2e-12 us | 2e-11 us | 7e-11 us | 3e-10 us | 9.6e-10 us |
| Max Doppler drift gap among UEs | 0.001 ppm | 0.002 ppm | 0.005 ppm | 0.010 ppm | 0.020 ppm |