3GPP TSG RAN WG1 #116			R1-2401313

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	8.5

Title:	On maintenance for coverage enhancements

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In the contribution we discuss the maintenance issues for further NR coverage enhancements.

# Remaining issues on PRACH

Based on the feature lead summary of the previous discussions in the last RAN1 115 meeting [1], at least the following remaining issues may be discussed:

- Notification to higher layers to suspend power ramping counter.

- How to apply PRACH mask.

- Coupling between PRACH repetition and Msg3 repetition (including power offset between PRACH and Msg3).

In the following, we discuss these remaining issues.

## Notification to higher layers to suspend power ramping counter

The following agreement was made in RAN1-115 meeting for further down-selection [2]:

Agreement For multiple PRACH transmissions, down-select one of the following options at RAN1#116: - Option 1: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped or with reduced transmit power. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in any of PRACH occasions are dropped or with reduced transmit power. - Option 1a: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission on part of PRACH occasions are dropped or when PRACH transmission in any of PRACH occasions is with reduced transmit power. - Option 2: - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped or with reduced transmit power. Note: this implies it’s up to UE implementation. - Option 2a: - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is with reduced transmit power. - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped. - Option 3: - Layer 1 notifies higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are dropped. - Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in all of PRACH occasions are with reduced transmit power. Note: whether any of the above options have specification impact is a separate discussion.

In legacy PRACH, if a PRACH transmission is dropped, Layer 1 notifies the higher layers to suspend the power ramping counter. However, if PRACH is transmitted with reduced power, Layer 1 may or may not notify the higher layers to suspend the counter, up to UE implementation. This rationale can be explained by the fact that a dropped PRACH transmission cannot be received naturally, therefore suspending the power ramping counter is required. On the other hand, if PRACH is transmitted with reduced power, the base station may still be likely to detect this PRACH transmissions depending on how much of the PRACH power is reduced. Therefore, it is reasonable to allow UE to decide by implementation whether the power ramping counter should be suspended or not.

Similar line of thought can be applied for multiple PRACH transmissions. For example, if UE drops some the of multiple PRACH transmissions or transmits with reduced power, the base station can still decode the PRACH transmission(s) that are not dropped. In other words, Layer 1 should not be required to notify higher layers to suspend the power ramping counter. Therefore, Option1 and Option2a are not preferred options. Option1a and Option3 are similar in the sense that Layer1 is required to notify higher layers only if all PRACH transmissions are dropped. Although this is reasonable approach, Option2 can also have the similar behaviour. It ma y be preferrable to leave this decision to UE implementation as Layer1 needs to make a decision for many of the conditions (e.g., whether one or more PRACH transmissions dropped and if yes, how many; whether any of the PRACH transmissions with reduced power and if yes, how much power, etc.) by implementation anyways. We propose to leave it to UE implementation.

Proposal 1: Support Option 2.

- Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped or with reduced transmit power. Note: this implies it’s up to UE implementation

## Applying PRACH mask

The following agreement was made in RAN1-115 meeting for further down-selection [2]:

Agreement For multiple PRACH transmissions with indication of PRACH mask index, down-select one of the following options at RAN1#116 - Option 1: UE applies PRACH mask prior to RO group determination. RO group is determined based on the ROs indicated by the PRACH mask index. - Option 2: UE applies PRACH mask after RO group determination. UE transmits PRACH with Npreamblerep preamble repetitions only on a RO group with all the ROs indicated by the mask. - Option 3: UE applies PRACH mask after RO group determination. UE transmits PRACH with Npreamblerep preamble repetitions only on a RO group where at least one RO of this RO group is indicated by the mask - Option 4: UE applies PRACH mask after RO group determination. The PRACH mask index indicates one or multiple RO groups for multiple PRACH transmission. - Note: this implies the PRACH mask index indicates the RO group instead of RO index(es).

On PRACH mask with multiple PRACH transmissions, there are two potential issues. One issue is the RO group determination. If the PRACH mask is applied before the RO group determination, RO groups will need to be determined based on the PRACH mask index, which may cause unnecessary complexity. Therefore, it is preferred to apply PRACH mask after the RO group determination. However, the issue here is that there may not be enough RO groups left after the mask is applied. This scenario should be avoided. Therefore, the best option might be if the mask index points to RO groups instead of each RO for multiple PRACH transmissions.

Proposal 2: Support Option 4.

- UE applies PRACH mask after RO group determination. The PRACH mask index indicates one or multiple RO groups for multiple PRACH transmission.

## Coupling between PRACH repetition and Msg3 repetition

Another remaining issue after RAN1-115 is whether any coupling between PRACH repetitions and Msg3 repetition is needed. In our view this discussion is out of the work scope. However, companies also discussed a specific issue on the power offset between PRACH and Msg3 as there is already a legacy power offset relationship between these transmissions.

The discussed issue is basically whether more than one msg3-DeltaPreamble values need to be configured for different numbers of multiple PRACH transmissions. It is not clear if the motivation to consider such multiple values requires necessary change in the maintenance phase. In our view, a single value for msg3-DeltaPreamble is sufficient and can be applied as power offset in case of any number of multiple PRACH transmissions. We don’t see any need for a correction to the specification.

Proposal 3: No need to have multiple power offset values between PRACH and Msg3.

# Remaining issues on Dynamic Waveform Switching

# Conclusions

We have the following proposals:

Proposal 1: Support Option 2.

- Layer 1 may notify higher layers to suspend the corresponding power ramping counter when PRACH transmission in at least one PRACH occasion is dropped or with reduced transmit power. Note: this implies it’s up to UE implementation

Proposal 2: Support Option 4.

- UE applies PRACH mask after RO group determination. The PRACH mask index indicates one or multiple RO groups for multiple PRACH transmission.

Proposal 3: No need to have multiple power offset values between PRACH and Msg3.

# References

- R1-2312677, FL Summary #5 on remaining issues for PRACH coverage enhancements, Moderator (China Telecom), RAN1#115, Chicago, November 2023.

- Chairman’s notes, RAN1-115, Chicago, November 2023