3GPP TSG RAN WG1 #116		R1-2401320

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	8.2

Source: 	MediaTek Inc.

Title: 	On maintenance for NR sidelink evolution

Document for:	Discussion/Decision

Introduction

In this contribution, we provide some TP including the following issues

- Rel-18 remining higher layer parameter

- S-RSSI measurement for SL-U

Discussions

Rel-18 higher layer parameter

After discussion in RAN1 #115, there is one remaining higher layer parameter for Rel-18 SL-U: intraCellGuardBandsSL-List, and the controversial issue for this parameter is whether the value of 0 should be excluded from the value range of nrofCRBs as copied below

RAN1 #115 – higher layer parameter - intraCellGuardBandsSL-List: value range SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16 FFS: whether to exclude 0 from the value range of nrofCRBs.

For legacy NR-U, the value of 0 is included in the value range of nrofCRBs, which is used to indicate a UE that no intra-cell guard-bands are configured for the carrier. In this case, per NR-U, the UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an UL bandwidth part as described in clause 7 of [8], and the UE fails to access any of the channels of the UL bandwidth part.

Different from NR-U that the IE of IE intraCellGuardBandsUL-List or intraCellGuardBandsDL-List is configured per UE, for SL-U, it would be configured per SL BWP. In that case, the spectrum utilization efficiency would be very low if 0 is configured as the nrofCRBs. To that point, we slightly prefer to exclude 0 from the value range of nrofCRBs.

Proposal 1: For the value range of Rel-18 higher layer parameter intraCellGuardBandsSL-List, support exclude 0 from the value range of nrofCRBs.

SL-RSSI measurement for SL-U

In RAN1 #115, a LS (R1-2400019) from RAN4 informed RAN1 an agreement was achieved in RAN4 to specify the SL-RSSI measurement requirements in SL-U for congestion control purpose:

R1-2400019 - LS on SL-U RSSI measurement from RAN4 Agreement: For congestion control purpose, SL-RSSI measurements use the OFDM symbols starting from the next symbol of the 2nd candidate starting symbol.

Correspondingly, a TP is needed for TS 38.215

Proposal 2: Regarding SL-RSSI measurement for congestion control purpose in SL-U, endorse the following TP with corresponding CR fields

TP #1 for SL-RSSI measurement for SL-U

| Reason for change: | A LS from RAN4 informed an agreement related to SL-RSSI measurement for congestion control purpose in SL-U, which needs to be reflected in the corresponding specification of TS 38.215 |
| --- | --- |
| Summary of change: | In clause 5.1.25 of TS 38.215, update the SL-RSSI measurement for congestion control purpose in SL-U |
| Consequences if not approved: | The agreement in RAN4 is not captured |

The Text Proposal for section 5.1.25 of TS 38.215 is provided as below

----------------------------------------------TP to TS 38.215 clause 5.1.25 starts-----------------------------------------

5.1.25	Sidelink received signal strength indicator (SL RSSI)

<<< UNCHANGED PARTS OMITTED >>>

| Definition | Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol, or starting from the next symbol of the 2nd candidate starting symbol for congestion control purpose of SL on shared spectrum. For frequency range 1, the reference point for the SL RSSI shall be the antenna connector of the UE. For frequency range 2, SL RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL RSSI value shall not be lower than the corresponding SL RSSI of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

<<< UNCHANGED PARTS OMITTED >>>

-----------------------------------------------TP to TS 38.215 clause 5.1.25 ends-----------------------------------------

Summary

Proposal 1: For the value range of Rel-18 higher layer parameter intraCellGuardBandsSL-List, support exclude 0 from the value range of nrofCRBs.

Proposal 2: Regarding SL-RSSI measurement for congestion control purpose in SL-U, endorse the following TP with corresponding CR fields

TP #1 for SL-RSSI measurement for SL-U

| Reason for change: | A LS from RAN4 informed an agreement related to SL-RSSI measurement for congestion control purpose in SL-U, which needs to be reflected in the corresponding specification of TS 38.215 |
| --- | --- |
| Summary of change: | In clause 5.1.25 of TS 38.215, update the SL-RSSI measurement for congestion control purpose in SL-U |
| Consequences if not approved: | The agreement in RAN4 is not captured |

The Text Proposal for section 5.1.25 of TS 38.215 is provided as below

----------------------------------------------TP to TS 38.215 clause 5.1.25 starts-----------------------------------------

5.1.25	Sidelink received signal strength indicator (SL RSSI)

<<< UNCHANGED PARTS OMITTED >>>

| Definition | Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol, or starting from the next symbol of the 2nd candidate starting symbol for congestion control purpose of SL on shared spectrum. For frequency range 1, the reference point for the SL RSSI shall be the antenna connector of the UE. For frequency range 2, SL RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL RSSI value shall not be lower than the corresponding SL RSSI of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

<<< UNCHANGED PARTS OMITTED >>>

-----------------------------------------------TP to TS 38.215 clause 5.1.25 ends-----------------------------------------

Reference

[1] Chair's notes RAN1#115, Nov. 2023, Chicago, USA.