3GPP TSG RAN WG1 #116			                  R1-2401335

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.6.1

Source: 	LG Electronics

Title: 	Discussion on LP-WUS and LP-SS design

Document for:	Discussion and decision

- Introduction

A new WI on low-power wake-up signal and receiver for NR was approved in RAN#102 [1], where the WID objectives are identified as shown below.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103

In this contribution, we discuss and provide views on LP-WUS and LP-SS design for NR.

- Discussions on LP-WUS design

During the SI period, the following target use cases were identified in TR 38.869 [2] with consideration of various scenarios for LP-WUS/WUR.

- IoT cases, e.g., industrial wireless sensors, controllers, actuators

- Wearable cases, e.g., smart watches, rings, eHealth related devices, and medical monitoring devices

- eMBB cases, e.g., XR/smart glasses, smart phones

Each use case may have different characteristics in terms of power-sensitivity, form-factor size, latency requirement, mobility, etc. Accordingly, the data rate and robustness required for each use case may be different. In order to satisfy these various requirements, RAN1 should strive to design LP-WUS configurable sufficiently.

2.1. LP-WUS signal structure

OOK-1 and/or OOK-4 can be considered as a candidate waveform for LP-WUS. Among these two, OOK-1 waveform can be more robust to timing error because it uses only single time segment within OFDM symbol. On the other hand, OOK-4 using multiple time segments can provide a range of spectral efficiencies while robustness to timing error decreases with the increasing of number of segments. Moreover, these waveforms can be transmitted with overlaid OFDM sequence where the sequence can be designed differently depending on the purpose of use. Considering these multiple types of LP-WUS transmission, it is desirable to include a preamble part that precedes the message part of LP-WUS. LP-WUR can be provided the useful information (e.g., OOK modulation method, data rate, payload size) about message part to be transmitted subsequently through the preamble part. Moreover, depending on UE implementation, this information may help reduce the detection/decoding complexity of LP-WUR. Additionally, a certain type of CRC can be attached to the message part of LP-WUS to reduce the FAR. However, the length of CRC to be used needs to be carefully determined depending on the size of the message part to be transmitted through LP-WUS and the use of CRC needs to be optional to reduce transmission overhead.

Proposal #1: Specify the LP-WUS structure including preamble part, message part and CRC

- Preamble part: Configurations on message part can be included

- Message part: UE ID or sub-group ID can be included

- CRC part: It can be optionally attached according to the length of message part

2.2. Overlaid sequence

According to objective in WID, LP-WUS is transmitted through OOK (OOK-1 and/or OOK-4) based waveform with overlaid OFDM sequence(s). The overlaid OFDM sequence can be decoded by LP-WUR via time-domain correlation without FFT or frequency-domain correlation with FFT. However, it is worth noting that not all LP-WUR can detect/decode the overlaid OFDM sequences. Considering the various use cases in which LP-WUS can be utilized, the LP-WUR which can only perform energy detection over OOK symbols should not be overlooked. From this perspective, LP-WUR can be broadly classified into two types in terms of whether overlaid sequences can decode or not, as follows:

- LP-WUR type 1: LP-WUR capable of energy detection only

- LP-WUR type 2: LP-WUR capable of decoding (overlaid) OFDM sequence.

With two types of LP-WUR above, we consider the utilization of overlaid OFDM sequence into three cases as follows.

- Case 1: As specified in WID, in order for the same information to be delivered irrespective of LP-WUR type, the same information as that transmitted through OOK symbols can be transmitted in the overlaid sequences. For example, the UE ID can be transmitted through both the OOK symbols and the overlaid sequence, and each of LP-WUR type 1 and LP-WUR type 2 can detect the UE ID at their convenient method.

- Case 2: A known sequence or reference sequence can be used as overlaid sequence. In this case, the information which is delivered through OOK symbols (e.g., UE ID) is not carried by overlaid sequences. However, in this case, in order to increase the usability of LP-WUS, it is conceivable to load a series of information helpful for LP-WUR. For example, cell information can be delivered by reference sequence to mitigate inter-cell interference for LP-WUR type 2.

- Case 3: The last case is to deliver additional information through the overlaid sequence. In general, LP-WUR type 2 has better capabilities compared to LP-WUR Type 1. Delivering additional information in overlaid sequences is the way of taking full advantage of LP-WUR type 2. gNB can provide improved services with LP-WUS in this case. However, this case may be limited to Connected mode only.

Proposal #2: Discuss on which type of information is delivered by overlaid OFDM sequence

Another discussion point is how to map the overlaid sequence(s) to the effective samples of OFDM symbol(s). Since the sampling rate of LP-WUR may be different for each UE, LP-WUR with poor sampling rate may not decode the overlaid sequence when the overlaid sequence is mapped to OFDM samples in high rate. For example, if each bit of the overlaid sequence is mapped to each sample of the OFDM symbol while the sampling rate of LP-WUR is lower than that of MR, LP-WUR may not be able to acquire all bits of the overlaid sequence. To support various sampling rates of LP-WUR, it is desirable that each bit of the overlaid sequence is mapped to a group of OFDM samples. The length of the overlaid sequence can be determined based on the number of samples in a OFDM symbol to which each bit of the overlaid sequence is mapped.

Proposal #3: Discuss on the sequence length (and detailed mapping to OFDM symbol) of overlaid OFDM sequence considering LP-WUR sampling rate

2.3. Separate transmission occasions for preamble and message part of LP-WUS

UE ID or UE group ID may be transmitted through LP-WUS. When transmitting the UE ID through OOK-1 based OOK symbols, a large number of OFDM symbols should be used for LP-WUS transmission. Moreover, if Manchester encoding is applied, the number of OFDM symbols required to transmit LP-WUS becomes quite large. It is undesirable since such a long transmission cause a low resource utilization for NW, and the time exposed to interference is prolonged, which may cause performance degradation. Additionally, the decoding complexity in LP-WUR may be increased.

If LP-WUS consists of a preamble part and a message part carrying the UE ID, it is desirable to transmit the preamble part separately from the message part. For example, the preamble can be transmitted with a specific time gap from the message part. For another example, message part can be transmitted within a specific time window from the preamble part. With these, the potential disadvantages when the preamble and message parts are transmitted continuously for a long time can be avoided. To further guarantee the flexibilities from NW side, the preamble part and message part can be configured as a separate monitoring occasions. It would be better to note that the monitoring occasions of LP-WUS message part can be determined based on UE ID or UE (sub) group ID for paging. By determining the MO according to the UE ID or UE group ID, the amount of information that should be delivered in the message part can be reduced. Details on this are described in our companion contribution [3]. In this scenario, a separate transmissions of preamble part and message part of LP-WUS can be more beneficial.

Proposal #4: Discuss on the separate transmissions for preamble part and message part of LP-WUS

2.4. Bandwidth and frequency location configurations

LP-WUS bandwidth and frequency location can be determined configurable for better scheduling flexibility at gNB. However, if they are configured fully flexible, LP-WUR would monitor LP-WUS in multiple frequency locations and bandwidth dynamically. This may require high monitoring and decoding complexity of LP-WUR, which may cause increased power consumption at LP-WUR.

To achieve the moderate NW flexibility and LP-WUR complexity at the same time, the bandwidth and frequency location of LP-WUS can be configured in association with BWP for MR or NR signal/channel for the cell (e.g., SSB). For example, LP-WUS can be configured with the same bandwidth and/or frequency location as initial BWP or SSB for Idle/Inactive mode UE. As another example for Connected mode UE, the frequency location of LP-WUS can be configured by frequency offset from the lowest RB/RE of the active BWP and the bandwidth of LP-WUS can be configured so that LP-WUS is fully overlapped with the active BWP of UE.

Alternatively, RAN1 can introduce the dedicated resources in frequency for LP-WUS transmission. Within the resources, the bandwidth and frequency location of LP-WUS can be configured with full flexibilities. It is noted that the dedicated resources are in common for multiple UEs (e.g., all or a group of Idle/Inactive UEs should monitor LP-WUS only within the frequency resources). This can also keep both NW flexibility and LP-WUS complexity at a reasonable level.

Proposal #5: Discuss on the bandwidth and frequency location of LP-WUS with consideration of NW flexibility and LP-WUR complexity

2.5. Considerations on CP duration

Another discussion point is how to transmit LP-WUS waveform in CP duration of MR OFDM symbol. Since the LP-WUS and NR signals during the CP duration can interfere with each other, whether and how the LP-WUS is transmitted in CP duration may impact the reception performance of frequency multiplexed NR signal. In particular, since the length of each OOK symbol decreases as M increases in OOK-4, the processing of OOK symbols overlapping with the CP duration may affect performance. Two different transmission methods can be considered. First, LP-WUS is omitted in CP duration of MR OFDM symbol and LP-WUR may perform the envelop detection except for the CP duration. In this case, however, LP-WUR should know the staring time of CP duration and the length of CP duration. It may require the accurate sync requirement for LP-WUR and potential signaling issue to indicate SCS of NR signal/channel to LP-WUR. Alternatively, LP-WUS is transmitted in CP duration of MR OFDM symbol and LP-WUR performs the envelop detection in MR OFDM symbol duration including CP. In this case, the performance degradation in CP duration may be expected, while LP-WUR has the advantage of not having to accurately detect the CP duration. That is, the sync requirement for LP-WUS/LP-WUR may be relaxed, and the potential signaling for indicating SCS of NR signal/channel may be reduced.

Proposal #6: Study on whether LP-WUS is transmitted in CP duration of MR OFDM symbol for NR signal/channel

Specifically, we present the example structure for OOK-4. The following figure shows the structure of OFDM symbol including CP and 4 OOK symbols that transmits 4 bits per OFDM symbol.

Figure 1 Different OOK symbol structures within OFDM symbol

Option A consists of 4 OOK symbols in OFDM symbol period excluding CP. In LP-WUR, after removing the CP, each OOK symbol can be decoded by envelop detection. Option B consists of 4 OOK symbols in OFDM symbol period including CP. In LP-WUR, the effort of removing the CP can be saved, but since there is no CP, LP-WUS signal can interfere with the NR signal/channel transmitted together. Option C and Option D can be considered as a way to solve the problem of Option B. Option C is a method in which the OFDM symbol including the CP is composed of 4 OOK symbols, but the OOK symbol overlapping with CP is not used to transmit LP-WUS data. Since the 404κ samples shown in the figure is not used for transmission, the data rate may be slightly lower than other options. Option D consists of 4 OOK symbols except for CP length at front and rear parts of the OFDM symbol to maintain orthogonality of OFDM. Both CP parts can be filled with zeros or the same dummy samples.

For each option, the length of OOK symbol can be different from other options. However, the performance of envelop detector due to such slight difference in the number of samples would not be significant. We propose to discuss above 4 candidate options to define and construct OOK symbols within OFDM symbol for OOK-4.

Proposal #7: Study on how to define/construct OOK symbols within OFDM symbol for OOK-4

- Discussions on LP-SS design

3.1. LP-SS signal structure

It is desirable that LP-SS is as simple as possible to accommodate low-power and low-complexity non-coherent receiver (LP-WUR). Since, as described in WID objective, LP-SS is based on the same waveform of LP-WUS using OOK-1 and/or OOK-4 waveform, LP-SS can be transmitted consecutively with ON and OFF symbols. There are two candidate options for LP-SS design as discussed in SI:

- Option 1: LP-SS is transmitted as sequence-based signal(s) with consecutive OOK symbols

- Option 2: LP-SS is transmitted as message-based signal(s) with consecutive OOK symbols

In our view, Option 1 (especially with known sequence) should be considered as LP-SS transmission. At first, the one of main purposes of LP-SS is time/frequency synchronization at LP-WUR. For this purpose, the sequence-based signal can be beneficial than message-based signal. Second, Option 2 would have high FAR (or even failure of decoding) and high decoding complexity of LP-SS than Option 1. In addition, the amount of information delivered by LP-SS is not so much or even zero. In this perspective, the sequence-based signal with consecutive OOK symbols can be a reasonable option for LP-SS transmission.

Proposal #8: Specify the sequence-based signal with consecutive OOK symbols for LP-SS

LP-SS can be used for both time/frequency synchronization and serving cell RRM measurement at LP-WUR. With that, there are two consideration points to design LP-SS sequence. Firstly, for the purpose of synchronization and RRM measurement, the sequence used for optimal performance may be different. For example, LP-SS used for RRM measurement may be beneficial with the sequence that has equal power within the sequence duration and LP-SS used for synchronization may be beneficial with the sequence which has good correlation properties. However, it should be noted that the use of two types of sequences for different purposes may require higher complexity to both network and UE side. Next, as described in WID, periodic LP-SS needs to be specified during WI. Common periodic LP-SS can be utilized for both purpose of LP-SS though, the periodicity of each purpose (synchronization or RRM measurement) may be configured separately since the minimum required number of LP-SS transmission for each purpose can be different. In this perspective, defining different occasion for synchronization and RRM measurement could be beneficial than using different sequences for different purposes. For example, considering channel condition or gNB implementation, gNB may indicate different periodicity for each purpose. Additionally, even though same sequence is used for both synchronization and RRM measurement, the length of the sequence might differ. For example, gNB might configure LP-SS of 20ms periodicity is used for synchronization and LP-SS of 32ms periodicity is used for RRM measurement or every periodic LP-SS is used for synchronization and aperiodic LP-SS is used for RRM measurement. For periodic LP-SS, to achieve sync/RRM measurement requirement, the length of the sequence could be longer than aperiodic LP-SS which is transmitted as part of LP-WUS.

Proposal #9: Discuss on the required LP-SS periodicity for synchronization and RRM measurement purpose separately

Another potential discussion point is whether to overlay OFDM sequence over OOK symbols for LP-SS or not. Unlike LP-WUS, applying overlaid OFDM sequence to LP-SS needs to be discussed in WI. There can be many benefits for using overlaid OFDM sequences. Using overlaid OFDM sequence may increase coverage for LP-WUR which can correlate OFDM sequence in time or frequency domain or can deliver additional information. For example, LP-WUS monitoring occasion may be indicated by the overlaid OFDM sequence. However, since LP-SS may not need to deliver large amount of information, additional information may not be needed. Also, there are many methods to increase coverage which may have more advantages than using overlaid OFDM sequence. For example, increasing coverage with overlaid OFDM sequence may cause coverage imbalance issue between LP-WURs that can receive/decode OFDM sequence or not. Additionally, the complexity for gNB and LP-WUR may increase by applying overlaid OFDM sequence for LP-SS. Comparing to the amount of the resource increase requirement, a discussion about the benefits obtained by overlaid OFDM sequence is needed. In this perspective, the necessity of overlaid OFDM sequence needs to be discussed.

Proposal #10: Consider LP-SS without overlaid OFDM sequence as starting point. The necessity of overlaid OFDM sequence on LP-SS can be discussed with consideration of its performance benefit

3.2. Time domain transmission and occasions of LP-SS

As agreed in RAN1#112, the following three options for LP-SS transmission can be considered:

-	Option 1: Aperiodic signal transmitted as part of LP-WUS

-	Option 2: Periodic signal transmitted separately from LP-WUS

-	Option 3: Option 1 + Option 2

Regarding Option 1, as discussed in SI, it may be difficult to ensure time synchronization at LP-WUR with only aperiodic LP-SS which is transmitted as part of LP-WUS. For example, when LP-WUS transmission is not made for long duration, timing might be lost for LP-WUR. In this case, periodic LP-SS is needed for coarse synchronization to maintain time sync of LP-WUR. Also, for RRM offloading to the LR, periodic LP-SS might be needed to guarantee the RRM measurement requirement.

When LP-SS is transmitted along with LP-WUS, synchronization or RRM measurement requirement may not be satisfied since LP-SS is not guaranteed periodically. In this case, by adjusting aperiodic LP-SS occasion would be helpful to the aforementioned problems. For example, even though LP-SS is configured as aperiodic signal (as part of LP-WUS) gNB can configure the LP-SS occasion for synchronization and RRM measurement operation. Figure 2 shows an example of this case. As aforementioned, aperiodic LP-SS is transmitted in LP-WUS occasion since it needs to be transmitted as part of LP-WUS. However, in this case, when aperiodic LP-SS is sent along with LP-WUS, gNB can configure a certain occasion that is mandatory to transmit LP-SS to maintain the time sync or RRM measurement of LP-WUR as shown in Figure 2.

Periodic LP-SS is sent separately as LP-WUS as show in Figure 2. Since aperiodic LP-SS which is sent similar as preamble of LP-WUS is absent, both synchronization and RRM measurement need to be done with periodic LP-SS.

However, transmitting periodic LP-SS frequently may be burden to gNB. To lead less burden to gNB, transmitting both aperiodic and periodic LP-SS would be a straightforward solution. For example, when both aperiodic and periodic LP-SS is transmitted, periodic LP-SS could be sparsely transmitted and setting a certain occasion for aperiodic LP-SS may solve the issue for synchronization or RRM measurement. Additionally, as shown in Figure 2, periodic LP-SS and aperiodic LP-SS may use different sequence or even different length of the sequence for their use cases respectively.

Figure 2 LP-SS occasion with 3 different transmission schemes

Proposal #11: Discuss how to configure the LP-SS transmission/monitoring occasions (together with LP-WUS monitoring occasions) considering synchronization and RRM measurement

3.3. Bandwidth and frequency location of LP-SS

Similar to LP-WUS, the bandwidth and frequency location of LP-SS can be determined configurable for better scheduling flexibility at gNB. To achieve a moderate flexibility and complexity, LP-SS bandwidth can be as same as PSS/SSS in SSB. There are advantages in some aspects when frequency location of LP-SS is configured dependently to PSS/SSS. For example, RRM measurement by LP-SS could be done with a similar frequency regardless of LP-WUR can receive existing PSS/SSS or not. And it can save the gNB effort to perform RF retuning when LP-SS are transmitted next to SSB. For instance, the frequency location of LP-SS can be configured based on the PSS/SSS frequency location. In this perspective, frequency location of LP-SS may be configured according to the PSS/SSS location.

Proposal #12: Discuss on the bandwidth and frequency location of LP-SS with consideration of NW flexibility and LP-WUR complexity

3.4. Considerations on inter-cell interference

LP-SS should be configured as a cell-specific signal that could be used for serving cell RRM measurement. Meanwhile, if a UE supporting LP-WUS is located at the cell edge, synchronization and RRM measurement through LP-SS may be affected by UE due to inter-cell interference. Following options could be considered for inter-cell interference mitigation.

-	Option 1: LP-SS transmission on different frequency according to physical cell ID

-	Option 2: LP-SS transmission with different cyclic shift according to physical cell ID

For Option 1, the frequency location of LP-SS can be differently determined based on physical cell ID. For Option 2, different cyclic shift can be determined based on physical cell ID. In this way, the inter-cell interference can be avoided by transmitting LP-SS based on the differentiation of each cell. For inter-cell interference mitigation, above simple options can be a starting point.

Proposal #13: Consider the inter-cell interference mitigation technique for LP-SS design

- Conclusions

In this contribution, we have discussed on the various aspects for LP-WUS and LP-SS design, and the followings are proposed.

Proposal #1: Specify the LP-WUS structure including preamble part, message part and CRC

- Preamble part: Configurations on message part can be included

- Message part: UE ID or sub-group ID can be included

- CRC part: It can be optionally attached according to the length of message part

Proposal #2: Discuss on which type of information is delivered by overlaid OFDM sequence

Proposal #3: Discuss on the sequence length (and detailed mapping to OFDM symbol) of overlaid OFDM sequence considering LP-WUR sampling rate

Proposal #4: Discuss on the separate transmissions for preamble part and message part of LP-WUS

Proposal #5: Discuss on the bandwidth and frequency location of LP-WUS with consideration of NW flexibility and LP-WUR complexity

Proposal #6: Study on whether LP-WUS is transmitted in CP duration of MR OFDM symbol for NR signal/channel

Proposal #7: Study on how to define/construct OOK symbols within OFDM symbol for OOK-4

Proposal #8: Specify the sequence-based signal with consecutive OOK symbols for LP-SS

Proposal #9: Discuss on the required LP-SS periodicity for synchronization and RRM measurement purpose separately

Proposal #10: Consider LP-SS without overlaid OFDM sequence as starting point. The necessity of overlaid OFDM sequence on LP-SS can be discussed with consideration of its performance benefit

Proposal #11: Discuss how to configure the LP-SS transmission/monitoring occasions (together with LP-WUS monitoring occasions) considering synchronization and RRM measurement

Proposal #12: Discuss on the bandwidth and frequency location of LP-SS with consideration of NW flexibility and LP-WUR complexity

Proposal #13: Consider the inter-cell interference mitigation technique for LP-SS design

- References

RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)” CMCC (Moderator), RAN#102

RP-233068, “Study on low-power wake up signal and receiver for NR”, 3GPP TR 38.869 V2.0.0

R1-2401336, “Discussion on LP-WUS operation in IDLE/INACTIVE modes”, LG Electronics, RAN1#116