3GPP TSG RAN WG1 #116	R1-2401348

Athens, Greece, Feb 26 - Mar 1, 2024

Agenda item:		9.6.2

Source:	Nordic Semiconductor ASA

Title:					On LP-WUS operation in IDLE/Inactive

Document for:		Discussion and Decision

# Introduction

In RAN#102, work item in RP-234056 has been approved.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103

In this contribution, we focus on LP-WUS procedures.

# Discussion

## Monitoring procedures in IDLE/INACTIVE

Agreement Study further pros and cons of the following monitoring behaviours of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time RAN plenary guidance - At least duty-cycled monitoring of LP-WUS is supported

Duty-cycle monitoring has been prioritized by RAN plenary. Continuous monitoring can be achieved by configuring MOs of LP-WUS with slot/symbols granularity. It is already clear that LP-WUS will be multiple symbols long even if payload is kept small. One aspect to discuss further is whether MOs can overlap or not. In other words, whether minimum LP-WUS periodicity will be bounded by LP-WUS length or not.

Proposal-1: UE may be configured with monitoring occasions (MOs) for LP-WUS with symbol granularity. Discuss further whether Mos for a single UE can overlap or not.

## Procedure upon wake-up

During SI the following options were on the table: - Option 1: perform PO monitoring, and afterwards follow legacy procedures. - Option 1’: introduce dynamic PO - Option 2: perform PEI monitoring, and afterwards follow legacy procedures. - Option 3: transmit PRACH for initial access, and follow legacy procedures. - Option 4: monitor system information, and follow legacy procedures RAN2 - After waking up by a LP-WUS, capture the below solutions in the TR: - Alt 1.1: UE could monitor paging DCI/paging; - Alt 1.2: UE could monitor PEI, if configured and supported; FFS details on using LP-WUS and PEI together, e.g. subgrouping - FFS Alt 2: UE could perform random access directly, FFS on whether and what condition/requirement is needed. R2 assumes that this require that LP-WUS includes UE_ID or equivalent. (Depends on LP-WUS capacity to carry information) - For Alt.1 above, after waking up by a LP-WUS, RAN2 assumes the baseline is the UE monitors the legacy PO.

Option 1 is a baseline and should be specified at first. Any latency improvements on top of Option 1 may not be well justified, especially since wake-up ramp-up times were studied to be 0.5-1s. Considering 1.28s DRX cycle, PRACH (Option3) or dynamic PO (Option 1’) may provide on average at most 0.64s advantage. It is not clear whether any specification effort would be justified.

Option 2 would provide improved FAR, but also increase power consumption and latency. Option 2 could be considered after Option 1 is specified.

We believe that decision on whether Option 2, Option 3 or Option 1’ are specified should be left to RAN2. For now, RAN1 focuses on Option 1 until further progress in RAN2.

RAN1 role in this topic is to decide whether any restriction on the LP-WUS MO position is required or not. If gNB configures LP-WUS MO taking into account UE’s wake-up time, the latency is minimized. On the other hand, as discussion in 9.6.1 it is also beneficial to align LP-WUS MOs with position of LP-SS.

Further aspect is whether multiple POs can be associated with a single LP-WUS MO, like it is in PEI. Given limited payload LP-WUS can carry and for the sake of simplicity, there should be one-to-one mapping between LP-WUS MO and PO.

Proposal-2:

- RAN1 to focus on Option 1 design, and

  - discuss whether any restriction on position LP-WUS with respect to associated PO are required.

  - discuss association between PO MO and LP-WUS MO.

- RAN2 to decide on latency enhancement, if any

## Activation procedures

Agreement - For Idle/Inactive mode, following options for activation and deactivation of LP-WUS monitoring by LP-WUR for a UE can be considered for study - Alt 1a: - gNB transmits legacy paging indication and LP-WUS - UE activation and/or deactivation of LP-WUS WUS monitoring is up to UE implementation. - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient. - Alt 1b: - gNB transmits legacy paging indication and LP-WUS - UE activation and/or deactivation of LP-WUS monitoring is based on preconfigured criteria - This behavior may apply based on channel condition, e.g. when coverage is sufficient/insufficient. - Alt 2: - activation and/or deactivation of LP-WUS monitoring in a cell is based on signalling. - Paging misdetection performance shall not be impacted. RAN2 agreement - Entry/exit condition(s) of using LP-WUS is configured in SIB. - FFS via RRC dedicated signaling, e.g. by RRC release. - Entry condition(s) of using LP-WUS include at least good serving cell quality, e.g. the serving cell quality measurement on LR and/or serving cell quality measurement on MR is better than configured threshold(s) in SIB. Other condition(s) is not precluded/FFS. - UE stops using LP-WUS when exit condition(s) configured in SIB is fulfilled. The exit condition(s) includes at least out of coverage of LP signaling, e.g. the serving cell quality measured by LR is less than the configured threshold in SIB, FFS on measurement on MR.

RAN2 agreed during SI that at least threshold conditions are defined in SIB for entry and exist criteria. This means that Alt 1b has been de-facto agreed in RAN2, and whether UE monitors PEI or PO is transparent to network. To ensure that gNB is capable to configure correct entry and exit criteria. RAN4 requirements for LP-WUS coverage should be defined and it should be ensured that UE is capable to measure at least RSRP-metric based on LP-SS. While these aspects are in the territory of RAN2 and RAN4, RAN1 role here is to ensure that LP-SS design enables sufficient accuracy of measurement by LR, i.e. to design LP-SS to be long enough, and or enable UE to average over multiple LP-SS occurrences.

When UE gets to cell-edge, S(I)NR starts to drop rapidly. What should be the target S(I)NR should be further discussed. Targeting target-S(I)NR= -3dB with 90% accuracy could be a starting point.

Observation-1: RAN1 needs to decide what is the target S(I)NR for RRM measurement accuracy. -3dB should be a starting point.

# Conclusions

In this contribution we discussed issues related to LP-WUS signal design and had observations and proposals:

Proposal-1: UE may be configured with monitoring occasions (MOs) for LP-WUS with symbol granularity. Discuss further whether MOs for a single UE can overlap or not.

Proposal-2:

- RAN1 to focus on Option 1 design, and

  - discuss whether any restriction on position LP-WUS with respect to associated PO are required.

  - discuss association between PO MO and LP-WUS MO.

- RAN2 to decide on latency enhancement, if any

Observation-1: RAN1 needs to decide what is the target S(I)NR for RRM measurement accuracy. -3dB should be a starting point.