3GPP TSG RAN WG1 #116	                                   					       												                                    R1-2401360

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	8.12.6

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR network energy savings

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion [116-R18-UE_features] during RAN1 #116. According to the Chair’s Notes:

[116-R18-UE_features] Email discussion on Rel-18 UE features – Ralf (AT&T) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed and/or agreed during RAN1 #116 within the scope of [116-R18-UE_features]. All proposals are based on the latest RAN1 UE features list for Rel-18 in [1].

# Summary of Contributions Submitted to RAN1 #116

The following is the moderator’s summary of contributions submitted to RAN1 #116 in this agenda item.

| 42. Netw_Energy_NR | 42-1 | Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for periodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP | FFS | Yes |  | UE does not support spatial domain adaptation for periodicCSI reporting | Per band | No | No | N/A | Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4} Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4} Note: Components 6 and 7 are signaled per BC FFS: different values for joint operation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 42. Netw_Energy_NR; 42-1; Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for periodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP; Yes; UE does not support spatial domain adaptation for periodicCSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4} Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4} Note: Components 6 and 7 are signaled per BC; Optional with capability signaling. |
| Huawei/HiSilicon [3] | - For semi-persistent CSI reporting, the candidate values for configured number of sub-configurations and reported CSIs are to be determined. As it was agreed that “Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH” and “Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH”, we propose as follows: - For 42-1a and 42-2a, - The candidate values for L: 2, 3, 4, 5, 6, 7 and 8. - The candidate values for N: 1, 2, 3 and 4. - For aperiodic CSI reporting, the candidate values for reported CSIs are to be determined. The same values as semi-persistent CSI reporting can be used. - For 42-1b and 42-2b, - The candidate values for N: 1, 2, 3 and 4. - Regarding the prerequisite feature groups, since we are specifically design the NES feature with willingness of supporting deferent values from a legacy CSI report capability, there does not expect strong dependences among these FGs, and components 4)~7) are reported for each of these FGs respectively, so no prerequisite feature groups are needed. - Regarding whether to set different values for joint operation, from UE implementation perspective, the newly introduced FG is mainly related to UE capabilities, similar to the one defined in FG 2-33, 2-35 in Rel-15, about CSI measurement, CSI processing and CSI reporting. The difference between SD/PD only operation and joint operation mainly lays on the implementation at gNB side and configurations (to UE), while the difference for UE would be small, e.g. the supported maximum number of CSI reports per BWP, CPU counting or active resource/antenna ports counting are kept the same among different configurations for adaptation. Therefore, there is no justification to set different candidate values for joint operation. Proposal 3: For FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, - No prerequisite feature groups are needed. Proposal 4: For joint operation of SD and PD, - Remove the FFS in 42-1/42-1b/42-2/42-2b - Add a note in 42-1/42-1b/42-2/42-2b that same candidate values are used for joint operation of SD and PD, if a UE supports both SD and PD. |
| Vivo [4] |  |
| ZTE [5] | The following FG 23-7-1 is the ‘Basic Features of CSI Enhancement for Multi-TRP’, which is introduced in R17. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 2, Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 3. CSI report mode selection of mode 1 with X=0 and/or mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses 5. Supported codebook modes for NCJT CSI. The FG 23-7-1 is also about CSI enhancement, which does not set a ‘Prerequisite feature groups’. Given this, for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, the ‘Prerequisite feature groups’ doesn’t need, either. Proposal 2: ‘Prerequisite feature groups’ is not needed for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b. |
| CATT [6] | The support of the spatial and power domain adaptation requires additional UE CSI measurements and reports when some TxRUs are turned off. A number of TxRUs turning off and generating a new number of antenna ports or beam pattern is indicated as an antenna sub-configuration of the full antenna configuration. The additional CSI reports of sub-configurations with each CSI report associated with one sub-configuration will be fed back to the gNB for dynamic link adaptation. Multiple CSI reports associated with full antenna configuration and sub-configurations are included in all CSI report type; they are aperiodic CSI report, semi-persistent CSI report, and periodic CSI report. The UE capability of CSI measurements for A-CSI, SP-CSI and P-CSI feedbacks should be the same without any differentiation in the UE capability since the measurements of configured CSI-RS resource(s) are the same. In RAN1#115, the maximum number of SP-CSI measurements were agreed to be the same as that of P-CSI. Since the additional CSI measurements of antenna sub-configuration would be provisioned by the UE at each band. The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands and based on each UE (per UE) regardless if it is Type 1 or Type 2 antenna adaptation. For Type 1 antenna adaptation, the additional number of CSI measurements for CSI feedbacks in a CSI report is based on the measurements of a CSI-RS resource set. The maximum number of CSI measurements N from a CSI-RS resource set should be based on UE capability in additional CSI measurements and computation. The maximum number of CSI measurements N in a CSI report would depend on the number of network operation of spatial adaptation for network energy saving. The spatial domain adaptation relies on the ON/OFF of the TxRU to achieve the network energy saving. The transition time of the TxRU ON/OFF would be the deciding factor. The power amplifier of the TxRU would demands range of ms to stabilize the Tx power setting and would not be able to dynamically shutting down and turning on. Thus, the TxRU ON/OFF would not be substantially frequent at the gNB for spatial adaptation. The number of the CSI measurements of sub-configurations should not be large. Thus, the maximum number of CSI measurements of antenna sub-configurations in a CSI report should be set to 4. The maximum number of sub-configurations L for UE to support should be based on the possibility of spatial and power adaptations by the network. The level of the power adaptations would not see much network energy saving gain. Thus, the majority of the spatial and power adaptation is different spatial domain configurations. The maximum number of spatial and power domain sub-configurations should be set to 8. For joint operation of spatial and power domain adaptation, the maximum number of sub-configuration and the CSI reports should remain the same since the joint operation should not increase the additional complexity. Proposal 1: The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands for each UE (per UE). Proposal 2: The maximum number of antenna sub-configurations LMax for UE to support should be set to 8 with candidate values {1, 2, 4, 8} for spatial and/or power domain adaptation. Proposal 3: The maximum number of sub-configuration and the CSI reports for joint operation of spatial and power domain adaptation for NES should remain the same. |
| Samsung [7] | - Regarding FFS: different values for joint operation in Note The remaining issue is how to determine values for joint operation. For joint operation of SD and PD adaptation, RAN1 had made agreement to support joint operation for NES in RAN1#113. And, UE can have different capability values for each SD and PD adaptation. In this regard, even if UE supports both capabilities, there will be ambiguity on UE capability for joint operation, because UE can have different capabilities between SD and PD adaptation. Therefore, we propose to define new independent UE feature for joint operation of SD and PD adaptation for P/SP/AP CSI reporting. Proposal 1: Define new UE feature of joint operation of SD and PD adaptation. - Regarding FFS in Prerequisite feature groups FG2-33 and FG2-35 can be reused for all FGs related to SD and PD adaptation as baseline. Hence we suggest to include FG2-33 and FG2-35 as prerequisite feature groups of all FGs related to SD and PD adaptation. Proposal 2: For all FGs related to SD and PD adaptation, add FG2-33 and FG2-35 as prerequisite feature groups. - Regarding Component 9 Regarding the component 9 in FGs, it is clear that the value is for each report type, and this is associated with UE capability defined in FG2-35. The value of component 9 will be used instead of the values for component 1 and 2 in FG2-35, when CSI reporting with sub-configuration for SD and PD adaptation is configured. In case of UE is configured with Rel-18 NES CSI report and legacy CSI report, to be clear the intention of component 9, we propose to add Note in FG42-1, FG42-1b, FG42-2 and FG42-2b. Proposal 3: Add a note in FG42-1, FG42-1b, FG42-2 and FG42-2b that ‘Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration’. - Regarding Component 4, 5, 6, and 7 In terms of components related to maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs, it is still unclear how to determine maximum value and how to count number of simultaneous NZP-CSI-RS resource and CSI-RS ports per CC or in active BWPs across all CCs not only when both Rel-18 CSI report and legacy CSI report are configured but also when different types of adaptations or CSI reporting are configured on each CCs and/or each active BWPs in a CC. First, to count NZP-CSI-RS resource and CSI-RS ports, it would be reasonable to count all NZP-CSI-RS resource and CSI-RS ports for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b. Proposal 4: Add a note in all FGs that ‘Note: for the purpose of component-4, 5, 6 and 7 calculation: NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b’. Regarding to determine maximum value of components, we propose to use one value for all these FGs. It would be straightforward way to inherit the philosophy applied for FG2-33 which is a legacy counterpart of FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. Proposal 5: Add a note in all FGs that ‘Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b’. - Aspects to be newly considered Additionally, there are still ambiguity on how to count the number of CSI report(s) processed by UE simultaneously per CC or across all CCs. It is related to how many CSI reports can be processed simultaneously, and the number of CSI report(s) processed by UE simultaneously are defined in FG2-35. During Rel-18 NES discussion, to calculate the occupied number of CPUs, each CSI report corresponding to sub-configuration is counted as legacy CSI report. In this regard, the CSI report(s) corresponding to sub-configuration should be counted for legacy UE capability in FG2-35. And, the total number of CSI report(s) processed by UE simultaneously per CC or across all CCs should not be larger than the value of FG2-35. Therefore, we propose to add new components and note in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b as follow: - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. Proposal 6: Define new components and note for the maximum number of CPUs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. |
| Apple [8] | 42. Netw_Energy_NR; 42-1; Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for periodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP; 2-33, 2-35; Yes; UE does not support spatial domain adaptation for periodicCSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4} Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4} Note: Components 6 and 7 are signaled per BC; Optional with capability signaling. |
| NTT DOCOMO, INC. [9] | For FG42-1, there are two remaining FFS points, one is about prerequisite FG(s) and another is about potential different values for joint SD/PD operation. Regarding prerequisite FG(s) for FG42-1, the discussion point would be about the relationship among FG42-1/1a/1b, i.e., whether/which one of FGs is prerequisite for other FG(s). Although we originally prefer to have single FG for spatial domain adaptation with allowing UE to report different values for periodic/semi-persistent/aperiodic CSI reporting so that NW can configure any CSI reporting type for the UE supporting spatial domain adaptation, it was agreed to have separate FGs for periodic/semi-persistent/aperiodic CSI reporting. In Rel-15, basic CSI feedback capability FG2-32 covers periodic/aperiodic CSI reporting, while FG2-32a/b for semi-persistent CSI reporting are optional capabilities. Therefore, we prefer to have FG42-1 or FG42-1b as prerequisite FG for FG42-1a, while FG42-1 and FG42-1b would not require any prerequisite FG (basic CSI reporting capability FG2-32 is anyway mandatory). Proposal 6: FG42-1 does not require any prerequisite FG. Regarding the potential different values for joint SD/PD operation, we think it is simpler to define separate FGs for joint SD/PD operation so that UE can report same or different values for joint SD/PD operation from the values for SD operation and for PD operation. Proposal 7: FG42-3/3a/3b are defined for joint SD/PD operation. - The note “FFS: different values for joint operation” can be removed from FG42-1/1b/2/2b. |
| Ericsson [10] | For joint SD and PD operation, there are at least two options: - (Option 1) If UE indicates support for SD operation (e.g., 42-1b) and the UE also indicates support for PD operation (e.g., 42-2b), then the UE also supports joint SD+PD operation. - (Option 2) Introduce a separate capability for indicating joint SD + PD operation. We suggest adopting Option 2, with corresponding FGs for spatial domain and power domain as pre-requisites. New FGs with full components/component values for joint SD+PD operation are shown in Annex A though we note it is possible to rely on indicated pre-requisites to derive certain component values. Also, other aspects (e.g., value of N, etc) that may need common handling with existing FGs are left out of initial FG definition. Those are addressed together for both existing and new FGs in subsequent proposals. For NES, introduce new FGs for joint spatial and power domain adaptation with CSI feedback based on CSI report sub-configuration(s). - (New) FG 42-3 for periodic CSI reporting with pre-requisites:42-1 and 42-2. - (New) FG 42-3a for semi-persistent CSI reporting with pre-requisites:42-1a and 42-2a - (New) FG 42-3b for aperiodic CSI reporting with pre-requisites:42-1b and 42-2b 42. Netw_Energy_NR; (New) 42-3; Joint spatial and Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs and one power offset for periodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP; 42-1 and 42-2; Yes; UE does not support joint spatial and power domain adaptation for periodicCSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1 and PD, SD-type2 and PD, both (SD-type1 and PD) and (SD-type2 and PD)} Component 2 candidate value: {2,3,4} Component 4 candidate value: {1, 2, 3 … 32} Component 5 candidate value: {8, 16, 24, … 128 } Component 6 candidate value: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4}; Optional with capability signaling. 42. Netw_Energy_NR; (New) 42-3a; Joint spatial and Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs and one power offset f for semi-persistent CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook; 42-1b and 42-2b; Yes; UE does not support joint spatial and power domain adaptation for semi-persistent CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1 and PD, SD-type2 and PD, both (SD-type1 and PD) and (SD-type2 and PD)} Component 2 candidate values: FFS Component 3 candidate values: {1, 2, 3 … 32} Component 4 candidate values: {8, 16, 24, … 128 } Component 5 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 6 candidate values: {8, 16, 24, …, 248, 256} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 5 and 6 are signaled per BC; Optional with capability signaling. 42. Netw_Energy_NR; (New) 42-3b; Joint spatial and Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs and one power offset for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP; 42-1b and 42-2b; Yes; UE does not support joint spatial and power domain adaptation for aperiodic CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1 and PD, SD-type2 and PD, both (SD-type1 and PD) and (SD-type2 and PD)} Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values: FFS Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128 } Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} Note: Components 6 and 7 are signaled per BC; Optional with capability signaling. Some notes related to remaining issues for existing FGs (and applicable to new FGs also). - For component related to “Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration”, there is no need for defining candidate values related to N. For aperiodic/semi-persistent CSI reporting, UE should support all values of N up to Lmax, i.e. value of N up to Lmax is supported by the UE. - The components 4,5,6,7 can be updated to reflect the NES condition for applying the per-CC and across-CCs limits. - Pre-requisites: For 42-1/1a/1b/2/2a/2b: adding 2-33 and 2-35 is sufficient. Since these two are mandatory with capability signaling, it is strictly not necessary to explicitly reflect them in the FG table. For NES, FGs 42-1a/42-1b/42-2a/42-2b/(New 42-3a)/(New 42-3b), for component 3 (Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration), - Update the Note to reflect “Value of N: 1 <=N<=Lmax” - There is no need for defining candidate values for component 3 For NES, FGs 42-1/42-1a/42-1b/42-1/42-2a/42-2b/(New 42-3a)/(New 42-3b), update the components 4,5,6,7 as follows: - 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of simultaneous NZP-CSI-RS resources per CC - 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs |
| Sharp [11] |  |
| ETRI [12] |  |
| Vodafone [13] |  |
| LG Electronics [14] |  |
| Qualcomm Incorporated [15] | 42-1; Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for periodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP; FFS 2-35; Yes; UE does not support spatial domain adaptation for periodic CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4} Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4} Note: Components 6 and 7 are signaled per BC FFS: different values for joint operation Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain.; Optional with capability signaling. |
| MediaTek Inc. [16] |  |

| 42. Netw_Energy_NR | 42-1a | Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for semi-persistent CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook | FFS | Yes |  | UE does not support spatial domain adaptation for semi-persistent CSI reporting | Per band | No | No | N/A | Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: FFS Component 3 candidate values: FFS Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128} Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 6 and 7 are signaled per BC | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 42. Netw_Energy_NR; 42-1a; Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for semi-persistent CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook; 42-1; Yes; UE does not support spatial domain adaptation for semi-persistent CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4} Component 3 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128} Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 6 and 7 are signaled per BC; Optional with capability signaling. |
| Huawei/HiSilicon [3] | - For semi-persistent CSI reporting, the candidate values for configured number of sub-configurations and reported CSIs are to be determined. As it was agreed that “Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH” and “Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH”, we propose as follows: - For 42-1a and 42-2a, - The candidate values for L: 2, 3, 4, 5, 6, 7 and 8. - The candidate values for N: 1, 2, 3 and 4. - For aperiodic CSI reporting, the candidate values for reported CSIs are to be determined. The same values as semi-persistent CSI reporting can be used. - For 42-1b and 42-2b, - The candidate values for N: 1, 2, 3 and 4. - Regarding the prerequisite feature groups, since we are specifically design the NES feature with willingness of supporting deferent values from a legacy CSI report capability, there does not expect strong dependences among these FGs, and components 4)~7) are reported for each of these FGs respectively, so no prerequisite feature groups are needed. - Regarding whether to set different values for joint operation, from UE implementation perspective, the newly introduced FG is mainly related to UE capabilities, similar to the one defined in FG 2-33, 2-35 in Rel-15, about CSI measurement, CSI processing and CSI reporting. The difference between SD/PD only operation and joint operation mainly lays on the implementation at gNB side and configurations (to UE), while the difference for UE would be small, e.g. the supported maximum number of CSI reports per BWP, CPU counting or active resource/antenna ports counting are kept the same among different configurations for adaptation. Therefore, there is no justification to set different candidate values for joint operation. Proposal 1: For FG 42-1a/42-2a, - The candidate values for L: 2, 3, 4, 5, 6, 7 and 8. - The candidate values for N: 1, 2, 3 and 4. Proposal 3: For FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, - No prerequisite feature groups are needed. |
| Vivo [4] |  |
| ZTE [5] | For semi-persistent CSI reports and aperiodic CSI reports, the same candidate values of N can be used. And the maximum candidate value of N can refer to the maximum value of Lmax of periodic CSI report, which is 4. The minimum candidate value of N should be 1 to support single CSI sub-report. Proposal 1: The candidate values of N for SP-CSI report and AP-CSI report are {1,2,3,4}. The following FG 23-7-1 is the ‘Basic Features of CSI Enhancement for Multi-TRP’, which is introduced in R17. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 2, Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 3. CSI report mode selection of mode 1 with X=0 and/or mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses 5. Supported codebook modes for NCJT CSI. The FG 23-7-1 is also about CSI enhancement, which does not set a ‘Prerequisite feature groups’. Given this, for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, the ‘Prerequisite feature groups’ doesn’t need, either. Proposal 2: ‘Prerequisite feature groups’ is not needed for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b. |
| CATT [6] | The support of the spatial and power domain adaptation requires additional UE CSI measurements and reports when some TxRUs are turned off. A number of TxRUs turning off and generating a new number of antenna ports or beam pattern is indicated as an antenna sub-configuration of the full antenna configuration. The additional CSI reports of sub-configurations with each CSI report associated with one sub-configuration will be fed back to the gNB for dynamic link adaptation. Multiple CSI reports associated with full antenna configuration and sub-configurations are included in all CSI report type; they are aperiodic CSI report, semi-persistent CSI report, and periodic CSI report. The UE capability of CSI measurements for A-CSI, SP-CSI and P-CSI feedbacks should be the same without any differentiation in the UE capability since the measurements of configured CSI-RS resource(s) are the same. In RAN1#115, the maximum number of SP-CSI measurements were agreed to be the same as that of P-CSI. Since the additional CSI measurements of antenna sub-configuration would be provisioned by the UE at each band. The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands and based on each UE (per UE) regardless if it is Type 1 or Type 2 antenna adaptation. For Type 1 antenna adaptation, the additional number of CSI measurements for CSI feedbacks in a CSI report is based on the measurements of a CSI-RS resource set. The maximum number of CSI measurements N from a CSI-RS resource set should be based on UE capability in additional CSI measurements and computation. The maximum number of CSI measurements N in a CSI report would depend on the number of network operation of spatial adaptation for network energy saving. The spatial domain adaptation relies on the ON/OFF of the TxRU to achieve the network energy saving. The transition time of the TxRU ON/OFF would be the deciding factor. The power amplifier of the TxRU would demands range of ms to stabilize the Tx power setting and would not be able to dynamically shutting down and turning on. Thus, the TxRU ON/OFF would not be substantially frequent at the gNB for spatial adaptation. The number of the CSI measurements of sub-configurations should not be large. Thus, the maximum number of CSI measurements of antenna sub-configurations in a CSI report should be set to 4. The maximum number of sub-configurations L for UE to support should be based on the possibility of spatial and power adaptations by the network. The level of the power adaptations would not see much network energy saving gain. Thus, the majority of the spatial and power adaptation is different spatial domain configurations. The maximum number of spatial and power domain sub-configurations should be set to 8. For joint operation of spatial and power domain adaptation, the maximum number of sub-configuration and the CSI reports should remain the same since the joint operation should not increase the additional complexity. Proposal 1: The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands for each UE (per UE). Proposal 2: The maximum number of antenna sub-configurations LMax for UE to support should be set to 8 with candidate values {1, 2, 4, 8} for spatial and/or power domain adaptation. Proposal 3: The maximum number of sub-configuration and the CSI reports for joint operation of spatial and power domain adaptation for NES should remain the same. |
| Samsung [7] | - Regarding FFS in Prerequisite feature groups FG2-33 and FG2-35 can be reused for all FGs related to SD and PD adaptation as baseline. Hence we suggest to include FG2-33 and FG2-35 as prerequisite feature groups of all FGs related to SD and PD adaptation. Proposal 2: For all FGs related to SD and PD adaptation, add FG2-33 and FG2-35 as prerequisite feature groups. - Regarding Component 4, 5, 6, and 7 In terms of components related to maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs, it is still unclear how to determine maximum value and how to count number of simultaneous NZP-CSI-RS resource and CSI-RS ports per CC or in active BWPs across all CCs not only when both Rel-18 CSI report and legacy CSI report are configured but also when different types of adaptations or CSI reporting are configured on each CCs and/or each active BWPs in a CC. First, to count NZP-CSI-RS resource and CSI-RS ports, it would be reasonable to count all NZP-CSI-RS resource and CSI-RS ports for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b. Proposal 4: Add a note in all FGs that ‘Note: for the purpose of component-4, 5, 6 and 7 calculation: NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b’. Regarding to determine maximum value of components, we propose to use one value for all these FGs. It would be straightforward way to inherit the philosophy applied for FG2-33 which is a legacy counterpart of FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. Proposal 5: Add a note in all FGs that ‘Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b’. - Aspects to be newly considered Additionally, there are still ambiguity on how to count the number of CSI report(s) processed by UE simultaneously per CC or across all CCs. It is related to how many CSI reports can be processed simultaneously, and the number of CSI report(s) processed by UE simultaneously are defined in FG2-35. During Rel-18 NES discussion, to calculate the occupied number of CPUs, each CSI report corresponding to sub-configuration is counted as legacy CSI report. In this regard, the CSI report(s) corresponding to sub-configuration should be counted for legacy UE capability in FG2-35. And, the total number of CSI report(s) processed by UE simultaneously per CC or across all CCs should not be larger than the value of FG2-35. Therefore, we propose to add new components and note in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b as follow: - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. Proposal 6: Define new components and note for the maximum number of CPUs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. |
| Apple [8] | 42. Netw_Energy_NR; 42-1a; Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for semi-persistent CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP; 2-33, 2-35; Yes; UE does not support spatial domain adaptation for semi-persistent CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2, 3, 4} Component 3 candidate values: {1, 2, 3, 4} Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128} Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Component 9 candidate values: {1, 2, 3, 4} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 6 and 7 are signaled per BC; Optional with capability signaling. |
| NTT DOCOMO, INC. [9] | For FG42-1a, there are three remaining FFS points, one is about prerequisite FG(s) and other two points are about candidate values for component 2/3. Our views on the prerequisite FG(s) for FG42-1a were already provided above, and accordingly we propose to have FG42-1 or FG42-1b as prerequisite FG for FG42-1a. Regarding candidate values for component 2 (The max number of sub-configurations Lmax in one CSI report configuration for semi-persistent CSI reporting), candidate values for periodic CSI reporting (component 2 of FG42-1) and for aperiodic CSI reporting (component 2 of FG42-1b) can be referred. {2, 3, 4} are candidate values for periodic CSI reporting and {2,3,4,5,6,7,8} are candidate values for aperiodic CSI reporting. For periodic CSI reporting, since the value of Lmax equals to max number of CSIs N in one periodic CSI report, and hence candidate values are limited to lower values compared with aperiodic CSI reporting case. As candidate values for max number of CSIs N in one SP-CSI report can be reported in component 3, candidate values for component 2 of FG42-1a can be same as those for FG42-1b. Regarding candidate values for component 3 (Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration), it was agreed at RAN1#114bis that the maximum value of N is no larger than 4 for SP-CSI reporting on PUCCH. So, candidate values {1,2,3,4} for SP/A-CSI report can be considered. Proposal 8: - FG42-1a has one of {42-1, 42-1b} as prerequisite FG. - Candidate values for component 2 of FG42-1a are {2,3,4,5,6,7,8}. - Candidate values for component 3 of FG42-1a are {1,2,3,4}. |
| Ericsson [10] | Some notes related to remaining issues for existing FGs (and applicable to new FGs also). - For component related to “Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration”, there is no need for defining candidate values related to N. For aperiodic/semi-persistent CSI reporting, UE should support all values of N up to Lmax, i.e. value of N up to Lmax is supported by the UE. - The components 4,5,6,7 can be updated to reflect the NES condition for applying the per-CC and across-CCs limits. - Pre-requisites: For 42-1/1a/1b/2/2a/2b: adding 2-33 and 2-35 is sufficient. Since these two are mandatory with capability signaling, it is strictly not necessary to explicitly reflect them in the FG table. For NES, FGs 42-1a/42-1b/42-2a/42-2b/(New 42-3a)/(New 42-3b), for component 3 (Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration), - Update the Note to reflect “Value of N: 1 <=N<=Lmax” - There is no need for defining candidate values for component 3 For NES, FGs 42-1/42-1a/42-1b/42-1/42-2a/42-2b/(New 42-3a)/(New 42-3b), update the components 4,5,6,7 as follows: - 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of simultaneous NZP-CSI-RS resources per CC - 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs |
| Sharp [11] |  |
| ETRI [12] |  |
| Vodafone [13] |  |
| LG Electronics [14] | - Candidate values for Lmax and N In RAN1#115 meeting, the following agreement was made for L_max values for P/AP-CSI reporting. Agreement: The following candidate values for Lmax in FGs 42-1/42-1a/42-1b/42-2/42-2a/42-2b are agreed FG; Lmax. SD P; {2,3,4}. SD AP; {2,3,4,5,6,7,8}. PD P; {2,3,4}. PD AP; {2,3,4,5,6,7,8}.. The remaining issue is to determine Lmax for SP-CSI reporting. It should be noted that 8-bit bitmap is allocated for each of sub-configurations in “Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE” that is introduced for Rel-18 NES feature. Thus, the candidate value range for Lmax could be 2 to 8 for semi-persistent CSI reporting. Proposal #1: For Lmax in FGs 42-1a (spatial domain & semi-persistent CSI reporting) and 42-2a (power domain & semi-persistent CSI reporting), the candidate values are {2,3,4,5,6,7,8}. In RAN1#114bis meeting, the following two NOTEs were agreed for semi-persistent CSI reporting. - Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH - Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH In our view, UE doesn’t need to report the candidate value for N in addition to Lmax. If a UE indicates support of up to Lmax sub-configurations at least for AP-CSI reporting, the UE should be able to report any N (<=Lmax) sub-configuration(s). However, especially for SP-CSI reporting, if the candidate values of Lmax are 2 to 8 and the above NOTE for N is considered, one method is to determine N as minimum of 4 and Lmax. Proposal #2: Do not introduce UE capability signaling for N in FGs 42-1a, 42-1b, 42-2a, and 42-2b. - For 42-1b and 42-2b, UE supports N up to Lmax. - For 42-1a and 42-2a, UE supports N up to min(4,Lmax). |
| Qualcomm Incorporated [15] | 42-1a; Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for semi-persistent CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook; FFS 2-32a or 2-32b; Yes; UE does not support spatial domain adaptation for semi-persistent CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: FFS {2,3,4,5,6,7,8} Component 3 candidate values: FFS {2, 3, 4} Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128} Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 6 and 7 are signaled per BC Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs; Optional with capability signaling. |
| MediaTek Inc. [16] |  |

| 42. Netw_Energy_NR | 42-1b | Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP | FFS | Yes |  | UE does not support spatial domain adaptation for aperiodic CSI reporting | Per band | No | No | N/A | Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values FFS Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11,12} FFS: different values for joint operation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 42. Netw_Energy_NR; 42-1b; Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP; 42-1; Yes; UE does not support spatial domain adaptation for aperiodic CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11,12}; Optional with capability signaling. |
| Huawei/HiSilicon [3] | - For semi-persistent CSI reporting, the candidate values for configured number of sub-configurations and reported CSIs are to be determined. As it was agreed that “Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH” and “Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH”, we propose as follows: - For 42-1a and 42-2a, - The candidate values for L: 2, 3, 4, 5, 6, 7 and 8. - The candidate values for N: 1, 2, 3 and 4. - For aperiodic CSI reporting, the candidate values for reported CSIs are to be determined. The same values as semi-persistent CSI reporting can be used. - For 42-1b and 42-2b, - The candidate values for N: 1, 2, 3 and 4. - Regarding the prerequisite feature groups, since we are specifically design the NES feature with willingness of supporting deferent values from a legacy CSI report capability, there does not expect strong dependences among these FGs, and components 4)~7) are reported for each of these FGs respectively, so no prerequisite feature groups are needed. - Regarding whether to set different values for joint operation, from UE implementation perspective, the newly introduced FG is mainly related to UE capabilities, similar to the one defined in FG 2-33, 2-35 in Rel-15, about CSI measurement, CSI processing and CSI reporting. The difference between SD/PD only operation and joint operation mainly lays on the implementation at gNB side and configurations (to UE), while the difference for UE would be small, e.g. the supported maximum number of CSI reports per BWP, CPU counting or active resource/antenna ports counting are kept the same among different configurations for adaptation. Therefore, there is no justification to set different candidate values for joint operation. Proposal 2: For FG 42-1b/42-2b, - The candidate values for N: 1, 2, 3 and 4. Proposal 3: For FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, - No prerequisite feature groups are needed. |
| Vivo [4] |  |
| ZTE [5] | For semi-persistent CSI reports and aperiodic CSI reports, the same candidate values of N can be used. And the maximum candidate value of N can refer to the maximum value of Lmax of periodic CSI report, which is 4. The minimum candidate value of N should be 1 to support single CSI sub-report. Proposal 1: The candidate values of N for SP-CSI report and AP-CSI report are {1,2,3,4}. The following FG 23-7-1 is the ‘Basic Features of CSI Enhancement for Multi-TRP’, which is introduced in R17. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 2, Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 3. CSI report mode selection of mode 1 with X=0 and/or mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses 5. Supported codebook modes for NCJT CSI. The FG 23-7-1 is also about CSI enhancement, which does not set a ‘Prerequisite feature groups’. Given this, for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, the ‘Prerequisite feature groups’ doesn’t need, either. Proposal 2: ‘Prerequisite feature groups’ is not needed for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b. |
| CATT [6] | The support of the spatial and power domain adaptation requires additional UE CSI measurements and reports when some TxRUs are turned off. A number of TxRUs turning off and generating a new number of antenna ports or beam pattern is indicated as an antenna sub-configuration of the full antenna configuration. The additional CSI reports of sub-configurations with each CSI report associated with one sub-configuration will be fed back to the gNB for dynamic link adaptation. Multiple CSI reports associated with full antenna configuration and sub-configurations are included in all CSI report type; they are aperiodic CSI report, semi-persistent CSI report, and periodic CSI report. The UE capability of CSI measurements for A-CSI, SP-CSI and P-CSI feedbacks should be the same without any differentiation in the UE capability since the measurements of configured CSI-RS resource(s) are the same. In RAN1#115, the maximum number of SP-CSI measurements were agreed to be the same as that of P-CSI. Since the additional CSI measurements of antenna sub-configuration would be provisioned by the UE at each band. The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands and based on each UE (per UE) regardless if it is Type 1 or Type 2 antenna adaptation. For Type 1 antenna adaptation, the additional number of CSI measurements for CSI feedbacks in a CSI report is based on the measurements of a CSI-RS resource set. The maximum number of CSI measurements N from a CSI-RS resource set should be based on UE capability in additional CSI measurements and computation. The maximum number of CSI measurements N in a CSI report would depend on the number of network operation of spatial adaptation for network energy saving. The spatial domain adaptation relies on the ON/OFF of the TxRU to achieve the network energy saving. The transition time of the TxRU ON/OFF would be the deciding factor. The power amplifier of the TxRU would demands range of ms to stabilize the Tx power setting and would not be able to dynamically shutting down and turning on. Thus, the TxRU ON/OFF would not be substantially frequent at the gNB for spatial adaptation. The number of the CSI measurements of sub-configurations should not be large. Thus, the maximum number of CSI measurements of antenna sub-configurations in a CSI report should be set to 4. The maximum number of sub-configurations L for UE to support should be based on the possibility of spatial and power adaptations by the network. The level of the power adaptations would not see much network energy saving gain. Thus, the majority of the spatial and power adaptation is different spatial domain configurations. The maximum number of spatial and power domain sub-configurations should be set to 8. For joint operation of spatial and power domain adaptation, the maximum number of sub-configuration and the CSI reports should remain the same since the joint operation should not increase the additional complexity. Proposal 1: The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands for each UE (per UE). Proposal 2: The maximum number of antenna sub-configurations LMax for UE to support should be set to 8 with candidate values {1, 2, 4, 8} for spatial and/or power domain adaptation. Proposal 3: The maximum number of sub-configuration and the CSI reports for joint operation of spatial and power domain adaptation for NES should remain the same. |
| Samsung [7] | - Regarding FFS: different values for joint operation in Note The remaining issue is how to determine values for joint operation. For joint operation of SD and PD adaptation, RAN1 had made agreement to support joint operation for NES in RAN1#113. And, UE can have different capability values for each SD and PD adaptation. In this regard, even if UE supports both capabilities, there will be ambiguity on UE capability for joint operation, because UE can have different capabilities between SD and PD adaptation. Therefore, we propose to define new independent UE feature for joint operation of SD and PD adaptation for P/SP/AP CSI reporting. Proposal 1: Define new UE feature of joint operation of SD and PD adaptation. - Regarding FFS in Prerequisite feature groups FG2-33 and FG2-35 can be reused for all FGs related to SD and PD adaptation as baseline. Hence we suggest to include FG2-33 and FG2-35 as prerequisite feature groups of all FGs related to SD and PD adaptation. Proposal 2: For all FGs related to SD and PD adaptation, add FG2-33 and FG2-35 as prerequisite feature groups. - Regarding Component 9 Regarding the component 9 in FGs, it is clear that the value is for each report type, and this is associated with UE capability defined in FG2-35. The value of component 9 will be used instead of the values for component 1 and 2 in FG2-35, when CSI reporting with sub-configuration for SD and PD adaptation is configured. In case of UE is configured with Rel-18 NES CSI report and legacy CSI report, to be clear the intention of component 9, we propose to add Note in FG42-1, FG42-1b, FG42-2 and FG42-2b. Proposal 3: Add a note in FG42-1, FG42-1b, FG42-2 and FG42-2b that ‘Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration’. - Regarding Component 4, 5, 6, and 7 In terms of components related to maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs, it is still unclear how to determine maximum value and how to count number of simultaneous NZP-CSI-RS resource and CSI-RS ports per CC or in active BWPs across all CCs not only when both Rel-18 CSI report and legacy CSI report are configured but also when different types of adaptations or CSI reporting are configured on each CCs and/or each active BWPs in a CC. First, to count NZP-CSI-RS resource and CSI-RS ports, it would be reasonable to count all NZP-CSI-RS resource and CSI-RS ports for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b. Proposal 4: Add a note in all FGs that ‘Note: for the purpose of component-4, 5, 6 and 7 calculation: NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b’. Regarding to determine maximum value of components, we propose to use one value for all these FGs. It would be straightforward way to inherit the philosophy applied for FG2-33 which is a legacy counterpart of FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. Proposal 5: Add a note in all FGs that ‘Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b’. - Aspects to be newly considered Additionally, there are still ambiguity on how to count the number of CSI report(s) processed by UE simultaneously per CC or across all CCs. It is related to how many CSI reports can be processed simultaneously, and the number of CSI report(s) processed by UE simultaneously are defined in FG2-35. During Rel-18 NES discussion, to calculate the occupied number of CPUs, each CSI report corresponding to sub-configuration is counted as legacy CSI report. In this regard, the CSI report(s) corresponding to sub-configuration should be counted for legacy UE capability in FG2-35. And, the total number of CSI report(s) processed by UE simultaneously per CC or across all CCs should not be larger than the value of FG2-35. Therefore, we propose to add new components and note in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b as follow: - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. Proposal 6: Define new components and note for the maximum number of CPUs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. |
| Apple [8] | 42. Netw_Energy_NR; 42-1b; Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI sub-report(s) included in one CSI report where each CSI sub-report corresponds to one sub-configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP; 2-33, 2-35; Yes; UE does not support spatial domain adaptation for aperiodic CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values: {2, 3, 4, 5, 6, 7, 8} Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11,12}; Optional with capability signaling. |
| NTT DOCOMO, INC. [9] | For FG42-1b, there are three remaining FFS points, first one is about prerequisite FG(s), second one is about potential different values for joint SD/PD operation and third one is about candidate values for component 3. As our views on those three points are already provided for FG42-1/1a, following proposals for FG42-1b can be made. Proposal 9: - FG42-1b does not require any prerequisite FG. - Candidate values for component 3 of FG42-1b are {1,2,3,4}. |
| Ericsson [10] | Some notes related to remaining issues for existing FGs (and applicable to new FGs also). - For component related to “Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration”, there is no need for defining candidate values related to N. For aperiodic/semi-persistent CSI reporting, UE should support all values of N up to Lmax, i.e. value of N up to Lmax is supported by the UE. - The components 4,5,6,7 can be updated to reflect the NES condition for applying the per-CC and across-CCs limits. - Pre-requisites: For 42-1/1a/1b/2/2a/2b: adding 2-33 and 2-35 is sufficient. Since these two are mandatory with capability signaling, it is strictly not necessary to explicitly reflect them in the FG table. For NES, FGs 42-1a/42-1b/42-2a/42-2b/(New 42-3a)/(New 42-3b), for component 3 (Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration), - Update the Note to reflect “Value of N: 1 <=N<=Lmax” - There is no need for defining candidate values for component 3 For NES, FGs 42-1/42-1a/42-1b/42-1/42-2a/42-2b/(New 42-3a)/(New 42-3b), update the components 4,5,6,7 as follows: - 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of simultaneous NZP-CSI-RS resources per CC - 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs |
| Sharp [11] |  |
| ETRI [12] |  |
| Vodafone [13] |  |
| LG Electronics [14] | In RAN1#114bis meeting, the following two NOTEs were agreed for semi-persistent CSI reporting. - Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH - Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH In our view, UE doesn’t need to report the candidate value for N in addition to Lmax. If a UE indicates support of up to Lmax sub-configurations at least for AP-CSI reporting, the UE should be able to report any N (<=Lmax) sub-configuration(s). However, especially for SP-CSI reporting, if the candidate values of Lmax are 2 to 8 and the above NOTE for N is considered, one method is to determine N as minimum of 4 and Lmax. Proposal #2: Do not introduce UE capability signaling for N in FGs 42-1a, 42-1b, 42-2a, and 42-2b. - For 42-1b and 42-2b, UE supports N up to Lmax. - For 42-1a and 42-2a, UE supports N up to min(4,Lmax). |
| Qualcomm Incorporated [15] | 42-1b; Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP; FFS 2-35; Yes; UE does not support spatial domain adaptation for aperiodic CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values FFS {2, 3, 4} Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11,12} FFS: different values for joint operation Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain.; Optional with capability signaling. |
| MediaTek Inc. [16] |  |

| 42. Netw_Energy_NR | 42-2 | Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset foraperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP | FFS | Yes |  | UE does not support power domain adaptation for periodicCSI reporting | Per band | No | No | N/A | Component 2 candidate value: {2,3,4} Component 4 candidate value: {1, 2, 3 … 32} Component 5 candidate value: {8, 16, 24, … 128 } Component 6 candidate value: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4} FFS: different values for joint operation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 42. Netw_Energy_NR; 42-2; Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset foraperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP; Yes; UE does not support power domain adaptation for periodicCSI reporting; Per band; No; No; N/A; Component 2 candidate value: {2,3,4} Component 4 candidate value: {1, 2, 3 … 32} Component 5 candidate value: {8, 16, 24, … 128 } Component 6 candidate value: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4}; Optional with capability signaling. |
| Huawei/HiSilicon [3] | - For semi-persistent CSI reporting, the candidate values for configured number of sub-configurations and reported CSIs are to be determined. As it was agreed that “Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH” and “Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH”, we propose as follows: - For 42-1a and 42-2a, - The candidate values for L: 2, 3, 4, 5, 6, 7 and 8. - The candidate values for N: 1, 2, 3 and 4. - For aperiodic CSI reporting, the candidate values for reported CSIs are to be determined. The same values as semi-persistent CSI reporting can be used. - For 42-1b and 42-2b, - The candidate values for N: 1, 2, 3 and 4. - Regarding the prerequisite feature groups, since we are specifically design the NES feature with willingness of supporting deferent values from a legacy CSI report capability, there does not expect strong dependences among these FGs, and components 4)~7) are reported for each of these FGs respectively, so no prerequisite feature groups are needed. - Regarding whether to set different values for joint operation, from UE implementation perspective, the newly introduced FG is mainly related to UE capabilities, similar to the one defined in FG 2-33, 2-35 in Rel-15, about CSI measurement, CSI processing and CSI reporting. The difference between SD/PD only operation and joint operation mainly lays on the implementation at gNB side and configurations (to UE), while the difference for UE would be small, e.g. the supported maximum number of CSI reports per BWP, CPU counting or active resource/antenna ports counting are kept the same among different configurations for adaptation. Therefore, there is no justification to set different candidate values for joint operation. Proposal 3: For FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, - No prerequisite feature groups are needed. Proposal 4: For joint operation of SD and PD, - Remove the FFS in 42-1/42-1b/42-2/42-2b - Add a note in 42-1/42-1b/42-2/42-2b that same candidate values are used for joint operation of SD and PD, if a UE supports both SD and PD. |
| Vivo [4] |  |
| ZTE [5] | The following FG 23-7-1 is the ‘Basic Features of CSI Enhancement for Multi-TRP’, which is introduced in R17. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 2, Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 3. CSI report mode selection of mode 1 with X=0 and/or mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses 5. Supported codebook modes for NCJT CSI. The FG 23-7-1 is also about CSI enhancement, which does not set a ‘Prerequisite feature groups’. Given this, for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, the ‘Prerequisite feature groups’ doesn’t need, either. Proposal 2: ‘Prerequisite feature groups’ is not needed for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b. |
| CATT [6] | The support of the spatial and power domain adaptation requires additional UE CSI measurements and reports when some TxRUs are turned off. A number of TxRUs turning off and generating a new number of antenna ports or beam pattern is indicated as an antenna sub-configuration of the full antenna configuration. The additional CSI reports of sub-configurations with each CSI report associated with one sub-configuration will be fed back to the gNB for dynamic link adaptation. Multiple CSI reports associated with full antenna configuration and sub-configurations are included in all CSI report type; they are aperiodic CSI report, semi-persistent CSI report, and periodic CSI report. The UE capability of CSI measurements for A-CSI, SP-CSI and P-CSI feedbacks should be the same without any differentiation in the UE capability since the measurements of configured CSI-RS resource(s) are the same. In RAN1#115, the maximum number of SP-CSI measurements were agreed to be the same as that of P-CSI. Since the additional CSI measurements of antenna sub-configuration would be provisioned by the UE at each band. The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands and based on each UE (per UE) regardless if it is Type 1 or Type 2 antenna adaptation. For Type 1 antenna adaptation, the additional number of CSI measurements for CSI feedbacks in a CSI report is based on the measurements of a CSI-RS resource set. The maximum number of CSI measurements N from a CSI-RS resource set should be based on UE capability in additional CSI measurements and computation. The maximum number of CSI measurements N in a CSI report would depend on the number of network operation of spatial adaptation for network energy saving. The spatial domain adaptation relies on the ON/OFF of the TxRU to achieve the network energy saving. The transition time of the TxRU ON/OFF would be the deciding factor. The power amplifier of the TxRU would demands range of ms to stabilize the Tx power setting and would not be able to dynamically shutting down and turning on. Thus, the TxRU ON/OFF would not be substantially frequent at the gNB for spatial adaptation. The number of the CSI measurements of sub-configurations should not be large. Thus, the maximum number of CSI measurements of antenna sub-configurations in a CSI report should be set to 4. The maximum number of sub-configurations L for UE to support should be based on the possibility of spatial and power adaptations by the network. The level of the power adaptations would not see much network energy saving gain. Thus, the majority of the spatial and power adaptation is different spatial domain configurations. The maximum number of spatial and power domain sub-configurations should be set to 8. For joint operation of spatial and power domain adaptation, the maximum number of sub-configuration and the CSI reports should remain the same since the joint operation should not increase the additional complexity. Proposal 1: The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands for each UE (per UE). Proposal 2: The maximum number of antenna sub-configurations LMax for UE to support should be set to 8 with candidate values {1, 2, 4, 8} for spatial and/or power domain adaptation. Proposal 3: The maximum number of sub-configuration and the CSI reports for joint operation of spatial and power domain adaptation for NES should remain the same. |
| Samsung [7] | - Regarding FFS: different values for joint operation in Note The remaining issue is how to determine values for joint operation. For joint operation of SD and PD adaptation, RAN1 had made agreement to support joint operation for NES in RAN1#113. And, UE can have different capability values for each SD and PD adaptation. In this regard, even if UE supports both capabilities, there will be ambiguity on UE capability for joint operation, because UE can have different capabilities between SD and PD adaptation. Therefore, we propose to define new independent UE feature for joint operation of SD and PD adaptation for P/SP/AP CSI reporting. Proposal 1: Define new UE feature of joint operation of SD and PD adaptation. - Regarding FFS in Prerequisite feature groups FG2-33 and FG2-35 can be reused for all FGs related to SD and PD adaptation as baseline. Hence we suggest to include FG2-33 and FG2-35 as prerequisite feature groups of all FGs related to SD and PD adaptation. Proposal 2: For all FGs related to SD and PD adaptation, add FG2-33 and FG2-35 as prerequisite feature groups. - Regarding Component 9 Regarding the component 9 in FGs, it is clear that the value is for each report type, and this is associated with UE capability defined in FG2-35. The value of component 9 will be used instead of the values for component 1 and 2 in FG2-35, when CSI reporting with sub-configuration for SD and PD adaptation is configured. In case of UE is configured with Rel-18 NES CSI report and legacy CSI report, to be clear the intention of component 9, we propose to add Note in FG42-1, FG42-1b, FG42-2 and FG42-2b. Proposal 3: Add a note in FG42-1, FG42-1b, FG42-2 and FG42-2b that ‘Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration’. - Regarding Component 4, 5, 6, and 7 In terms of components related to maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs, it is still unclear how to determine maximum value and how to count number of simultaneous NZP-CSI-RS resource and CSI-RS ports per CC or in active BWPs across all CCs not only when both Rel-18 CSI report and legacy CSI report are configured but also when different types of adaptations or CSI reporting are configured on each CCs and/or each active BWPs in a CC. First, to count NZP-CSI-RS resource and CSI-RS ports, it would be reasonable to count all NZP-CSI-RS resource and CSI-RS ports for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b. Proposal 4: Add a note in all FGs that ‘Note: for the purpose of component-4, 5, 6 and 7 calculation: NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b’. Regarding to determine maximum value of components, we propose to use one value for all these FGs. It would be straightforward way to inherit the philosophy applied for FG2-33 which is a legacy counterpart of FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. Proposal 5: Add a note in all FGs that ‘Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b’. - Aspects to be newly considered Additionally, there are still ambiguity on how to count the number of CSI report(s) processed by UE simultaneously per CC or across all CCs. It is related to how many CSI reports can be processed simultaneously, and the number of CSI report(s) processed by UE simultaneously are defined in FG2-35. During Rel-18 NES discussion, to calculate the occupied number of CPUs, each CSI report corresponding to sub-configuration is counted as legacy CSI report. In this regard, the CSI report(s) corresponding to sub-configuration should be counted for legacy UE capability in FG2-35. And, the total number of CSI report(s) processed by UE simultaneously per CC or across all CCs should not be larger than the value of FG2-35. Therefore, we propose to add new components and note in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b as follow: - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. Proposal 6: Define new components and note for the maximum number of CPUs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. |
| Apple [8] | 42. Netw_Energy_NR; 42-2; Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset foraperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP; 2-33, 2-35; Yes; UE does not support power domain adaptation for periodicCSI reporting; Per band; No; No; N/A; Component 2 candidate value: {2,3,4} Component 4 candidate value: {1, 2, 3 … 32} Component 5 candidate value: {8, 16, 24, … 128 } Component 6 candidate value: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4}; Optional with capability signaling. |
| NTT DOCOMO, INC. [9] | For FG42-2, there are two remaining FFS points same as those for FG42-1. Our views on 42-1 are applicable to 42-2, and hence following proposal for FG42-2 can be made. Proposal 10: FG42-2 does not require any prerequisite FG. |
| Ericsson [10] | For joint SD and PD operation, there are at least two options: - (Option 1) If UE indicates support for SD operation (e.g., 42-1b) and the UE also indicates support for PD operation (e.g., 42-2b), then the UE also supports joint SD+PD operation. - (Option 2) Introduce a separate capability for indicating joint SD + PD operation. We suggest adopting Option 2, with corresponding FGs for spatial domain and power domain as pre-requisites. New FGs with full components/component values for joint SD+PD operation are shown in Annex A though we note it is possible to rely on indicated pre-requisites to derive certain component values. Also, other aspects (e.g., value of N, etc) that may need common handling with existing FGs are left out of initial FG definition. Those are addressed together for both existing and new FGs in subsequent proposals. For NES, introduce new FGs for joint spatial and power domain adaptation with CSI feedback based on CSI report sub-configuration(s). - (New) FG 42-3 for periodic CSI reporting with pre-requisites:42-1 and 42-2. - (New) FG 42-3a for semi-persistent CSI reporting with pre-requisites:42-1a and 42-2a - (New) FG 42-3b for aperiodic CSI reporting with pre-requisites:42-1b and 42-2b 42. Netw_Energy_NR; (New) 42-3; Joint spatial and Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs and one power offset for periodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP; 42-1 and 42-2; Yes; UE does not support joint spatial and power domain adaptation for periodicCSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1 and PD, SD-type2 and PD, both (SD-type1 and PD) and (SD-type2 and PD)} Component 2 candidate value: {2,3,4} Component 4 candidate value: {1, 2, 3 … 32} Component 5 candidate value: {8, 16, 24, … 128 } Component 6 candidate value: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4}; Optional with capability signaling. 42. Netw_Energy_NR; (New) 42-3a; Joint spatial and Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs and one power offset f for semi-persistent CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook; 42-1b and 42-2b; Yes; UE does not support joint spatial and power domain adaptation for semi-persistent CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1 and PD, SD-type2 and PD, both (SD-type1 and PD) and (SD-type2 and PD)} Component 2 candidate values: FFS Component 3 candidate values: {1, 2, 3 … 32} Component 4 candidate values: {8, 16, 24, … 128 } Component 5 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 6 candidate values: {8, 16, 24, …, 248, 256} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 5 and 6 are signaled per BC; Optional with capability signaling. 42. Netw_Energy_NR; (New) 42-3b; Joint spatial and Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs and one power offset for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP; 42-1b and 42-2b; Yes; UE does not support joint spatial and power domain adaptation for aperiodic CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {SD-type1 and PD, SD-type2 and PD, both (SD-type1 and PD) and (SD-type2 and PD)} Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values: FFS Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128 } Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} Note: Components 6 and 7 are signaled per BC; Optional with capability signaling. Some notes related to remaining issues for existing FGs (and applicable to new FGs also). - For component related to “Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration”, there is no need for defining candidate values related to N. For aperiodic/semi-persistent CSI reporting, UE should support all values of N up to Lmax, i.e. value of N up to Lmax is supported by the UE. - The components 4,5,6,7 can be updated to reflect the NES condition for applying the per-CC and across-CCs limits. - Pre-requisites: For 42-1/1a/1b/2/2a/2b: adding 2-33 and 2-35 is sufficient. Since these two are mandatory with capability signaling, it is strictly not necessary to explicitly reflect them in the FG table. For NES, FGs 42-1a/42-1b/42-2a/42-2b/(New 42-3a)/(New 42-3b), for component 3 (Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration), - Update the Note to reflect “Value of N: 1 <=N<=Lmax” - There is no need for defining candidate values for component 3 For NES, FGs 42-1/42-1a/42-1b/42-1/42-2a/42-2b/(New 42-3a)/(New 42-3b), update the components 4,5,6,7 as follows: - 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of simultaneous NZP-CSI-RS resources per CC - 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs |
| Sharp [11] |  |
| ETRI [12] |  |
| Vodafone [13] |  |
| LG Electronics [14] |  |
| Qualcomm Incorporated [15] | 42-2; Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset foraperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP; FFS 2-35; Yes; UE does not support power domain adaptation for periodic CSI reporting; Per band; No; No; N/A; Component 2 candidate value: {2,3,4} Component 4 candidate value: {1, 2, 3 … 32} Component 5 candidate value: {8, 16, 24, … 128 } Component 6 candidate value: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4} FFS: different values for joint operation Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain.; Optional with capability signaling. |
| MediaTek Inc. [16] |  |

| 42. Netw_Energy_NR | 42-2a | Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting | Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for semi-persistent CSI reporting 1. The max number of sub-configurations Lmax in one CSI report configuration 2. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. 3. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 4. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 6. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Support of single-panel type 1 codebook | FFS | Yes |  | UE does not support power domain adaptation for semi-persistent CSI reporting | Per band | No | No | N/A | Component 1 candidate values: FFS Component 2 candidate values: FFS Component 3 candidate values: {1, 2, 3 … 32} Component 4 candidate values: {8, 16, 24, … 128 } Component 5 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 6 candidate values: {8, 16, 24, …, 248, 256} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 5 and 6 are signaled per BC | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 42. Netw_Energy_NR; 42-2a; Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting; Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for semi-persistent CSI reporting 1. The max number of sub-configurations Lmax in one CSI report configuration 2. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. 3. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 4. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 6. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Support of single-panel type 1 codebook; 42-2; Yes; UE does not support power domain adaptation for semi-persistent CSI reporting; Per band; No; No; N/A; Component 1 candidate values: FFS Component 2 candidate values: FFS Component 3 candidate values: {1, 2, 3 … 32} Component 4 candidate values: {8, 16, 24, … 128 } Component 5 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 6 candidate values: {8, 16, 24, …, 248, 256} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 5 and 6 are signaled per BC; Optional with capability signaling. |
| Huawei/HiSilicon [3] | - For semi-persistent CSI reporting, the candidate values for configured number of sub-configurations and reported CSIs are to be determined. As it was agreed that “Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH” and “Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH”, we propose as follows: - For 42-1a and 42-2a, - The candidate values for L: 2, 3, 4, 5, 6, 7 and 8. - The candidate values for N: 1, 2, 3 and 4. - For aperiodic CSI reporting, the candidate values for reported CSIs are to be determined. The same values as semi-persistent CSI reporting can be used. - For 42-1b and 42-2b, - The candidate values for N: 1, 2, 3 and 4. - Regarding the prerequisite feature groups, since we are specifically design the NES feature with willingness of supporting deferent values from a legacy CSI report capability, there does not expect strong dependences among these FGs, and components 4)~7) are reported for each of these FGs respectively, so no prerequisite feature groups are needed. - Regarding whether to set different values for joint operation, from UE implementation perspective, the newly introduced FG is mainly related to UE capabilities, similar to the one defined in FG 2-33, 2-35 in Rel-15, about CSI measurement, CSI processing and CSI reporting. The difference between SD/PD only operation and joint operation mainly lays on the implementation at gNB side and configurations (to UE), while the difference for UE would be small, e.g. the supported maximum number of CSI reports per BWP, CPU counting or active resource/antenna ports counting are kept the same among different configurations for adaptation. Therefore, there is no justification to set different candidate values for joint operation. Proposal 1: For FG 42-1a/42-2a, - The candidate values for L: 2, 3, 4, 5, 6, 7 and 8. - The candidate values for N: 1, 2, 3 and 4. Proposal 3: For FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, - No prerequisite feature groups are needed. |
| Vivo [4] |  |
| ZTE [5] | For semi-persistent CSI reports and aperiodic CSI reports, the same candidate values of N can be used. And the maximum candidate value of N can refer to the maximum value of Lmax of periodic CSI report, which is 4. The minimum candidate value of N should be 1 to support single CSI sub-report. Proposal 1: The candidate values of N for SP-CSI report and AP-CSI report are {1,2,3,4}. The following FG 23-7-1 is the ‘Basic Features of CSI Enhancement for Multi-TRP’, which is introduced in R17. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 2, Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 3. CSI report mode selection of mode 1 with X=0 and/or mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses 5. Supported codebook modes for NCJT CSI. The FG 23-7-1 is also about CSI enhancement, which does not set a ‘Prerequisite feature groups’. Given this, for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, the ‘Prerequisite feature groups’ doesn’t need, either. Proposal 2: ‘Prerequisite feature groups’ is not needed for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b. |
| CATT [6] | The support of the spatial and power domain adaptation requires additional UE CSI measurements and reports when some TxRUs are turned off. A number of TxRUs turning off and generating a new number of antenna ports or beam pattern is indicated as an antenna sub-configuration of the full antenna configuration. The additional CSI reports of sub-configurations with each CSI report associated with one sub-configuration will be fed back to the gNB for dynamic link adaptation. Multiple CSI reports associated with full antenna configuration and sub-configurations are included in all CSI report type; they are aperiodic CSI report, semi-persistent CSI report, and periodic CSI report. The UE capability of CSI measurements for A-CSI, SP-CSI and P-CSI feedbacks should be the same without any differentiation in the UE capability since the measurements of configured CSI-RS resource(s) are the same. In RAN1#115, the maximum number of SP-CSI measurements were agreed to be the same as that of P-CSI. Since the additional CSI measurements of antenna sub-configuration would be provisioned by the UE at each band. The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands and based on each UE (per UE) regardless if it is Type 1 or Type 2 antenna adaptation. For Type 1 antenna adaptation, the additional number of CSI measurements for CSI feedbacks in a CSI report is based on the measurements of a CSI-RS resource set. The maximum number of CSI measurements N from a CSI-RS resource set should be based on UE capability in additional CSI measurements and computation. The maximum number of CSI measurements N in a CSI report would depend on the number of network operation of spatial adaptation for network energy saving. The spatial domain adaptation relies on the ON/OFF of the TxRU to achieve the network energy saving. The transition time of the TxRU ON/OFF would be the deciding factor. The power amplifier of the TxRU would demands range of ms to stabilize the Tx power setting and would not be able to dynamically shutting down and turning on. Thus, the TxRU ON/OFF would not be substantially frequent at the gNB for spatial adaptation. The number of the CSI measurements of sub-configurations should not be large. Thus, the maximum number of CSI measurements of antenna sub-configurations in a CSI report should be set to 4. The maximum number of sub-configurations L for UE to support should be based on the possibility of spatial and power adaptations by the network. The level of the power adaptations would not see much network energy saving gain. Thus, the majority of the spatial and power adaptation is different spatial domain configurations. The maximum number of spatial and power domain sub-configurations should be set to 8. For joint operation of spatial and power domain adaptation, the maximum number of sub-configuration and the CSI reports should remain the same since the joint operation should not increase the additional complexity. Proposal 1: The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands for each UE (per UE). Proposal 2: The maximum number of antenna sub-configurations LMax for UE to support should be set to 8 with candidate values {1, 2, 4, 8} for spatial and/or power domain adaptation. Proposal 3: The maximum number of sub-configuration and the CSI reports for joint operation of spatial and power domain adaptation for NES should remain the same. |
| Samsung [7] | - Regarding FFS in Prerequisite feature groups FG2-33 and FG2-35 can be reused for all FGs related to SD and PD adaptation as baseline. Hence we suggest to include FG2-33 and FG2-35 as prerequisite feature groups of all FGs related to SD and PD adaptation. Proposal 2: For all FGs related to SD and PD adaptation, add FG2-33 and FG2-35 as prerequisite feature groups. - Regarding Component 4, 5, 6, and 7 In terms of components related to maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs, it is still unclear how to determine maximum value and how to count number of simultaneous NZP-CSI-RS resource and CSI-RS ports per CC or in active BWPs across all CCs not only when both Rel-18 CSI report and legacy CSI report are configured but also when different types of adaptations or CSI reporting are configured on each CCs and/or each active BWPs in a CC. First, to count NZP-CSI-RS resource and CSI-RS ports, it would be reasonable to count all NZP-CSI-RS resource and CSI-RS ports for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b. Proposal 4: Add a note in all FGs that ‘Note: for the purpose of component-4, 5, 6 and 7 calculation: NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b’. Regarding to determine maximum value of components, we propose to use one value for all these FGs. It would be straightforward way to inherit the philosophy applied for FG2-33 which is a legacy counterpart of FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. Proposal 5: Add a note in all FGs that ‘Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b’. - Aspects to be newly considered Additionally, there are still ambiguity on how to count the number of CSI report(s) processed by UE simultaneously per CC or across all CCs. It is related to how many CSI reports can be processed simultaneously, and the number of CSI report(s) processed by UE simultaneously are defined in FG2-35. During Rel-18 NES discussion, to calculate the occupied number of CPUs, each CSI report corresponding to sub-configuration is counted as legacy CSI report. In this regard, the CSI report(s) corresponding to sub-configuration should be counted for legacy UE capability in FG2-35. And, the total number of CSI report(s) processed by UE simultaneously per CC or across all CCs should not be larger than the value of FG2-35. Therefore, we propose to add new components and note in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b as follow: - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. Proposal 6: Define new components and note for the maximum number of CPUs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. |
| Apple [8] | 42. Netw_Energy_NR; 42-2a; Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting; Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for semi-persistent CSI reporting 1. The max number of sub-configurations Lmax in one CSI report configuration 2. Report of N CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. 3. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 4. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 6. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Support of single-panel type 1 codebook 8. Supported total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP; 2-33, 2-35; Yes; UE does not support power domain adaptation for semi-persistent CSI reporting; Per band; No; No; N/A; Component 1 candidate values: {2, 3, 4} Component 2 candidate values: {1, 2, 3, 4} Component 3 candidate values: {1, 2, 3 … 32} Component 4 candidate values: {8, 16, 24, … 128 } Component 5 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 6 candidate values: {8, 16, 24, …, 248, 256} Component 8 candidate values: {1, 2, 3, 4} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 5 and 6 are signaled per BC; Optional with capability signaling. |
| NTT DOCOMO, INC. [9] | For FG42-2a, there are three remaining FFS points same as those for FG42-1a. Our views on 42-1a are applicable to 42-2a, and hence proposal for FG42-2a can be made. In addition, component index for FG42-2a is not aligned with those for FG42-2/2b, as it may be just editorial error, it should be fixed so that same component index is used for same content across 42-2/2a/2b. Proposal 11: - FG42-2a has one of {42-2, 42-2b} as prerequisite FG. - For FG42-2a, component index 1 is applied to “Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for semi-persistent CSI reporting”, and other component indices are incremented by 1. - Candidate values for component 2 of FG42-2a are {2,3,4,5,6,7,8}. - Candidate values for component 3 of FG42-2a are {1,2,3,4}. |
| Ericsson [10] | Some notes related to remaining issues for existing FGs (and applicable to new FGs also). - For component related to “Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration”, there is no need for defining candidate values related to N. For aperiodic/semi-persistent CSI reporting, UE should support all values of N up to Lmax, i.e. value of N up to Lmax is supported by the UE. - The components 4,5,6,7 can be updated to reflect the NES condition for applying the per-CC and across-CCs limits. - Pre-requisites: For 42-1/1a/1b/2/2a/2b: adding 2-33 and 2-35 is sufficient. Since these two are mandatory with capability signaling, it is strictly not necessary to explicitly reflect them in the FG table. For NES, FGs 42-1a/42-1b/42-2a/42-2b/(New 42-3a)/(New 42-3b), for component 3 (Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration), - Update the Note to reflect “Value of N: 1 <=N<=Lmax” - There is no need for defining candidate values for component 3 For NES, FGs 42-1/42-1a/42-1b/42-1/42-2a/42-2b/(New 42-3a)/(New 42-3b), update the components 4,5,6,7 as follows: - 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of simultaneous NZP-CSI-RS resources per CC - 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs |
| Sharp [11] |  |
| ETRI [12] |  |
| Vodafone [13] |  |
| LG Electronics [14] | - Candidate values for Lmax and N In RAN1#115 meeting, the following agreement was made for L_max values for P/AP-CSI reporting. Agreement: The following candidate values for Lmax in FGs 42-1/42-1a/42-1b/42-2/42-2a/42-2b are agreed FG; Lmax. SD P; {2,3,4}. SD AP; {2,3,4,5,6,7,8}. PD P; {2,3,4}. PD AP; {2,3,4,5,6,7,8}.. The remaining issue is to determine Lmax for SP-CSI reporting. It should be noted that 8-bit bitmap is allocated for each of sub-configurations in “Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE” that is introduced for Rel-18 NES feature. Thus, the candidate value range for Lmax could be 2 to 8 for semi-persistent CSI reporting. Proposal #1: For Lmax in FGs 42-1a (spatial domain & semi-persistent CSI reporting) and 42-2a (power domain & semi-persistent CSI reporting), the candidate values are {2,3,4,5,6,7,8}. In RAN1#114bis meeting, the following two NOTEs were agreed for semi-persistent CSI reporting. - Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH - Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH In our view, UE doesn’t need to report the candidate value for N in addition to Lmax. If a UE indicates support of up to Lmax sub-configurations at least for AP-CSI reporting, the UE should be able to report any N (<=Lmax) sub-configuration(s). However, especially for SP-CSI reporting, if the candidate values of Lmax are 2 to 8 and the above NOTE for N is considered, one method is to determine N as minimum of 4 and Lmax. Proposal #2: Do not introduce UE capability signaling for N in FGs 42-1a, 42-1b, 42-2a, and 42-2b. - For 42-1b and 42-2b, UE supports N up to Lmax. - For 42-1a and 42-2a, UE supports N up to min(4,Lmax). |
| Qualcomm Incorporated [15] | 42-2a; Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting; Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for semi-persistent CSI reporting 1. The max number of sub-configurations Lmax in one CSI report configuration 2. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. 3. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 4. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 6. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Support of single-panel type 1 codebook; FFS 2-32a or 2-32b; Yes; UE does not support power domain adaptation for semi-persistent CSI reporting; Per band; No; No; N/A; Component 1 candidate values: FFS {2,3,4,5,6,7,8} Component 2 candidate values: FFS {2, 3, 4} Component 3 candidate values: {1, 2, 3 … 32} Component 4 candidate values: {8, 16, 24, … 128 } Component 5 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 6 candidate values: {8, 16, 24, …, 248, 256} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 5 and 6 are signaled per BC Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs; Optional with capability signaling. |
| MediaTek Inc. [16] |  |

| 42. Netw_Energy_NR | 42-2b | Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP | FFS | Yes |  | UE does not support power domain adaptation for aperiodic CSI reporting | Per band | No | No | N/A | Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values: FFS Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128 } Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} Note: Components 6 and 7 are signaled per BC FFS: different values for joint operation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] | 42. Netw_Energy_NR; 42-2b; Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP; 42-2; Yes; UE does not support power domain adaptation for aperiodic CSI reporting; Per band; No; No; N/A; Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values: FFS Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128 } Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} Note: Components 6 and 7 are signaled per BC; Optional with capability signaling. |
| Huawei/HiSilicon [3] | - For semi-persistent CSI reporting, the candidate values for configured number of sub-configurations and reported CSIs are to be determined. As it was agreed that “Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH” and “Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH”, we propose as follows: - For 42-1a and 42-2a, - The candidate values for L: 2, 3, 4, 5, 6, 7 and 8. - The candidate values for N: 1, 2, 3 and 4. - For aperiodic CSI reporting, the candidate values for reported CSIs are to be determined. The same values as semi-persistent CSI reporting can be used. - For 42-1b and 42-2b, - The candidate values for N: 1, 2, 3 and 4. - Regarding the prerequisite feature groups, since we are specifically design the NES feature with willingness of supporting deferent values from a legacy CSI report capability, there does not expect strong dependences among these FGs, and components 4)~7) are reported for each of these FGs respectively, so no prerequisite feature groups are needed. - Regarding whether to set different values for joint operation, from UE implementation perspective, the newly introduced FG is mainly related to UE capabilities, similar to the one defined in FG 2-33, 2-35 in Rel-15, about CSI measurement, CSI processing and CSI reporting. The difference between SD/PD only operation and joint operation mainly lays on the implementation at gNB side and configurations (to UE), while the difference for UE would be small, e.g. the supported maximum number of CSI reports per BWP, CPU counting or active resource/antenna ports counting are kept the same among different configurations for adaptation. Therefore, there is no justification to set different candidate values for joint operation. Proposal 2: For FG 42-1b/42-2b, - The candidate values for N: 1, 2, 3 and 4. Proposal 3: For FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, - No prerequisite feature groups are needed. |
| Vivo [4] |  |
| ZTE [5] | For semi-persistent CSI reports and aperiodic CSI reports, the same candidate values of N can be used. And the maximum candidate value of N can refer to the maximum value of Lmax of periodic CSI report, which is 4. The minimum candidate value of N should be 1 to support single CSI sub-report. Proposal 1: The candidate values of N for SP-CSI report and AP-CSI report are {1,2,3,4}. The following FG 23-7-1 is the ‘Basic Features of CSI Enhancement for Multi-TRP’, which is introduced in R17. 23. NR_FeMIMO; 23-7-1; Basic Features of CSI Enhancement for Multi-TRP; 1. Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 2, Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max 3. CSI report mode selection of mode 1 with X=0 and/or mode 2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses 5. Supported codebook modes for NCJT CSI. The FG 23-7-1 is also about CSI enhancement, which does not set a ‘Prerequisite feature groups’. Given this, for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b, the ‘Prerequisite feature groups’ doesn’t need, either. Proposal 2: ‘Prerequisite feature groups’ is not needed for FG 42-1/42-1a/42-1b/42-2/42-2a/42-2b. |
| CATT [6] | The support of the spatial and power domain adaptation requires additional UE CSI measurements and reports when some TxRUs are turned off. A number of TxRUs turning off and generating a new number of antenna ports or beam pattern is indicated as an antenna sub-configuration of the full antenna configuration. The additional CSI reports of sub-configurations with each CSI report associated with one sub-configuration will be fed back to the gNB for dynamic link adaptation. Multiple CSI reports associated with full antenna configuration and sub-configurations are included in all CSI report type; they are aperiodic CSI report, semi-persistent CSI report, and periodic CSI report. The UE capability of CSI measurements for A-CSI, SP-CSI and P-CSI feedbacks should be the same without any differentiation in the UE capability since the measurements of configured CSI-RS resource(s) are the same. In RAN1#115, the maximum number of SP-CSI measurements were agreed to be the same as that of P-CSI. Since the additional CSI measurements of antenna sub-configuration would be provisioned by the UE at each band. The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands and based on each UE (per UE) regardless if it is Type 1 or Type 2 antenna adaptation. For Type 1 antenna adaptation, the additional number of CSI measurements for CSI feedbacks in a CSI report is based on the measurements of a CSI-RS resource set. The maximum number of CSI measurements N from a CSI-RS resource set should be based on UE capability in additional CSI measurements and computation. The maximum number of CSI measurements N in a CSI report would depend on the number of network operation of spatial adaptation for network energy saving. The spatial domain adaptation relies on the ON/OFF of the TxRU to achieve the network energy saving. The transition time of the TxRU ON/OFF would be the deciding factor. The power amplifier of the TxRU would demands range of ms to stabilize the Tx power setting and would not be able to dynamically shutting down and turning on. Thus, the TxRU ON/OFF would not be substantially frequent at the gNB for spatial adaptation. The number of the CSI measurements of sub-configurations should not be large. Thus, the maximum number of CSI measurements of antenna sub-configurations in a CSI report should be set to 4. The maximum number of sub-configurations L for UE to support should be based on the possibility of spatial and power adaptations by the network. The level of the power adaptations would not see much network energy saving gain. Thus, the majority of the spatial and power adaptation is different spatial domain configurations. The maximum number of spatial and power domain sub-configurations should be set to 8. For joint operation of spatial and power domain adaptation, the maximum number of sub-configuration and the CSI reports should remain the same since the joint operation should not increase the additional complexity. Proposal 1: The UE capability of additional CSI feedbacks for antenna sub-configurations should be same cross bands for each UE (per UE). Proposal 2: The maximum number of antenna sub-configurations LMax for UE to support should be set to 8 with candidate values {1, 2, 4, 8} for spatial and/or power domain adaptation. Proposal 3: The maximum number of sub-configuration and the CSI reports for joint operation of spatial and power domain adaptation for NES should remain the same. |
| Samsung [7] | - Regarding FFS: different values for joint operation in Note The remaining issue is how to determine values for joint operation. For joint operation of SD and PD adaptation, RAN1 had made agreement to support joint operation for NES in RAN1#113. And, UE can have different capability values for each SD and PD adaptation. In this regard, even if UE supports both capabilities, there will be ambiguity on UE capability for joint operation, because UE can have different capabilities between SD and PD adaptation. Therefore, we propose to define new independent UE feature for joint operation of SD and PD adaptation for P/SP/AP CSI reporting. Proposal 1: Define new UE feature of joint operation of SD and PD adaptation. - Regarding FFS in Prerequisite feature groups FG2-33 and FG2-35 can be reused for all FGs related to SD and PD adaptation as baseline. Hence we suggest to include FG2-33 and FG2-35 as prerequisite feature groups of all FGs related to SD and PD adaptation. Proposal 2: For all FGs related to SD and PD adaptation, add FG2-33 and FG2-35 as prerequisite feature groups. - Regarding Component 9 Regarding the component 9 in FGs, it is clear that the value is for each report type, and this is associated with UE capability defined in FG2-35. The value of component 9 will be used instead of the values for component 1 and 2 in FG2-35, when CSI reporting with sub-configuration for SD and PD adaptation is configured. In case of UE is configured with Rel-18 NES CSI report and legacy CSI report, to be clear the intention of component 9, we propose to add Note in FG42-1, FG42-1b, FG42-2 and FG42-2b. Proposal 3: Add a note in FG42-1, FG42-1b, FG42-2 and FG42-2b that ‘Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration’. - Regarding Component 4, 5, 6, and 7 In terms of components related to maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs, it is still unclear how to determine maximum value and how to count number of simultaneous NZP-CSI-RS resource and CSI-RS ports per CC or in active BWPs across all CCs not only when both Rel-18 CSI report and legacy CSI report are configured but also when different types of adaptations or CSI reporting are configured on each CCs and/or each active BWPs in a CC. First, to count NZP-CSI-RS resource and CSI-RS ports, it would be reasonable to count all NZP-CSI-RS resource and CSI-RS ports for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b. Proposal 4: Add a note in all FGs that ‘Note: for the purpose of component-4, 5, 6 and 7 calculation: NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b’. Regarding to determine maximum value of components, we propose to use one value for all these FGs. It would be straightforward way to inherit the philosophy applied for FG2-33 which is a legacy counterpart of FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. Proposal 5: Add a note in all FGs that ‘Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resource and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b’. - Aspects to be newly considered Additionally, there are still ambiguity on how to count the number of CSI report(s) processed by UE simultaneously per CC or across all CCs. It is related to how many CSI reports can be processed simultaneously, and the number of CSI report(s) processed by UE simultaneously are defined in FG2-35. During Rel-18 NES discussion, to calculate the occupied number of CPUs, each CSI report corresponding to sub-configuration is counted as legacy CSI report. In this regard, the CSI report(s) corresponding to sub-configuration should be counted for legacy UE capability in FG2-35. And, the total number of CSI report(s) processed by UE simultaneously per CC or across all CCs should not be larger than the value of FG2-35. Therefore, we propose to add new components and note in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b as follow: - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. Proposal 6: Define new components and note for the maximum number of CPUs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b. - UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations. - Candidate values: {from 1 to 8} - UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations. - Candidate values: {from 5 to 32} - Note: Maximum number of CPUs is reported in FG 2-35. |
| Apple [8] | 42. Netw_Energy_NR; 42-2b; Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI sub-report(s) included in one CSI report where each CSI sub-report corresponds to one sub-configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP; 2-33, 2-35; Yes; UE does not support power domain adaptation for aperiodic CSI reporting; Per band; No; No; N/A; Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values: {2, 3, 4, 5, 6, 7, 8} Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128 } Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} Note: Components 6 and 7 are signaled per BC; Optional with capability signaling. |
| NTT DOCOMO, INC. [9] | For FG42-2b, there are three remaining FFS points same as those for FG42-1b. Our views on 42-1b are applicable to 42-2b, and hence following proposal for FG42-2b can be made. Proposal 12: - FG42-2b does not require any prerequisite FG. - Candidate values for component 3 of FG42-2b are {1,2,3,4}. |
| Ericsson [10] | Some notes related to remaining issues for existing FGs (and applicable to new FGs also). - For component related to “Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration”, there is no need for defining candidate values related to N. For aperiodic/semi-persistent CSI reporting, UE should support all values of N up to Lmax, i.e. value of N up to Lmax is supported by the UE. - The components 4,5,6,7 can be updated to reflect the NES condition for applying the per-CC and across-CCs limits. - Pre-requisites: For 42-1/1a/1b/2/2a/2b: adding 2-33 and 2-35 is sufficient. Since these two are mandatory with capability signaling, it is strictly not necessary to explicitly reflect them in the FG table. For NES, FGs 42-1a/42-1b/42-2a/42-2b/(New 42-3a)/(New 42-3b), for component 3 (Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration), - Update the Note to reflect “Value of N: 1 <=N<=Lmax” - There is no need for defining candidate values for component 3 For NES, FGs 42-1/42-1a/42-1b/42-1/42-2a/42-2b/(New 42-3a)/(New 42-3b), update the components 4,5,6,7 as follows: - 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of simultaneous NZP-CSI-RS resources per CC - 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs |
| Sharp [11] |  |
| ETRI [12] |  |
| Vodafone [13] |  |
| LG Electronics [14] | In RAN1#114bis meeting, the following two NOTEs were agreed for semi-persistent CSI reporting. - Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH - Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH In our view, UE doesn’t need to report the candidate value for N in addition to Lmax. If a UE indicates support of up to Lmax sub-configurations at least for AP-CSI reporting, the UE should be able to report any N (<=Lmax) sub-configuration(s). However, especially for SP-CSI reporting, if the candidate values of Lmax are 2 to 8 and the above NOTE for N is considered, one method is to determine N as minimum of 4 and Lmax. Proposal #2: Do not introduce UE capability signaling for N in FGs 42-1a, 42-1b, 42-2a, and 42-2b. - For 42-1b and 42-2b, UE supports N up to Lmax. - For 42-1a and 42-2a, UE supports N up to min(4,Lmax). |
| Qualcomm Incorporated [15] | 42-2b; Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting; 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration 4. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP; FFS 2-35; Yes; UE does not support power domain adaptation for aperiodic CSI reporting; Per band; No; No; N/A; Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values: FFS {2, 3, 4} Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128 } Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} Note: Components 6 and 7 are signaled per BC FFS: different values for joint operation Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain.; Optional with capability signaling. |
| MediaTek Inc. [16] |  |

Other

| Company | Summary |
| --- | --- |
| Nokia/Nokia Shanghai Bell [2] |  |
| Huawei/HiSilicon [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| CATT [6] |  |
| Samsung [7] | The new mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} in a slot has been introduced for Rel-18 NES. The new UE feature should be introduced to reflect mixed codebook combination in a slot. Therefore, we suggest to add additional new FG42-X to indicate the UE supports mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} in a slot. Proposal 7: Define new UE feature to indicate the UE supports mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} in a slot. |
| Apple [8] | As for SD/PD adaptation, one CSI reporting can comprise one or more sub-reports. The UE implementation of a sub-report can reuse that of a CSI reporting such as CSI computation and memory. In order to commercialize NES features in the market as early as possible, there needs capability signaling to be able to implement the features with no or minimal hardware impact to UE. Towards this end, the components were introduced in RAN1 #115 of the supported total number of P/AP-CSI reporting settings without sub-configurations plus the total number of sub-configurations across P/SP-CSI report settings with sub-configurations per BWP. It was initially proposed in [5] that the components were for P/SP/AP-CSI reporting, but the rearrangements during online discussion to split the FGs to periodic, semi-persistent, and aperiodic CSI reporting individually have missed the case of SP-CSI reporting. In addition, two new parameters related to Ncpu (i.e. simultaneousCSI-ReportsPerCC and simultaneousCSI-ReportsAllCC) have been missed in UE feature list. If these new capabilties are not introduced, there could be ‘under-reporting’ issues to support SD/PD adaptation considering hardware limitation as discussed in [7]. This is illustrated in Figure 1. Figure 1 Missing components for CSI report to support SD/PD adaptation. Therefore, we propose the following capability signaling. Proposal 1-1: Introduce the following capability signaling - 1/ Supported total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP (per-band) - 2/ simultaneousCSI-ReportsPerCC-r18 (per-band) - Indicates the number of CSI report(s) for which the UE can measure and process reference signals simultaneously in a CC of the band for which this capability is provided. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsPerCC-r18 includes the beam report, and CSI report without sub-configurations plus sub-configurations across CSI reports with sub-configurations - 3/ simultaneousCSI-ReportsAllCC-r18 (per-BC) - Indicates whether the UE supports CSI report framework and the number of CSI report(s) which the UE can simultaneously process across all CCs, and across MCG and SCG in case of NR-DC. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsAllCC-r18 includes the beam report, and CSI report without sub-configurations plus sub-configurations across CSI reports with sub-configurations. This parameter may further limit simultaneousCSI-ReportsPerCC-r18 in MIMO-ParametersPerBand and Phy-ParametersFRX-Diff for each band in a given band combination. 42. Netw_Energy_NR; 42-4; simultaneousCSI-ReportsPerCC-r18; 1. Indicates the number of CSI report(s) for which the UE can measure and process reference signals simultaneously in a CC of the band for which this capability is provided. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsPerCC-r18 includes the beam report, and CSI report without sub-configurations plus sub-configurations across CSI reports with sub-configurations; Yes; UE does not support spatial or power domain adaptation; Per band; No; No; N/A; Component 1 candidate values: {1, 2, 3, 4, 5, 6, 7, 8}; Optional with capability signaling. 42. Netw_Energy_NR; 42-4a; simultaneousCSI-ReportsAllCC-r18; 1. Indicates whether the UE supports CSI report framework and the number of CSI report(s) which the UE can simultaneously process across all CCs, and across MCG and SCG in case of NR-DC. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsAllCC-r18 includes the beam report, and CSI report without sub-configurations plus sub-configurations across CSI reports with sub-configurations. This parameter may further limit simultaneousCSI-ReportsPerCC-r18 in MIMO-ParametersPerBand and Phy-ParametersFRX-Diff for each band in a given band combination.; Yes; UE does not support spatial or power domain adaptation; Per BC; No; No; N/A; Component 1 candidate values: {5, 6, 7, ..., 32}; Optional with capability signaling. Regarding prerequisite for each FG, no association between Rel-18 FGs (e.g. between P/SP/AP-CSI reporting, between SD and PD adaptations, etc.) is desirable. There is no strong motivation to tight features each other. With the same reason, we also propose the separate capability for joint operation of SD and PD adaptation. On the other hand, FG 2-33 (CSI-RS and CSI-IM reception for CSI feedback) and FG 2-35 (CSI report framework) are deemed necessary as prerequisite since UE needs to report both Rel-15/16 and Rel-18 capability signaling given that UE does not know network version (i.e. for backward compatibility reason). Proposal 1-2: Prerequisites for all SD and PD adaptations (FG 42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b) are FG 2-33 (CSI-RS and CSI-IM reception for CSI feedback) and FG 2-35 (CSI report framework). Proposal 1-3: Separate capability for joint operation of SD and PD adaptation is introduced. 42. Netw_Energy_NR; 42-3; Joint operation of spatial and power domain adaptation for periodic CSI reporting; 1. support of joint operation of spatial and power domain adaptation for periodic CSI reporting; 42-1, 42-2; Yes; UE does not support joint operation of spatial and power domain adaptation for periodic CSI reporting; Per band; No; No; N/A; Optional with capability signaling. 42. Netw_Energy_NR; 42-3a; Joint operation of spatial and power domain adaptation for semi-persistent CSI reporting; 1. support of joint operation of spatial and power domain adaptation for semi-persistent CSI reporting; 42-1a, 42-2a; Yes; UE does not support joint operation of spatial and power domain adaptation for semi-persistent CSI reporting; Per band; No; No; N/A; Optional with capability signaling. 42. Netw_Energy_NR; 42-3b; Joint operation of spatial and power domain adaptation for aperiodic CSI reporting; 1. support of joint operation of spatial and power domain adaptation for aperiodic CSI reporting; 42-1b, 42-2b; Yes; UE does not support joint operation of spatial and power domain adaptation for aperiodic CSI reporting; Per band; No; No; N/A; Optional with capability signaling. Some component candidate values have not been finalized yet. Our proposed values are shown below together. Proposal 1-4: The following component candidate values are proposed. - 42-1a: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting - 2. The max number of sub-configurations Lmax in one CSI report configuration - [Apple] Component 2 candidate values: {2, 3, 4} - 3. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. - [Apple] Component 3 candidate values: {1, 2, 3, 4} - 42-1b: Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting - 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration - [Apple] Component 3 candidate values: {2, 3, 4, 5, 6, 7, 8} - 42-2a: Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting - 1. The max number of sub-configurations Lmax in one CSI report configuration - [Apple] Component 2 candidate values: {2, 3, 4} - 2. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration. - [Apple] Component 3 candidate values: {1, 2, 3, 4} - 42-2b: Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting - 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration - [Apple] Component 3 candidate values: {2, 3, 4, 5, 6, 7, 8} On the following component: 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration We understand the wording came from one agreement made in RAN1 #112b: Agreement For a CSI report config with L sub-configuration(s), support a framework that enables a UE to report N CSI(s) in one reporting instance where the N CSI(s) are associated with N sub-configuration(s) from L (where 1≤ N≤ L) and each CSI corresponds to one sub-configuration. Later, we have introduced ‘sub-report’ in CSI reporting during CR phase (e.g. see TS38.213 CR [8]). The current wording may mislead the definitions and therefore we suggest to update the descriptions of components. Proposal 1-5: The following compoments are updated as follows: - Report of N CSI sub-report(s) included in one AP/SP-CSI report where each CSI sub-report corresponds to one sub-configuration. |
| NTT DOCOMO, INC. [9] |  |
| Ericsson [10] |  |
| Sharp [11] |  |
| ETRI [12] |  |
| Vodafone [13] |  |
| LG Electronics [14] | - Mixed codebook support Agreement (RAN1#114) For Type 1 SD for multi-panel case, - Introduce a new mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} in R18 for FG codebookComboParameterAddition (indicating the UE supports the mixed codebook combinations in a slot) - Note: gNB can configure either Type 1 single panel codebook or Type 1 multi-panel codebook for a sub-configuration from one or multiple sub-configurations within one CSI report configuration if a UE reports support of multi-panel operation.. According to the above RAN1 agreement that a new mixed codebook combination is introduced in Rel-18, the corresponding UE behavior is described in 214 specifation, as highlighted below. ##### 5.2.1.4.2 Report quantity configurations <Unchanged texts omitted> If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by the higher layer parameter [csi-ReportSubConfigList]: - the UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with [ABC], each sub-configuration can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'.. However, it doesn’t seem that the current UE feature list for NES reflects the previous RAN1 agreement and UE behavior defined in 214 spec. Thus, we suggest to add one more FG corresponding to the support of mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null}. Proposal #3: Add one more FG or component to support the new mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} in a slot for type 1 SD adaptation, according to RAN1 agreement made in RAN1#114. |
| Qualcomm Incorporated [15] | On a new mixed codebook combination, RAN1#114 made the following agreements on introducing a new mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} in R18 for FG codebookComboParameterAddition (indicating the UE supports the mixed codebook combinations in a slot) in order to support the configuration of either Type 1 single panel codebook or Type 1 multi-panel codebook for a sub-configuration from one or multiple sub-configurations within one CSI report configuration if a UE reports support of multi-panel operation. Agreement (RAN1#114) For Type 1 SD for multi-panel case, - Introduce a new mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} in R18 for FG codebookComboParameterAddition (indicating the UE supports the mixed codebook combinations in a slot) - Note: gNB can configure either Type 1 single panel codebook or Type 1 multi-panel codebook for a sub-configuration from one or multiple sub-configurations within one CSI report configuration if a UE reports support of multi-panel operation. In the current specification, the codebook combination FG is per band and per band combination as being specified in the two features codebookComboParametersAddition-r16 and codebookComboParametersAdditionPerBC-r16, respectively, as follows: Therefore, to properly capture the above agreement, we make the following proposal. Proposal 1.2: RAN1 introduces the following new feature group in supporting a new mixed codebook combination for Type 1 spatial domain adaptation. 42-x; Mixed codebook combination for spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration; - Indicate the support of active CSI-RS resources and ports for mixed codebook types in any slot. The following codebook combination is a possible mixed codebook combination {Codebook1, Codebook2, Codebook3} for UE supporting CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration. - {Type 1 Single Panel, Type 1 Multi Panel, Null} - Indicate the list of supported CSI-RS resources across all bands in a band combination by referring to codebookVariantsList for the mixed codebook types. The following parameters are included in codebookVariantsList for each code book type: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource across all bands within a band combination; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs within a band combination, simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs within a band combination, simultaneously. - The UE supporting this feature shall indicate the support of CodebookComboParametersAddition-r16 and the support of multi-panel operation.; 42-1 or 42-1a or 42-1b; Yes; UE does not support the mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} for spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration; Per band for component 1 Per BC for component 2; No; No; N/A; - Note: Component 1 candidate values of 42-1, 42-1a, 42-1b is {SD-type1, SD-type1and2}.; Optional with capability signaling. |
| MediaTek Inc. [16] |  |

# Discussion Items during RAN1 #116

After review of contributions submitted to RAN1 #116 in this agenda item, the following topics were identified by the moderator for discussion during RAN1 #116.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MediaTek1 | MediaTek t-doc revision is R1-2401471, uploaded on Feb. 20. In particular, we would like to highlight UE can preport different set of values for components 4 – 7 for SD-type1, SD-type2 and PD. Requiring UE to report the same values across SD and PD features to accommodate joint operation would be too conservative to exploit UE capability. We would suggest allowing different reported values for SD-type1, SD-type2 and PD while using a rule, e.g., taking minimum, for determining the capability limit the corresponding joint operations. |

# Issue 1: FG 42-1

After review of contributions submitted to RAN1 #116 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 42. Netw_Energy_NR | 42-1 | Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for periodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations | FFS | Yes |  | UE does not support spatial domain adaptation for periodicCSI reporting | Per band | No | No | N/A | Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4} Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4} Component 10 candidate values: {1, 2, 3, …, 8} Component 11 candidate values: {5, 6, …, 32} Note: Components 6 and 7 are signaled per BC FFS: different values for joint operation Note: If a UE supports both spatial domain and power domain adaptation, the same candidate values are used for joint operation of spatial domain and power domain adaptation Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain Note: Maximum number of CPUs is reported in FG 2-35 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | 1/ On the new additions for component 10 and 11: 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations Our understanding is the proposals are duplicated with Issue 8. It can mislead total Ncpu is represented by ‘legacy Ncpu’ + ‘new Ncpu (component 10/11). Therefore, we propose to adopt the proposals in Issue 8 rather than new additions of component 10/11. 2/ On the new addition of note: Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration We suggest the following modification. Note: Component 9 is used instead of the values in maxNumberPeriodicCSI-PerBWP-ForCSI-Report in FG2-35 when CSI report configuration includes at least one subconfigurationthe corresponding Rel-18 CSI report configuration 3/ On the new addition:Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b From offline discussion with Samsung, we understand this comes from different definition of components 4/5/6/7 than legacy definition. In other words, legacy definition considers all (P/SP/AP-CSI reporting) whereas the current components 4/5/6/7 are defined individually for P-, SP-, and AP- CSI reporting. One way can be to move out components 4/5/6/7 to (new) separate FG. This requires at least two (new) separate FGs (one for SD and the other for PD). In case joint operation of SD+PD is defined as separate FG, we need at least three (new) separate FGs (one for SD, another for PD, and the other for SD+PD). 5/ On the following note: Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain We think it would restrict UE capability unnecessarily and this note may make thing confused. If we introduce separate FG for joint operation of SD+PD, component 9 can be added for the new FGs and we do not need this note (our preference). 6/ On the following note: Note: Maximum number of CPUs is reported in FG 2-35 This is related to our comment 1/ above. Thus, if Issue 8 is agreed, this note is not needed in our opinion. |
| Qualcomm | - New addition to components 4-7 “In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier,” seems unnecessary since it does not provide additional information. Suggest to not have it. - For the following new components, the proponent seems propoposing to replicate components 4-5 of FG 2-35. However, it does not reflect the meaning of the components 4-5 of FG 2-35 which describe the counting for all P/SP/A CSI and any latency class and codebook type; including report for beam management. We prefer to not have these new components in this FG. 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations - The following new note is unclear. It is our understanding that for any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively. Hence, the 4th new note should be needed. Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b - We don’t think the restriction in the following note is needed. Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b - For prequiste of the FG, we propose to have FG 2-33 (or FG 2-35 if new FG is not introduced). |
| Samsung | 1) On the new additions for component 10 and 11: 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations Our intention to add this is to clarify how many CSI reports can be processed simultaneously, and the number of CSI report(s) processed by UE simultaneously as in defined in FG2-35. Given the discussion, we are fine with removing this and continue to discuss in Issue 8. 2) On the new addition of note: Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration We are also OK with the modified proposal from Apple. 3) On the new addition: Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b Our intention to add this is to clarify how to determine maximum value and how to count number of simultaneous NZP-CSI-RS resource and CSI-RS ports per CC or in active BWPs across all CCs not only when both Rel-18 CSI report and legacy CSI report are configured but also when different types of adaptations or CSI reporting are configured on each CCs and/or each active BWPs in a CC. If it can be clarified, we are open to other options. 4) On the following note: Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain We have similar view as Apple. Our preference is to introduce separate FG for joint operation of SD+PD. 5) On the following note: Note: Maximum number of CPUs is reported in FG 2-35 We have similar view as Apple. No need to have it if FG in issue 8 is agreed. - Prequiste of the FG, we propose to have FG 2-33 and FG 2-35 because they are basic features for CSI report and reception when NW does not support Rel-18 NES. |
| LG Electronics | - As pointed out by Apple, it needs to be clarified whether NEWly added components 10 and 11 are overlapped with Issue 8 or not. If overlapped, we prefer to introduce NEW FGs as proposed by moderator in Issue 8. - For the following note, our understanding is that relevant previous FFS point is only for component 4 to 7. In that sense, we would suggest the following modification. Note: If a UE supports both spatial domain and power domain adaptation, the same candidate values for components 4-7 are used for joint operation of spatial domain and power domain adaptation - It seems that the following two notes are overlapped and one of them can be removed. Note: If a UE supports both spatial domain and power domain adaptation, the same candidate values for components 4-7 are used for joint operation of spatial domain and power domain adaptation Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b - For the following note, FG 42-1a and FG 42-2a also need to be added as follows. Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b - We fail to understand the following note. For example, if UE reports N as component 4 (supported maximum number of simultaneous NZP-CSI-RS resources per CC) in FG 42-1 and K as “Supported maximum number of simultaneous NZP-CSI-RS resources per CC” in FG 2-33, UE will apply N for a CC where CSI report containing sub-configurations is configured while UE will apply K for a CC where CSI report containing sub-configuration is not configured. In that sense, we don’t think the following note is needed. Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs |
| Huawei, HiSilicon | 1/ Our understanding is that components 9/10 is for sub-report only, while proposal in Issue 8 seems to be for CSI report level, where a repot may or may not have sub-report. Is it correct understanding? 2/ we support FL proposal that is Not to have separate FG for joint SD+PD. 3/ we are not clear on the note: Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 4/ On the below Note, we think it is against the spirit to allow NES with advanced UE capability, if reported by a UE. This seems to be reflected in Issue 8 on the other hand. Note: Maximum number of CPUs is reported in FG 2-35 5/ We support FL proposal Not to have pre-requiest FG. Since the functionality for R18 NES include special case of e.g. 1 sub-report, it does not matter much whether a UE has to be support 2-33/35 in our view. |
| Ericsson1 | - New addition to components 4,5,6,7 : we support to add it as it clarifies the case for which these components apply - Components 10, 11 : We do not support. - We do not support the following additions to the Note column i.e. Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain Note: Maximum number of CPUs is reported in FG 2-35 |
| MediaTek1 | The following note only clarifies the candidate values for joint SD and PD operation: “Note: If a UE supports both spatial domain and power domain adaptation, the same candidate values are used for joint operation of spatial domain and power domain adaptation” But is it not clear whether there are separated capability reports for SD-type1 & PD and SD-type2 & PD. Given we have already significant amount of reported values for adaptation types, reporting types, and per-BC values for components 6 and 7, we would like to suggest no separated reporting for joint SD and PD operation. The note can be revised to reflect that the capability limits of components 4-7 of joint SD and PD are determined by taking minimum over the corresponding reported values: “Note: If a UE supports both spatial domain and power domain adaptation, UE capability limit for component 4/5/6/7 is the minimum of the reported value for power domain adaptation and that for the correspond type of spatial domain adaptation (SD-type1 or SD-type2).” |

# Issue 2: FG 42-1a

After review of contributions submitted to RAN1 #116 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 42. Netw_Energy_NR | 42-1a | Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for semi-persistent CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations | FFS | Yes |  | UE does not support spatial domain adaptation for semi-persistent CSI reporting | Per band | No | No | N/A | Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: FFS {2,3,4,5,6,7,8} Component 3 candidate values: FFS {2,3,4} Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128} Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Component 9 candidate values: {1, 2, 3, 4} Component 10 candidate values: {1, 2, 3, …, 8} Component 11 candidate values: {5, 6, …, 32} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 6 and 7 are signaled per BC Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | 1/ On the new additions for component 10 and 11: 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations Our understanding is the proposals are duplicated with Issue 8. It can mislead total Ncpu is represented by ‘legacy Ncpu’ + ‘new Ncpu (component 10/11). Therefore, we propose to adopt the proposals in Issue 8 rather than new additions of component 10/11. 2/ On the new addition of note: Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration We suggest the following modification. Note: Component 9 is used instead of the values in maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report in FG2-35 when CSI report configuration includes at least one subconfigurationthe corresponding Rel-18 CSI report configuration 3/ On the new addition:Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b From offline discussion with Samsung, we understand this comes from different definition of components 4/5/6/7 than legacy definition. In other words, legacy definition considers all (P/SP/AP-CSI reporting) whereas the current components 4/5/6/7 are defined individually for P-, SP-, and AP- CSI reporting. One way can be to move out components 4/5/6/7 to (new) separate FG. This requires at least two (new) separate FGs (one for SD and the other for PD). In case joint operation of SD+PD is defined as separate FG, we need at least three (new) separate FGs (one for SD, another for PD, and the other for SD+PD). |
| Qualcomm | Same comments as the ones in Issue 1. |
| Samsung | Same comments as the ones in Issue 1. |
| LG Electronics | Same comments as the ones in Issue 1. |
| Huawei, HiSilicon | In addition to our comments for Issue 1, particularly for SP-CSI reporting, we propose to add candidate value 1 for Component 3. Duing the discussion phase of WI, there is a use case identified for a UE only supporting 1 sub-report, i.e. for just dynamically changing the to-be-reported CSI one by one => Component 3 candidate values: FFS {1, 2,3,4} |
| Ericsson1 | Same comments as for Issue 1, and For component 3, We do not support defining candidate values of {2,3,4}. It should simply reflect 1 <=N<=Lmax |

# Issue 3: FG 42-1b

After review of contributions submitted to RAN1 #116 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 42. Netw_Energy_NR | 42-1b | Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration/list of CSI-RS IDs for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI sub-report(s) included in one CSI report where each CSI sub-report corresponds to one sub-configuration 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations | FFS | Yes |  | UE does not support spatial domain adaptation for aperiodic CSI reporting | Per band | No | No | N/A | Component 1 candidate values: {SD-type1, SD-type2, SD-type1and2} Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4} FFS Component 4 candidate values: SD Type 1: {1, 2, 3 … 32} SD Type 2: {1, 2, 3 … 32} Component 5 candidate values: SD Type 1: {8, 16, 24, … 128 } SD Type 2: {8, 16, 24, … 128 } Component 6 candidate values: SD Type 1: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} SD Type 2: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: SD Type 1: {8, 16, 24, …, 248, 256} SD Type 2: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11,12} Component 10 candidate values: {1, 2, 3, …, 8} Component 11 candidate values: {5, 6, …, 32} FFS: different values for joint operation Note: If a UE supports both spatial domain and power domain adaptation, the same candidate values are used for joint operation of spatial domain and power domain adaptation Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | 1/ On the new additions for component 10 and 11: 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations Our understanding is the proposals are duplicated with Issue 8. It can mislead total Ncpu is represented by ‘legacy Ncpu’ + ‘new Ncpu (component 10/11). Therefore, we propose to adopt the proposals in Issue 8 rather than new additions of component 10/11. 2/ On the new addition of note: Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration We suggest the following modification. Note: Component 9 is used instead of the values in maxNumberAperiodicCSI-PerBWP-ForCSI-Report or maxNumberAperiodicCSI-PerBWP-ForCSI-Report-r16 in FG2-35 when CSI report configuration includes at least one subconfigurationthe corresponding Rel-18 CSI report configuration 3/ On the new addition:Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b From offline discussion with Samsung, we understand this comes from different definition of components 4/5/6/7 than legacy definition. In other words, legacy definition considers all (P/SP/AP-CSI reporting) whereas the current components 4/5/6/7 are defined individually for P-, SP-, and AP- CSI reporting. One way can be to move out components 4/5/6/7 to (new) separate FG. This requires at least two (new) separate FGs (one for SD and the other for PD). In case joint operation of SD+PD is defined as separate FG, we need at least three (new) separate FGs (one for SD, another for PD, and the other for SD+PD). 5/ On the following note: Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain We think it would restrict UE capability unnecessarily and this note may make thing confused. If we introduce separate FG for joint operation of SD+PD, component 9 can be added for the new FGs and we do not need this note (our preference). |
| Qualcomm | Same comments as the ones in Issue 1. |
| Samsung | Same comments as the ones in Issue 1. |
| LG Electronics | Same comments as the ones in Issue 1. |
| Ericsson1 | Same comments as for Issue 1, and For component 3 candidate value, we do not support defining candidate values of {1,2,3,4}. It should simply reflect 1 <=N<=Lmax. We do not support candidate value of N=1, which means only UE supports reporting of a single hypothesis only. |
| MediaTek1 | Same comments as the ones in Issue 1. |

# Issue 4: FG 42-2

After review of contributions submitted to RAN1 #116 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 42. Netw_Energy_NR | 42-2 | Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset foraperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations | FFS | Yes |  | UE does not support power domain adaptation for periodicCSI reporting | Per band | No | No | N/A | Component 2 candidate value: {2,3,4} Component 4 candidate value: {1, 2, 3 … 32} Component 5 candidate value: {8, 16, 24, … 128 } Component 6 candidate value: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate value: {8, 16, 24, …, 248, 256} Note: Components 6 and 7 are signaled per BC Component 9 candidate values: {2, 3, 4} Component 10 candidate values: {1, 2, 3, …, 8} Component 11 candidate values: {5, 6, …, 32} FFS: different values for joint operation Note: If a UE supports both spatial domain and power domain adaptation, the same candidate values are used for joint operation of spatial domain and power domain adaptation Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | 1/ On the new additions for component 10 and 11: 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations Our understanding is the proposals are duplicated with Issue 8. It can mislead total Ncpu is represented by ‘legacy Ncpu’ + ‘new Ncpu (component 10/11). Therefore, we propose to adopt the proposals in Issue 8 rather than new additions of component 10/11. 2/ On the new addition of note: Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration We suggest the following modification. Note: Component 9 is used instead of the values in maxNumberPeriodicCSI-PerBWP-ForCSI-Report in FG2-35 when CSI report configuration includes at least one subconfigurationthe corresponding Rel-18 CSI report configuration 3/ On the new addition:Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b From offline discussion with Samsung, we understand this comes from different definition of components 4/5/6/7 than legacy definition. In other words, legacy definition considers all (P/SP/AP-CSI reporting) whereas the current components 4/5/6/7 are defined individually for P-, SP-, and AP- CSI reporting. One way can be to move out components 4/5/6/7 to (new) separate FG. This requires at least two (new) separate FGs (one for SD and the other for PD). In case joint operation of SD+PD is defined as separate FG, we need at least three (new) separate FGs (one for SD, another for PD, and the other for SD+PD). 5/ On the following note: Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain We think it would restrict UE capability unnecessarily and this note may make thing confused. If we introduce separate FG for joint operation of SD+PD, component 9 can be added for the new FGs and we do not need this note (our preference). |
| Qualcomm | Same comments as the ones in Issue 1. |
| Samsung | Same comments as the ones in Issue 1. |
| LG Electronics | Same comments as the ones in Issue 1. |
| Ericsson1 | Same comments as for Issue 1. |
| MediaTek1 | Same comments as the ones in Issue 1. |

# Issue 5: FG 42-2a

After review of contributions submitted to RAN1 #116 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 42. Netw_Energy_NR | 42-2a | Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting | Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for semi-persistent CSI reporting 1. The max number of sub-configurations Lmax in one CSI report configuration 2. Report of N CSI sub-report(s) included in one SP-CSI report where each CSI sub-report corresponds to one sub-configuration. 3. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of simultaneous NZP-CSI-RS resources per CC 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Support of single-panel type 1 codebook 8. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 9. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations 10. Supported total number of semi-persistent CSI reporting settings without sub-configurations plus the total number of sub-configurations across CSI report settings with sub-configurations per BWP | FFS | Yes |  | UE does not support power domain adaptation for semi-persistent CSI reporting | Per band | No | No | N/A | Component 1 candidate values: FFS {2,3,4,5,6,7,8} Component 2 candidate values: FFS {2,3,4} Component 3 candidate values: {1, 2, 3 … 32} Component 4 candidate values: {8, 16, 24, … 128 } Component 5 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 6 candidate values: {8, 16, 24, …, 248, 256} Component 8 candidate values: {1, 2, 3, …, 8} Component 9 candidate values: {5, 6, …, 32} Component 10 candidate values: {1, 2, 3, 4} Note: Maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH Note: Maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH Note: Components 5 and 6 are signaled per BC Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | 1/ On the new additions for component 10 and 11: 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations Our understanding is the proposals are duplicated with Issue 8. It can mislead total Ncpu is represented by ‘legacy Ncpu’ + ‘new Ncpu (component 10/11). Therefore, we propose to adopt the proposals in Issue 8 rather than new additions of component 10/11. 2/ On the new addition of note: Note: Component 8 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration We suggest the following modification. Note: Component 8 is used instead of the values in maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report in FG2-35 when CSI report configuration includes at least one subconfigurationthe corresponding Rel-18 CSI report configuration 3/ On the new addition:Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b From offline discussion with Samsung, we understand this comes from different definition of components 4/5/6/7 than legacy definition. In other words, legacy definition considers all (P/SP/AP-CSI reporting) whereas the current components 4/5/6/7 are defined individually for P-, SP-, and AP- CSI reporting. One way can be to move out components 4/5/6/7 to (new) separate FG. This requires at least two (new) separate FGs (one for SD and the other for PD). In case joint operation of SD+PD is defined as separate FG, we need at least three (new) separate FGs (one for SD, another for PD, and the other for SD+PD). |
| Qualcomm | Same comments as the ones in Issue 1. |
| Samsung | Same comments as the ones in Issue 1. |
| LG Electronics | Same comments as the ones in Issue 1. |
| Huawei, HiSilicon | Similarly, Component 2 candidate values: FFS {1, 2,3,4} |
| Ericsson1 | Same comments as for Issue 1, and For component 3 candidate value, we do not support defining candidate values of {2,3,4}. It should simply reflect 1 <=N<=Lmax. |

# Issue 6: FG 42-2b

After review of contributions submitted to RAN1 #116 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 42. Netw_Energy_NR | 42-2b | Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting | 1. Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for aperiodic CSI reporting 2. The max number of sub-configurations Lmax in one CSI report configuration 3. Report of N CSI sub-report(s) included in one CSI report where each CSI sub-report corresponds to one sub-configuration 4. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of simultaneous NZP-CSI-RS resources per CC 5. In case a CSI report configuration(s) containing sub-configuration(s) is configured for a carrier, for that carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 6. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. In case a CSI report configuration(s) containing sub-configuration(s) is configured for at least one carrier, supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Support of single-panel type 1 codebook 9. Supported total number of aperiodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across aperiodic CSI report settings with sub-configurations per BWP 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations | FFS | Yes |  | UE does not support power domain adaptation for aperiodic CSI reporting | Per band | No | No | N/A | Component 2 candidate values: {2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4} FFS Component 4 candidate values: {1, 2, 3 … 32} Component 5 candidate values: {8, 16, 24, … 128 } Component 6 candidate values: {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} Component 7 candidate values: {8, 16, 24, …, 248, 256} Component 9 candidate values: {2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12} Component 10 candidate values: {1, 2, 3, …, 8} Component 11 candidate values: {5, 6, …, 32} Note: Components 6 and 7 are signaled per BC FFS: different values for joint operation Note: If a UE supports both spatial domain and power domain adaptation, the same candidate values are used for joint operation of spatial domain and power domain adaptation Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration Note: For the purpose of component 4, 5, 6 and 7 calculation, NZP-CSI-RS resource and CSI-RS ports are counted for legacy reporting settings, Rel-18 reporting settings across all reporting types and all types of adaptation similar as component 9 of FG42-1, 42-1b, 42-2, 42-2b Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b Note: For any slot, the CSI-RS resource/port counting in components 4-7 is also accounted towards the following components in FG 2-33, respectively: - Supported maximum number of simultaneous NZP-CSI-RS resources per CC - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC - Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | 1/ On the new additions for component 10 and 11: 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations Our understanding is the proposals are duplicated with Issue 8. It can mislead total Ncpu is represented by ‘legacy Ncpu’ + ‘new Ncpu (component 10/11). Therefore, we propose to adopt the proposals in Issue 8 rather than new additions of component 10/11. 2/ On the new addition of note: Note: Component 9 is used instead of the values in FG2-35 when CSI report configuration includes the corresponding Rel-18 CSI report configuration We suggest the following modification. Note: Component 9 is used instead of the values in maxNumberAperiodicCSI-PerBWP-ForCSI-Report or maxNumberAperiodicCSI-PerBWP-ForCSI-Report-r16 in FG2-35 when CSI report configuration includes at least one subconfigurationthe corresponding Rel-18 CSI report configuration 3/ On the new addition:Note: The UE shall declare the same value for maximum number of simultaneous NZP-CSI-RS resources and maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC or in active BWPs across all CCs in FG42-1, 42-1a, 42-1b, 42-2, 42-2a, 42-2b From offline discussion with Samsung, we understand this comes from different definition of components 4/5/6/7 than legacy definition. In other words, legacy definition considers all (P/SP/AP-CSI reporting) whereas the current components 4/5/6/7 are defined individually for P-, SP-, and AP- CSI reporting. One way can be to move out components 4/5/6/7 to (new) separate FG. This requires at least two (new) separate FGs (one for SD and the other for PD). In case joint operation of SD+PD is defined as separate FG, we need at least three (new) separate FGs (one for SD, another for PD, and the other for SD+PD). 5/ On the following note: Note: For joint spatial and power domain adaptation operation, the UE reports Component 9 for either spatial domain adaptation or power domain adaptation. The reported value is used for the supported total number of periodic CSI reporting settings without sub-configurations plus the total number of sub-configurations across periodic CSI report settings with sub-configurations per BWP for joint spatial and power domain We think it would restrict UE capability unnecessarily and this note may make thing confused. If we introduce separate FG for joint operation of SD+PD, component 9 can be added for the new FGs and we do not need this note (our preference). |
| Qualcomm | Same comments as the ones in Issue 1. |
| Samsung | Same comments as the ones in Issue 1. |
| LG Electronics | Same comments as the ones in Issue 1. |
| Ericsson1 | Same comments as for Issue 1, and For component 3 candidate value, we do not support defining candidate values of {1,2,3,4}. It should simply reflect 1 <=N<=Lmax. We do not support candidate value of N=1, which means only UE supports reporting of a single hypothesis only. |
| MediaTek1 | Same comments as the ones in Issue 1. |

# Issue 7: Mixed Codebook

After review of contributions submitted to RAN1 #116 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following new row/FG

| 42-x | Mixed codebook combination for spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration | 1. Indicate the support of active CSI-RS resources and ports for mixed codebook types in any slot. The following codebook combination is a possible mixed codebook combination {Codebook1, Codebook2, Codebook3} for UE supporting CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration. 2. Indicate the list of supported CSI-RS resources across all bands in a band combination by referring to codebookVariantsList for the mixed codebook types. The following parameters are included in codebookVariantsList for each code book type: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource across all bands within a band combination; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs within a band combination, simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs within a band combination, simultaneously. 3. The UE supporting this feature shall indicate the support of CodebookComboParametersAddition-r16 and the support of multi-panel operation. | 42-1 or 42-1a or 42-1b | Yes |  | UE does not support the mixed codebook combination {Type 1 Single Panel, Type 1 Multi Panel, Null} for spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s), each containing one port subset configuration | Per band for component 1 Per BC for component 2 | No | No | N/A | Component 1 candidate values: {Type 1 Single Panel, Type 1 Multi Panel, Null}} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Ok |

# Issue 8: Simultaneous CSI Reports

After review of contributions submitted to RAN1 #116 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following new row/FG

| 42. Netw_Energy_NR | 42-y1 | simultaneousCSI-ReportsPerCC-r18 | 1. Indicates the number of CSI report(s) for which the UE can measure and process reference signals simultaneously in a CC of the band for which this capability is provided. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsPerCC-r18 includes the beam report, and CSI report without sub-configurations plus sub-configurations across CSI reports with sub-configurations |  | Yes |  | UE does not support spatial or power domain adaptation | Per band | No | No | N/A | Component 1 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. Netw_Energy_NR | 42-y2 | simultaneousCSI-ReportsAllCC-r18 | 1. Indicates whether the UE supports CSI report framework and the number of CSI report(s) which the UE can simultaneously process across all CCs, and across MCG and SCG in case of NR-DC. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsAllCC-r18 includes the beam report, and CSI report without sub-configurations plus sub-configurations across CSI reports with sub-configurations. This parameter may further limit simultaneousCSI-ReportsPerCC-r18 in MIMO-ParametersPerBand and Phy-ParametersFRX-Diff for each band in a given band combination. |  | Yes |  | UE does not support spatial or power domain adaptation | Per BC | No | No | N/A | Component 1 candidate values: {5, 6, 7, ..., 32} | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple2 | Support |
| Samsung | OK |
| LG Electronics | OK |
| Ericsson1 | We are open to this, but would like understand what the under-reporting issue. It seems this allows UE to under-report CPU for NES. |
| Apple2 | There can be under-reporting issue without these capabilities, but there is no under-reporting issue with these capabilities. UE cannot report the new capability parameter value smaller than old capability parameter. For instance, if UE reports simultaneousCSI-ReportsPerCC=4 (legacy parameter) and simultaneousCSI-ReportsPerCC-r18=3, - From simultaneousCSI-ReportsPerCC=4, UE says I support 4 CSI reports for which the UE can measure and process reference signals simultaneously in a CC. - From simultaneousCSI-ReportsPerCC=3, UE says I support 3 CSI reports (without sub-configuration) for which the UE can measure and process reference signals simultaneously in a CC. In this case, UE does not know which value between 4 and 3 is used for Ncpu in case of CSI report without sub-configuration since UE doesn’t know in which release the network operates based. Therefore, we think it doesn’t work if UE reports smaller value of new signaling than that of old signaling. |
| MediaTek1 | Question for clarification: Whether there is duplication with the following new components 10 and 11? 10. UE can process Y CSI report(s) simultaneously in a CC. CSI reports corresponds to sub-configurations 11. UE can process X CSI report(s) simultaneously across all CCs. CSI reports corresponds to sub-configurations |

# Conclusion

Agreements reached during RAN1 #116 as part of this agenda item are summarized in [17].

# References

- R1-2312705, Corrected RAN1 UE features list for Rel-18 NR after RAN1#115, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2400086, Remaining issues of UE Features for Other Topics B (NES, MobEnh, NCR, IoT-NTN, NR-NTN, BWP_wor), Nokia/Nokia Shanghai Bell

- R1-2400149, UE features for other Rel-18 WIs (Topics B), Huawei/HiSilicon

- R1-2400231, Discussion on Rel-18 UE features for NES, Mobility Enh and NR NTN, vivo

- R1-2400354, Discussion on UE features for Rel-18 Topic-B, ZTE

- R1-2400417, UE features for other Rel-18 work items (Topics B), CATT

- R1-2400717, UE features for other Rel-18 work items (Topics B), Samsung

- R1-2401000, On UE features for Rel-18 work items in Topic B Apple

- R1-2401106, Discussion on UE features for Mobility, NES, BWP without restriction, and NR-NTN, NTT DOCOMO, INC.

- R1-2401142, Rel-18 UE features topics set B, Ericsson

- R1-2401170, UE features for NR NTN, Sharp

- R1-2401222, Discussion on UE features for other Rel-18 work items (Topics B), ETRI

- R1-2401252, Discussion on BWP Without Restriction updates, Vodafone

- R1-2401325, Discussion on UE features for NES, LG Electronics

- R1-2401429, UE features for other Rel-18 work items (Topics B), Qualcomm Incorporated

- R1-2401314, UE features for other Rel-18 work items (Topics B), MediaTek Inc.

- R1-2401620, Session Notes of AI 8.12.6, Ad-Hoc Chair (AT&T)