3GPP TSG RAN WG1 #116						                       R1-2401367

Athens, Greece, 26 February – 01 March, 2024

Agenda Item:	9.1.3.1

Source:	AT&T

Title:	Discussion on AI/ML for CSI prediction

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP had agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. One of the identified use cases include:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

Two sub-use cases have been identified for CSI feedback enhancements:

- CSI compression with two-sided model

- Time domain CSI prediction using UE sided model

In RAN#102 as part of Rel-19 Work Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [10], it was agreed to continue to further study CSI prediction to improve performance gains by addressing aspects that required further study as captured in the TR 38.843.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24):

- CSI feedback enhancement [RAN1]:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

  - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

In this contribution, we will discuss the specification aspects of time domain CSI prediction to improve performance gain.

# Potential specification impact for time domain CSI prediction using UE sided model

For CSI prediction, specification impact was discussed in RAN1#114[9], and the following observations and agreements were made:

Observation

In CSI prediction using UE sided model use case, at least the following aspects have been proposed by companies on data collection, including:

- Signaling and procedures for the data collection

  - data collection indicated by NW

  - Requested from UE for data collection

- CSI-RS configuration

- Assistance information for categorizing the data, if needed

  - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side.

Agreement

For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM:

- Type 1:

  - UE calculate the performance metric(s)

  - UE reports performance monitoring output that facilitates functionality fallback decision at the network

    - Performance monitoring output details can be further defined

    - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed).

  - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

- Type 2:

  - UE reports predicted CSI and/or the corresponding ground truth

  - NW calculates the performance metrics.

  - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

- Type 3:

  - UE calculate the performance metric(s)

  - UE report performance metric(s) to the NW

  - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

- Functionality selection/activation/ deactivation/switching what is defined for other UE side use cases can be reused, if applicable.

- Configuration and procedure for performance monitoring

  - CSI-RS configuration for performance monitoring

- Performance metric including at least intermediate KPI (e.g., NMSE or SGCS)

- UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report.

- Note: down selection is not precluded.

- Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

In RAN1#114-bis[9] the following agreement was made combining the functionality based LCM and model ID based LCM.

Agreement

Model-ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

In previous meetings we significantly reduced the scope of the CSI prediction by only studying a limited specification impact. As we have been provided with sufficient time to continue the study, we need to study the full specification impact to achieve the maximum performance gains.

Proposal 1: In CSI prediction using UE sided model use case, study the necessity, feasibility, potential specification impact for at least the following aspects.

- Data collection

- Performance monitoring for AI/ML model activation/deactivation/switching/fallback

- CSI configuration and report

- LCM

  - Aspects specified as condition for functionality based LCM

  - Aspects that are considered as assistance information.

In the current CSI prediction, the main aim is to improve the CSI estimate for the UE in fast moving vehicles with a highly varying channel. Generally, CSI prediction was not considered for the stationary or slow-moving UE as their channel is relatively stable so there is a very low gain by predicting the missing CSI values. However, CSI prediction can still be used to monitor the channel of these stationary or slow-moving UE and reduce the CSI reporting frequency. The CSI prediction model can predict and fill out the missing values between the CSI samples. By reducing the CSI reporting frequency, we will be able to reduce the reporting overhead and we can achieve significantly better performance results even for stationary and slow speed UE.

Proposal 2: Study CSI prediction for slow moving or stationary UE, where CSI prediction both predicts the CSI values and selects the CSI reporting frequency for the UE.

One of the main issues raised regarding the evaluation of CSI prediction in the previous meetings was regarding the limited number of generalization studies and limited results using spatial consistency. Therefore, we propose the following to have more simulation studies for CSI predictions.

Proposal 3: For the evaluation of the AI/ML based CSI prediction sub use cases, study the performance gains for the following aspects.

- Channel model for simulations

  - Spatial consistency

  - More deterministic channel model, e.g., ray tracing

- Generalization for the following aspects

  - Scalability over different network configurations for a given site

  - Scalability over different user distributions for different deployment scenarios

  - Scalability over mixed speed distributions

Further details are needed regarding the performance metrics used for performance monitoring as well as reporting model capability. These are inline with the specification impacts that were agreed to be included for CSI compression.

Proposal 4: For time domain CSI prediction using UE sided model, consider specification impact needed to support the co-existence and fallback between AI/ML model and legacy non-AI/ML based CSI feedback mode.

Proposal 5: For time domain CSI prediction using UE sided model, include at least the following options for performance monitoring metrics/methods:

- Intermediate KPIs as monitoring metrics (e.g., SGCS)

- Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK).

- Legacy CSI based monitoring: schemes using additional legacy CSI reporting.

- Other monitoring solutions, at least including the following option:

  - Input or Output data-based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection.

Proposal 6: Evaluate the performance monitoring accuracy for CSI prediction for different performance monitoring metrics.

Proposal 7: For the UE sided CSI prediction, include the following specification impacts for the following aspects.

- Reporting model capability of CSI prediction (processing time, max future predicted time step, etc)

- gNB and UE’s alignment on prediction related time domain configuration information

- CSI reporting (e.g., Batch CSI report for current and past CSI) and CSI-RS configurations

For the CSI prediction we need to select the different configuration that can be used in functionality or model ID based LCM. We need to discuss which aspects are to be included in functionality-based LCM and which in model ID based LCM and what are granularities for these configurations.

Proposal 8: For CSI prediction using UE sided model consider the following configurations and their granularity that will be signaled and the corresponding specification impact for the AI/ML model LCM.

- UE speed

- Frequency PRBs

- Prediction window

- Observation window

- Scenario (UMa etc.)

- Performance requirement/monitoring

- Other additional configurations

Finally, we need to have an agreement regarding additional conditions and how they can be used to assist the UE for the AI/ML model LCM purposes.

Proposal 9: For time domain CSI prediction using UE sided model, consider which aspects should be considered as additional conditions, and how to indicate them to assist the UE with AI/ML model LCM.

# Conclusion

In this contribution, we discussed a sub-use case on CSI feedback enhancements related to time domain CSI prediction using UE sided model. We made the following proposals.

Proposal 1: In CSI prediction using UE sided model use case, study the necessity, feasibility, potential specification impact for at least the following aspects.

- Data collection

- Performance monitoring for AI/ML model activation/deactivation/switching/fallback

- CSI configuration and report

- LCM

  - Aspects specified as condition for functionality based LCM

  - Aspects that are considered as assistance information.

Proposal 2: Study CSI prediction for slow moving or stationary UE, where CSI prediction both predicts the CSI values and selects the CSI reporting frequency for the UE.

Proposal 3: For the evaluation of the AI/ML based CSI prediction sub use cases, study the performance gains for the following aspects.

- Channel model for simulations

  - Spatial consistency

  - More deterministic channel model, e.g. ray tracing

- Generalization for the following aspects

  - Scalability over different network configurations for a given site

  - Scalability over different user distributions for different deployment scenarios

  - Scalability over mixed speed distributions

Proposal 4: For time domain CSI prediction using UE sided model, consider specification impact needed to support the co-existence and fallback between AI/ML model and legacy non-AI/ML based CSI feedback mode.

Proposal 5: For time domain CSI prediction using UE sided model, include at least the following options for performance monitoring metrics/methods:

- Intermediate KPIs as monitoring metrics (e.g., SGCS)

- Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK).

- Legacy CSI based monitoring: schemes using additional legacy CSI reporting.

- Other monitoring solutions, at least including the following option:

  - Input or Output data-based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection.

Proposal 6: Evaluate the performance monitoring accuracy for CSI prediction for different performance monitoring metrics.

Proposal 7: For the UE sided CSI prediction, include the following specification impacts for the following aspects.

- Reporting model capability of CSI prediction (processing time, max future predicted time step, etc)

- gNB and UE’s alignment on prediction related time domain configuration information

- CSI reporting (e.g., Batch CSI report for current and past CSI) and CSI-RS configurations

Proposal 8: For CSI prediction using UE sided model consider the following configurations and their granularity that will be signaled and the corresponding specification impact for the AI/ML model LCM.

- UE speed

- Frequency PRBs

- Prediction window

- Observation window

- Scenario (UMa etc.)

- Performance requirement/monitoring

- Other additional configurations

Proposal 9: For time domain CSI prediction using UE sided model, consider which aspects should be considered as additional conditions, and how to indicate them to assist the UE with AI/ML model LCM.

# References

[1] RP-213599 Study on AI/ML for Air-Interface, RAN#94e.

[2] RAN1#109-e Chair’s Notes, e-meeting May 9-20th, 2022

[3] RAN1#110 Chair’s Notes, August 22nd – 26th, 2022.

[4] RAN1#110bis-e Chair’s Notes, October 10th – 19th, 2022.

[5] RAN1#111 Chair’s Notes, November 14th – 18th, 2022.

[6] RAN1#112 Chair’s Notes, February 27th – Mar 3rd, 2023.

[7] RAN1#112bis-e, Chair’s notes, e-meeting, April 2023.

[8] RAN#100, meeting report, June 2023.

[9] Chair’s notes RAN1#114bis, Toulouse, France, October 2023.

[10] RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface