3GPP TSG-RAN WG1 Meeting #116	R1-2401381

Athens, Greece, 26 February - 1 March, 2024

Title:	Draft reply LS on MAC agreement for SL positioning

Response to:	R1-2400008 (R2-2313848) LS on MAC agreements for SL positioning

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	Huawei, HiSilicon [to be RAN1]

To:	RAN2

Cc:	RAN3, RAN4

Contact Person:

#### Name:	Jinhuan Xia

Tel. Number:

E-mail Address:	jinhuan.xia@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 thanks RAN2 for providing the progress on the MAC specification for SL positioning and the questions. RAN1 discussed the questions and provide the following responses.

Q1: On the maximum number of parallel SL-PRS transmission

  - What is the maximum total number of parallel SL-PRS transmissions on SL-PRS shared/dedicated resource pool?

  - What is the maximum number of parallel SL-PRS transmission supported on a SL-PRS shared resource pool and SL-PRS dedicated resource pool, respectively?

A1: RAN1 understands the “parallel SL-PRS transmission” refers to the concept of the HARQ process for communication for the same destination.

  - With regards to the maximum total number parallel SL-PRS transmission on SL-PRS shared/dedicated resource pool and the maximum number of parallel SL-PRS transmission supported on a SL-PRS shared resource pool and SL-PRS dedicated resource pool, respectively, RAN1 concluded that parallel SL-PRS transmission is not supported, since there is no HARQ process ID associated with SL-PRS transmission.

Q2: When SL-PRS is transmitted on a SL-PRS shared resource pool where PSFCH is configured, if the associated PSSCH transmission is positively acknowledged, should the UE continue to perform SL-PRS retransmission?

A2: If the associated PSSCH transmission is positively acknowledged, UE may still continue to perform SL-PRS retransmission on the shared SL-PRS resource pool when needed if the number of retransmissions does not exceed the maximum number of retransmissions provided by congestion control.

2. Actions:

To RAN2

ACTION: RAN1 respectfully requests RAN2 to take above response into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #116-bis	15 Apr – 19 Apr 2024		Changsha, CN

TSG-RAN WG1 Meeting #117	20 May – 24 May 2024		Fukuoka, JP