3GPP TSG RAN WG1 #116	R1-2401409

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	7.1

Source: 	Qualcomm Incorporated

Title: 	Issues on SRS carrier switching

Document for:	Discussion and Decision

# Background

In RAN1#116, RAN1 made the following agreements regarding SRS carrier switching:

Conclusion

Minimum support for aperiodic SRS carrier switching triggered by DCI 1_1/1_2 requires the capabilities

- srs-CarrierSwitch (feature group 2-56) and its prerequisite(s) as specified in 38.306.

- Basic DL NR-NR CA operation, supportedBandCombinationList  (FG 6-5), e.g. since 38.214 section 6.2.1.3 requires switching among carriers of serving cells.

- srs-TxSwitch (FG 2-55), as inferred by the similar requirement for format 2_3 in 38.214 section 6.1.2.3.

If cross carrier scheduling is to be used for SRS carrier switching triggered by DCI 1_1/1_2, the additional capability is needed:

- crossCarrierScheduling-SameSCS (FG 6-10) if triggering cell and target cell for A-SRS CS are not the same.

Agreement

Alt 1 from RAN#114, “RAN1 concludes that DCI 1_1 and 1_2 triggering are supported, subject to existing UE capabilities, for aperiodic SRS carrier switching in Rel-15” is agreed as updated, and no new UE capability is defined for the support of DCI 1_1 and 1_2 triggering for aperiodic SRS carrier switching in the case where SRS is triggered by a single DCI.

- FG 22-8, if indicated, also applies to SRS carrier switching (according to current specifications)

[some parts omitted]

R1-2312673	Correction for SRS carrier switching for DCI formats 1_1 and 1_2	Ericsson, T-Mobile USA, Verizon, Samsung

(Rel-17, 38.214, CR0476, Cat F) is agreed.

Based on the discussion in previous meetings, there are two pending issues:

- How to handle conflicting triggering between two DCIs.

- Applicability of “available slots”.

In this contribution, we present our view on these two issues.

# Conflicting triggering between two DCIs

During RAN1#114b, the following issue was brought up: If multiple CCs are triggered from 2_3, the specification has a clear text describing how the prioritization / ordering is done, while there are no rules defined with respect to collision of SRS CS between multiple 1_1/1_2 or between 1_1/1_2 and 2_3.

For reference, the explicit comment made on RAN1#114b on this issue is as follows [1]:

In case of more than one PUSCH-less carriers configured with SRS carrier switching, it is possible that two DL DCIs trigger overlapped SRS carrier switching. There is no specified UE behaviour on carrier priority for this case yet, which is different from the trigger of DCI 2_3. Additionally, it seems unclear whether carrier priority is needed when a trigger of DCI 2_3 and a trigger of DL DCI overlap. Therefore, we suggest to narrow down the case for no new capability as “in the case where DCI 2_3 is not configured and/or only one uplink carrier is configured with SRS carrier switching.”

While we acknowledge the issue of collision of triggering between multiple DCIs, in our view it would be natural for RAN1 to conclude that the case of conflicting SRS CS is an error case and, therefore, the UE behavior would not be defined in the problematic cases defined above.

Therefore, we make the following proposal:

Proposal 1: RAN1 concludes that a UE is not expected to receive multiple DCIs triggering colliding SRS CS with the same source CC received in different DCIs.

# Available slot operation

In RAN1#114 the following proposal was brought forward, which was further discussed in RAN1#115 without conclusion:

Proposal#4 (RAN1#114bis) Further discuss in RAN1#115 if available slot operation can be applied to aperiodic SRS carrier switching triggered by DCI 1_1/1_2.

The main issue of “available slot operation” resides in its use with DCI format 2_3, since there is no DCI field that would allow to indicate the actual slot. However, for the case where availableSlotOffsetList has a single entry, no DCI indication is needed and therefore DCI format 2_3 would support this operation. In line with this reasoning, we make the following proposed conclusion:

Proposal 2:  Conclusion: SRS triggering with DCI format 2_3 supports available slot operation for the case where availableSlotOffsetList has a single entry.

- No CR is needed.

# Summary

In this contribution we discussed remaining issues on SRS CS. We made the following proposals:

Proposal 1: RAN1 concludes that a UE is not expected to receive multiple DCIs triggering colliding SRS CS with the same source CC received in different DCIs.

Proposal 2:  Conclusion: SRS triggering with DCI format 2_3 supports available slot operation for the case where availableSlotOffsetList has a single entry.

- No CR is needed.

# References

[1] R1-2310680 Summary#3 of NR Rel-15/16 maintenance discussion for aperiodic triggering support for SRS carrier switching, Moderator (Ericsson), RAN WG1 Meeting #114bis