3GPP TSG RAN WG1 #116			R1-2401422

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	8.10

Source: 	Qualcomm Incorporated

Title: 	Maintenance on NR NTN enhancements

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss several remaining issues on NR NTN enhancements.

# UE RX-TX measurements

In RAN1#115, a working assumption was made to ensure consistent determination of the beginning of a subframe when the associated PRS and SRS are not transmitted in the beginning of a subframe. We propose to confirm the working assumption.

Proposal 1: Confirm the following working assumption:

Working assumption

For UE RX-TX measurements in NTN, the time of the beginning of a subframe is determined by assuming the time durations of the OFDM symbols at the receiver are the same as defined in 38.211.

To reflect the above proposal, we propose the following text proposals on TS38.215.

Proposal 2: Adopt TP1 for TS 38.215

### ------------------------------------------------------- Start of TP for TS 38.215  (TP1) ------------------------------------------------------------

### 5.1.30	UE Rx – Tx time difference

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS or CSI-RS for tracking resources, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. The time of the beginning of a subframe is determined by assuming the time durations of the OFDM symbols at the receiver are the same as defined in 38.211. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

*** Unchanged parts are omitted ***

### 5.2.3	gNB Rx – Tx time difference

| Definition | The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources can be used to determine the start of one subframe containing SRS. The time of the beginning of a subframe is determined by assuming the time durations of the OFDM symbols at the receiver are the same as defined in 38.211. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector. In NTN, the gNB Rx – Tx time difference at the uplink time synchronization reference point [5] is reported. |
| --- | --- |

### ------------------------------------------------------------ End of TP for TS 38.215 ------------------------------------------------------------

# Application time of cell DRX in NTN

Cell DTX and cell DRX are defined in Rel-18. In addition, a PDCCH in DCI format 2_9 can be used to activate and deactivate cell DTX and/or cell DRX. A change in activation or deactivation indicated in slot m is applied in slot m+d, where d is the number of slots in 3 ms. As shown in Figure 1, the above application time of the cell DRX activion/deaction can have a problem in NR NTN due to large timing advance. In particular, a UE’s UL transmission for slot m+4 with 15 KHz SCS may occur before slot m on which the UE receives the deactivation command.

Figure 1.  Application time of a cell DRX activation/deactivation indicated in slot m.

To solve the above problem, Koffset can be used to delay the application time of cell DRX. To ensure consistent application time for all the UEs address by a DCI format 2_9, cell specific Koffset is preferred.

Proposal 3: Adopt TP2 for TS 38.213

### --------------------------------------------------------- Start of TP for TS 38.213 (TP2) ------------------------------------------------------------

## 11.5	Adaptation of cell operation

*** Unchanged parts are omitted ***

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in activation or deactivation of a current cell DTX operation or cell DRX operation for a second serving cell, the UE operates on the second serving cell according to the indicated cell DTX operation or cell DRX operation starting from a slot on the active DL BWP or on the active UL BWP of the second serving cell, respectively, that is not before the beginning of the slot m+d on the active DL BWP of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1. When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in activation or deactivation of a current cell DRX operation for a second serving cell, the UE operates on the second serving cell according to the indicated cell DRX operation starting from a slot on the active UL BWP of the second serving cell that is not before the beginning of the slot m+d+Kcell,offset∙2μ on the active DL BWP of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1,  Kcell,offset is the cellSpecificKoffset if provided for the second serving cell or 0 otherwise, m is the SCS configuration of the active DL BWP of the first serving  cell, and for a cell that is provided with higher-layer ephemeris parameters the starting time of a slot is the starting time of the slot at the uplink time synchronization reference point of the cell.

*** Unchanged parts are omitted ***

### ------------------------------------------------------ End of TP for TS 38.213 ------------------------------------------------------------

# Summary

In this contribution, we have discussed two remaining issues for NR NTN enhancements and proposed corresponding TPs. We made the following proposals:

Proposal 1: Confirm the following working assumption:

Working assumption

For UE RX-TX measurements in NTN, the time of the beginning of a subframe is determined by assuming the time durations of the OFDM symbols at the receiver are the same as defined in 38.211.

Proposal 2: Adopt TP1 for TS 38.215

Proposal 3: Adopt TP2 for TS 38.213