3GPP TSG RAN WG1 #116			R1-2401427

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	8.12.4

Source: 	Qualcomm

Title: 	UE features for MC enhancements

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our views on UE features for MC enhancements.

- UE features for Multi-cell PDSCH/PUSCH scheduling with a DCI

  - FG49-1

Regarding the note “[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]”, we are OK to confirm the note. This must anyway be a common understanding.

Proposal 1:

- In FG49-1, confirm the note “[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]”

  - FG49-2

Regarding the note “[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]”, we are OK to confirm the note. This must anyway be a common understanding.

Regarding whether to keep “[Up to]” for component 9), we understand the intention and discussion. Since both descriptions exist in the past, we think either is OK. Given the fact that the UE capability is to process up to two DCI format 0_3 and up to two unicast UL DCI formats, it would be more sense to keep the “[Up to]”.

Proposal 2:

- In FG49-2,

  - Confirm the note “[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]”

  - Confirm the “[Up to]” in component 9

  - FG49-2b

Regarding whether to keep “[Up to]” for component 9), we understand the intention and discussion. Since both descriptions exist in the past, we think either is OK. Given the fact that the UE capability is to process up to two DCI format 0_3 and up to two unicast UL DCI formats, it would be more sense to keep the “[Up to]”.

Proposal 3:

- In FG49-2b,

  - Confirm the “[Up to]” in component 9

  - FG49-3x/3y

For legacy cross-carrier scheduling from lower SCS to higher SCS, support of multiple unicast DCIs to process per slot of scheduling cell has been defined in Rel-16 (FG18-5c, 18-5d). FG49-3x/3y should be corresponding FGs for DCI format 0_3/1_3. In FG18-5c/5d, the support is reported per SCS combination between the scheduling cell and the scheduled cell. Same prinriple should apply to DCI FG49-3x/3y.

Proposal 4:

- FG49-3x indicates Advanced UE capability for larger number of unicast DL DCI for FG49-1b

  - Component: The number of DCI format 1_3 to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3 is X per slot of scheduling cell for the set of cells

    - For lower to higher SCS,

      - Candidate values of X

        - {1, 2, 4} for (12, 120), (15, 60), (30, 120)

        - {2} for (15, 30), (30,60), (60, 120)

  - The value of X is reported per combination of {scheduling cell SCS, scheduled cell(s) SCS} of FG49-1b

- FG49-3y indicates Advanced UE capability for larger number of unicast UL DCI for FG49-2b

  - Component: The number of DCI format 0_3 to process for a set of cells configured for multi-cell PUSCH scheduling by a DCI format 0_3 is X per slot of scheduling cell for the set of cells

    - For lower to higher SCS,

      - Candidate values of X

        - {1, 2, 4} for (12, 120), (15, 60), (30, 120)

        - {2} for (15, 30), (30,60), (60, 120)

  - The value of X is reported per combination of {scheduling cell SCS, scheduled cell(s) SCS} of FG49-2b

  - Other features

In the following, we describe some additional optional features necessary for multi-cell scheduling by a DCI format 1_3 or DCI format 0_3. For those listed below, there is no corresponding FG for DCI format 1_3 or 0_3 – therefore, new FGs are anyway necessary.

    - Priority indicator

RAN1 agreed to support priority indicator for DCI format 0_3 and 1_3. There are UE features for priority indicator in legacy DCI formats as FG11-4, 11-4a, 11-4b, 12-1, and 12-1a. However, these legacy FGs are not applicable to DCI format 1_3 and DCI format 0_3.

- FG11-4/11-4a are for the case where only DCI format 0_1/1_1 or only DCI format 0_2/1_2 is configured.

- FG11-4b indicates support of operation with mixed DCI formats (1_1 and 1_2) with priority indication field.

- FG12-1 is for the case where dynamic indication of priority level of dynamic PUSCH with a single DCI format. Although there is no specific description of which DCI format this applies, it is clear from FG12-1a that this single DCI format is either DCI format 0_1 or DCI format 0_2.

- FG12-1a indicates support of operation with mixed DCI formats (0_1 and 0_2) with priority indication field.

We need new FGs to accommodate DCI format 1_3 and DCI format 0_3 with priority indication field.

For mixed DCI formats with priority indication field with DCI format 1_3 or 0_3, we think separate FGs are necessary. We do not think it is necessary to support three DCI formats with priority indication fields (1_3 + 1_1 + 1_2, or 0_3 + 0_1 + 0_2). The mixed DCI formats with priority indication field for Rel-18 multi-cell scheduling can be 1_3 + (1_1 or 1_2), and 0_3 + (0_1 or 0_2).

Proposal 5:

- UE features for DL priority indicator in a DCI format 1_3 should be introduced:

  - FG49-6: Two HARQ-ACK codebooks with different priorities with up to one sub-slot based HARQ-ACK codebook enabled for DCI format 1_3 (similar to FG11-4)

  - FG49-7: Two HARQ-ACK codebooks with different priorities with two sub-slot based HARQ-ACK codebooks enabled for DCI format 1_3 (similar to FG11-4a)

  - FG49-8: Mixed DCI formats including DCI format 1_3 for DL priority indication in a BWP

    - Support of priority indication field in DCI formats (1_1 or 1_2) and 1_3 (similar to FG11-4b)

Proposal 6:

- UE features for UL priority indicator in a DCI format 0_3 should be introduced:

  - FG49-9: UL priority indication in DCI with DCI format 0_3

    - Support of priority indicator field configured in DCI format 0_3 (similar to FG12-1)

  - FG49-10: Mixed DCI formats including DCI format 0_3 for UL priority indication

    - Support priority indication field in DCI formats (0_1 or 0_2) and 0_3 (similar to FG12-1a)

    - PHY priority handling for one-shot HARQ-ACK by DCI 1_3

PHY priority handling (same as FG25-5) for one-shot HARQ-ACK feedback triggered by DCI format 1_3 should be enabled by another FG.

Proposal 7:

- UE feature for PHY priority handling for one-shot HARQ-ACK feedback by DCI 1_3 should be introduced:

  - FG49-11: PHY priority handling for one-shot HARQ-ACK feedback by DCI 1_3:

    - Support transmission of Type-3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI format 1_3 (similar to FG25-5)

    - HARQ-ACK re-transmission

FG25-7 specifies that HARQ-ACK re-transmission can be triggered by DCI format 1_1 and DCI format 1_2. To enable this by using DCI format 1_3, corresponding new FG is necessary.

Proposal 8:

- FG49-12: HARQ-ACK re-transmission triggered by DCI format 1_3 (similar to FG25-7)

    - SCell dormancy indication

FG18-5 indicates support of SCell dormancy indication by DCI format 0_1/1_1. There must be a corresponding FG for SCell dormancy indication by DCI format 0_3/1_3. Since now we are willing to enable multi-cell scheduling for DL and UL separately, it is preferred to have the feature for DL and UL separately.

Proposal 9:

- UE features for SCell dormancy indication within active time by DCI format 1_3 and DCI format 0_3 should be introduced

  - FG49-13: SCell dormancy indication within active time by DCI 1_3 (similar to FG18-5)

  - FG49-14: SCell dormancy indication within active time by DCI 0_3 (similar to FG18-5)

    - Cross slot scheduling

FG19-2 indicates support of cross slot scheduling with minimum scheduling offset K0/K2 by DCI format 0_1/1_1. It is necessary to have another FG indicating support of dynamic indication of applicable minimum scheduling restriction by DCI format 0_3 and 1_3.

Proposal 10:

- UE features for cross-slot scheduling by DCI format 1_3 and DCI format 0_3 should be introduced

  - FG49-15: Dynamic indication of applicable minimum scheduling restriction by DCI format 1_3 (similar to FG19-2)

  - FG49-16: Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3 (similar to FG19-2)

    - Unified-TCI indication by a DCI

FG23-1-1b and FG23-10-1b specify the UE capabilieis for unified TCI with joint and separate DL/UL TCI updates by a DCI format. According to their description, the FGs are limited to DCI format 1_1. In order to support the features by DCI format 1_3, it is necessary to introduce the FGs for joint DL/UL TCI update and for separate DL/UL TCI update, respectively.

Proposal 11:

- UE features for Unified-TCI indication by DCI format 1_3 should be introduced

  - FG49-17: Unified TCI with joint DL/UL TCI update with DCI-based TCI state indication for DCI format 1_3:

    - TCI state indication for update and activation

      - b) MAC-CE + DCI-based TCI state indication (use of DCI format 1_3 with DL assignment(s)),

      - c) MAC-CE + DCI-based TCI state indication (use of DCI format 1_3 without DL assignment)

    - The min beam application time in Y symbols per SCS

    - The max number of MAC-CE activated joint TCI states per CC in a band

  - FG49-18: Unified TCI with separate DL/UL TCI update with DCI-based TCI state indication for DCI format 1_3:

    - TCI state indication for update and activation

      - b) MAC-CE + DCI-based TCI state indication (use of DCI format 1_3 with DL assignment(s)),

      - c) MAC-CE + DCI-based TCI state indication (use of DCI format 1_3 without DL assignment)

    - The min beam application time in Y symbols per SCS

    - The max number of MAC-CE activated DL TCI states per CC in a band

    - The max number of MAC-CE activated UL TCI states per CC in a band

    - PDCCH monitoring for 480kHz/960kHz in FR2-2

Rel-17 NR_ext_to_71GHz supported SCS 480kHz/960kHz. However, the basic FG for FR2-2 (FG24-1) was only for 120kHz. Support for PDCCH monitoring for 480kHz/960kHz were optional FGs, 24-4, 24-4f, 24-5, 24-5f. For these SCSs, PDCCH monitoring is multi-slot based.

FGs49-1/1a/1b/2/2a/2b are for the case where the UE is able to monitor PDCCH in every slots. To discuss SCS 480kHz/960kHz, we would need additional FGs. Details of these additional FGs can be discussed once the FGs 49-1, 49-1a, 49-1b, 49-2, 49-2a, and 49-2b are clearer.

Proposal 12:

- Clarify that FR2-2 in FG49-1, 49-1a, 49-1b, 49-2, 49-2a, and 49-2b are for SCS of up to 120kHz.

- Specify separate FGs for PDCCH monitoring with DCI format 1_3/0_3 for 480kHz and 960kHz.

  - Details of the separate FGs are to be discussed once FG49-1, 49-1a, 49-1b, 49-2, 49-2a, and 49-2b are stable.

- Proposed UE feature list for MC-Enh

  - UE features for Multi-cell PDSCH/PUSCH scheduling

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type: value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} 7) Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 49-1 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field 10) The number of unicast DL DCIs to process per slot of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3 - One DCI format 1_3 for the set of cells and, - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3 11) Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases - 1) Search space set configuration for DCI format 1_3 for the set of cells is provided only on the scheduling cell, or; - 2) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells - UE supporting FG 49-1 can additionally report whether the UE support following case - 3) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells 12) When multiple component 3 values are reported and if scheduling cell is not included in the set of cells, support multi-cell PDSCH scheduling by DCI format 1_3 from one carrier type, indicated in component 3, to another carrier type, indicated in component 3, for the following scheduling cases: - FR1 licensed TDD to FR1 unlicensed TDD - FR2-1 to FR2-2 - UE can additionally report the support of {FR1 licensed FDD from/to FR1 licensed TDD} Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | Per BC | N/A | N/A | N/A | Note: Support of CCS with DL DCI formats 1_1/1_2 is according to FG 6-10 | Optional with capability signaling |
| 49. NR_MC_enh | 49-1b | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} 7) Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 49-1b 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field 10) The number of unicast DL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3 - One DCI format 1_3 for the set of cells and, - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3 - For low-to-high SCS, N = 1. - For high-to-low SCS, N is based on pair of (scheduling CC SCS, scheduled CC SCS): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15) 11) Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases - 2) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | Per BC | N/A | N/A | N/A | Note: Support of CCS with DL DCI formats 1_1/1_2 is according to FG 18-5 | Optional with capability signaling |
| 49. NR_MC_enh | 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type:value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields 9) The number of unicast UL DCIs to process per slot of scheduling cell for a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3 - For FDD scheduling cell - Up to one DCI format 0_3 for the set of cells and, - Up to one unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells - For a cell in a set of cells, no more than one DCI scheduling PUSCH for the cell - For TDD scheduling cell - Up to two DCI format 0_3 for the set of cells and, - Up to two unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells - For a cell in a set of cells, no more than two DCI scheduling PUSCH for the cell 10) Monitoring SS set(s) for DCI format 0_3 for a set of cells for the following cases - 1) Search space set configuration for DCI format 0_3 for the set of cells is provided only on the scheduling cell, or; - 2) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells - UE supporting FG 49-2 can additionally report whether the UE support following case - 3) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells 11) - When multiple component 3 values are reported and if scheduling cell is not included in the set of cells, support multi-cell PUSCH scheduling by DCI format 0_3 from one carrier type, indicated in component 3, to another carrier type, indicated in component 3, for the following scheduling cases: - FR1 licensed TDD to FR1 unlicensed TDD - FR2-1 to FR2-2 - UE can additionally report the support of {FR1 licensed FDD from/to FR1 licensed TDD} Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | Per BC | N/A | N/A | N/A | Note: Support of CCS with UL DCI formats 0_1/0_2 is according to FG 6-10 | Optional with capability signaling |
| 49. NR_MC_enh | 49-2b | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields 9) The number of unicast UL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3 - For FDD scheduling cell - Up to one DCI format 0_3 for the set of cells and, - Up to one unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells - For a cell in a set of cells, no more than one DCI scheduling PUSCH for the cell - For TDD scheduling cell - Up to two DCI format 0_3 for the set of cells and, - Up to two unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells - For a cell in a set of cells, no more than two DCI scheduling PUSCH for the cell - For low-to-high SCS, N = 1. - For high-to-low SCS, N is based on pair of (scheduling CC SCS, scheduled CC SCS): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15) 10) Monitoring SS set(s) for DCI format 0_3 for a set of cells for the following cases - 2) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | Per BC | N/A | N/A | N/A | Note: Support of CCS with UL DCI formats 0_1/0_2 is according to FG 18-5b | Optional with capability signaling |
| 49. NR_MC_enh | 49-3x | Advanced UE capability for larger number of unicast DL DCI for FG49-1b | The number of DCI format 1_3 to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3 is X per slot of scheduling cell for the set of cells - For lower to higher SCS, - Candidate values of X - {1, 2, 4} for (12, 120), (15, 60), (30, 120) - {2} for (15, 30), (30,60), (60, 120) | 49-1b |  |  |  | Per {scheduling cell SCS, scheduled cell(s) SCS } of FG49-1b |  |  |  |  |  |
| 49. NR_MC_enh | 49-3y | Advanced UE capability for larger number of unicast UL DCI for FG49-2b | The number of DCI format 0_3 to process for a set of cells configured for multi-cell PUSCH scheduling by a DCI format 0_3 is X per slot of scheduling cell for the set of cells - For lower to higher SCS, - Candidate values of X - {1, 2, 4} for (12, 120), (15, 60), (30, 120) - {2} for (15, 30), (30,60), (60, 120) | 49-2b |  |  |  | Per {scheduling cell SCS, scheduled cell(s) SCS } of FG49-2b |  |  |  |  |  |
| 49. NR_MC_enh | 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | At least one of {49-1, 49-1b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4b | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | At least one of {49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4c | Configurable Type-1A fields for DCI format 0_3/1_3 | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4d | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 0_3 2) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 without scheduling a PDSCH using a reserved FDRA value | At least one of {49-1, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 3 HARQ codebook triggered by DCI format 1_3 | Per BC | N/A | N/A | N/A | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signaling |
| 49. NR_MC_enh | 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_3 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_3 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot | At least one of {49-1, 49-1b} | Yes |  | UE does not support HARQ feedback based on enhanced Type 3 HARQ codebook triggered by DCI format 1_3 | Per BC | N/A | N/A | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured Candidate values for component 5 is: {1, 2, 3, 4, 5, 6, 7}. | Optional with capability signaling |

Other necessary FGs:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 49-6 | Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook simultaneously constructed for supporting HARQ-ACK codebooks with different priorities by DCI format 1_3 | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed with the restriction up to one sub-slot based HARQ-ACK codebook. 2. Supports separate PUCCH configuration for different HARQ-ACK codebooks. 3. Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4. Supports a DCI format 1_3 scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_3/1_3 is configured per BWP. 5. Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook and UCI-OnPUSCH for different HARQ-ACK codebooks. 6. Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot Candidate values for the component 6 of FG49-6 is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration 7. Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels for HARQ-ACK |  |  |  |  |  |  |  |  | Similar to FG11-4 |  |
|  | 49-7 | Two HARQ-ACK codebooks with two sub-slot based HARQ-ACK codebook simultaneously constructed for supporting HARQ-ACK codebooks with different priorities by DCI format 1_3 | 1. Supports two subslot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2. Supports separate PUCCH configuration for different HARQ-ACK codebooks. 3. Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4. Supports a DCI format 1_3 scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_3/1_3 is configured in USS per BWP. 5. Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook and UCI-OnPUSCH for different HARQ-ACK codebooks. 6. Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot. 7. Candidate values for the component 6 of FG49-7 is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration. |  |  |  |  |  |  |  |  | Similar to FG11-4a |  |
|  | 49-8 | DL priority indication in DCI with mixed DCI formats including DCI format 1_3 | Support of priority indicator field configured in DCI formats 1_3 and 1_1/1_2 in a BWP when configured to monitor both DCI formats 1_3 and 1_1/1_2 in the BWP |  |  |  |  |  |  |  |  | Similar to FG11-4b |  |
|  | 49-9 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer for DCI format 0_3 | Support intra-UE multiplexing/prioritization of overlapping PUCCH/PUCCH and PUCCH/PUSCH with two priority levels in physical layer (PHY) for DCI format 0_3 1) Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format 2) Multiplexing/prioritization between UL channels/signals with the same PHY priority level 3) Prioritization between UL channels/signals with different PHY priority levels 4) Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. 5) Additional number of symbols (d2) of the preparation time needed for the high priority UL transmission that cancels a low priority UL transmission |  |  |  |  |  |  |  |  | Similar to FG12-1 |  |
|  | 49-10 | UL priority indication in DCI with mixed DCI formats including DCI format 0_3 | Support of priority indicator field configured in DCI formats 0_3 and 0_1/0_2 in a BWP when configured to monitor both DCI formats 0_3 and 0_1/0_2 in the BWP |  |  |  |  |  |  |  |  | Similar to FG12-1a |  |
|  | 49-11 | PHY priority handling for one-shot HARQ-ACK feedback triggered by DCI format 1_3 | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI format 1_3 |  |  |  |  |  |  |  |  | Similar to FG25-5 |  |
|  | 49-12 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_3 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 49-6) 3. Supported minimum value M for the HARQ re-tx offset 4. Supported maximum value N for the HARQ re-tx offset |  |  |  |  |  |  |  |  | Similar to FG25-7 |  |
|  | 49-13 | SCell dormancy indication in DCI format 0_3 | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_3 |  |  |  |  |  |  |  |  | Similar to FG18-5 |  |
|  | 49-14 | SCell dormancy indication in DCI format 1_3 | Support for SCell dormancy indication sent within the active time on PCell with DCI format 1_3 |  |  |  |  |  |  |  |  | Similar to FG18-5 |  |
|  | 49-15 | Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3 | 1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3 2) minimumSchedulingOffset K2 configuration for PUSCH 3) Support of extended value range for aperiodic CSI-RS triggering offset |  |  |  |  |  |  |  |  | Similar to FG19-2 |  |
|  | 49-16 | Dynamic indication of applicable minimum scheduling restriction by DCI format 1_3 | 1) Dynamic indication of applicable minimum scheduling restriction by DCI format 1_3 2) minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset |  |  |  |  |  |  |  |  | Similar to FG19-2 |  |
|  | 49-17 | Unified TCI with joint DL/UL TCI update by DCI format 1_3 for intra- and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC | - TCI state indication for update and activation b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 without DL assignment) - The minimum beam application time in Y symbols per SCS - The maximum number of MAC-CE activated joint TCI states per CC in a band | 23-1-1 |  |  |  |  |  |  |  | Similar to FG23-1-1b |  |
|  | 49-18 | Unified TCI with separate DL/UL TCI update by DCI format 1_3 for intra-cell beam management with more than one MAC-CE activated separate TCI state per CC | - TCI state indication for update and activation b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 without DL assignment) - The minimum beam application time in Y symbols per SCS - The maximum number of MAC-CE activated DL TCI states per CC in a band - The maximum number of MAC-CE activated UL TCI states per CC in a band | 23-10-1 |  |  |  |  |  |  |  | Similar to FG23-10-1b |  |